3GPP TSG-RAN1 Meeting #97	R1-1907208

# Reno, USA, May 13- May 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | <CR#> | rev | <Rev#> | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on mapping from virtual to physical resource blocks for PUSCH scheduled by RAR UL grant and Msg3 retransmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Subclause 6.3.1.7 of TS38.211 describes the mapping from virtual to physical resource blocks for msg3 corresponding to the msg3 initial transmission and retransmission in CBRA. Except for msg3, virtual resource block n is mapped to physical resource block n. According to the TS38.211, it seems that, for PUSCH sheduled by RAR UL grant in CFRA, virtual resource block n is also mapped to physical resoruce bolck n. However, TS38.213 specifies that the frequency allocation determining for PUSCH scheduled by RAR UL grant in both CBRA and CFRA are same. Therefore, current TS38.211 miss capturing the correct mapping from virtual to physical resoruce blocks for PUSCH scheduled by RAR UL grant in CFRA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the same mapping from virtual to physical resource blocks are applied to the PUSCH transmission scheduled by RAR UL grant and corresponding msg3 retransmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between TS38.211 and TS 38.213 regarding the mapping from virtual to physical resource bolcks for PUSCH scheduled by RAR UL grant in CFRA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis - If UE implements the change and gNB does not, or gNB implements the change but UE does not, there will be a different understanding between UE and gNB on the mapping from virtual to physical resource blocks for PUSCH scheduled by RAR UL grant in CFRA. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.7	Mapping from virtual to physical resource blocks

Virtual resource blocks shall be mapped to physical resource blocks according to non-interleaved mapping.

For non-interleaved VRB-to-PRB mapping, virtual resource block n is mapped to physical resource block n except for  a PUSCH transmission scheduled by a RAR UL grant or corresponding PUSCH retransmission scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI in active uplink bandwidth part i starting at NBWP,istart, including all resource blocks of the initial uplink bandwidth part starting at NBWP,0start, and having the same subcarrier spacing and cyclic prefix as the initial uplink bandwidth part, in which case virtual resource block n is mapped to physical resource block n+NBWP,0start-NBWP,istart.