3GPP TSG RAN WG1 Meeting #97			R1-1907210

Reno, USA, May 13th – 17th, 2019

Source:	Sharp

Title:	Downlink structure and procedure for NR-U operation

Agenda Item:	7.2.2.1.2

Document for:	Discussion and Decision

Background

During NR-U study phase, the use of a signal of which detection facilitates power saving and/or transmission burst detection was actively discussed, and the following agreements were made in RAN1#AH1901 [1].

Agreement: - The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection). - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design. - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst - FFS: Potential enhancements to DMRS design to address issues with detection probability - The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

In RAN1#96bis, partial slot design was discussed and the following outcomes were made [2].

Agreement: Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

Regarding COT indication, the following was captured in TR38.889 [3].

In addition to the functionalities provided by DCI format 2_0 in Rel-15 NR, indication of the COT structure in the time domain has been identified as being beneficial.

In this contribution we present our views on downlink structure and procedure for NR-U operation.

Discussions

NR-U operation without using partial slots

Dynamic indication of PDCCH monitoring occasions facilitates to utilize time domain resources efficiently even when a successful LBT is done in a middle of a slot. On the other hand, network may choose to operate without the initial partial slot (i.e. COT starts always from the slot boundary) but using full slots only. For example, FBE based operation doesn’t require the initial partial slot operation due to the fixed (and periodic) LBT opportunity. In such operation, Rel-15 PDCCH monitoring occasion configuration is sufficient, and the signal which facilitates power saving on PDCCH monitoring is not necessary.

Observation 1:

- At least for full slot based NR-U operation (e.g. in case of FBE based operation), Rel-15 PDCCH monitoring occasion configuration is sufficient, and the signal which facilitates power saving on PDCCH monitoring is not necessary.

In RAN1#AH1901 it was agreed that the UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst. It was also noted that the power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. If the DMRS detection performance is not as good as PDCCH decoding performance requirement, the UE has to try to decode PDCCHs anyway, and this means the DMRS detection does not provide power saving by not performing blind decodes. Therefore, wideband DMRS should be used in PDCCH monitoring occasions of initial partial slots so that sufficient DMRS detection accuracy is achieved.

Proposal 1:

- Wideband DMRS should be used in PDCCH monitoring occasions of initial partial slots.

Downlink partial slot transmission

In RAN1#94bis it was discussed how to handle partial slot. During offline discussions two different aspects were considered. One is PDSCH mapping, while the other is PDCCH monitoring interval. At first they were mixed up, but people tried to separate those two aspects. As an outcomes from the offline discussions the following options have been identified as possible candidates for PDSCH transmission in the partial slot at least for the first PDSCH(s) transmitted in the DL transmission burst.

- Option 1: PDSCH(s) as in Rel-15 NR

- Option 2: Punctured PDSCH depending on LBT outcome

- Option 3: PDSCH mapping type B with durations other than 2/4/7 symbols

- Option 4: PDSCH across slot boundary

In the offline discussions during RAN1#96, performance evaluation results presented in [4] were reviewed. According to the evaluation results, Option 1 achieves the worst BLER performance due to the largest PDCCH and DMRS overhead. On the other hand, it was commented that the other options than Option 1 need more analyses including specification impacts.

According to the discussions so far, even for Option 1 people seems to have different assumptions in terms of PDSCH processing for initial partial slots. Figure 1 shows PDSCH processing assumptions for each option. Option 1 has two possible PDSCH processing assumptions. Option 3 and Option 3+4 are similar but not the same in terms of a combination of the length of the first mini slot and the successful channel access timing.

Option 1 with PDSCH processing assumption A Option 1 with PDSCH processing assumption B

Option 3

Figure 1: Initial partial slot structures and PDCCH/PDSCH processing

Pros and cons of each assumption are summarized in Table 1.

Option 1 with PDSCH processing assumption A: The first assumption for Option 1 is that, before a successful channel access, the network prepares several sets of PDCCH and mini-slot based PDSCH which constitute a single full slot. If the network gets a successful channel access in a middle of an initial slot, the network gives up transmitting all the sets which are fully or partially eliminated due to LBT procedure, while the network can still transmit the remaining sets. As either PDCCH/PDSCH contents or transmission timing do not change, no special handling over Rel-15 type B PDSCH mapping with Case 2 PDCCH monitoring is necessary. On the other hand, since the network needs to put PDCCH on every single possible starting position prior to LBT, the number of possible starting positions should be limited. This limitation causes the necessity of longer reservation signal transmissions before PDCCH occasion. If the number of possible starting positions increases to minimize the reservation signal overhead, on the contrary, it results in rapid increase of PDCCH and DMRS overhead. Furthermore, in order to fully utilize initial partial slot resources, UEs need to have Case 2 PDCCH monitoring capability, since actually transmitted PDCCHs may appear multiple times within the initial partial slot.

Option 1 with PDSCH processing assumption B: The second assumption for Option 1 is that, before a successful channel access, the network prepares a single mini slot with a fixed time domain length (or a single full slot) before a successful channel access. If the network gets a successful channel access in a middle of an initial slot, the network starts the transmission of the prepared mini slot in the middle of the slot. PDCCH/PDSCH transmission timing change but their durations do not change. If the rest of the initial partial slot consists of 2, 4, 7 OFDM symbols or a combination of them, the remaining time slot can be filled with mini-slot(s). Otherwise, the gNB needs to transmit reservation signal so that the transmission burst is contiguous. Therefore, Option 1 with PDSCH processing assumption B also has a drawback in terms of overhead, even though its overhead is not as large as the one in Option 1 with PDSCH processing assumption A. In addition, this option also requires UEs to have Case 2 PDCCH monitoring capability.

Option 3: PDSCH processing for Option 3 is that the network prepares a single mini slot with a fixed time domain length (or a single full slot) before a successful channel access. If the network gets a successful channel access in a middle of an initial slot, the network starts the transmission of the prepared mini slot in the middle of the slot. PDCCH/PDSCH transmission timing change but their durations do not change. Therefore, PDSCH processing is almost the same as for Option 1 with PDSCH processing assumption B. The difference is that Option 3 supports mini slot lengths other than 2, 4, and 7 OFDM symbols. Therefore, Option 3 can avoid unnecessary reservation signal transmissions and is able to fully utilize the rest of resources after the first mini slot transmission. In contrast, this difference causes a specification impact that possible mini slot lengths other than 2, 4, and 7 OFDM symbols need to be added. If the first mini slot length is fixed to the small number of OFDM symbol compared to a full slot length, the complexity may be same as Case 2 PDCCH monitoring, i.e. Option 1. If the first mini slot length gets closer to a full slot, the complexity becomes similar to Option 2.

Table 1: Pros/Cons of PDSCH processing assumptions A, B and C.

|  | PDCCH generation at gNB side | PDSCH handling | PDCCH occasions | Overhead due to PDCCH, DMRS and/or reservation | SPEC impact |
| --- | --- | --- | --- | --- | --- |
| Option 1 with PDSCH processing assumption A | Prior to LBT | Same as normal scheduling | Multiple occasions per slot | Largest: - Each possible starting position causes PDCCH and DMRS overhead | None |
| Option 1 with PDSCH processing assumption B | After LBT | Same as normal scheduling | Multiple occasions per slot | Large: - More than one set of PDCCH and DMRS per slot. - Reservation signal is needed if the number of remaining OFDM symbols is not aligned with Rel-15 mini-slot length(s) | None |
| Option 3 | After LBT | Same as normal scheduling | A single or multiple occasions per slot | Fair: - Depending on the length of the 1st mini-slot, more than one set of PDCCH and DMRS per slot. - No need of reservation signal to fill the remaining OFDM symbols | Marginal: - Modifications on SLIV to add mini-slot lengths other than 2,4,7 |

In addition, it was proposed that RMSI PDSCH support another length than 2/4/7 symbols so as to have enough coding rate for the RMSI carried in a DRS burst. If the RMSI supports other length, support of Option 3 for unicast PDSCH may not cause much increase of UE complexity.

Based on the above analyses, it is good to support PDSCH processing assumption B (i.e. Option 3).

Proposal 2:

- NR-U should support PDSCH mapping type B with durations other than 2/4/7 symbols (i.e. Option 3).

If Option 3 is supported, the network needs to update PDCCH contents after getting the channel access. Compared with the PDCCH preparation, DMRS generation is much easier, and therefore the network should have no problem to change the DMRS sequence depending on channel access timing. Hence, Rel-15 DMRS sequence generation should be used for NR-U.

Proposal 3:

- Rel-15 DMRS sequence generation should be used for NR-U.

DL/UL direction indication and COT structure indication

On one hand, NR-U semi-static slot format configuration may not make much sense, since actual transmission start timing has to be adjusted depending on LBT result. On the other hand, in RAN1#94bis it was agreed that, in addition to the functionalities provided by DCI format 2_0 in Rel-15 NR, indication of the COT structure in the time domain has been identified as being beneficial. However, SFI PDCCH monitoring with a long period does not work well, because the maximum COT length is limited. Therefore, more flexible monitoring occasion of SFI indication should be considered. In addition, an SFI for a given COT should be able to be signaled multiple times within the COT. Otherwise, if the SFI suffers from interferences from hidden nodes, whole COT may become unavailable. Given that Rel-15 SFI configuration is reused, different SFI entries have to be prepared for different SFI indication timing even if DL/UL configuration within the COT does not change. It causes the problem that the SFI entries cannot be fully utilized for providing flexibility of COT structures.

Proposal 4:

- An SFI for a given COT should be able to be signaled multiple times within the COT.

In SI phase it was identified to be beneficial to have COT structure indication on top of SFI indication. The gNB does not know when it gets the channel again after finishing the current COT. Therefore, it does not make sense to indicate SFI of outside the current COT.

Observation 2:

- It does not make sense to indicate SFI of outside the current COT.

There are two benefits for UEs to be signaled with COT structures.

- Presence of CSI-RS which is valid for CSI measurement.

- UL durations where the UEs are allowed to perform a simple LBT for their UL transmissions.

For both cases, the DL/UL indication is sufficient, and it is unclear if the UE needs to be able to aware of when the gNB gets the channel and when the gNB’s COT ends.

Proposal 5:

- SFI indicates DL/UL/Flexible configuration of inside the COT in which the SFI is sent.

As discussed in Section 2.1 there are several options for BWP-based operation within a carrier with bandwidth larger than 20 MHz. Given that Option 3 is adopted (i.e. Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB), COT may need to be defined for each 20MHz bandwidth. The reason is that, even when a PDSCH is transmitted in one of LBT bandwidth, some other part within the same BWP may still be occupied by another node. Therefore, it is beneficial to support frequency domain occupancy indication for NR-U. An easiest way is to put SFI PDCCH in every LBT bandwidth, and each indicates all or a part of the COT structure in the respective LBT bandwidth. Another option is to put a single SFI PDCCH which indicates time and frequency domain occupancy in the BWP.

Proposal 6:

- Frequency domain channel occupancy indication should be supported.

Conclusion

In this contribution we present our views on downlink structure and procedure for NR-U operation, and we make the following proposals and observations:

Observation 1:

- At least for full slot based NR-U operation (e.g. in case of FBE based operation), Rel-15 PDCCH monitoring occasion configuration is sufficient, and the signal which facilitates power saving on PDCCH monitoring is not necessary.

Proposal 1:

- Wideband DMRS should be used in PDCCH monitoring occasions of initial partial slots.

Proposal 2:

- NR-U should support PDSCH mapping type B with durations other than 2/4/7 symbols (i.e. Option 3).

Proposal 3:

- Rel-15 DMRS sequence generation should be used for NR-U.

Proposal 4:

- An SFI for a given COT should be able to be signaled multiple times within the COT.

Observation 2:

- It does not make sense to indicate SFI of outside the current COT.

Proposal 5:

- SFI indicates DL/UL/Flexible configuration of inside the COT in which the SFI is sent.

Proposal 6:

- Frequency domain channel occupancy indication should be supported.

References

- 3GPP RAN1#AH1901 Chairman’s note, January 2019.

- 3GPP RAN1#96bis Chairman’s note, April 2019.

- 3GPP TR38.889 v16.0.0 (2018-12) “Study on NR-based access to unlicensed spectrum”.

- R1-1903241, “On DL signals and channels”, Nokia, Nokia Shanghai Bell, RAN1#96, February 2019.