3GPP TSG RAN WG1 #97	R1-1907233

Reno, USA, May 13th – 17th, 2019

Agenda item:	7.2.4.3

Source:			ITRI

Title:			Discussion on Synchronization Issues and Mechanisms for NR V2X Sidelink

Document for:	Discussion and Decision

- Introduction

NR V2X SI was completed in February 2019, and the new NR V2X work item [1] was approved in RAN#83. Sidelink synchronization mechanism is one of the objectives for the new work item of NR V2X, and the sidelink synchronization mechanism related objectives can be shown as follows:

4.1 Objective of SI or Core part WI or Testing part WI

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

- Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

- Resource allocation [RAN1, RAN2]

…….

- Sidelink synchronization mechanism as per the study outcome [RAN1, RAN2]

- Procedures selecting synchronization reference

- S-SSB and procedures to transmit and receive it, including when GNSS and gNB/eNB are unavailable

- Use of RS for sidelink synchronization if specification impact is identified

- Solution for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks

……….

- Sidelink physical layer procedures as per the study outcome

……………..

For the NR V2X SI, the sidelink synchronization mechanism related agreements at RAN1#94 [2] are shown as follows,

Agreements:

- NR V2X Sidelink Synchronization includes at least the following

  - Sidelink synchronization signal(s)

  - PSBCH

  - Sidelink synchronization sources and procedure(s)

    - Study potential synchronization sources –GNSS, gNB, eNB, UE, LTE UE

      - Note: this doesn’t mean all of them are to be supported

Besides, the sidelink synchronization mechanism related agreements at RAN1#94bis [3] are shown as follows,

Agreements:

At least GNSS, gNB, NR UE, and eNB are supported as the synchronization source for NR V2X.

eNB as a synchronization source for NR V2X UEs supporting LTE Uu/PC5 or Uu only (no change to the eNB behaviour)

Whether a source is supported is for further NR V2X UE capability consideration

Agreements:

- The design of NR V2X sidelink synchronization signals and PSBCH uses NR SSB structure as the starting point with the following properties,

  - NR V2X synchronization signals include sidelink PSS (S-PSS) and sidelink SSS (S-SSS) and are structured with PSBCH in a block format (S-SSB)

Moreover, the sidelink synchronization mechanism related agreements at RAN1#95 [4] are shown as follows,

Agreements:

- The aspects of synchronization sequence for NR V2X to be considered for the evaluation include,

- The length of S-PSS and S-SSS sequences

- If and how to distinguish from NR Uu PSS and SSS sequences

- The number of NR SL-SSID targeted in the design of NR V2X S-PSS/S-SSS

  - Use cases of NR SL-SSID should be addressed

Agreements:

- Using the below table as a starting point for evaluation assumptions for sidelink synchronization LLS.

  - Detection probability of S-PSS/S-SSS

  - Decoding BLER of PSBCH

  - Check further offline regarding UE speeds (absolute vs. relative, including current channel model assumptions in the TR)  on Friday, confirmed to be relative speed and thus, the speeds in the table below need to be doubled

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 6 GHz | 30 GHz |
| Channel Model | CDL channel models |  |
| Subcarrier Spacing(s) | 15, 30, 60 kHz | 60, 120 kHz |
| SNR Range | > -6 dB | > -6 dB |
| UE Speed | 3 km/h, 120 km/h (mandatory) 30km/h, 250 km/h (optional) | 3 km/hr, 120 km/h (mandatory) |
| Interference model | Scenario 1: no interference Scenario 2: effect of interference includes in the model | Scenario 1: no interference |
| Initial Frequency Offset | TX: Uniform distribution within [-5, 5] ppm of nominal carrier frequency RX: Uniform distribution within [-5, 5] ppm of nominal carrier frequency |  |

Agreements:

- The study of NR V2X synchronization includes synchronization based on S-SSB

- The study also includes use of other sidelink signals/channels (e.g., other RSs in the SL, using PSSCH, using PSCCH, etc.) for the sidelink synchronization

Furthermore, the sidelink synchronization mechanism related agreements at RAN1 Ad-Hoc 1901 [5] are shown as follows,

Agreements:

- For NR SLSS, as the baseline:

  - The sequence type for S-PSS is the same type as the M-sequence used for NR-PSS

  - The sequence type for S-SSS is the same type as the Gold sequence for NR-SSS

Agreements:

- Combination 1:

  - Time domain: 2 symbol of length-127 S-PSS, 2 symbol of length-127 S-SSS

  - Frequency domain:11 or 12 RBs

  - BW containing S-SSB:

    - 2.5 MHz for 15 kHz SCS

    - 5 MHz for 30 kHz SCS

    - 10 MHz for 60 kHz SCS

    - 20 MHz for 120 kHz SCS

- Combination 2:

  - Time domain: 2 symbol of length-127 S-PSS, 2 symbol of length-127 S-SSS

  - Frequency domain:  20 RBs

  - BW containing S-SSB:

    - 5 MHz for 15 kHz SCS

    - 10 MHz for 30 kHz SCS

    - 20 MHz for 60 kHz SCS

    - 40 MHz for 120 kHz SCS

- Combination 3:

  - Time domain: 1 symbol of length-127 S-PSS, 1 symbol of length-127 S-SSS

  - Frequency domain:  20 RBs

  - BW containing S-SSB:

    - 5 MHz for 15 kHz SCS

    - 10 MHz for 30 kHz SCS

    - 20 MHz for 60 kHz SCS

    - 40 MHz for 120 kHz SCS

- Combination 4:

  - Time domain: 1 symbol of length-255 S-PSS, 1 symbol of length-255 S-SSS

  - Frequency domain:  24 RBs

  - BW containing S-SSB:

    - 5 MHz for 15 kHz SCS

    - 10 MHz for 30 kHz SCS

    - 20 MHz for 60 kHz SCS

    - 40 MHz for 120 kHz SCS

- Other combinations are not precluded.

- Note:  Company should specify the assumptions, such as total energy per SSB, when the performance results are compared between different combinations.

Agreements:

- At least for single-carrier operation:

- For the SL synchronization procedure, each type of synchronization reference has a respective sync priority

  - FFS the priority between eNB and gNB (if necessary)

- For the SL synchronization procedure, among the available references, a UE selects the synchronization reference with the highest priority as the reference to derive its transmission timing

  - FFS other potential usage

  - FFS how to handle the case when there are two or more references of a same priority to be selected as the highest priority

Then, the sidelink synchronization mechanism related agreements at RAN1#96 [6] are shown as follows,

Agreements:

- Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured.

  - The following table is a working assumption

Agreements:

- NR V2X supports using a sidelink RS for synchronization purpose

  - Applicable only on unlicensed (ITS) carrier with no network deployment on this carrier

  - This RS is not a standalone RS and not part of SLSS.

  - This RS will not appear in the synchronization procedure for the selection of sync sources.

  - RS used for the synchronization purpose would not impact any sidelink RS design

  - FFS: Whether this RS is DM RS or other RS

  - FFS: Whether this could be achieved by UE implementation

  - FFS: Specification impact

For the NR V2X WI, the sidelink synchronization mechanism related agreements at RAN1#96bis [7] are shown as follows,

Agreements:

- RAN1 will consider the design of NR SLSS where

  - S-PSS of LTE and NR candidates have similar PAPR.

  - S-SSS of LTE and NR candidates have similar PAPR.

  - SLSS design should take into consideration PAPR difference between S-PSS and S-SSS.

Agreements:

- In NR V2X, S-SSB bandwidth is 11RBs.

  - PSBCH spans 11RBs.

- The S-SSB is designed following combination 1.

  - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS

  - Two symbols are used for each of S-PSS and S-SSS, respectively.

Agreements:

- For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC.

This paper is the revision of R1-1905078. In this paper, we will focus on sidelink synchronization issues and mechanisms discussion for NR V2X. At first, we will introduce the issues and mechanisms on synchronization procedures including synchronization reference selection, eNB/gNB timing misalignment, and synchronization signal forwarding. Then, we will discuss the issues and mechanisms on the design of sidelink synchronization signal block (S-SSB) for NR V2X, especially for the considerations of sidelink PSS (S-PSS) design and the high speed case agreed in RAN1#95. Besides, sidelink RS based synchronization is introduced from our point of view. Finally, we will investigate the sidelink synchronization issues due to different subcarrier spacing (SCS).

- Discussion

In this section, we will discuss the issues and mechanisms on the synchronization procedures including synchronization reference selection, eNB/gNB timing misalignment, and synchronization signal forwarding, the design of S-SSB with the considerations of S-PSS design and the high speed case agreed in RAN1#95, and sidelink synchronization issues due to sidelink RS based synchronization and different SCS .

2.1 Issues and mechanisms on synchronization procedures

In RAN1#96 meeting, it is agreed that whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured, and the synchronization source priority table including GNSS-based synchronization and gNB/eNB-based synchronization is a working assumption, as shown in section 1. Because the working assumption is based on the synchronization source priority agreed in Rel-12 D2D and Rel-14 LTE-V2X, we confirm the working assumption.

Proposal 1: The working assumption for the synchronization source priority table including GNSS-based synchronization and gNB/eNB-based synchronization is confirmed.

Besides, the use of GNSS-based synchronization and gNB/eNB-based synchronization can be based on that in Rel-14 LTE V2X, where the related synchronization source priority agreements in LTE-based V2X Services WI in [8] are shown as follows,

Agreements:

- If the UE detects no eNB in a carrier which is (pre-)configured as the carrier which potentially includes eNBs used as sync reference, the following priority rules should be applied:

  - P1: GNSS

  - P2: the following UE has the same priority:

    - UE directly synchronized to GNSS

    - UE directly synchronized to eNB

  - P3: the following UE has the same priority:

    - UE indirectly synchronized to GNSS (if RAN1 decides to differentiate between direct and indirect synchronization to GNSS)

    - UE indirectly synchronized to eNB

  - P4: the remaining UEs have the lowest priority.

- The priority order between P1 and P2 can be (pre-)configured.

  - Note: It is RAN1 understanding that P1 will be prioritized over P2 if the eNB prioritizes GNSS over eNB timing.

Agreements:

- If the UE detects an eNB in a carrier which is (pre-)configured as the carrier which potentially includes eNBs used as sync reference,

  - If GNSS is prioritized over eNB by eNB configuration, UE directly or indirectly synchronized to GNSS has a higher priority than eNB.

Hence, we propose that GNSS–based synchronization in the synchronization source priority table should be used if the UE detects no gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference. Similarly, we also propose that gNB/eNB-based synchronization in the synchronization source priority table should be used if the UE detects an gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference.

Proposal 2: The GNSS–based synchronization in the synchronization source priority table should be used if the UE detects no gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference.

Proposal 3: The gNB/eNB-based synchronization in the synchronization source priority table should be used if the UE detects an gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference.

Besides, it is agreed in RAN1 Ad-Hoc 1901 meeting that a UE selects the synchronization reference with the highest priority as the reference to derive its transmission timing. When there are two or more synchronization references of a same priority to be selected as the highest priority, some mechanisms are needed to handle the case. For example, the synchronization reference that the received synchronization signal with largest signal strength is transmitted by it can be selected when there are two or more synchronization references of a same priority to be selected as the highest priority.

Proposal 4: The synchronization reference that the received synchronization signal with largest signal strength is transmitted by it can be selected when there are two or more synchronization references of a same priority to be selected as the highest priority

If the same synchronization source priority is pre-configured for eNB and gNB, the NR V2X UE can choose eNB or gNB as synchronization reference by itself when the NR V2X UE receives synchronization signals from both eNB and gNB. As mentioned above, the NR V2X UE can select the synchronization reference that the received synchronization signal with largest signal strength is transmitted by it. However, for this case, the NR V2X UEs may select different synchronization sources as synchronization references when the NR V2X UEs are located around the middle region between the eNB coverage and gNB coverage. Then, the NR V2X UEs may follow different synchronization reference timings and the timing misalignment issue may occur for the NR V2X communication among these NR V2X UEs, when eNB and gNB have different reference timings. Hence, some mechanisms are needed to handle the timing misalignment issue between eNB and gNB. For example, some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB. Therefore, we propose that some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB if the same synchronization source priority is pre-configured for eNB and gNB.

Proposal 5: In order to overcome timing misalignment issue between eNB and gNB, some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB if the same synchronization source priority is pre-configured for eNB and gNB.

After a NR V2X UE selects a synchronization source and follows its reference timing, the NR V2X UE needs to decide to become a new synchronization source and forward the synchronization signal or not. Sidelink synchronization signal (SLSS) forwarding/transmission can extend the coverage of the network synchronization. However, when the number of misaligned reference timings distributed by SLSS transmission increases, interference among misaligned timings will also become severe. Hence, to keep the number of synchronization sources to a minimum can eliminate such interference and maintain the unity of a synchronous network. Therefore, we propose that sidelink synchronization signal forwarding should be considered to extend coverage for NR V2X, the details are FFS.

Proposal 6: Sidelink synchronization signal forwarding should be considered to extend coverage for NR V2X, where the details are FFS.

2.2 Issues and mechanisms on the design of Sidelink SS Block

From the sidelink synchronization mechanism related agreements shown in section 1, the design for NR V2X synchronization signals, including sidelink PSS (S-PSS)/ sidelink SSS (S-SSS) and PSBCH, can refer to that agreed in NR SI/WI.

The agreements and working assumption related to sequence design for NR-PSS and NR-SSS in Rel-15 NR WI in [9-10] are shown as follows,

Agreements:

- Confirm following working assumptions on NR-PSS as agreements

  - Number of PSS sequences: 3

    - PSS sequence details:

      - Frequency domain-based pure BPSK M sequence (fixing the time/freq. offset ambiguity)

        - 1 polynomial: Decimal 145 (i.e. g(x) = x7 + x4 + 1)

        - In freq. domain 3 cyclic shifts (0, 43, 86) to get the 3 PSS signals

        - Initial poly shift register value: 1110110

Working assumption:

- NR-SSS sequence design is 1 polynomial with 127 cyclic shifts, and 1 another polynomial with 9 cyclic shifts

    - Two generator polynomials will be defined for m-sequences, and cyclic shift according to NR-cell ID is applied to each m-sequence

    - Two polynomials are generated by g0(x) = x7 + x4 + 1 and g1(x) = x7 + x + 1

      - Initial state is [0000001]

    - The cyclic shift values m0 and m1 are jointly determined by the cell IDs carried by NR PSS (i.e., NID(2)=0, 1, 2) and NR SSS (i.e., NID(1)=0, 1, …, 335)), where the cell ID is given by NIDcell=3NID(1)+NID(2)

      - m0=3NID(1)112+NID(2)

      - m1=NID1   mod 112+m0+1

Agreement:

Confirm Working Assumption for NR-SSS design from RAN1#89.

In RAN1 Ad-Hoc 1901 meeting, it is agreed that the sequence type for S-PSS is the same type as the M-sequence used for NR-PSS and the sequence type for S-SSS is the same type as the Gold sequence for NR-SSS. Besides, in last RAN1 meeting, it is agreed that length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS. However, some parameters in NR-PSS and NR-SSS, including the number of sequences and the values of cyclic shift, should be revised in order to meet the purposes and requirements for NR V2X. In the following, we will show the simulation results of the peak-to-average power ration (PAPR), cubic metric (CM), and autocorrelation for LTE-PSS and NR-PSS. Besides, based on the simulation results, we will provide our viewpoint for the design of S-PSS.

Large PAPR/ CM will introduce non-linear distortion when the transmitted signals pass through the power amplifier. Fig. 1(a) and 1(b) show the simulated PAPR & raw CM values in dB for LTE-PSS with all root indices and NR-PSS with all cyclic shift indices, respectively. The LTE-PSS is the Zadoff-Chu sequence with length 63, and the NR-PSS is the mentioned above M-sequence with length 127. From the simulation results, we can observe that several conjugated root pairs {(1,62),(8,55),(16,47),(25,38),(29.34), (31,32)} for LTE-PSS are with smaller values of both PAPR and raw CM. However, several conjugated root pairs {(9,54),(18,45),(27,36)} for LTE-PSS are with larger values of both PAPR and raw CM. Compare to LTE-PSS, the values of both PAPR and raw CM for NR-PSS with all cyclic shifts are smaller.

Observation 1: LTE-PSS only with some specific conjugated root pairs have good PAPR and raw CM properties.

Observation 2: NR-PSS with all cyclic shifts have good PAPR and raw CM properties.

(a)                                                 (b)

Figure 1. PAPR & raw CM values for (a) LTE-PSS with all root indices and (b) NR-PSS with all cyclic shift indices

Besides, we use the peak to side lobe ratio (PSR) to evaluate the autocorrelation of LTE-PSS and NR-PSS, where the PSR is defined as the ratio of the first tallest autocorrelation peak value to the second tallest autocorrelation peak value. The autocorrelation is better when the corresponding PSR is larger. Fig. 2(a) and 2(b) show the simulated PSR values in dB at different carrier frequency offset values (CFOs) for LTE-PSS with all root indices and NR-PSS with all cyclic shift indices, respectively. From the simulation results, we can observe that the PSR values of LTE-PSS with all roots are sensitive to CFOs. However, the PSR values of NR-PSS with all cyclic shifts are robust to CFOs when CFOs are less than 0.5 subcarrier spacing (SCS).

Observation 3: The autocorrelation values of LTE-PSS with all roots are sensitive to CFOs, while the autocorrelation values of NR-PSS with all cyclic shifts are robust to CFOs when the CFOs are less than 0.5 SCS.

(a)                                                  (b)

Figure 2. PSR of Autocorrelation at different CFOs for (a) LTE-PSS with all root indices and (b) NR-PSS with all cyclic shift indices

Proposal 7: The sequence types and length for NR-PSS and NR-SSS are agreed to be reused for S-PSS and S-SSS in NR V2X. However, some parameters, including the number of sequences and the values of cyclic shift, should be revised for S-PSS and S-SSS in order to meet the purposes and requirements for NR V2X, where the details are FFS.

In 5G NR, the PBCH payload consists of MIB and the additional timing information. The NR PBCH content can be classified into the following information,

- System timing: SFN, half radio frame;

- TDD configuration

- Priority information

- Initial access: subCarrierSpacingCommon, ssb-SubcarrierOffset, dmrs-TypeA-Position, pdcch-ConfigSIB1, MSB of the subcarrier offset between SSB and the common resource block grid (FR1) / SSB index(FR2);

- Cell related: cellBarred, intraFreqReselection;

In last RAN1 meeting, it is agreed that the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC for the evaluation of PSBCH performance. In order to meet the purposes and requirements for NR V2X, the content of PSBCH for NR V2X can reuse that in PBCH for 5G NR, and the details are FFS.

Proposal 8:  The content of PSBCH for NR V2X are FFS.

Another important issue for the design of S-SSB is the adoption of subcarrier spacing in the S-SSB, especially in the cases of high UE speed & high carrier frequency that may introduce severe inter-carrier interference and hence significantly degrade system performance.

It is agreed that S-SSB has the same numerology, which includes subcarrier spacing (SCS) and CP length, as that of control and data channels for a given carrier in RAN1#95 meeting. Besides, in RAN1#95 meeting, several values of carrier frequency, subcarrier spacing, and UE speed are also agreed in the evaluation assumptions for sidelink synchronization LLS including subcarrier spacing of 15, 30, 60 kHz and UE speed of up to 120km/h (mandatory) and 250km/h (optional) are used for carrier frequency of 6GHz. Also, it is agreed that subcarrier spacing of 60, 120 kHz and UE speed of up to 120km/h (mandatory) are used for carrier frequency of 30GHz.

In the cases of high UE speed & high carrier frequency for NR V2X, for example carrier frequency is 6GHz and the UE speed is 120km/h (the maximum absolute vehicle speed may be 240km/h), the normalized Doppler frequency defined as fd*TU where fd is the maximum Doppler frequency and TU is the useful symbol duration (TU=66.67 sec when subcarrier spacing is 15 kHz) will be up to 8.8% when subcarrier spacing is 15 kHz. And the normalized Doppler frequency which is up to about 0.1 will significantly degrade system performance, as shown in section 2.2 in [11].

From above-mentioned analysis, subcarrier spacing of 15kHz is not appropriate to be adopted for the case that carrier frequency is 6GHz and the UE speed is 120km/h because  is 8.8%. Similarly, subcarrier spacing of 15kHz and 30kHz are not appropriate to be adopted for the case that carrier frequency is 6GHz and the UE speed is 250km/h because is 18.4% and 9.2%, respectively. Besides, subcarrier spacing of 60kHz is also not appropriate to be adopted for the case that carrier frequency is 30GHz and the UE speed is 120km/h because  is 11%. Therefore, adoption of larger subcarrier spacing should be considered in the cases of high UE speed & high carrier frequency to alleviate inter-carrier interference due to Doppler effect for NR V2X.

Proposal 9: In the design of S-SSB, adoption of larger subcarrier spacing should be considered in the cases of high UE speed & high carrier frequency to alleviate inter-carrier interference due to Doppler effect for NR V2X.

2.3 Discussion on the Sidelink RS based Synchronization

In last RAN1 meeting, it is agreed that NR V2X supports using a sidelink RS for synchronization purpose. Sidelink RS based synchronization is beneficial for sidelink synchronization when the UEs cannot detect any gNB/eNB synchronization signals, sidelink synchronization siganls (SLSS) and GNSS synchronization signals. The RS adopted for Sidelink RS based synchronization can be DM RS or other RS as long as it can be detected accurately, for example, the length of the RS is large enough. Besides, in order to reduce specification impact, no new RS is supported and the Sidelink RS based synchronization procedure is achieved by UE implementation.

Proposal 10: For Sidelink RS based synchronization, the adopted RS can be DM RS or other RS as long as the length of the RS is large enough. Besides, in order to reduce specification impact, no new RS is supported and the Sidelink RS based synchronization procedure is achieved by UE implementation.

2.4 Considerations on the Sidelink Synchronization Issues due to Different SCS

Compare to LTE, support of multiple numerologies is one of the key features for NR. In order to meet the low latency requirements for NR V2X, multiple SCS will be supported for NR V2X. And the SCS related agreements on the physical channel for NR V2X are shown in [2] as follows,

Agreements:

- RAN1 to continue study on the physical channel considering at least the following aspects:

- Subcarrier spacing

  - Candidates for further study are:

    - FR1: 15 kHz, 30 kHz, 60 kHz, 120 kHz

    - FR2: 30 kHz, 60 kHz, 120 kHz, 240 kHz

  - Companies are encouraged to consider the potential issues and benefit of introducing new subcarrier spacing.

Besides, in RAN1#95 meeting, subcarrier spacing of 15, 30, 60 kHz for carrier frequency of 6GHz in FR1 and subcarrier spacing of 60, 120 kHz for carrier frequency of 30GHz in FR2 are agreed in the evaluation assumptions for sidelink synchronization LLS.

From above agreements, multiple SCS may be supported for NR V2X SLSS/PSBCH transmission. Hence, there will be a case that NR V2X UEs located in different cells transmit SLSS/PSBCH with different subcarrier spacing, or in-coverage NR V2X UEs and out-of-coverage NR V2X UEs transmit SLSS/PSBCH with different subcarrier spacing. In this case, a new synchronization mechanism should be considered for NR V2X.

Proposal 11: For a case that NR V2X UEs located in different cells transmit SLSS/PSBCH with different subcarrier spacing, or in-coverage NR V2X UEs and out-of-coverage NR V2X UEs transmit SLSS/PSBCH with different subcarrier spacing, a new synchronization mechanism should be considered for NR V2X. The details are FFS.

- Conclusion

In this contribution, we discuss sidelink synchronization for NR V2X. Based on the analysis, we have the following observations and proposals:

Proposal 1: The working assumption for the synchronization source priority table including GNSS-based synchronization and gNB/eNB-based synchronization is confirmed.

Proposal 2: The GNSS–based synchronization in the synchronization source priority table should be used if the UE detects no gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference.

Proposal 3: The gNB/eNB-based synchronization in the synchronization source priority table should be used if the UE detects an gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference.

Proposal 4: The synchronization reference that the received synchronization signal with largest signal strength is transmitted by it can be selected when there are two or more synchronization references of a same priority to be selected as the highest priority

Proposal 5: In order to overcome timing misalignment issue between eNB and gNB, some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB if the same synchronization source priority is pre-configured for eNB and gNB.

Proposal 6: Sidelink synchronization signal forwarding should be considered to extend coverage for NR V2X, where the details are FFS.

Observation 1: LTE-PSS only with some specific conjugated root pairs have good PAPR and raw CM properties.

Observation 2: NR-PSS with all cyclic shifts have good PAPR and raw CM properties.

Observation 3: The autocorrelation values of LTE-PSS with all roots are sensitive to CFOs, while the autocorrelation values of NR-PSS with all cyclic shifts are robust to CFOs when the CFOs are less than 0.5 SCS.

Proposal 7: The sequence types and length for NR-PSS and NR-SSS are agreed to be reused for S-PSS and S-SSS in NR V2X. However, some parameters, including the number of sequences and the values of cyclic shift, should be revised for S-PSS and S-SSS in order to meet the purposes and requirements for NR V2X, where the details are FFS.

Proposal 8:  The content of PSBCH for NR V2X are FFS.

Proposal 9: In the design of S-SSB, adoption of larger subcarrier spacing should be considered in the cases of high UE speed & high carrier frequency to alleviate inter-carrier interference due to Doppler effect for NR V2X.

Proposal 10: For Sidelink RS based synchronization, the adopted RS can be DM RS or other RS as long as the length of the RS is large enough. Besides, in order to reduce specification impact, no new RS is supported and the Sidelink RS based synchronization procedure is achieved by UE implementation.

Proposal 11: For a case that NR V2X UEs located in different cells transmit SLSS/PSBCH with different subcarrier spacing, or in-coverage NR V2X UEs and out-of-coverage NR V2X UEs transmit SLSS/PSBCH with different subcarrier spacing, a new synchronization mechanism should be considered for NR V2X. The details are FFS.

4.	Reference

- RP-190766, “New WID on 5G V2X with NR sidelink”, 3GPP TSG RAN Meeting #83, Shenzhen, P. R. China, March 18 -21, 2019.

- Chairman notes, 3GPP RAN1 #94, Aug. 20 - 24, 2018

- Chairman notes, 3GPP RAN1 #94bis, Oct. 8 - 12, 2018

- Chairman notes, 3GPP RAN1 #95, Nov. 12 - 16, 2018

- Chairman notes, 3GPP RAN1 NR Ad-Hoc 1901, Jan. 21 - 25, 2019

- Chairman notes, 3GPP RAN1 #96, Feb. 25 - Mar. 1, 2019

- Chairman notes, 3GPP RAN1 #96bis, Apr. 8 - 12, 2019

- Chairman notes, 3GPP RAN1 #86bis, Oct. 10 - 14, 2016

- Chairman notes, 3GPP RAN1 #89, May 15 - 19, 2017

- Chairman notes, 3GPP RAN1 NR Ad-Hoc#2, June 27 - 30, 2017

- R1-1905078, “On Sidelink Synchronization Issues and Mechanisms for NR V2X,” proposed by ITRI, Apr. 2019.