3GPP TSG-RAN WG1 Meeting #97	R1-1907256

May 13th  – May 17th,  2019

Reno, USA

Agenda item:	7.2.1.2

Source: 	Qualcomm Incorporated

Title: 	Procedures for Two-Step RACH

Document for:		Discussion and Decision

# 1. 	Introduction

At RAN #82 meeting, two-step RACH was approved as a new WI for NR Rel-16, which involves both RAN1 and RAN2 efforts to specify contention-based random access procedures applicable to both licensed and unlicensed spectrum [1].

In RAN1 #96 and #96B meetings, the following agreements have been made on the procedures related to two-step RACH [2-3]:

Agreements:

For the relation of PRACH resources between 2-step and 4-step RACH, the network has the flexibility to configure the following options:

- Option 1: Separate ROs are configured for 2-step and 4-step RACH

- Option 2: Shared RO but separate preambles for 2-step and 4-step RACH

Agreements:

Further study the granularity of the time advance command, if supported in MsgB:

- Based on the subcarrier spacing of MsgA PUSCH using a 12-bit TA command, where the granularity of the TA command is determined according to the following table.

| Subcarrier Spacing (kHz) of the PUSCH | Unit |
| --- | --- |
| 15 | 16*64 Tc |
| 30 | 8*64 Tc |
| 60 | 4*64 Tc |
| 120 | 2*64 Tc |

- Other options/variations are not precluded

Agreements:

For 2-step RACH preamble power control parameter configuration, further study and down select from the following options:

- Option 1: Power control parameters can be separately configured for 2-step and 4-step RACH.

  - If a power control parameter is not configured for 2-step RACH, the corresponding power control parameter of 4-step RACH is used instead for 2-step.

- Option 2: The corresponding power control parameter of 2-step RACH preamble follows that of 4-step RACH preamble.

Agreements:

For the determination of the PUSCH Tx power, further study at least the following components including possible down selection:

- An offset relative to the preamble received target power

  - Option 1: Offset configured for 2-step RACH

  - Option 2: Offset is the release 15 delta_preamble_msg3

  - Option 3: Offset is the release 15 delta_preamble_msg3 + configurable delta

- An offset relative to the MsgA PRACH Tx power for the MsgA PUSCH Tx power configured for 2-step RACH.

- Transmission bandwidth of MsgA PUSCH

- MsgA PUSCH Transport format (ΔTF). Further study the following options for further down selection

  - Option 1: deltaMCS configured for 2-step separate from 4-step

  - Option 2: reuse deltaMCS of 4-step RACH

- Preamble received target power.

- Pathloss. Further study the following options for further down selection

  - Option 1: Full pathloss compensation (α = 1)

  - Option 2: Partial pathloss compensation alpha configured for 2-step separate from that of 4-step RACH.

  - Option 4.3: Partial pathloss compensation using msg3-alpha.

- RS resource index for pathloss estimation.

- Total power ramp-up requested by higher layers for MsgA PUSCH Tx:

  - Option 1: from the first to the current MsgA PUSCH transmission (∆Prampuprequested).

  - Option 2: from the first to the latest random access MsgA preamble transmission (∆Prampuprequested).

- Power reduction priority rule in CA/DC

Agreements:

- For MsgA Tx beam selection further study at least the following options:

  - Option 1: The MsgA PRACH and MsgA PUSCH use the same Tx spatial filter (beam).

  - Option 2: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) up to UE implementation.

    - No spec impact expected.

    - Note: in 4-step RACH it is up to UE implementation to decide the beams for Msg1 and Msg3.

  - Option 3: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) under network control/assistance.

- MsgA retransmission, if supported, is defined as a retransmission of MsgA PRACH (with a re-selection of preamble) and MsgA PUSCH. Further study at the following options:

  - Option 1: Using the same payload for MsgA PUSCH.

  - Option 2: MsgA PUSCH payload can be different.

  - FFS: Conditions for MsgA retransmission and relation to fall back.

- FFS: retransmission of PUSCH only.

- FFS: retransmission of PRACH only.

In this contribution, we will discuss the remaining issues on two-step RACH related procedures. The channel structure and design considerations for msgA and msgB are discussed in a companion paper [4].

# 2. 	Design Objectives for MsgA and MsgB

Table 1 lists the physical signals and channels used by msgA and msgB, as well as the contents of each message type. Figure 1 shows an example of contention-based two-step RACH procedure.

- Before starting two-step RACH, UE receives and processes SSB/SIB/RS from the serving gNB.

  - The configuration information for two-step RACH is carried by system information (SI).

  - For RRC_CONNECTED UE, the configuration information can also be carried by RRC signaling.

- Based on the configuration information for two-step RACH as well as the contents of message A (msgA), UE will select a RACH occasion (RO) [2] and a PUSCH resource unit (PRU) [3], and transmit the preamble and payload on the selected RO and PRU, respectively.

  - To record the number of two-step RACH re-attempts, a counter for msgA retransmissions can be configured on UE side. The maximum value of this counter is configured by the network.

- After receiving msgA, gNB will process the preamble first.

  - If the msgA preamble can be successfully detected, gNB will continue processing the payload of msgA. After that, a message B (msgB) will be transmitted to the two-step RACH UE within the RAR window. The contents of msgB [1, 4, 11] depend on the outcome of msgA processing:

    - If the payload of msgA can be successfully decoded, msgB will include at least the contention resolution information (e.g. a unique UE identifier).

    - If the payload of msgA cannot be decoded, msgB will include at least the RAPID and the UL grant for retransmitting the payload part of msgA.

      - If UE can successfully decode the RAPID and UL grant, it will fall-back to four-step RACH procedure, by re-transmitting the payload part of msgA on the granted resources.

  - If the msgA preamble fails to be detected, gNB will not transmit anything to the two-step RACH UE.

- After msgA is transmitted, a RAR window will be configured on UE side. msgB will be monitored within the RAR window [12]. The behaviors and procedures of UE depend on the outcome of msgB reception [4].

  - Before the RAR window expires, if UE can decode the msgB in response to its latest msgA transmission/re-transmission (or detect the contention resolution information associated with the identity of the UE), the two-step RACH procedure is considered complete with success.

  - When the RAR window expires, if UE does not receive a msgB and the counter of re-transmissions is less than the maximum value, UE will re-transmit its msgA preamble and payload, which is considered as a re-attempt of two-step RACH procedure.

  - When the RAR window expires, if no msgB is received and the counter of msgA re-transmissions has reached its maximum, the two-step RACH procedure is considered as ended with failure.

Therefore, we have the following proposal:

Proposal 1:  In order to operate two-step RACH in all RRC states and all cell sizes supported by NR Rel-15, the following design objectives should be satisfied:

- configurable waveform, MCS and contents should be supported in msgA transmission, wherein the payload can carry the equivalent content of msg3 in four-step RACH, UP data, MAC CE and piggybacked UCI;

- msgA should support preamble formats specified in NR for both small cells and large cells;

- msgB should enable the completion of two-step RACH procedure when both preamble and payload of msgA are correctly detected, or request re-transmission of payload on granted resources if msgA preamble detection is successful but msgA payload decoding fails.

- Preamble partitioning and/or UCI can be used to indicate the MCS and resource allocation of msgA payload. When UCI is supported, rate matching of msgA payload needs to be specified.

Table 1: Physical Signals and Channels Employed by Two-Step RACH

Figure 1:  Example of Two-Step RACH Procedure

# 3. 	RO and PO Design for msgA

The resource allocation for msgA should be configurable, which is able to support different preamble formats (long and short PRACH preamble sequences), different payload size (a few bytes to a few hundred bytes) and configurable MCS [1-4].

The resource allocation for msgB should also be configurable, which depends at least on the contents/payload size, cell coverage, traffic load as well as the processing outcome of msgA [1-4, 7].

## 3.1   RO Configurations for msgA Preamble

Similar to msg1 transmission in four-step RACH, the pool of preamble sequences can be partitioned into mutually exclusive subsets, and each subset can be associated with one or multiple POs of msgA. The POs  can be configured with different resource size to accommodate different waveform, MCS and payload size.

In NR Rel-15, the RACH occasion (RO) of msg1 is associated with a suitable SSB beam. In two-step RACH, the transmission of msgA preamble can re-use the beam association rule of four-step RACH [2]. Examples of two-step RACH RO configuration are shown in Table 2. Both separate and shared RO can be configured for two-step RACH. When a RO is shared between two-step RACH and four-step RACH, the pool of PRACH preambles can be partitioned into mutually exclusive subsets, which are used by different RACH procedures.

Table 2: Examples of Two-Step RACH RO Configurations

## 3.2   PO Configuration for msgA Payload

Due to the bursty traffic pattern and the nature of contention-based random access, it is inefficient to dedicate one PO to a single UE. To improve the spectral efficiency, one PO can be shared by UEs with similar payload size, MCS and waveform. To accommodate different payload size, MCS and waveform, multiple POs can be configured within a RO period, which can be grouped into DMRS/PUSCH resource set with distinctive configurations in payload size/MCS/waveform.

Feasible choices of MCS and PO size for msgA payload transmission should be specified for each RRC state, based on the use cases of two-step RACH and the deployment supported by NR Rel-15. A discussion on the resource size of PO and the coverage considerations of msgA can be found in our companion paper on channel structure and the references therein [3].  In addition, LLS can be used to refine the feasible choices for MCS and payload size [4].

As a simplification of resource size specification for PO, a PUSCH resource element definition in time-frequency domains can be introduced, similar to the CCE definition of PDCCH in NR Rel-15 [5].

Based on the CIoT traffic model  [9], Figure 2 shows the statistical distribution of packet arrives for different cell size and deployment density, which can be considered as a reference for resource allocation of PO:

- ISD=1.732 km, device deployment density=106 devices/km2;

- ISD=25 km, device deployment density=5×103 devices/km2.

Figure 2:  Number of Packet Arrivals for Different Cell Size and Device Deployment Density

## 3.3   RO to PO Mapping

For shared RO between two-step RACH and four-step RACH, preamble partitioning can be used to reduce the complexity of implementation. For msgA PUSCH transmission, waveform different from that of msg3 can be supported. For a RO dedicated to two-step RACH (i.e. separate RO), preamble partitioning can also be employed to indicate different PO configurations, wherein a PO can be configured with specific combination of waveform/MCS/payload size.

An example of preamble partitioning and RO to PO mapping is shown in Figure 3, wherein the pool of preamble sequences is partitioned into two subsets, A and B. Each subset is associated with a pre-configured PUSCH resource unit, which comprises one or multiple PO. Consequently, UE selecting preamble subset A will transmit its payload using DMRS/PUSCH ResourceSet A (PUSCH resource unit type A), whereas UE selecting preamble sequences from subset B will transmit its payload using DMRS/PUSCH ResourceSet B (PUSCH resource unit type B). The size/dimension of ResourceSet A and B can be similar or different, and each ResourceSet can be configured with one or multiple POs. Therefore, the partitioning of preamble sequences and RO to PO mapping can be leveraged to indicate/configure different waveform/MCS/payload size for msgA transmission.

On the other hand, constrained resource mapping can be considered for msgA transmission on asynchronous UL with large timing offset (e.g. msgA transmission in RRC_IDLE or RRC_INACTIVE state without a valid TA). As shown in Figure 4, the RO and PO for asynchronous PUSCH transmission can be configured to be adjacent in time and overlapping (fully or partially) in frequency. As a result, the msgA preamble can be used by gNB to enhance the performance of channel estimation and PUSCH decoding.

Figure 3: Preamble Partitioning and RO to PO Mapping

Figure 4: Constrained Resource Mapping for Async UL

Proposal 2:  The following designs for RO and PO should be supported:

- multiple POs can be configured per PO configuration period, to accommodate different waveform, MCS and payload size;

- multiple two-step RACH UEs can share the same PO, if their msgA transmission use similar configurations for waveform/MCS/payload size;

- to simplify resource size specification for PO, PUSCH resource element in time-frequency domains can be defined;

- resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain;

- constrained resource mapping can be configured for asynchronous msgA transmission with potentially large timing offset, wherein the corresponding RO and PO are adjacent in time and overlapping in frequency;

- it is beneficial to balance the resource allocations for RO and PO to achieve a good trade-off in resource utilization efficiency and collision probability.

# 4.	Timing Control Procedures for msgA

As stated in the WID [1], two-step RACH shall be able to operate regardless of whether the UE has a valid TA or not. Typically, UE in RRC_INACTIVE or RRC_IDLE state may not be UL synchronized with gNB. There are multiple solutions to improve the performance of two-step RACH and reduce the transceiver implementation complexities for asynchronous UL without a valid TA.

- Option 1: UE can apply a timing adjustment, say Tadj, based on DL measurements for RSRP and/or positioning.  Tadj can be applied to both preamble and payload of msgA, as shown in Figure 5; or to the payload part of msgA only, as shown by Figure 6. The positioning measurements can be based on SSB/SIB, or additional RS such as PRS and CSI-RS.

- Option 2: gNB can estimate the timing offset of UL transmission by processing the msgA preamble. Based on the timing offset estimation, gNB can perform timing adjustment and/or UE grouping. Multiple processing windows can be applied to UE groups characterized by different range of timing offsets, as illustrated in Figure 7.

Figure 5: UE Assisted Timing Adjustment for Both Preamble and Payload of msgA

Figure 6: UE-Assisted Timing Adjustment for Payload Part of msgA Only

Figure 7: Timing Offset Estimation and Adjustment by gNB

Proposal 3: Both UE-assisted timing adjustment and gNB-assisted timing adjustment can be applied to msgA transmission or reception.

- For UE-assisted timing adjustment, the UL timing offset estimation can be obtained from DL measurements.

- For gNB-assisted timing adjustment, the UL timing offset can be obtained at least from the msgA preamble processing.

# 5.	Power Control Procedures for msgA

Since msgA preamble will re-use the NR Rel-15 PRACH design [1], and a configurable transmission gap can be inserted between msgA preamble and payload [4], it is unnecessary to apply different power control parameters for msgA preamble. The power control parameters specified for msg1 in NR Rel-15 can be re-used for msgA preamble transmission.

In NR Rel-15, if a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c, using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE can determine the PUSCH transmission power in PUSCH transmission occasion i as

where,

- is the UE configured maximum output power defined in TS 38.101;

- is a parameter composed of the sum of a component  and a component  where ;

- is the pathloss compensation factor;

- and μ denote the number of resource blocks and SCS configuration, respectively, which represent the PUSCH bandwidth dependent power control offset;

- is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index  for the active DL BWP of serving cell c;

- represents the transport format dependent power control offset;

The payload transmission in msgA can re-use the NR Rel-15 PUSCH power control mechanism outlined above. To minimize the impacts on specification and provide sufficient flexibility for configurable payload size and MCS of msgA, we have the following proposal:

Proposal 4: The following designs for power control procedures can be considered for msgA transmission in two-step RACH:

- OLPC should be applied for msgA preamble and payload transmission;

- power control parameters specified for msg1 in NR Rel-15 can be re-used for msgA preamble;

- in determining the value of,  a network configured offset ∆f,c(j) can be applied on top of PO_PRE;

- the PUSCH bandwidth dependent power control offset in NR Rel-15 should be re-used;

- the PUSCH transport-format dependent power control offset in NR Rel-15 should be re-used;

- partial pathloss compensation should be supported and configured by network;

- PUSCH-PathlossReferenceRS-Id can re-use NR Rel-15 design; additional RS resources, such as DMRS/SIB and PRS, can also be configured to improve the accuracy of pathloss measurements;

- when UE capability supports DC/CA, TX power allocation rules should be specified across multiple carriers;

- when msgA retransmissions are associated with beam switching, power ramping procedures should be considered jointly with beam switching procedures.

# 6.	Fall-Back and Retransmission Procedures for Two-Step RACH

Both two-step RACH and four-step RACH procedures should be supported in NR Rel-16 and beyond. UE can make RACH type selection at the beginning of RACH procedure, or after a few re-attempts of RACH procedure of the same type.

Proposal 5: Both fall-back and re-attempt should be supported in two-step RACH. RACH type selection can be supported at the beginning of a RACH procedure, or after the number of re-attempts for  two-step RACH goes beyond the maximum value configured by the network.

- Two-step RACH can fall-back to four-step RACH, when the msgA preamble detection is successful but the msgA payload decoding fails. To trigger fall-back to four-step RACH, gNB needs to send the RAR in msgB, which includes at least the RAPID, the timing advance command and the UL grant of the fall-back UE.

- msgA can be re-transmitted if the UE does not receive a msgB (RAR or contention resolution) within the RAR window, and the counter of msgA retransmissions has not reached its maximum value configured by the network. When msgA is re-transmitted, the UE will transmit both preamble and payload. Both the preamble and payload can be different from the last transmission of msgA.

# 7. 	Beam Management and UCI Considerations

In NR Rel-15, each RO is associated with a corresponding DL SSB index, so that UE’s choice of RO can indicate UE’s preferred DL beam to the gNB. UEs with beam-correspondence also use this DL beam for the msg1 and msg3 transmissions. This behavior can naturally be extended for the case of two-step RACH, which leads to the following proposal:

Proposal 6: Allow configurations in which UE can determine its UL beam for msgA preamble in the same manner as for four-step RACH, and use the same UL beam for both preamble and payload of msgA.

In NR Rel-15, PUSCH slot-repetition is RRC-configured, and thus, enabling slot-repetition for msg3 would have involved either configuration prior to RACH (in SIB1) implying every msg3 transmission has the same repetition factor, or extending the UL grant field in the msg2 RAR payload to dynamically indicate the repetition factor. Thus, Rel-15 msg3 does not support slot-repetition, i.e., msg3 PUSCH is always in a single slot. For two-step RACH on the other hand, as part of PO configuration, different POs could easily be configured with different slot repetition factors. This allows for a flexible tradeoff between RACH overhead and time-diversity robustness. Further, the different slot repetitions could then be transmitted with different transmit beams. This provides further robustness from beam-diversity. The transmit beams could be determined by a configured association with other SSB-indices. Note that NR Rel-15 rules could still be reused for preamble re-transmissions. Based on this, we propose the following:

Proposal 7: Allow configuration of POs with slot-repetition, and allow different repeated slots to use different transmit beams.

Early reporting of proper uplink control information (UCI) can be utilized by network to improve system performance. For one example, this has been utilized in the so-called preamble partitioning of NR Rel-15 RACH procedure. With this configuration, a UE can choose between group A and group B of preambles to provide a one-bit uplink control information to assist network in scheduling msg3.

Within 3GPP, attempt for more accurate and more detailed UCI reporting during random access procedure was made in LTE NB-IoT/eMTC, which finally led to the adoption of including CQI report as RRC message in msg3 in LTE Rel-15. In addition, more CQI reports are being added for more operation modes. More bits of UCI can provide more accurate/detailed information which allows gNB to have more scheduling flexibility as well as better link adaptation. One example use case is that UCI can include beam index indication, potentially along with the corresponding L1-RSRP.

In NR Rel-15, gNB indicates SSB to RO mapping in RMSI and UE can send RACH corresponding to any suitable SSB. Depending on the RO configuration and latency requirement, UE may not afford to wait for the later RO corresponding to a better SSB to send msg1. Instead, UE may want to transmit msg1 corresponding to the earlier suitable SSB. However, with current NR Rel-15 solution, UE has to stay with the beam used for msg1. These two options are listed in Figure 8.

Figure 8: Showing RO Selection Options for a NR Rel-15 UE

We can observe that Option 1 allows lower latency at the cost of a suboptimal beam, while Option 2 allows a better beam while suffering a latency penalty. With two-step RACH where msgA includes PUSCH resources, it is possible for UE to include UCI report in msgA, as the WID indicated. This allows UE to send msgA at earlier time instance while indicating the preferred beam (potentially along with the corresponding L1-RSRP) in UCI. Once gNB detects UCI, gNB could use the preferred beam for subsequent transmission to achieve better link quality. In this way, UE can benefit from both better beam and low latency, as shown in Figure 9.

Figure 9: Improving Both Latency and Beam Quality by Including UCI in msgA

UCI may be included in msgA either using NR Rel-15 piggybacking mechanism for UCI-on-PUSCH, or within the payload, i.e., within the MAC PDU or as a MAC-CE. The same mechanism may then be extended to include UCI in msg3 after fallback from two-step RACH to four-step RACH. This leads to the following proposal:

Proposal 8: Allow UCI reporting in msgA.

# 8. 	Conclusions

In this contribution, the procedures related to two-step RACH, which target both licensed spectrum (FR1 and FR2) and unlicensed spectrum (FR1), has been discussed. To conclude, we have the following proposals:

Proposal 1:  In order to operate two-step RACH in all RRC states and all cell sizes supported by NR Rel-15, the following design objectives should be satisfied:

- configurable waveform, MCS and contents should be supported in msgA transmission, wherein the payload can carry the equivalent content of msg3 in four-step RACH, UP data, MAC CE and piggybacked UCI;

- msgA should support preamble formats specified in NR for both small cells and large cells;

- msgB should enable the completion of two-step RACH procedure when both preamble and payload of msgA are correctly detected, or request re-transmission of payload on granted resources if msgA preamble detection is successful but msgA payload decoding fails.

- Preamble partitioning and/or UCI can be used to indicate the MCS and resource allocation of msgA payload. When UCI is supported, rate matching of msgA payload needs to be specified.

Proposal 2:  The following designs for RO and PO should be supported:

- multiple POs can be configured per PO configuration period, to accommodate different waveform, MCS and payload size;

- multiple two-step RACH UEs can share the same PO, if their msgA transmission use similar configurations for waveform/MCS/payload size;

- to simplify resource size specification for PO, PUSCH resource element in time-frequency domains can be defined;

- resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain;

- constrained resource mapping can be configured for asynchronous msgA transmission with potentially large timing offset, wherein the corresponding RO and PO are adjacent in time and overlapping in frequency;

- it is beneficial to balance the resource allocations for RO and PO to achieve a good trade-off in resource utilization efficiency and collision probability.

Proposal 3: Both UE-assisted timing adjustment and gNB-assisted timing adjustment can be applied to msgA transmission or reception.

- For UE-assisted timing adjustment, the UL timing offset estimation can be obtained from DL measurements.

- For gNB-assisted timing adjustment, the UL timing offset can be obtained at least from the msgA preamble processing.

Proposal 4: The following designs for power control procedures can be considered for msgA transmission in two-step RACH:

- OLPC should be applied for msgA preamble and payload transmission;

- power control parameters specified for msg1 in NR Rel-15 can be re-used for msgA preamble;

- in determining the value of,  a network configured offset ∆f,c(j) can be applied on top of PO_PRE;

- the PUSCH bandwidth dependent power control offset in NR Rel-15 should be re-used;

- the PUSCH transport-format dependent power control offset in NR Rel-15 should be re-used;

- partial pathloss compensation should be supported and configured by network;

- PUSCH-PathlossReferenceRS-Id can re-use NR Rel-15 design; additional RS resources, such as DMRS/SIB and PRS, can also be configured to improve the accuracy of pathloss measurements;

- when UE capability supports DC/CA, TX power allocation rules should be specified across multiple carriers;

- when msgA retransmissions are associated with beam switching, power ramping procedures should be considered jointly with beam switching procedures.

Proposal 5: Both fall-back and re-attempt should be supported in two-step RACH. RACH type selection can be supported at the beginning of a RACH procedure, or after the number of re-attempts for  two-step RACH goes beyond the maximum value configured by the network.

- Two-step RACH can fall-back to four-step RACH, when the msgA preamble detection is successful but the msgA payload decoding fails. To trigger fall-back to four-step RACH, gNB needs to send the RAR in msgB, which includes at least the RAPID, the timing advance command and the UL grant of the fall-back UE.

- msgA can be re-transmitted if the UE does not receive a msgB (RAR or contention resolution) within the RAR window, and the counter of msgA retransmissions has not reached its maximum value configured by the network. When msgA is re-transmitted, the UE will transmit both preamble and payload. Both the preamble and payload can be different from the last transmission of msgA.

Proposal 6: Allow configurations in which UE can determine its UL beam for msgA preamble in the same manner as for four-step RACH, and use the same UL beam for both preamble and payload of msgA.

Proposal 7: Allow configuration of POs with slot-repetition, and allow different repeated slots to use different transmit beams.

Proposal 8: Allow UCI reporting in msgA.

# References

[1] RP-182894, “New Work Item: Two-Step RACH for NR,” ZTE, 3GPP TSG RAN #82, Sorrento, Italy.

[2] 3GPP TSG RAN WG1 Meeting #96, “RAN1 Chairman’s Notes,” Athens, Greece.

[3] 3GPP TSG RAN WG1 Meeting #96B, “RAN1 Chairman’s Notes,” Xi’an, China.

[4] R1-1907255, “Channel Structure for Two-step RACH,” Qualcomm, 3GPP TSG RAN1 #97, Reno, USA.

[5] 3GPP TS 38.211 V15.4.0, “NR; Physical Channels and Modulation (Release 15),” December 2018.

[6] R2-1905472, “LS on MsgA Content Size,” 3GPP TSG RAN WG2 Meeting #105B, Xi’an, China.

[7] R2-1907911, “MsgB Content in Two-Step RACH,” Qualcomm, 3GPP TSG RAN WG2 Meeting #106, Reno, USA.

[8] R2-1907910, “Two-Step RACH Fall Back to Four-Step RACH,” Qualcomm, 3GPP TSG RAN WG2 Meeting #106, Reno, USA

[9] 3GPP TR 45.820, V13.1.0, “Cellular System Support for Ultra-Low Complexity and Low Throughput Internet of Things,” November 2015.