3GPP TSG RAN WG1 #97	R1- 1907346

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.9.2

Source:	Apple Inc.

Title:	Cross Slot Scheduling for UE Power Saving

Document for:	Discussion, Decision

# Introduction

During the last RAN1#96b meeting, we made following agreements regarding the cross slot scheduling technique.

Agreements: - Regarding aperiodic CSI-RS triggering, at least if a UE is operated with cross-slot scheduling based power saving, - If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states and the PDCCH SCS is equal to the CSI-RS SCS, specification allows the aperiodic CSI-RS triggering offset to be set to a non-zero value. Agreements: - For an active DL and an active UL BWP, a UE can be indicated via signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured) where the signalling type is to be down-selected from: - Alt 1: MAC-CE based - Alt 2: L1 based - FFS: How to determine the minimum applicable value if explicit value is not provided. Agreements: Possible candidate indication methods to adapt the minimum applicable value of K0 (or K2) for an active DL (or UL) BWP, where the indication method is to be selected from: - Alt 1: Indication of a subset of TDRA entries, e.g., bit-map based indication - Alt 2: Indication of one active table from multiple configured TDRA tables - Alt 3: Indication of the minimum applicable value - Note: Other option is not precluded Note: PDCCH monitoring case 1-1 is prioritized for the design. FFS: Whether and how the minimum applicable K0 (or K2) value of the active DL (or UL) BWP is also applied to cross-BWP scheduling Agreements: Possible candidate indication methods to adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, where the indication method is to be selected from: - Alt 1: Implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated - Alt 2: Indication of the minimum applicable value - Note: Other option is not precluded Note: PDCCH monitoring case 1-1 is prioritized for the design. Agreements: - The adaptation on the minimum applicable value of K0 does not apply to at least the following cases: RNTI; PDCCH search space. SI-RNTI; Type0 common. SI-RNTI; Type0A common. RA-RNTI, TC-RNTI; Type1 common. P-RNTI; Type2 common. Conclusion: Companies are encouraged to further investigate whether to apply the adaptation for at least the following cases: - K0 related: RNTI; PDCCH search space. C-RNTI, CS-RNTI. MCS-C-RNTI. - K2 related: RNTI; PDCCH search space. PUSCH scheduled by MAC RAR as described in subclause 8.2 of TS 38.213. C-RNTI, TC-RNTI, CS-RNTI. MCS-C-RNTI.

In this contribution, we discuss the potential enhancements on cross-slot scheduling techniques in R16.

# Cross Slot Scheduling in R16

## Signaling Type

It has been agreed in RAN1#96b meeting that the dynamic indication of minimum K0, K2 and aperiodic CSI-RS triggering offset is to be supported. The remaining issue is to determine signaling type to be supported. Two alternatives were discussed last time and to be down selected.

Agreements: - For an active DL and an active UL BWP, a UE can be indicated via signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured) where the signalling type is to be down-selected from: - Alt 1: MAC-CE based - Alt 2: L1 based - FFS: How to determine the minimum applicable value if explicit value is not provided.

The decision should be made taking into account required reliability and requirement for latency of the signaling of interest.

Reliability

What is the signaled is the minimum applicable values (of K0,K2, CSI-RS triggering offset) which limits the potentially indicatable values of K0, K2, and CSI-RS triggering offset in DCI. If there is any signaling error for minimum applicable values change, then gNB and UE would have different understanding on what could be signaled. This could be potentially problematic depending on the design of potential TDRA entry change mechanism. If UE and gNB are completely out of sync of what is meant by Time domain resource assignment field value, then the signaling requires higher reliability to avoid such case. If potential TDRA entry change mechanism is designed such that it is robust to such error, then, lower reliability (DCI based) could be accepted. Otherwise, more reliable approach (MAC-CE based) is preferred.

Observation 1

- The choice of signaling type depends on required reliability, which is again depends on the potential TDRA entry change mechanism (i.e., indication method).

- If potential TDRA entry change mechanism is robust to signaling error, then DCI based approach is acceptable.

- If potential TDRA entry change mechanism is less robust to signaling error, then MAC-CE based approach should be used.

Latency

The required latency for dynamic switching depends on how fast gNB wants to change the value. In most eMBB applications, the additional delay (of 1ms or more) incurred due to cross slot scheduling would be unnoticeable to end users. So, we think, for most applications, static configuration of TDRA table with minimum K0 value larger than 0 (or any other positive number) should be good enough for power saving.

The one use case which may require the dynamic switching is the case with very low latency requirement. In this case, low latency requirement necessarily have gNB to do same slot scheduling. The same slot scheduling (i.e., with K0=0) requires UE to buffer potential PDSCH symbols received before PDCCH decoding end time. When, there is no active arrival of traffic, avoiding the unnecessary PDSCH symbol buffering could help UE power saving. However, for the use case requiring low latency, this kind of dynamic switching based on L1 would be less attractive if there is any switching delay required.

Observation 2

- The dynamic switching between same slot and cross slot scheduling could be useful for UE power saving only for the use case requiring low latency. But, if there is any switching delay required, then L1 based switching would be less attractive.

Proposal 1

- The decision of signaling type is to be determined after indication method is determined.

## Indication Method

Regarding indication method for adapting minimum applicable value, we made following agreement with at least three different approaches listed below.

Agreements: Possible candidate indication methods to adapt the minimum applicable value of K0 (or K2) for an active DL (or UL) BWP, where the indication method is to be selected from: - Alt 1: Indication of a subset of TDRA entries, e.g., bit-map based indication - Alt 2: Indication of one active table from multiple configured TDRA tables - Alt 3: Indication of the minimum applicable value - Note: Other option is not precluded Note: PDCCH monitoring case 1-1 is prioritized for the design. FFS: Whether and how the minimum applicable K0 (or K2) value of the active DL (or UL) BWP is also applied to cross-BWP scheduling

Fundamentally, what needs to be indicated the set of indicatable TDRA entries with minimum K0, K2 values larger than zero. As long as UE is aware of the minimum value of K0 is larger than 0, it can avoid unnecessary PDSCH buffering. We discuss three methods for indication.

Alt1: Set Indication

- Network configures a TDRA table per BWP according to R15 procedure.

- Configuration (per BWP)

  - The TDRA entries in the TDRA table forms a universal set in the BWP.

  - Network can further specify different subsets of the universal set.

  - Each code point of DCI field for signaling corresponds to one set.

- Set indication

  - Network can send one DCI code point to indicate a set of TDRA entries of which member could be potentially indicated by scheduling DCIs.

This is could be best explained by Figure 1, where two TDRA entry sets are configured based on RRC configured per-BWP TDRA table. The first set (set0) includes all the entries (including yellow and green) in the TDRA table. In this example, K0=0 is included in set0. Thus, UE should be prepared receive same slot scheduling with K0=0 when potentially indicatable set is set0. The second set (set1) includes only green TDRA entries. In this case, all the K0 values are larger than 4, which make sure UE avoid unnecessary PDSCH buffering.

Figure 1 The pre-configured TDRA entry sets and mapping between DCI field value and TDRA entry sets.

There are several benefits of this approach. First, signaling overhead is low. A single bit is required to support two sets. Second, it is flexible since each set can include arbitrary set of TDRA entries. The minimum applicable K0 could be derived as the minimum K0 value from the set indicated. In the given example in Figure 1, the derived minimum applicable value for K0 would be 0 and 4 for set0 and set1 respectively.

Alt2 : Table Indication

The second approach Alt2 of multiple TDRA tables support is in principle very similar approach. In this approach, different table is different set. However, considering that most critical entry for avoiding unnecessary PDSCH buffering is K0=0, there could be many TDRA entries shared between two tables. This is redundancy in RRC signaling which is basically signaling overhead to both gNB and UE.

Alt3 : Value Indication

The third approach Alt3 is a straightforward method indicating minimum applicable value. Compared to Alt1, it requires more bits (e.g., 4 bits), and the difference gets larger when multiple cells are considered. The signaled minimum applicable value could be used to update existing TDRA table such that K0 values in all entries are updated as max(K0_min, K0).

This approach basically allows all the entries be indictable, which keeps high flexibility in scheduler. It turns out that this approach of keeping all entries indicatable is good solution for cross BWP switching, which is explained later. Figure 2 shows the updated TDRA table, where K0_min (derived minimum applicable value) is used to update TDRA table. After K0_min is derived from signaling, the actual K0 value applied is updated with max(K0_min, K0) value.

Figure 2 TDRA table updated with derived minimum applicable K0 ( K0_min ).

Proposal 2

- As a cross slot indication method, support Alt1 or Alt3.

## Consideration of Cross BWP scheduling

To support cross BWP scheduling with minimum applicable values for K0 and K2, following aspects need to be considered.

(A1) The derived minimum applicable value and BWP switching delay (TBWPswitchDelay)

(A2) Potentially indicatable TDRA entry set in target (indicated) BWP

(A3) Different SCS

### Aspect (A1) : The Derived Minimum Applicable Value and BWP Switching Delay

For cross slot scheduling purpose, if we consider the case of minimum K0 ≤ TBWPswitchDelay, then there is no need to take into account of aspect (A1) since current R15 prohibits gNB to do cross BWP scheduling with PDSCH starting before BWP switching delay, as shown below.

“A UE does not expect to detect a DCI format 1_1 or a DCI format 0_1 indicating respectively an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change [TS 38.133].”

Table 1 shows the BWP switching delay currently defined in 38.133. So, for all cases below, the cross BWP scheduling will naturally avoid scheduling with K0 < TBWPswitchDelay.

Table 1 BWP switch delay [4]

However, if minimum K0 > TBWPswitchDelay, then aspect (A1) needs to be considered in cross BWP scheduling.

Observation 3

- If minimum applicable K0 ≤ TBWPswitchDelay, then there is no need to consider cross BWP scheduling with minimum applicable values in the TDRA table in target BWP since R15 UE is not supposed to receive cross BWP scheduling before n+TBWPswitchDelay, where n is the slot where DCI is received.

- If minimum applicable K0 > TBWPswitchDelay,, then minimum applicable value needs to be considered in TDRA table in target BWP for cross BWP scheduling.

We further discuss possible range of K0 in Section 2.7.

### Aspect (A2) : Potentially Indicatable TDRA Entry Set in Target BWP

In R15, the potentially indicatable TDRA entry in target BWP is limited by the bit length of the field of DCI in active BWP. Depending on the size of bit field in active and target BWP, either zero padding of truncation could be used. The limitation of indication occurs only when zero padding happens. For example, information field length in active BWP and target BWP are 3bits and 4bits respectively. Then, the TDRA entries which can be indicated in cross BWP scheduling DCI in active BWP is only the first 8 entries since MBS is zero padded with 0.

The problem here is that if (A1) is considered and the potentially indicatable entries in TDRA table in target BWP are further limited based on derived minimum applicable K0 value, then depending on the TDRA configuration, there could be no indicatable entries remained especially when entries are ordered in ascending order of K0 value as shown in Figure 3.

Figure 3 The case with no remaining TDRA entries after limitation based derived minimum K0 value

Simple solution is to order TDRA entries in descending order of K0 values. This will make sure at least one TDRA entries indicatable in target BWP (with largest K0 value). And, updating K0 values in TDRA table with max(K0_min, K0) also helps in this case.

Figure 4 TDRA table with entries sorted in descending order of K0 values

Observation 4

- If TDRA table is not ordered properly, it could be possible that there could no indictable TDRA entries left after limiting indicatable entries based on derived minimum applicable K0 or K2 value.

Proposal 3

- UE expects that TDRA tables are configured such that their entries are ordered in descending order of K0 value.

### Aspect (A3) : Different SCS

R15 allows BWP scheduling across BWP with different SCS. In this case, the K0 value indicated in data scheduling DCI is counted in terms of scheduled BWP. If minimum applicable K0/K2 is derived, this cannot be applied directly to TDRA table in target BWP due to different slot length. The derived value should be translated to K0/K2 value in terms of SCS of the target BWP.

Observation 5

- If derived minimum K0/K2 is to be applied to TDRA table in target BWP, then it should be translated and applied accordingly in terms of SCS of the target BWP.

## Adaptation on Aperiodic SRS slot Offset and/or K1 Value

The aperiodic SRS offset is fundamentally similar to K2. Both quantity describes the time gap between DCI and actual tx time (of PUSCH or SRS). The difference is the amount of BB processing for PUSCH and SRS since UE receives DCI requesting uplink transmission. Since SRS is relatively simple compared to PUSCH, the required preparation time for SRS could be shorter than that of PUSCH. So, it seems that there is no strong motivation for further delaying SRS transmission.

Proposal 4

- Do not limit aperiodic SRS slot offset.

The minimum K1 value could relax the processing speed of UE. Accordingly UE could use lower voltage. However, it should be noted that it prolongs the processing time for the TB, which could be negative in terms of power consumption. So, it is not very clear whether limiting K1 would lead to actual power decrease or increases. If UE could quickly finish PDSCH processing, it could potentially sleep early, which could save more power.

Observation 6

- It is not clear whether limiting minimum K0 (and relaxing PDSCH processing) would accrual lead to increase or decrease power consumption.

## TDRA Switching Delay

If a new minimum applicable K0/K2 is derived from gNB signaling in a slot, say N, then UE is supposed to apply this value at some point in time after the slot N. This is not only related to required switching time but also to the understanding between gNB and UE. The switching timing should be determined to make sure gNB and UE have the same understanding on potentially indicatable TDRA entries

Depending on the choice of signaling type, i.e., DCI or MAC-CE, the timing could be different. If signaling type is determined as DCI as shown in Figure 5, then the PDCCH decoding delay needs to be considered. The TDRA limitation cannot be applied before the corresponding PDCCH decoding finishes. Especially for the case with large minimum K0, the application time should be at least current minimum K0 value apart from the slot where DCI is received.

Observation 7

- If DCI is selected as signaling type, the application time of derived new minimum K0/K2 should be at least Z  slots apart from the slot where the DCI is received.

  - Z is at least current minimum K0/K2 value

Figure 5 The case with DCI as signaling type and application time of derived new K0/K2 value

If the signaling type is MAC-CE as shown in Figure 6, then application time could be later than the slot where ACK is transmitted. In this case, by the ACK transmission time, L1 finishes successful PHY decoding. However, L2 needs to process the MAC-CE and have to notify L1 about the TDRA table update based on derived minimum K0 value.

Observation 8

- If MAC-CE is selected as signaling type, the application time of derived minimum K0 and K2 should be [Y] slots from the slot where ACK is transmitted corresponding to the PDSCH containing the MAC-CE.

  - Y is FFS

Figure 6 The case with MAC-CE as signaling type and application time of derived new K0/K2 value

Proposal 5

- Define the application delay L of minimum applicable K0 value, where L is FFS.

## UE preferred K0 and K2

To better reflect UE’s preference on minimum K0 and K2 value, UE should be able to indicate it preferred set of TDRA table entry sets or minimum K0/K2 value.

Proposal 6

- NR supports that UE reports its preferred TDRA entry sets back to gNB among the entries in the RRC-configured TDRA table.

- NR supports UE preferred minimum K0/K2 value indication.

## Range of Derived Minimum Applicable K0 Value

We discuss possible range of derived minimum applicable K0 value and its implication in design. We see following different cases depending on value of K0 for SCS=15kHz.

| Minimum K0 value | Note |
| --- | --- |
| 0 | Allow same slot scheduling |
| 1 | This avoid TDRA entry with K0=0 only. This is good enough for UE to avoid unnecessary PDSCH buffering. |
| 1 < minimum K0 < TBWPswitchDelay | This range of minimum K0 may be required to relax PDCCH decoding time. But, the power saving gain of relaxing PDCCH decoding has never been studied so its gain is unknown. For cross BWP scheduling case, it is rather simple since there is no need to take into account TDRA entries in target BWP. |
| TBWPswitchDelay ≤ minimum K0 | This range of minimum K0 may be required to relax PDCCH decoding time. In cross BWP scheduling, TDRA entries in target BWP needs to be limited by minimum K0 in active BWP. Careful TDRA configuration and minimum K0 value configuration are required at gNB side to make sure there is indicatable TDRA entries in target BWP during cross BWP scheduling. |

Observation 9

- K0=0 needs to be avoided for avoiding unnecessary PDSCH buffering.

- Setting minimum K0 > 1 would allow relaxing PDCCH decoding time. However, no study has been done regarding this case. So, there is no known power saving gain for this case.

Proposal 7

- Derived minimum applicable K0 value could be at least 1.

- Further discuss whether configuring derived minimum applicable K0 larger than 1 would provide power saving gain.

# Conclusions

In this contribution, we have discussed cross slot scheduling techniques in R16. Based on discussion, we make following observations and proposals.

Observation 1

- The choice of signaling type depends on required reliability, which is again depends on the potential TDRA entry change mechanism (i.e., indication method).

- If potential TDRA entry change mechanism is robust to signaling error, then DCI based approach is acceptable.

- If potential TDRA entry change mechanism is less robust to signaling error, then MAC-CE based approach is preferred.

Observation 2

- The dynamic switching between same slot and cross slot could be useful for power saving only for the use case requiring low latency. But, if there is any switching delay required, then L1 based switching becomes less attractive.

Proposal 1

- The decision of signaling type is to be determined after indication method is determined.

Proposal 2

- As an cross slot indication method, support Alt1 or Alt3.

- In this case, the minimum applicable value is determined by the minimum K0/K2 value of the selected TDRA entry set.

Observation 3

- If minimum applicable K0 ≤ TBWPswitchDelay, then there is no need to consider cross BWP scheduling with minimum applicable values in the TDRA table in target BWP since R15 UE is not supposed to receive cross BWP scheduling before n+TBWPswitchDelay, where n is the slot where DCI is received.

- If minimum applicable K0 > TBWPswitchDelay,, then minimum applicable value needs to be considered in TDRA table in target BWP for cross BWP scheduling.

Observation 4

- If TDRA table is not ordered properly, it could be possible that there could no indictable TDRA entries left after limiting indicatable entries based on derived minimum applicable K0 or K2 value.

Proposal 3

- UE expects that TDRA tables is configured such that its entries are ordered in descending order of K0 value.

Observation 5

- If derived minimum K0/K2 is to be applied to TDRA table in target BWP, then it should be translated and applied accordingly to the SCS of the target BWP.

Proposal 4

- Do not limit aperiodic SRS slot offset.

Observation 6

- It is not clear whether limiting minimum K0 (and relaxing PDSCH processing) would accrual lead to increase or decrease power consumption.

Observation 7

- If DCI is selected as signaling type, the application time of derived new minimum K0/K2 should be at least Z  slots apart from the slot where the DCI is received.

  - Z is current minimum K0/K2 value

Observation 8

- If MAC-CE is selected as signaling type, the application time of derived minimum K0 and K2 should be [Y] slots from the slot where ACK is transmitted corresponding to the PDSCH containing the MAC-CE.

  - Y is FFS

Proposal 5

- Define the application delay L of minimum applicable K0 value, where L is FFS.

Proposal 6

- NR supports that UE reports its preferred TDRA entry sets back to gNB among the entries in the RRC-configured TDRA table.

- NR supports UE preferred minimum K0 value indication.

Observation 9

- K0=0 needs to be avoided for avoiding unnecessary PDSCH buffering.

- Setting minimum K0 > 1 would allow relaxing PDCCH decoding time. However, no study has been done regarding this case. So, there is no known power saving gain for this case.

Proposal 7

- Derived minimum applicable K0 value could be at least 1.

- Further discuss whether configuring derived minimum applicable K0 larger than 1 would provide power saving gain.

# References

- TR38.840 Study on UE Power Saving

- RP-190727 New WID: UE Power Saving in NR

- TR38.306 User Equipment (UE) radio access capabilities

- TS38.133 Requirements for support of radio resource management

- TS38.213 Physical layer procedures for control