3GPP TSG RAN WG1 Meeting #97 	R1-1907369

Reno, USA, 13th – 17th May 2019

Title:	On Downlink Performance in NTN

Source: 	Fraunhofer IIS, Fraunhofer HHI

Type:	Discussion

Document for:	Approval

Agenda Item:	7.2.5.1

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

In June 2018, the study item “Study on NR to support non-terrestrial networks” was completed. Potential impacts have been identified (see [1] for the full list) and solutions are currently investigated in the study item “Solutions on NR to support non-terrestrial networks”. From a RAN1 perspective, TUs have been allocated starting from April 2019 to November 2019. The following objectives for RAN 1 have been approved in [2] :

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1].

This contribution focuses on the issue of performance assessment in NR NTN downlink, where the high power amplifier (HPA) of the satellite transmitter typically introduces significant non-linear distortion in the signal. We note necessary assumptions for gauging the impact of satellite HPA non-linearity on block error rate (BLER) performance and present initial link-level simulation results.

# NR Downlink Performance in NTN

As satellite systems are power-limited, the required amplification of the downlink transmissions might necessitate operation extending significantly into the non-linear region of the satellite HPAs [3], [4]. Effect of such non-linearity on the performance of NR NTN downlink is likely to be more pronounced, considering the high peak-to-average power ratio (PAPR) of OFDM waveforms.

### Link-Level Simulation Assumptions

In order to focus solely on the impact of the HPA non-linearity on BLER performance, the link-level simulations we do are with SISO transmission under AWGN channel setting. DMRS-based channel estimation is enabled so that any phase rotation introduced by the HPA model is mitigated before decoding. The simulation settings are listed in Table 1.

Table 1: Settings

| Parameters |  | Values |
| --- | --- | --- |
| Bandwidth |  | 30 MHz |
| SCS |  | 15 KHz |
| Total # of PRBs |  | 160 |
| PRB bandwidth |  | 28.8 MHz |
| Desired allocation | # of RBs | 15 |
|  | # of data REs | 1960 |
|  | Modulation | 16 QAM |
|  | LDPC rate | 0.35 |
|  | TB size | 2816 |
|  | LDPC graph index | 1 |
| HARQ |  | Disabled |
| TX impairment |  | Satellite PA nonlinearity: Tables 2 and 3 |
| Predistortion techniques |  | Disabled |
| Channel estimation |  | DMRS-based |
| Channel type |  | AWGN |

The high non-linearity introduced by HPAs might result in inter-carrier interference (ICI) that is contributed not only from the resource elements (RE) within the desired allocation (which is to be decoded), but also from those in other allocations intended for the multiple UEs per downlink beam. Therefore, to account for such ICI, REs other than that of the desired allocation are filled with arbitrary symbols drawn uniformly at random from the 16 QAM constellation.

Proposal 1: Link-level simulations considering satellite HPA non-linearity should model the ICI between multiple allocations in the same downlink beam, even under AWGN channel or ideal channel knowledge assumptions.

HPA non-linearity models are applied in the link-level simulations as follows: with the OFDM signal input to the HPA being xt, the amplified output signal is

xpat=Axt ej[∠xt + φ(t)] ,

where A . is in accordance with the amplitude modulation to amplitude modulation (AM/AM) characteristics of the HPA while φ(t) is as per the amplitude modulation to phase modulation (AM/PM) characteristics.

We apply the characteristics from the HPA models proposed by [3], namely, the models for Travelling Wave Tube Amplifier (TWTA) and Solid State Power Amplifier (SSPA) that are reproduced in Tables 2 and 3, respectively. As the dynamic range of the high PAPR OFDM input signals to the HPA would go beyond the input range specified in the tables given by [3], we resort to linear extrapolation of the AM/AM and AM/PM characteristics. And, to ensure reasonable slopes in the extrapolations, a few additional points (indicated in red) are introduced in Tables 2 and 3.

Table 2: AM/AM and AM/PM characteristics for linearized TWTA, proposed in [3]

| IBO (dB) | OBO (dB) | Phase (deg) |
| --- | --- | --- |
| -20 | -18 | 0 |
| -18 | -16 | 0 |
| -16 | -14 | -0.5 |
| -14 | -12 | -0.5 |
| -12 | -10 | -0.5 |
| -10 | -8 | -1 |
| -8 | -6 | -1 |
| -6 | -4 | -1 |
| -4 | -2 | -2.5 |
| -3 | -1 | -3 |
| -2 | -0.5 | -4 |
| -1 | -0.1 | -6 |
| 0 | 0 | -8 |
| 1 | -0.1 | -7 |
| 2 | -0.5 | -6 |
| 3 | -1 | -4 |
| 4 | -2 | -2 |
| 5 | -3 | -2 |

Table 3: Typical AM/AM and AM/PM characteristics for SSPA, proposed in [3]

| IBO (dB) | OBO (dB) | Phase (deg) |
| --- | --- | --- |
| -20 | -18.7 | 0 |
| -18 | -16.7 | 0 |
| -16 | -14.7 | 0.5 |
| -14 | -12.7 | 1 |
| -12 | -10.7 | 2.5 |
| -10 | -8.6 | 3.5 |
| -8 | -6.5 | 5 |
| -6 | -4.5 | 6 |
| -4 | -2.6 | 7 |
| -3 | -1.9 | 7 |
| -2 | -1.2 | 7 |
| -1 | -0.6 | 6.5 |
| 0 | 0 | 6 |
| 1 | 0.5 | 5 |
| 2 | 0.9 | 4 |
| 3 | 1.1 | 3.5 |
| 4 | 1.3 | 3 |
| 5 | 1.3 | 3 |

Plotted in Figs. 1—4 are instances of the foregoing input-output responses from our simulations, where the average IBO (input back-off) for the TWTA and SSPA cases were, respectively, 0.28 dB and 1.58 dB. It can be seen that the input range of the OFDM signals goes beyond the range given by the assumptions in [3]. Based on the input power ranges observed (e.g., Figs. 1—4) from simulations under relevant settings, we propose the following.

Fig. 1: TWTA AM/AM characteristics from Table 2.           Fig. 2: TWTA AM/PM characteristics from Table 2.

Fig. 3: SSPA AM/AM characteristics from Table 3.            Fig. 4: SSPA AM/PM characteristics from Table 3.

Proposal 2: Input value range of the HPA AM/AM and AM/PM characteristics specified for NR NTN downlink simulations shall cover the high dynamic range of the OFDM waveforms: recommended range is from -30 dB to 10 dB.

### Simulation Results

The IBO (input back-off) and OBO (output back-off) values in the results hereafter indicate the back-off applied to the transmit signal power on average. Specifically, they are computed as

IBO= Psat/Pin,

OBO= Psat/Pout,

where Psat is the output saturation power from the AM/AM characterises (0 dB for the TWTA from Table 2 and 1.3 dB for the SSPA from Table 3), Pin is the average input signal power to the amplifier and Pout is the average output signal power from the amplifier.

Since the power back-off shifts the signal more towards the linear region of the amplifier, Figs 5—6 shows improvement in BLER for given Es/N0, with Es being the energy per symbol before the power back-off. However, the actual performance assessment of the link must account for the loss in signal power due to the back-off, which is indicated by the results in Figs. 7—8, where the BLER is plotted against EsN0+OBO.

Fig. 5: BLER vs Es/N0 with TWTA (Table 2) for different average back-off values (in dB).

Fig. 6: BLER vs Es/N0 with SSPA (Table 3) for different average back off values (in dB).

Fig. 7: BLER vs EsN0+OBO with TWTA (Table 2) for different average back-off values (in dB).

Fig. 8: BLER vs EsN0+OBO with SSPA (Table 3) for different average back-off values (in dB).

A quantity that allows better demonstration of the link performance with changing back-off values is the total degradation (TD) incurred for achieving a given BLER, which is computed (in dB) as follows [5]

TDdB= EsN0dB- EsN0L,  dB+ OBOdB,

where EsN0L is the EsN0 required with a linear PA for achieving the same BLER. The TD values for a BLER of 10-2 is plotted in Fig. 9, for different OBOs. With the TWTA from Table 2, the minimum of TD = 3.54 dB is achieved with OBO = 2.76 dB while the more linear SSPA from Table 3 has a lower TD = 3.24 dB at OBO = 2.56 dB.

Fig. 9: Total degradation vs OBO with BLER = 10-2, for TWTA (Table 2) and SSPA (Table 3)

Observation 1: Satellite HPA non-linearity has a significant impact on the BLER performance in NR NTN DL.

Observation 2: The minimum total degradation for the SSPA is lower than that of the TWTA.

In Figs. 10—11, we examine the constellations without any AWGN under the best OBO value that gives the least degradation in Fig. 9. We observe that the ICI induced by the nonlinearity results in error vector magnitude (EVM) values of 23% and 21.8%, respectively, for the TWTA and the SSPA.

Fig. 10: TWTA with OBO = 2.76 dB; EVM = 23%               Fig. 11: SSPA with OBO = 1.58 dB; EVM = 21.8%

Observation 3: PAPR reduction of NR DL might be needed for better BLER performance in NTN scenarios.

Observation 4: The EVM requirements shall be relaxed for NR NTN scenarios.

Proposal 3: Appropriate HPA non-linearity modeling should be included in the link- and system-level simulation assumptions for NR NTN downlink.

# Conclusion

In this paper, the following observations and proposals have been made:

Proposal 1: Link-level simulations considering satellite HPA non-linearity should model the ICI between multiple allocations in the same downlink beam, even under AWGN channel or ideal channel knowledge assumptions.

Proposal 2: Input value range of the HPA AM/AM and AM/PM characteristics specified for NR NTN downlink simulations shall cover the high dynamic range of the OFDM waveforms: recommended range is from -30 dB to 10 dB.

Observation 1: Satellite HPA non-linearity has a significant impact on the BLER performance in NR NTN DL.

Observation 2: The minimum total degradation for the SSPA is lower than that of the TWTA.

Observation 3: PAPR reduction of NR DL might be needed for better BLER performance in NTN scenarios.

Observation 4: The EVM requirements shall be relaxed for NR NTN scenarios.

Proposal 3: Appropriate HPA non-linearity modeling should be included in the link- and system-level simulation assumptions for NR NTN downlink.

# References

| [1] | 3GPP TR 38.811, "Study on New Radio (NR) to support non-terrestrial networks". |
| --- | --- |
| [2] | RP-190710, "Study on solutions for NR to support non-terrestrial networks (NTN)", Thales, RAN #83, Shenzhen, China, March 2019 |
| [3] | R1-1905216, "Key satellite parameters and simulation assumptions for NTN" , ESA, RAN #96, Xi’an, China, April, 2019 |
| [4] | R1-1904712, "Discussion on PAPR in NTN", Huawei, HiSilicon, RAN #96, Xi’an, China, April, 2019 |
| [5] | B. F. Beidas and R. Iyer Seshadri, "OFDM-Like Signaling for Broadband Satellite Applications: Analysis and Advanced Compensation," in IEEE Trans. Commun. vol. 65, no. 10, pp. 4433-4445, Oct. 2017. |