3GPP TSG-RAN WG1 Meeting # 97	R1-19xxxxx

# Reno, USA,  May 13 - 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | XX | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on conditions of rate matching pattern overlapping with PDSCH DMRS symbols |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current RAN1 specifications, there is ambiguity for the applicability of rate-matching (RM) pattern when the RM pattern is overlapping with PDSCH DMRS symbols. Two alternative interpretations are: • Alternative 1: PDSCH DMRS REs indicated by the frequency domain resource assignment field in a DCI are always transmitted and not overlapping with any RM pattern. • Alternative 2: Only for available PDSCH PRB allocated by both DCI and RM patterns, its DMRS REs are transmitted. If transmitted, no overlapping with any RM pattern. Thus, it is unclear whether PDSCH resources indicated by a DCI can cover one or multiple PRBs that have been completely rate-matched out by a RM pattern. This example CR for Alt. 2 along with its companion CR for Alt. 1 intends to resolve the ambiguity one way or the other. In subclause 5.1.2.3 of TS 38.214, a UE may assume that the same precoding is applied to all allocated PDSCH resource if PRG is determined as “wideband”. In this case, the UE also assumes that contiguous PRBs are scheduled for PDSCH where DM-RS REs are always transmitted, i.e. these DM-RS REs are not overlapping with any RM pattern, which is better to be reconfirmed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the condition of determinating a collision between PDSCH DMRS REs and RM patterns. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the slot containing rate matching patterns or SSB, PDSCH scheduling for a UE with full-bandwidth DCI indication is impossible, thus resulting in big performance loss in PDSCH throughput and strict restriction to RM pattern applications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on rate-matching patterns for PDSCH. - If this CR is only adopted by network but not by UE, UE may response with HARQ NACK when it is scheduled with a PDSCH which contains some PRBs completely nulled out by a rate-matching pattern. In other scheduling cases, there is no interoperability issue. - If this CR is only adopted by UE but not by network, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

If  is determined as "wideband", a UE is not expected to handle the case where for a PRB indicated by the frequency domain resource assignment field in a DCI for PDSCH, its PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. Otherwise, the UE is not expected to handle the case where for a PRB declared above as available for PDSCH, its PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

<Unchanged parts are omitted>