3GPP TSG RAN WG1 #97	R1- 1907510

Reno, NV, USA, May 13th – 17th, 2019

Source:	Ericsson

Title:	UE and gNB measurements for NR positioning

Agenda Item:	7.2.10.3

Document for:	Discussion, Decision

# Introduction

The following were measurement related agreement in RAN WG1 #96 meeting.

Agreement:​ Intra-frequency and inter-frequency DL RSTD measurements (currently supported to be based at least on DL PRS) are supported in RRC_CONNECTED mode ​ ​Agreement:​ At least intra-frequency DL PRS-RSRP measurements are supported in RRC_CONNECTED mode​ - FFS: Inter-frequency DL PRS-RSRP measurements in RRC_CONNECTED mode​ Agreement:​ - The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements. ​ - A DL PRS Resource ID ​ - A subset of DL PRS Resource IDs from a single DL PRS Resource set​ - A DL PRS Resource set​ Agreement:​ - The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference.

The work item description for NR positioning support mandates RAN1 centric objectives for NR positioning. One of the objectives is to define UE and gNB based measurements for positioning to support RAT dependent positioning methods in different scenarios. These measurements are done over the reference signals that are defined (or will be defined) in NR. The NR positioning measurements will be defined over resources, resource sets mapped to occasions.

In this contribution, we present our view on positioning measurements that are required to be done either by a UE or a base station depending on the positioning method being employed. The measurements agreed in NR Rel. 15 are not precluded and any additional measurements that are deemed important for the RAT dependent positioning solutions are discussed.

In LTE, one positioning occasion defines a unit event over which the measurements that are required by positioning techniques are performed by a single UE or a single base station. Moreover, the positioning occasions in LTE can either be defined as a singular period or a burst of periods over which the measurements will be done. In NR, the corresponding unit event can be occasion of a resource within a resource set.

It is important to define the basic unit of measurements for positioning. To clearly explain and connect definitions, following points are made:

  - The UE RSTD measurement report should be based on PRS configurations.

  - The PRS configuration may specify occasions for the measurement.

  - The basic unit of measurement is an occasion of a resource. For example, a resource can be a beam.

  - Measurements can be made over single resource using single or multiple occasions.

  - Measurements over multiple resources would require multiple occasions.

# UE measurements

Figure 1 shows a typical the location Information Transfer operations between UE and location server for positioning. The location transfer operation includes measurements obtained by UE as requested, for different types of positioning methods.

Figure 1 LMF-initiated Location Information Transfer Procedure.

- The LMF sends an LPP Request Location Information message to the UE. This request includes measurements requested, including any needed measurement configuration information, and required response time.

- The UE obtains measurements as requested in step 1. The UE then sends an LPP Provide Location Information message to the LMF, before the Response Time provided in step (1) elapsed, and includes the obtained measurements.

# 2.1 UE RSTD measurements

Reference Signal Time Difference (RSTD) is one of the major measurements that UE may report to support multitude of positioning methods in NR. In this section, we elaborate on the definition of RSTD measurement and how UE should report this measurement.

The UE RSTD measurements can be used in methods like DL-TDOA, single base station based positioning, E-CID etc. As in LTE, a single RSTD measurement in NR can also be defined as relative timing difference between the neighbour cell j and the reference cell i, defined as TsubframeRxj – TsubframeRxi, where TsubframeRxj is the time when the UE receives the start of one subframe from cell j, TsubframeRxi is the time when the UE receives the corresponding start of one subframe from E-UTRAN cell i that is closest in time to the subframe received from cell j. The reference point for the observed subframe time difference shall be the antenna connector of the UE [refer 38.215]. Moreover, as in LTE, the RSTD measurements can include one or several paths of the received DL signals to provide the multipath timing profile. The measurements provide the multipath timing profile at individual base stations(intra-RSTD) and the relative timing between base stations (inter-RSTD) as shown in Figure 2. The peaks shown in the figure are the channel impulse response (CIR) peaks obtained from the received signal.

As shown in the Figure 2, RSTD measurements can be defined as intra-RSTD measurements and inter-RSTD measurements.

- Intra-RSTD measurements are time difference measurements of the multipath peaks referenced with the first arrived peak from the same base station at the considered UE. Number of intra-RSTD will be P-1, where P is the number of paths considered.

- The inter-RSTD measurements are time difference measurements between first arrived peaks of base stations referenced with the first arrived peak of the serving base station of the considered UE. If the number of base stations is N, number of inter-RSTD measurements will be N-1.

Figure 2 Intra-RSTD and Inter RSTD measurements

Large number of paths considered for intra-RSTD measurements, P, would be the rich reporting which can be used for positioning methods based on analyzing multipath components for UE positioning. The values of N and P can be decided based on the UE capabilities, and the corresponding max numbers are typical configured.

The resolution of the RSTD measurements by UE has to be specified. Precision of RSTD measurements w.r.t. the true values of these measurements should be set as requirement by the network. The network can configure number of strongest beams (Q) for which above measurements should be reported. Along with the beam/resource IDs.

Figure 3 Finer RSTD around the desired peak

It should be possible to focus the RSTD measurements around certain peaks, as shown in the Fig.3. As is well known that for positioning the first peak is of most importance. Hence finer RSTD measurements around the first peak would contribute more to the positioning accuracy than RSTD measurements of later peaks. The network should be able to configure finer RSTD measurements in the CIR at a UE.

The measurements and measurement reporting rates should be configured by the location server. This can be done on a scenario basis. For example, the location server may require higher rates of measurement reporting to be able to provide measurements to a moving vehicle.

There should be requirements set on the precision of measurements collected by the UE to be able to achieve desired positioning accuracy. The measurement precision requirement should be based on the time of arrival accuracy required for achieving positioning accuracy requirement. It should be noted that the RSTD measurement precision at the UE may be limited by the sampling clock’s discreteness in time measurement. Based on the time of arrival accuracy requested by the network, a UE may take measures like interpolation etc. to further refine its time measurement accuracy. The time of arrival accuracy required for achieving certain positioning accuracy should be evaluated using tighter constraints on the system parameters like number of base stations for positioning, geometry, use case driven etc. This is necessary to ensure positioning precision even with the most basic setup where estimating position is possible. The evaluation scenario agreed in 3GPP may sometime be very optimistic, like having large number of base stations for computing UE positioning. For example, 12 base stations in IOO scenario. The measurement precision should be sufficient to be able to meet positioning requirement in any scenario with even three base stations. High precision measurement alleviates the need for high complexity measurements such as TOA measurements with many hard to reach base stations.

The resolution of RSTD measurement reporting has to be specified.

UE should report inter-RSTD and intra-RSTD measurements for every pair of base stations and individual base stations configured for positioning.

The network should configure values N, P and Q for the measurements to be performed and reported by the UE.

Network should be able to configure finer RSTD measurements in CIR around any peak.

The measurement rates and measurement reporting rates should be configured by the network.

Requirements on the measurement precision (RSTD precision) should be set by the network based on the required time of arrival accuracy.

The RSTD measurement precision should be sufficient to be able to meet positioning requirement in any scenario with even three base stations.

# 2.2	UE RSRP measurements

The Reference Signal Received Power (RSRP) measurements are used for positioning methods like E-CID based positioning, received signal strength (RSS) based positioning etc. RSRP measurements over different reference signals provide measures of signal strength and quality for communication purposes. For positioning, the RSRP measurements can also provide measure of timing measurements. Power levels of every path being reported would help the network in improving the position estimation of the UE.

RSRP measurements should be defined over the specified NR-PRS signal and will be called NRPRS-RSRP. It could be defined as the linear average over the power contributions (in [W]) of the resource elements that carry PRS reference signals configured for positioning measurements within the considered measurement frequency bandwidth in the configured PRS occasions.

The RSRP measurements should be specified for every RSTD measurement corresponding to a peak in the CIR. The values of the peaks could be defined as the power contributions (in [W]) of the PRS resource elements for a certain delay value. As shown in the Fig.4, the power peaks can be reported as [h1 h2 … hp hp-1]. In LTE, additional path is reported by the UE which includes the RSTD values of additional path and the path quality measure [2].

In NR, in addition, dynamic range of every path should be reported. The dynamic range of every path can be defined as the difference of the peak power to the noise floor of the correlation output. The noise floor of the correlation output consists of thermal noise, diffuse scattering components, contribution due to nonlinearity, aliasing etc. So, it is important to know how clear is a peak with respect to the noise floor around it.

Dynamic range of peaks in the power delay profile should be reported along with the RSTD measurements on these peaks.

Figure 4 A typical CIR and the power measurements at every peak.

# 2.3	UE RX-TX time difference measurements

In order to support positioning method that rely on round trip time (RTT), where RTT defines the time duration when a DL PRS from BS is received by a UE and BS receive the UL PRS transmitted by UE that is triggered by the DL PRS reception at the UE side, the UE needs to do Rx-Tx measurements and report it back to the BS/LS. In addition, the UE should also report Rx-Tx measurement quality.

The UE Rx – Tx time difference can be defined as TUE-RX – TUE-TX, where TUE-RX is the UE received timing of downlink radio frame i from the serving cell, defined by the first detected path in time and TUE-TX is the UE transmit timing of uplink radio frame i. The reference point for the UE Rx – Tx time difference measurement shall be the UE antenna connector.

In RTT methods, systematic delay errors at the UE affects the positioning estimation more than in TDOA based methods. Similarly, knowledge of the delay distribution at the UE side can be used in the position estimation algorithms to improve the positioning accuracy. Hence, a measure of UE Rx-Tx measurement should also be reported along with the measurements. The measure can be the distribution of the UE Rx-Tx measurement. If assumed Gaussian, then it can be the mean and variance of the UE Rx-Tx measurement distribution.

A quality measure of UE Rx-Tx measurement should be reported by the UE along with the UE Rx-Tx measurements.

# 3	gNB measurements

To facilitate UL based positioning solutions, gNB needs to do the required time and angle measurements by using the UL reference signals. In [3] we have presented candidate reference signals that are suitable for UL NR positioning. In this section, we will elaborate on our view on measurements that gNB can perform within a positioning occasion for UE positioning.

# 3.1 gNB Rx-Tx time difference

The gNB Rx-Tx time difference measurement facilitates positioning solutions based on UL RTT exploiting either a single or multiple base stations. The gNB Rx-Tx time difference defines the time difference between when the gNB detects the first path in time of the UL radio frame with UL positioning reference signal and the timing when gNB transmit DL radio frame with DL positioning reference signal.

# 3.2 Timing advance (TADV)

Timing advance is one of the time measurements in UL direction that defines gNB Rx-Tx time difference with or without UE Rx-Tx time difference. Depending on the configuration, TADV can be defined/calculated as

TADV = gNB Rx-Tx time difference + UE Rx-Tx time difference

or

TADV = gNB Rx-Tx time difference.

# 3.3 Angle of Arrival (AoA)

In a beamformed UL positioning reference signal transmission, the gNB in addition to first path detection of the radio frame containing the UL positioning reference signal can also estimate the angle of arrival (AoA) of such signals. The AoA measurement is done by the gNB with respect to a reference direction. In positioning solutions like uTDOA, additional information based on AoA can enhance the TOA measurement accuracy. Moreover, a single base station based positioning solution can also exploit this info for UE localization.

# 3.4 UL Relative Time of Arrival (TUL-RTOA)

The UL relative time of arrival is a time measurement that can be done at gNB for the time difference between beginning of subframe i containing the UL positioning reference signal and the beginning of subframe j with UL positioning reference signal. This sort of measurement is required when network based positioning method is being employed.

Define gNB Rx-Tx time difference, timing advance, AoA and UL RTOA as gNB measurements for NR positioning.

# Conclusions

In the previous sections we made the following proposals:

- The resolution of RSTD measurement reporting has to be specified.

- UE should report inter-RSTD and intra-RSTD measurements for every pair of base stations and individual base stations configured for positioning.

The network should configure values N, P and Q for the measurements to be performed and reported by the UE.

Network should be able to configure finer RSTD measurements in CIR around any peak.

The measurement rates and measurement reporting rates should be configured by the network.

Requirements on the measurement precision (RSTD precision) should be set by the network based on the required time of arrival accuracy.

The RSTD measurement precision should be sufficient to be able to meet positioning requirement in any scenario with even three base stations.

Dynamic range of peaks in the power delay profile should be reported along with the RSTD measurements on these peaks.

A quality measure of UE Rx-Tx measurement should be reported by the UE along with the UE Rx-Tx measurements.

Define gNB Rx-Tx time difference, timing advance, AoA and UL RTOA as gNB measurements for NR positioning.

# References

- RP-182155, Study on NR positioning support, RAN1, Intel Corporation, Ericsson, June 2019.

- 3GPP TS 36.355 V14.2.0, LTE Positioning Protocol (LPP) (Release 14), June 2017

- R1-19NNN, DL and UL Reference Signals for NR Positioning, RAN1. Ericsson, April 2019.