3GPP TSG RAN WG1 #97																				R1- 1907511

Reno, NV, USA, May 13th – 17th, 2019

Source:	Ericsson

Title:	Physical-layer procedures in NR positioning

Agenda item:	7.2.10.2

Document for:	Discussion and decision

Introduction

During RAN1#96b, the discussion on positioning procedures started, with the following set of agreements:

Agreement: The DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE Agreement: For intra-frequency measurements: - The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. - Select from one of the following options for the measurement bandwidth - Option 1: The UE measurement is within the DL BWP configuration - Option 2: The UE can measure outside the DL BWP configuration - FFS: Scenarios when measurements gaps would need to be configured. - When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. Agreement: For positioning purposes, to assist UE to perform Rx beamforming, consider one or more of the following options: - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts. Agreement: For positioning purposes, with regard to the UL SRS transmission timing, at least the following options have been identified: - Option 1: No adjustment is made to the configured TA. That is, support only a TA configuration which is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols). - Option 2: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value can be provided by the UE to the network upon request. - Note: The adjustment value is not needed to be known at the measuring cell for the purpose of UL-AoA measurement - Option 3: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value is provided by the network to the UE. Agreement: For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified: - Option 1: The UL SRS Tx power is constant (i.e., no power control is supported). - FFS: The value of the constant Tx power and how it is determined - Option 2: The UL SRS Tx power is based on the existing power control procedure. - Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example: - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop). Agreement: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, consider one or more of at least the following options: - Option 1: Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS. - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS). - Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources. - Option 3: The Tx beam is fixed for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2. - FFS: How the fixed beam is selected

In this contribution, we discuss further the procedure discussion.

Procedures for positioning

## DL PRS related procedures

### DL PRS timing aspects

In any cell, it is of interest for the UE to know the relation between the DL PRS and the cell other reference signals. For example, QCL information can give input to the UE in terms of coherence time and delay distribution (average delay, delay spread)  which are useful to size the search window and processing parameters for receiving the PRS.  The UE monitors CSI-RS and SSB for measurement and beam management purpose so there is always a RS to relate the DL PRS to.

For positioning purposes, to assist UE to acquire the PRS timing, the DL PRS QCL type C relation to DL Reference Signal from a serving or neighboring cell can be reported to the UE in the PRS configuration

  - The reference signals QCL status to be reported can be at least SSB, CSI-RS

  - FFS other reference signals

### Bandwidth part operation

As confirmed by agreements in RAN1#96b, the DL PRS configured by a gNB is intended to be received by multiple UEs and consequently it is designed independently of the UE capabilities.  The UE will receive a request for measurement from the location server based on PRS and will respond based on its capabilities. During RAN1#96b, it was agreed to restrict measurements outside of the UE active  BWP and SCS to be confined to configured measurement gaps.  It was left to resolve whether the measurement map include BWP outside of the UE configured BWPs.

If the UE is requested to measure PRS during a measurement gap, there should be enough time to adjust to a new BWP outside of the configured BWPs, therefore, we would expect the UE to be able to measure outside the DL BWP configuration.

When configured with a measurement gap, the UE can measure the DL PRS resource outside the active BWP, including BWPs inside and outside the UE BWP  configuration

### Beam management aspects

In RAN1#96b, the following options were stated

For positioning purposes, to assist UE to perform Rx beamforming, consider one or more of the following options: - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts.

Clearly all 3 options should be supported depending on the the use case. For FR1, option 3 is the most likely case, while option 1 and 2 depend on whether the UE has past knowledge to assist PRS reception.

Support UE RX beam sweeping on downlink PRS

Similar to the case of timing acquisition, QCL information can assist the reception of PRS with other DL signals with QCL type D relation signaling a common RX beam between the PRS and other DL signal such as SSB or CSI-RS.

For positioning purpose to assist the UE to acquire the PRS Rx beam, the DL PRS QCL type D relation to other reference signals from a serving and neighbouring cell can be reported to the UE in the PRS configuration.

  - The reference signals QCL status to be reported can be at least SSB, CSI-RS

  - FFS other reference signals

UL SRS related procedures

Transmission timing and timing adjustments

UL timing is controlled via a TA adjustment to ensure all uplink transmission in a cell arrive aligned at the gNB. There has been discussion on whether to have separate TA values for the neighbor cell, independent of the serving cell, for the purpose of positioning. The issue with such methods is that if a separate TA value is of interest only if the TA required exceeds the CP length. However, at these values the transmitted SRS is no longer orthogonal in the serving cell, where the path loss to the gNB is the lowest and therefore the impact of loss of orthogonality will be perceived the most. Conversely, a serving cell-based TA that results in a  misaligned SRS in the neighboring cell will experience a comparatively large path loss, so that the added interference is low compared to the one a serving cell would experience with aneighbour cell-based TA.

TA for SRS is following the serving cell timing.

Power control

The previous meeting yielded the following options for power control of the UL SRS in positioning:

For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified:

For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified: - Option 1: The UL SRS Tx power is constant (i.e., no power control is supported). - FFS: The value of the constant Tx power and how it is determined - Option 2: The UL SRS Tx power is based on the existing power control procedure. - Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example: - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop).

SRS currently support an open loop power control in its RRC configuration parameters pathlossReferenceRS,p0 and alpha, and a closed loop dynamic component controlled by TPC commands in DCI. As previously discussed regarding TA, there is a significant risk to tie the power of the SRS to the non-serving cell, as this will significantly raise the ICI level in the serving cell.  Therefore it is suggested to maintain the current power control procedure, and address the hearability of the SRS in the neighbor cells with alternative methods, e.g. repetitions if needed.

For power control of SRS in positioning, select Option 2: The UL SRS Tx power is based on the existing power control procedure.

Beam management aspects

SRS in FR2 is expected to be beamformed toward the target enodeB (serving or neighbor). To assist establishing the best UE TX beam and nodeB RX beam,the  RRC configuration IE of SRS uses the  spatialRelationInfo, to signal a target serving nodeB RX beam to align SRS to. It is proposed to also reuse this method for the purpose of positioning and extend it to the neighbor cell.

The assistance data for UL SRS based positioning may include spatial information from neighbouring cells DL RS for the purpose of aligning the SRS to the neighbor cell RX beams.

- Details on how to send the information is FFS

Conclusions

In the previous sections we made the following   proposals:

Proposal 1	For positioning purposes, to assist UE to acquire the PRS timing, the DL PRS QCL type C relation to DL Reference Signal from a serving or neighboring cell can be reported to the UE in the PRS configuration

o	The reference signals QCL status to be reported can be at least SSB, CSI-RS

o	FFS other reference signals

Proposal 2	When configured with a measurement gap, the UE can measure the DL PRS resource outside the active BWP, including BWPs inside and outside the UE BWP  configuration

Proposal 3	Support UE RX beam sweeping on downlink PRS

Proposal 4	For positioning purpose to assist the UE to acquire the PRS Rx beam, the DL PRS QCL type D relation to other reference signals from a serving and neighbouring cell can be reported to the UE in the PRS configuration.

o	The reference signals QCL status to be reported can be at least SSB, CSI-RS

o	FFS other reference signals

Proposal 5	TA for SRS is following the serving cell timing.

Proposal 6	For power control of SRS in positioning, select Option 2: The UL SRS Tx power is based on the existing power control procedure.

Proposal 7	The assistance data for UL SRS based positioning may include spatial information from neighbouring cells DL RS for the purpose of aligning the SRS to the neighbor cell RX beams.

	Details on how to send the information is FFS

References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019