3GPP TSG RAN WG1 Meeting #97	R1-1907522

Reno, USA, 13th – 17th May, 2019

Agenda Item:	7.2.9.3

Source:	Huawei, HiSilicon

Title:	CC grouping operation for UE power saving

Document for:	Discussion and Decision

# Introduction

The work item of UE power saving was approved in RAN#83 meeting. The scope of the work item until June, 2019 is as following [1], which would be further revised/expanded in RAN#84 meeting.

- Specify power saving techniques with UE adaptation with focus in RRC_CONNECTED mode [RAN1, RAN4]

  - Specify the power saving techniques with power saving signal/channel

    - Specify the PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED

    - Note: this objective shall not duplicate DRX operation and impact to DRX is studied at RAN2

    - Note: Any change of PDCCH channel coding and payload interleaver  is not in the scope

  - Specify the procedure of cross-slot scheduling power saving techniques

    - Note: The procedure is in addition to Rel-15 cross-slot scheduling procedure

- Evaluate the required switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers [RAN4]

  - Note: Switching on/off the RF is part of the evaluation

Note:

- These objectives are RAN1/RAN4 focus and do not consider RAN2 impact.

- The objectives are subject to further update in RAN#84.  The update will be based on recommendations from the completion of RAN2 study and remaining RAN1 recommendations based on the conclusion of RAN1 study.

This contribution will discuss the design of power saving adaption based on the operation in a group of cell in power efficient way.

# Discussion

## Analysis on power adaptation based on CC-grouping

When UE has multiple component carriers activated in CA, even for the same number of CCs, the power consumption for operating on different sets of component carriers would correspond to different power consumption levels. For example in Figure 1, if CC1 and CC2 are activated the power consumption shall be lower than that for the case when CC1 and CC3 are activated. This is due to the RF bandwidth the UE can support or the number of RF chains used for CA, which depends on UE implementation. Therefore, component carriers can be considered to be grouped into CC-groups where the operation on component carriers in the same CC-group can share modules and correspond to a lower power consumption level.  If the suggestion on the CC-group, which depends on UE implementation, can be reported to gNB, then gNB can try to wake up and schedule the intra-group carriers for saving UE power consumption. From a UE perspective, the UE may switch off the RF chains and other parts of the modem that are to process the carriers belongs to other CC-groups. UE can report the CC-group information to gNB when CA is configured by gNB.

As shown in Figure 2, {CC1, CC2} can be reported as a CC group X and {CC3, CC4} as another CC group Y for UE power saving, respectively. Scheduling inside either group X or group Y can correspond to a low UE power state, and cross CC-group scheduling between these two groups corresponds to a higher UE power state. For example, UE can just use a narrower bandwidth of RF bandwidth1 and keep essential modules to process carriers in CC group X, which contains the primary carrier CC1.

Figure 1 Example of CC grouping for power saving based on UE implementation

From network perspective, gNB can pre-configure the CC groups considering both UE implementation based on UE assistance information and the arriving traffic. PDCCH monitoring on each component carrier can be indicated in the WUS DCI. As discussed in the companion paper [2], the WUS DCI, especially the UE-specific WUS DCI, is expected to have small DCI format size for better detection performance and less resource overhead. Considering that the component carriers in the same CC-group recommended by UE have similar power consumption level on the UE, the PDCCH monitoring indication per recommended CC-group would be a reasonable method to reduce/control the DCI format size.

As an example in Figure 3, 4 CC groups are configured for a UE: the CC group 1 only contains the primary carrier CC1, while CC group 2 and 3 contain component carriers from different width of RF bandwidth respectively. For saving UE power, most of the activated secondary carriers can remain in a low power state if there is no scheduling on them and UE can skip PDCCH monitoring on these component carriers. When there is downlink traffic arrived, the scheduler can indicate UE to monitor appropriate set of CC-groups based on the data amount: if just small amount of data needs to be scheduled, the gNB can indicate in the WUS DCI that the UE to just monitor PDCCH of component carriers in the CC group 1. Meanwhile, if a large amount of data arrives, the gNB can indicate the UE to monitor the PDCCHs of the component carriers from the CC-group 1 or CC-group2.

Figure 2 Indicating PDCCH monitoring per recommended CC-group in wake-up signals

## Benefits of power adaptation based on CC grouping

In the power model discussion in RAN1#95 meeting, the power consumption of 2CC is 1.7 times of the power consumption of 1CC and for the worst case in CA the power consumption of 4CC is 3.4 times of the power consumption of 1CC in downlink. For the worst case, the UE implementation regarding RF part can be shown in Figure 2, in which the RF is separated for between CC1/CC2 and CC3/CC4 and other modules cannot be shared, and meanwhile there is no scheduling restriction for the 4CCs. Therefore the power consumption of 4CC is consumed twice as much as the power consumption of 2CC.

Figure 3 Illustration of the worst case in CA

The total power consumption includes the power of RF part and baseband part. Table 1 shows the comparison between the inter-group scheduling and intra-group scheduling. When the same number of component carriers are scheduled, the power consumption of baseband processing is same, however, the power consumption of RF part for CC1 and CC3 would be larger than that for CC1 and CC2. If the gNB adopts the UE reported CC grouping information and schedules intra-group component carriers, the power consumption of 2CC is 1.7 times of the power consumption of 1CC; otherwise, the power consumption of 2CC would be 2~3.4 times of the power consumption of 1CC.

Table 1 Comparison between inter-group and intra-group scheduling

| Case | CC1 and CC3 scheduled (inter-group scheduling) | CC1 and CC2 scheduled (intra-group scheduling) |
| --- | --- | --- |
| Baseband processing | Same |  |
| RF bandwidth | large | small |
| Power consumption estimation | In the range of 2~3.4 times of power consumption to handle 1CC | 1.7 times of power consumption to handle 1CC |

Proposal 1: Support UE reporting recommended CC-grouping information

Proposal 2: Further consider per recommended CC-group indication if the indication of PDCCH monitoring on component carriers is introduced in WUS DCI.

# Conclusions

This contribution analyzes the benefit of CC-group reporting and the potential usage of CC-grouping in WUS DCI. Based on the discussions and analysis, and the following proposals are proposed.

Proposal 1: Support UE reporting recommended CC-grouping information

Proposal 2: Further consider per recommended CC-group indication if the indication of PDCCH monitoring on component carriers is introduced in WUS DCI.

# References

- RP-190727, New WID: UE Power Saving in NR, CATT.

- R1-1906005, PDCCH based power saving signal/channel, Huawei, HiSilicon.