3GPP TSG RAN WG1 Meeting #97		R1-1907583

Reno, USA, May 13th – 17th, 2019

Source:	Sierra Wireless

Title:	LTE-M Preconfigured UL Resources Summary RAN1 #97

Agenda Item:	6.2.1.2

Document for:	Discussion and Decision

# Introduction

The following tdoc provides a summary of the LTE-M PUR open issues, company positions and proposals for RAN1 #97.

Definitions

The following definitions are used in this document:

PUR Configuration:	A configuration that contains all required information for D-PUR transmissions. The configuration may include: the number of D-PUR allocations (see below), time/frequency resource(s), repetition, MCS, TBS, period, time offset, hopping, RNTI, power control, DMRS, scrambling code, timing advance validation mechanism configuration, and others.

PUR Parameter: 	A parameter which may not be included in the PUR configuration: Timing Advance

PUR Allocation: 	A single time/frequency D-PUR resource allocation.

PUR SS Window: 	The time period associated with a PUR allocation where the UE monitors MPDCCH.

# PUR Search Space

## ACK and HARQ

Issue: Whether the ACK and HARQ will use the same RNTI and search space is open.

For dedicated PUR in idle mode and for a given CE mode, the same sized DCI, the same PUR search space, and same RNTI is used for all DCI messages.

Supporting Companies: Nokia [9], Sierra Wireless, LGE

## CE Mode A and B

Issue: How CE Mode A and Mode B procedures apply to PUR the PUR SS is an open issue and how a mode changed is an open issue.

For dedicated PUR in idle mode, the UE uses the CE mode configured in last RRC_CONNECTED state. The legacy procedure for CE mode change performed in connected mode is reused.

Supporting Companies: Ericsson [1], Sierra Wireless [5], LGE [8]

## Start of PUR SS Window

Issue: if the PUR SS Window should start before or after the PUR transmission is an open issue.

For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission

FFS: value of x, and if x is specified or signaled

FFS: FD-FDD TDD UEs

FFS: support for PUR SS Window before PUR transmission

- Note: The PUR SS window is the time period associated with a PUR transmission which UE monitors MPDCCH.

Supporting Companies:  Lenovo [4], Sierra Wireless [5], ZTE [6], Nokia [9], Sony [11]

## PUR SS MPDCCH candidates

Issue: How are the PUR SS MPDCCH candidates (repetition and AL) determined is an open issue.

For dedicated PUR in idle mode,

The maximum mPDDCH repetitions, rmax-mPDCCH-PUR, is included in the PUR configuration

The PUR search space repetitions and AL candidates are derived using legacy USS rules where rmax= rmax-mPDCCH-PUR

Supporting Companies:  Sierra Wireless [5], Nokia [9], LGE

## Duration of PUR SS Window

Issue: How the duration of the PUR SS window is defined is open.

For dedicated PUR in idle mode, the duration of the PUR SS window is configured by eNB

How the duration is configured, and the possible values, will be decided by RAN2.

Supporting Companies: Sierra Wireless [5], ZTE [6], LGE

## UE Procedure after UE gets a PUR L1 ACK

Issue: What the UE should do after it gets a PUR ACK after a PUR transmission is an open issue.

Discussion: There are several possible choices:

    - Option 1: UE goes directly to idle

    - Option 2: UE continues to monitor the PUR SS Window until the end then goes to idle

    - Option 3: Depends on L1 ACK info - Huawei [2], Sierra Wireless [5], Sequans [15], ZTE

Feature lead conclusion:  Wait for RAN2 to decide on WA #1

## UE Procedure after missing ACK or UL grant

Issue: What a UE does if it gets nothing (neither UL Grant nor ACK) in the PUR SS Window after a PUR transmission is an open issue.

Discussion:  There are two options:

    - Option 1: UE must do EDT or legacy RA - Ericsson [1], Sierra Wireless [5]

    - Option 2:  UE optionally does EDT or legacy RA - ZTE [6]

    - Option 3: Leave for RAN2 – Nokia [8]

This issue is linked to how to keep eNB and UE in sync if L1 ACKs are missed which is discussed in section 7.1

Feature lead conclusion:  Wait for RAN2 to decide on WA #1

## Monitor PUR SS Window when UE doesn’t transmit

Issue: Whether the UE must still monitor the PUR SS window when it doesn’t transmit on its PUR allocation (i.e. PUR is skipped) is an open issue.

Option 1: Not required to monitor PUR SS - Ericsson [1], Lenovo [4], Sierra Wireless [5], ZTE [6]

Option 2: Configurable to monitor PUR SS – LGE [8], Nokia [8]

For dedicated PUR in idle mode, if a UE skips a PUR transmission, it is not mandated to monitor the associated PUR SS window.

Supporting Companies:  Ericsson [1], Lenovo [4], Sierra Wireless [5], ZTE [6]

## PUR SS collisions with Paging CSS

Issue: What a UE does if the PUR SS collides with paging CSS is open.

Option 1: Not required to monitor Paging CSS - Sierra Wireless [5], ZTE [6]

Option 2: Not required to monitor PUR SS – LGE [8]

Feature lead conclusion:  More study is needed

# Legacy DMRS pattern PUR configuration

It was agreed that the PUR configuration shall include

- Legacy DMRS pattern

Issue: The details of how the legacy DMRS pattern is configured is open.

Option 1: One DRMS - ZTE [6]

Option 2: > one DMRS - Lenovo [4], Sierra Wireless [5], Xiaomi [7], Nokia [9], Intel [10], Samsung [12], Qualcomm [13]

For full PRB allocations, the legacy DMRS pattern in the PUR configuration is one of the [8] legacy cyclicShift values defined in TS 36.211 in table 5.5.2.1.1-2.

FFS: Sub-PRB allocations

Supporting Companies: Lenovo [4], Sierra Wireless [5], Xiaomi [7], Nokia [9], Intel [10], Samsung [12], Qualcomm [13]

# Signaling RSRP Threshold(s) Options

How the service cell RSRP changes are to be validated will be discussed in RAN4 but there are two main options discussed:

Single Threshold Method:  abs(∆RSRP_Measured)  > ∆RSRP_Threshold

Two  Threshold Method: 	∆RSRP_ThNeg  < ∆RSRP  < ∆RSRP_ThPos

Issue: How the threshold(s) will be signaled or calculated are still open RAN1 issues.

The following options have been discussed in the tdocs:

Option 1: an equation - Sierra Wireless [5]

Option 2: a lookup table - Sierra Wireless [5], Sony [11]

Option 3: Signal thresholds - ZTE

Feature lead conclusion:  Wait for RAN4 to decide on validation equation

Void.

# TA validation Timing

Issue:  The maximum time between when the UE validates TA and the PUR allocation could be specified of left for UE implementation.

For dedicated PUR in idle mode, TA validation should be completed within X subframes before it’s PUR allocation.

FFS: value of X

FFS: whether TA validation should be completed at least Y subframes before the PUR allocation.

Supporting companies: Huawei [2], ZTE [6], Xiaomi [7], Sierra Wireless

# Asynchronous Data Transmissions

Issue:  For a UE with a PUR configuration, how to support asynchronously data transmissions open issue.

For dedicated PUR in idle mode, a UE with a PUR configuration may send data via legacy EDT or RACH process at anytime

FFS: what UE behaviour is if PUR allocation collides with USS

Supporting companies: Nokia [9]

# L1 ACK

The following working assumption was made in RAN1 #96bis:

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

FFS: Which PUR configurations and PUR parameters will be signaled via L1

FFS: Definition of PUR configurations and PUR parameters

- The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

## UE/eNB Synchronization Issue

Issue: If the L1 ACK contains information and the L1 ACK is not decoded by the UE, the UE and eNB will be out of sync. For example, if the number of repeats is changed in the L1 ACK but the UE failed to decode the L1 ACK, on the next PUR transmission, the UE will use the previous number of repeats where the eNB expected the new number of repeats.

Discussion: There are several options available to solve this synchronization issue:

Option 1: Do not include resource allocation information in the L1 ACK.

Option 2: If L1 ACK is not decoded after PUR transmission, UE must do EDT or legacy RA to get possible PUR re-configurations - Ericsson [1], Sierra Wireless [5]

Option 3: UE includes L1 ACK information in the next PUR transmission and only starts to use updated information if the L1 ACK is confirmed by the eNB on the subsequent PUR allocation.

Option 4: UE sends a PUCCH ACK after the L1 ACK  - ZTE

Option 5: eNB blindly detects in the next PUR transmission whether the L1 PUR configure update was successful  - LGE

Feature lead conclusion:  Wait for RAN2 to decide on WA #1

## L1 ACK Information

The following working assumption was made in RAN1 #96bis:

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

FFS: Which PUR configurations and PUR parameters will be signaled via L1

FFS: Definition of PUR configurations and PUR parameters

- The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

Issue: What PUR configuration or parameters should be included in the L1 PUR ACK is open.

Discussion: Based on previous online discussions, the following parameters have been considered for inclusion into the PUR L1 ACK:

Timing advance adjustment – Huawei [2], Sierra Wireless [5], ZTE [6], Nokia [9], Sony [11], Samsung [12]

UE TX power adjustment – Huawei [2] Sierra Wireless [5], ZTE [6], Nokia [9],

PUSCH Repetition adjustment– Huawei [2], Sierra Wireless [5], ZTE [6], Nokia [9], Sony [11], Asustek [15]

Downlink data or signaling to follow – Huawei [2], Sierra Wireless [5], Nokia [9], Sequans [14]

TA update in L2 – Sequans [14]

- Feature lead conclusion:  Wait for RAN2 to decide on WA #1

# Other topics

The following other topics and open issues were discussed in tdocs submitted to RAN1 #97 and should be further studied at the next RAN1 meeting:

## Core Topics:

PUR support with other features (e.g. larger TBS, 64 QAM, Early Ack indication, Extended CP)

LGE [8], Qualcomm [13], Nokia[9]

PUCCH Configuration – LGE [8], Qualcomm [13]

- UE procedure when skipping is disabled and UE has no data to transmit – ZTE [6]

- DL Scrambling for the PUR Search space – Qualcomm [13]

## Possible Enhancements:

- New mechanism to update TA

Not needed (EDT without data can be used to get TA): Ericsson [1], Sierra Wireless [5]

Simplified RACH: Huawei [2], ZTE [6], Xiaomi [7]

Early Termination of legacy RA: LGE [8]

- MTBG Support

No support:  Ericsson [1], Lenovo [4], Xiaomi [7], LGE [8]

Wait: Sierra Wireless [5]

- NACK Support

No: Ericsson [1], Sierra Wireless [5], Intel [10]

Yes:  Huawei [2], NTT Docomo [3], Lenovo [4], ZTE [6], LGE [8]

- Scheduling of retransmissions

Dynamic: Ericsson [1]

- CFS PUR Support

Yes: Lenovo [4], Sierra Wireless [5], Xiaomi [7], Nokia [9], Intel [10], Samsung [12]

No CE Mode A: Ericsson [1]

No: ZTE [6]

- CBS PUR Support

No: Ericsson [1], Lenovo [4], Sierra Wireless [5], Xiaomi [7], Samsung [12], LGE

Consider Support: Qualcomm [13], Fraunhofer

- Connected mode Support

No: Sierra Wireless [5], Samsung [12], LGE

Yes: Intel [10]

- CE mode B support:

No – Samsung [12]

- Support UE-group RNTI

Yes - Huawei [2]

No – ZTE [6]

- Power Control - Ericsson [1], Huawei [2], LGE

Frequency hopping support for MPDCCH and PDSCH – Qualcomm [13]

- Support PDCCH Order in PUR SS – ZTE [6], LGE [8]

## RAN2 topics:

PUR release mechanisms - Huawei [2], NTT DOCOMO [3], Lenovo [4], Samsung [12]

PUR reconfiguration request – Huawei [2], Sony [11]

PUR skipping configuration -Huawei [2], NTT DOCOMO [3]

PUR SS duration details – Nokia [9]

# Appendix I: LTE-M PUR Tdocs

R1-1905957	Support for transmission in preconfigured UL resources in LTE-MTC	Ericsson

R1-1905980	UL transmission in preconfigured resource	Huawei, HiSilicon

R1-1906187	Discussion on preconfigured UL resources in MTC	NTT DOCOMO, INC.

R1-1906278	UL transmission in preconfigured resources for eMTC	   Lenovo, Motorola Mobility

R1-1906460	LTE-M Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

R1-1906496	Support for transmission in preconfigured UL resources for MTC	ZTE

R1-1906575	Discussion on transmission in preconfigured UL resources	Beijing Xiaomi Software Tech

R1-1906682	Discussion on preconfigured UL resources in MTC	LG Electronics

R1-1906702	Transmission in preconfigured UL resources	Nokia, Nokia Shanghai Bell

R1-1906769	UL transmission in preconfigured resources for eMTC	 Intel Corporation

R1-1906845	Remaining issues in PUR	Sony

R1-1906893	Discussion on transmission in preconfigured UL resources for MTC	Samsung

R1-1906992	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-1907080	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

R1-1907356	Discussion on LTE-M PUR resource configuration in respect to radio condition	ASUSTEK COMPUTER (SHANGHAI)

# Appendix II: Agreements

## RAN1 #96bis

- Working Assumption#1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

- FFS: Which PUR configurations and PUR parameters will be signaled via L1

- FFS: Definition of PUR configurations and PUR parameters

- The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

- Working Assumption#2

- For dedicated PUR

- During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

  - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

- FFS if the UE monitors any additional RNTI which may be shared with other UEs.

- Note: The same RNTI may be used over non-overlapping time and/or frequency resources

- Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed.

- The UE monitors the MPDCCH for at least a time period after a PUR transmission

- FFS: Details of the time period

- FFS: UE behaviour if nothing is received in the time period

- FFS: If and how often UE monitors MPDCCH after a PUR allocation in which it has not transmitted

The value(s) of RSRP threshold(s) is UE specific

- The power control parameters within the PUR configuration, shall at least include:

- Target UL power level (P_0) for the PUR transmission

For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

## RAN1 #96

In idle mode, the TA validation configuration can include “PUR Time Alignment Timer”

- Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer

- Details on how to specify the “PUR Time Alignment Timer” is up to RAN2

In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes

- Above applies for the case where the UE is configured to use the serving cell change attribute

For dedicated PUR in idle mode, the Dedicated PUR ACK is at least sent on MPDCCH

- RAN2 can decide if a higher layer PUR ACK is also supported

For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration.

- PUR search space is the search space where UE monitors for MPDCCH

- FFS: Whether PUR search space is common or UE specific

When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures

- FFS whether only TA is acquired and then data sent on PUR is supported

- FFS other approaches to obtain a valid TA

When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied.

For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following

- Time domain resources including periodicity(s)

  - Note: also includes number of repetitions, number of RUs, starting position

- Frequency domain resources

- TBS(s)/MCS(s)

- Power control parameters

- Legacy DMRS pattern

In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission:

- Timing advance adjustment

- UE TX power adjustment

- FFS: Repetition adjustment for PUSCH

FFS: Whether the above update is done in L1 and/or higher layer

In idle mode, the PUR search space configuration includes at least the following:

- MPDCCH narrowband location

- MPDCCH repetitions and aggregation levels

- MPDCCH starting subframe periodicity (variable G)

- Starting subframe position (alpha_offset)

For dedicated PUR in idle mode, the PUR resource configuration includes at least the following

- A PUSCH frequency hopping indication to enable or disable legacy frequency hopping

In idle mode, a UE can be configured such that TA is always valid within a given cell.

- FFS: up to RAN2 how to implement e.g. PUR Time Alignment Timer = infinity

## RAN1 #95

For dedicated PUR in idle mode, the UE may skip UL transmissions.

- FFS: Resource release mechanism

- FFS: Whether or not to support mechanism to disable skipping by eNB

The following evaluation parameters for CFS PUR link level simulation are agreed.

| Parameter | Value/Description |
| --- | --- |
| Channel model | ETU 1Hz |
| Resource Bandwidth(s) | 1, 2, 3 or 6 PRBs (contiguous) for eMTC Proponent to provide details on resource allocation Same bandwidth for all UEs |
| Traffic Pattern | Fixed (N UEs transmit in the same resource) or (Optional) Bursty (N UEs may transmit in the same resource, with activity factor p for each of them) Note: For all traffic models, it is assumed that a UE has a dedicated DMRS Note: Final decision can be based on the results for both fixed and bursty traffic model |
| Number of repetitions in a resource | To be declared by proponent |
| Transport Block Size and Modulation Order | To be declared by proponent |
| Number of eNB Rx antennas | 2 as baseline, other values are not precluded |
| Metrics | BLER before HARQ Number of resources (PRB x subframes) used by the technique to serve a given number of UEs Baseline for comparison is dedicated PUR |
| MCL | 164, 144, 124dB MCL |
| UE grouping | All UEs in the same MCL level. |
| Processing Receiver at eNB | No SIC/joint decoding of UEs is assumed. MMSE single user decoding. Not NOMA. |
| DMRS collision | Each company to provide details on their DMRS assumption |
| Channel Estimation CFO Timing Error Power Imbalance | Realistic Uniform +/-50 Hz, +/-100 Hz (optional) +/-0.5 CP uniform, +/-1 CP uniform (optional) Uniform +/-[5 or 10] dB |

The evaluated proposals should be within the scope of the eMTC WI.

If multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process

- FFS: if multi-TB grant is enabled/supported

In idle mode, at least the following TA validation attributes are supported:

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

  - Based on RSRP measurement definition in existing Rel-15 TS36.214

Include in LS to RAN2, RAN4 to consider in their work.

The UE can be configured to use at least these TA validation attributes:

- Time Alignment Timer for idle mode

- Serving cell RSRP changes

- Note: the configuration shall support disabling of the TA validation attributes

Include in LS to RAN2, RAN4

For further study:

TA validation attributes:

- Subscription based UE differentiation (or Stationary indication in held in subscription)

- Cell specific indication where TA is valid within that cell

Include in LS to RAN2, RAN4:

RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode.

For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in MPDCCH search space

- FFS: Details on the search space (for example USS, CSS)

For dedicated PUR in idle mode, upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK

FFS: if ACK is sent on MPDCCH (layer 1) and/or PDSCH (layer 2/3)

Include in LS to RAN2, RAN4.

For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect

- an UL GRANT for retransmission on the MPDCCH, or

- FFS: a NACK, or

- FFS: no explicit ACK

Include in LS to RAN2, RAN4.

## RAN1#94bis

In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed):

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

- FFS Other attributes:

  - Neighbour cell RSRP change

  - TDOA of >=2 eNBs

  - TA History

  - Subscription based UE differentiation

  - Others not precluded (for example, attributes that need to be considered for high mobility UEs)

Note that UE power consumption should be taken into account for the FFS attributes

Dedicated preconfigured UL resource is defined as an PUSCH resource used by a single UE

  - PUSCH resource is time-frequency resource

  - Dedicated PUR is contention-free

Contention-free shared preconfigured UL resource (CFS PUR) is defined as an PUSCH resource simultaneously used by more than one UE

  - PUSCH resource is at least time-frequency resource

  - CFS PUR is contention-free

Contention-based shared preconfigured UL resource (CBS PUR) is defined as an PUSCH resource simultaneously used by more than one UE

  - PUSCH resource is at least time-frequency resource

  - CBS PUR is contention-based (CBS PUR may require contention resolution)

In idle mode, dedicated PUR is supported.

- Support for CFS PUR is FFS.

- Support for CBS PUR is FFS.

In IDLE mode, HARQ is supported for transmission in dedicated PUR

- A single HARQ process is supported,

  - FFS whether more than one HARQ processes are supported

- FFS: The design of the corresponding MPDCCH search space

For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported.

For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria

Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

## RAN1 #94

Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA

- FFS: Validation mechanism for TA

- FFS: How the pre-configured UL resources is acquired

For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed

Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types.

HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered:

  - Whether to support HARQ;

      - If supported, details of HARQ design including the number of HARQ processes;

  - Whether ACK/NACK is necessary

Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.