3GPP TSG-RAN WG1 Meeting # 97	R1-1907604

# Reno, USA,  May 13th - May 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1258 | rev |  | Current version: | 13.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Class A periodic CSI reporting using PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_EBF_FDMIMO-Core |  |  |  |  |  | Date: |  |  | 2019-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Class A periodic CSI reporting on PUCCH with Mode 1-1 contains the following reporting instances: - wideband CQI/PMI (i.e., 2nd PMI) reporting with periodicity Npd and offset NOFFSET,CQI - wideband first PMI reporting with with periodicity H'∙Npd and offset NOFFSET,CQI - RI reporting with with periodicity Npd∙MRI and offset NOFFSET,CQI+NOFFSET,RI The wideband first PMI reporting has a periodicity that is H' times that of wideband CQI/PMI reporting. Both the wideband first PMI reporting and the wideband CQI/PMI reporting has the same offset NOFFSET,CQI. Hence, there will be subframes where the wideband CQI/PMI report will collide with the wideband first PMI report. The current specification does not define how to handle the collision when the wideband first PMI report and the wideband CQI/PMI report belong to the same serving cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Class A periodic CSI reporting on PUCCH with Mode 1-1, clarify that when wideband first PMI report collides with wideband CQI/PMI report and when the two colliding reports belong to the same serving cell, the wideband CQI/PMI report is dropped. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.2.2	Periodic CSI Reporting using PUCCH

****************** Unchanged Parts Omitted ********************

The following CQI/PMI and RI reporting types with distinct periods and offsets are supported for the PUCCH CSI reporting modes given in Table 7.2.2-3:

-	Type 1 report supports CQI feedback for the UE selected sub-bands

-	Type 1a report supports subband CQI and second PMI feedback

-	Type 2, Type 2b, and Type 2c report supports wideband CQI and PMI feedback

-	Type 2a report supports wideband PMI feedback

-	Type 3 report supports RI feedback

-	Type 4 report supports wideband CQI

-	Type 5 report supports RI and wideband PMI feedback

-	Type 6 report supports RI and PTI feedback

- 	Type 7 report support CRI and RI feedback

-	Type 8 report supports CRI, RI and wideband PMI feedback

-	Type 9 report supports CRI, RI and PTI feedback

-	Type 10 report supports CRI feedback

For a UE configured in transmission mode 1-9 and for each serving cell, or for a UE configured in transmission mode 10 and for each CSI process in each serving cell, the periodicity  (in subframes) and offset  (in subframes) for CQI/PMI reporting are determined based on the parameter cqi-pmi-ConfigIndex () given in Table 7.2.2-1A for FDD or for FDD-TDD with primary cell frame structure 1 and Table 7.2.2-1C for TDD or for FDD-TDD and primary cell frame structure type 2. The periodicity  and relative offset  for RI reporting are determined based on the parameter ri-ConfigIndex () given in Table 7.2.2-1B. For a UE configured in transmission mode 9 and for each serving cell, or for a UE configured in transmission mode 10 and for each CSI process in each serving cell, if the UE is configured with parameter eMIMO-Type by higher layers, and eMIMO-Type is set to 'CLASS B', and the number of configured CSI-RS resources is more than one, when RI reporting is configured, the periodicity  for CRI reporting is determined based on the parameter cri-ConfigIndex () given in Table 7.2.2-1J. When the number of antenna ports in each configured CSI-RS resource is one, the periodicity  and relative offset for CRI reporting are determined based on the parameter cri-ConfigIndex () given in Table 7.2.2-1K. The parameters cqi-pmi-ConfigIndex, ri-ConfigIndex , and cri-ConfigIndex are configured by higher layer signalling. The relative reporting offset for RI  takes values from the set . If a UE is configured to report for more than one CSI subframe set then parameter cqi-pmi-ConfigIndex, ri-ConfigIndex and cri-ConfigIndex respectively correspond to the CQI/PMI, RI, and CRI periodicity and relative reporting offset for subframe set 1 and cqi-pmi-ConfigIndex2, ri-ConfigIndex2 and cri-ConfigIndex2 respectively correspond to the CQI/PMI, RI, and CRI periodicity and relative reporting offset for subframe set 2. For a UE configured with transmission mode 10, the parameters cqi-pmi-ConfigIndex , ri-ConfigIndex, cri-ConfigIndex, cqi-pmi-ConfigIndex2, ri-ConfigIndex2, and cri-ConfigIndex2 can be configured for each CSI process. A BL/CE UE is not expected to be configured with the parameter ri-ConfigIndex.

In the case where wideband CQI/PMI reporting is configured:

-	The reporting instances for wideband CQI/PMI are subframes satisfying .

-	For a UE configured in transmission mode 9 or 10, and UE configured with the parameter eMIMO-Type by higher layers, and eMIMO-Type set to 'CLASS A', the reporting interval of wideband first PMI reporting is an integer multiple  of period  (in subframes).

-	The reporting instances for wideband first PMI are subframes satisfying .

-	In case RI reporting is configured, the reporting interval of the RI reporting is an integer multiple  of period  (in subframes).

-	The reporting instances for RI are subframes satisfying .

-	In case CRI reporting is configured,

-	if the number of antenna ports in each configured CSI-RS resource is one,

-	the reporting interval of the CRI reporting is an integer multiple  of period  (in subframes)

-	The reporting instances for CRI are subframes satisfying .

-	otherwise

-	the reporting interval of the CRI reporting is an integer multiple  of period  (in subframes).

-	The reporting instances for CRI are subframes satisfying .

In the case where both wideband CQI/PMI and subband CQI (or subband CQI/second PMI for transmission modes 9 and 10) reporting are configured:

-	The reporting instances for wideband CQI/PMI and subband CQI (or subband CQI/second PMI for transmission modes 9 and 10) are subframes satisfying .

-	When PTI is not transmitted (due to not being configured) or the most recently transmitted PTI is equal to 1 for a UE configured in transmission modes 8 and 9, or for a UE configured in transmission mode 10 without a 'RI-reference CSI process' for a CSI process, or the transmitted PTI is equal to 1 reported in the most recent RI reporting instance for a CSI process when a UE is configured in transmission mode 10 with a 'RI-reference CSI process' for the CSI process, or the transmitted PTI is equal to 1 for a 'RI-reference CSI process' reported in the most recent RI reporting instance for a CSI process when a UE is configured in transmission mode 10 with the 'RI-reference CSI process' for the CSI process, and the most recent type 6 report for the CSI process is dropped:

-	The wideband CQI/ wideband PMI (or wideband CQI/wideband second PMI for transmission modes 8, 9 and 10) report has period , and is reported on the subframes satisfying . The integer  is defined as , where  is the number of bandwidth parts.

-	Between every two consecutive wideband CQI/ wideband PMI (or wideband CQI/wideband second PMI for transmission modes 8, 9 and 10) reports, the remaining  reporting instances are used in sequence for subband CQI (or subband CQI/second PMI for transmission modes 9 and 10) reports on  full cycles of bandwidth parts except when the gap between two consecutive wideband CQI/PMI reports contains less than  reporting instances due to a system frame number transition to 0, in which case the UE shall not transmit the remainder of the subband CQI (or subband CQI/second PMI for transmission modes 9 and 10) reports which have not been transmitted before the second of the two wideband CQI/ wideband PMI (or wideband CQI/wideband second PMI for transmission modes 8, 9 and 10) reports. Each full cycle of bandwidth parts shall be in increasing order starting from bandwidth part 0 to bandwidth part . The parameter is configured by higher-layer signalling.

-	When the most recently transmitted PTI is 0 for a UE configured in transmission modes 8 and 9 or for a UE configured in transmission mode 10 without a 'RI-reference CSI process' for a CSI process, or the transmitted PTI is 0 reported in the most recent RI reporting instance for a CSI process when a UE is configured in transmission mode 10 with a 'RI-reference CSI process' for the CSI process, or the transmitted PTI is 0 for a 'RI-reference CSI process' reported in the most recent RI reporting instance for a CSI process when a UE is configured in transmission mode 10 with the 'RI-reference CSI process' for the CSI process, and the most recent type 6 report for the CSI process is dropped:

-	The wideband first precoding matrix indicator report has period , and is reported on the subframes satisfying , where is signalled by higher layers.

-	Between every two consecutive wideband first precoding matrix indicator reports, the remaining reporting instances are used for a wideband second precoding matrix indicator with wideband CQI as described below

-	In case RI reporting is configured, the reporting interval of RI is  times the wideband CQI/PMI period , and RI is reported on the same PUCCH cyclic shift resource as both the wideband CQI/PMI and subband CQI reports.

-	The reporting instances for RI are subframes satisfying

-	In case CRI reporting is configured,

-	if the number of antenna ports in each configured CSI-RS resource is one,

-	the reporting interval of the CRI reporting is  times the wideband CQI/PMI period ,

-	The reporting instances for CRI are subframes satisfying .

-	otherwise

-	the reporting interval of the CRI reporting is  times the RI period  (in subframes).

-	The reporting instances for CRI are subframes satisfying

If a UE is not configured with higher layer parameter eMIMO-Type, or for a CSI process a UE is configured with higher layer parameter eMIMO-Type and eMIMO-Type is set to 'CLASS A', or for a CSI process a UE is configured with higher layer parameter eMIMO-Type and eMIMO-Type is set to 'CLASS B', and one configured CSI-RS resource, in case of collision of a CSI report with PUCCH reporting type 3, 5, or 6 of one serving cell with a CSI report with PUCCH reporting type 1, 1a, 2, 2a, 2b, 2c, or 4 of the same serving cell the latter CSI report with PUCCH reporting type (1, 1a, 2, 2a, 2b, 2c, or 4) has lower priority and is dropped.

If a UE is configured with higher layer parameter eMIMO-Type and eMIMO-Type is set to 'CLASS A' for a CSI process, in case of collision of a CSI report with PUCCH reporting type 2a of one serving cell with a CSI report with PUCCH reporting type 1, 1a, 2, 2b, 2c, or 4 of the same serving cell, the latter CSI report with PUCCH reporting type (1, 1a, 2, 2b, 2c, or 4) has lower priority and is dropped.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, for a CSI process, if a UE is configured with higher layer parameter eMIMO-Type and eMIMO-Type is set to 'CLASS B', and more than one configured CSI-RS resources, in case of collision of a CSI report with PUCCH reporting type 7, 8, 9, or 10 of one serving cell with a CSI report with PUCCH reporting type 1, 1a, 2, 2a, 2b, 2c, 3, 4, 5, or 6 of the same serving cell the latter CSI report with PUCCH reporting type (1, 1a, 2, 2a, 2b, 2c, 3, 4, 5, or 6) has lower priority and is dropped.

For a serving cell and UE configured in transmission mode 10, in case of collision between CSI reports of same serving cell with PUCCH reporting type of the same priority, and the CSI reports corresponding to different CSI processes, the CSI reports corresponding to all CSI processes except the CSI process with the lowest csi-ProcessId-r11 are dropped.

For a serving cell and UE configured in transmission mode 1-9 and configured with CSI subframe sets  and  by the higher layer parameter csi-SubframePatternConfig-r12 for the serving cell, in case of collision between CSI reports of same serving cell with PUCCH reporting type of the same priority, the CSI report corresponding to CSI subframe set  is dropped.

For a serving cell and UE configured in transmission mode 10 and configured with CSI subframe sets  and  by the higher layer parameter csi-SubframePatternConfig-r12 for the serving cell, in case of collision between CSI reports of same serving cell with PUCCH reporting type of the same priority and the CSI reports corresponding to CSI processes with same csi-ProcessId-r11, the CSI report corresponding to CSI subframe set  is dropped.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and if a PUCCH format 4 or format 5 resource for HARQ-ACK according to Table 10.1.2.2.3-1 cannot be determined, and if the UE is configured with more than one serving cell, the UE transmits a CSI report of only one serving cell in any given subframe. For a given subframe, in case of collision of a CSI report with PUCCH reporting type 7, 8, 9, or 10 of one serving cell with a CSI report with PUCCH reporting type 1, 1a, 2, 2a, 2b, 2c, 3, 4, 5 or 6 of another serving cell, the latter CSI with PUCCH reporting type (1, 1a, 2, 2a, 2b, 2c, 3, 4, 5 or 6) has lower priority and is dropped. For a given subframe, in case of collision of a CSI report with PUCCH reporting type 3, 5, 6, or 2a of one serving cell with a CSI report with PUCCH reporting type 1, 1a, 2, 2b, 2c, or 4 of another serving cell, the latter CSI with PUCCH reporting type (1, 1a, 2, 2b, 2c, or 4) has lower priority and is dropped. For a given subframe, in case of collision of CSI report with PUCCH reporting type 2, 2b, 2c, or 4 of one serving cell with CSI report with PUCCH reporting type 1 or 1a of another serving cell, the latter CSI report with PUCCH reporting type 1, or 1a has lower priority and is dropped.

For a given subframe and serving cells with UE configured in transmission mode 1-9, in case of collision between CSI reports of these different serving cells with PUCCH reporting type of the same priority, the CSI reports for all these serving cells except the serving cell with lowest ServCellIndex are dropped.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and if a PUCCH format 4 or format 5 resource for HARQ-ACK according to Table 10.1.2.2.3-1 cannot be determined, for a given subframe and serving cells with UE configured in transmission mode 10, in case of collision between CSI reports of different serving cells with PUCCH reporting type of the same priority and the CSI reports corresponding to CSI processes with same csi-ProcessId-r11, the CSI reports of all serving cells except the serving cell with lowest ServCellIndex are dropped.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and if a PUCCH format 4 or format 5 resource for HARQ-ACK according to Table 10.1.2.2.3-1 cannot be determined, for a given subframe and serving cells with UE configured in transmission mode 10, in case of collision between CSI reports of different serving cells with PUCCH reporting type of the same priority and the CSI reports corresponding to CSI processes with different csi-ProcessId-r11, the CSI reports of all serving cells except the serving cell with CSI reports corresponding to CSI process with the lowest csi-ProcessId-r11 are dropped.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and if a PUCCH format 4 or format 5 resource for HARQ-ACK according to Table 10.1.2.2.3-1 cannot be determined, for a given subframe, in case of collision between CSI report of a given serving cell with UE configured in transmission mode 1-9, and CSI report(s) corresponding to CSI process(es) of a different serving cell with the UE configured in transmission mode 10, and the CSI reports of the serving cells with PUCCH reporting type of the same priority, the CSI report(s) corresponding to CSI process(es) with csi-ProcessId-r11 > 1 of the different serving cell are dropped.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and if a PUCCH format 4 or format 5 resource for HARQ-ACK according to Table 10.1.2.2.3-1 cannot be determined, for a given subframe, in case of collision between CSI report of a given serving cell with UE configured in transmission mode 1-9, and CSI report corresponding to CSI process with csi-ProcessId-r11 = 1 of a different serving cell with the UE configured in transmission mode 10, and the CSI reports of the serving cells with PUCCH reporting type of the same priority, the CSI report of the serving cell with highest ServCellIndex is dropped.

See subclause 10.1 for UE behaviour regarding collision between CSI and HARQ-ACK and the corresponding PUCCH format assignment.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and if a PUCCH format 4 or format 5 resource for HARQ-ACK according to Table 10.1.2.2.3-1 cannot be determined, the CSI report of a given PUCCH reporting type shall be transmitted on the PUCCH resource  as defined in [3], where  is UE specific and configured by higher layers for each serving cell.

If a UE is not configured with higher layer parameter format4-MultiCSI-resourceConfiguration or format5-MultiCSI-resourceConfiguration, and

-	if the UE is not configured for simultaneous PUSCH and PUCCH transmission or,

-	if the UE is configured for simultaneous PUSCH and PUCCH transmission and not transmitting PUSCH,

in case of collision between CSI and positive SR in a same subframe, CSI is dropped.

****************** Unchanged Parts Omitted ********************