3GPP TSG RAN WG1 Meeting #97                       R1-1907620

Reno, USA, May 13th – 17th, 2019

Title:	[DRAFT] LS on LTE capabilities for FD-MIMO and 1024QAM

Release:	Rel-15

Work Item:	LTE_EBF_FDMIMO-Core, LTE_1024QAM_DL-Core, NR_newRAT-Core

Source:	Qualcomm Incorporated [RAN1]

To:	RAN2

# Contact Person:

# Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

Attachments:	None

# 1. Overall Description:

# 1.1 FD-MIMO

# In RAN1-96b, RAN1 reached the following agreement (which was included in previous LS to RAN2 (R1-1905576):

Agreement:

Adopt the following baseband capability signalling for Rel-13 FD-MIMO:

For a UE configured with FD-MIMO, for a band combination for which the UE does not report FD-MIMO capabilities, a configuration related to a set of CCs is supported by the UE if the following inequality is met

i ∈ configured CCswi∙li≤y

where

-  li is the maximum number of DL layers configured for CC i

- wi=x0,  if CC i is configured with FD-MIMO and li=2x1,  if CC i is configured with FD-MIMO and li=4x2,  if CC i is configured with FD-MIMO and li=81,   if CC i  is not configured with FD-MIMO

- y is the “total number of weighted layers” the UE can process

Notes:

- Legacy capability signalling can still be used after this capability is introduced (to e.g. signal a subset of supported band combinations).

- Detailed design of the capability signalling is left to RAN2

# In RAN1#97, RAN1 made the following additional agreement:

- Value of y (“Total number of weighted layers”) can also be signalled in EN-DC band combination. Signalling details are up to RAN2.

  - Note: this agreement applies to only LTE part of EN-DC

# 1.2 1024QAM

# In RAN1#97, RAN1 made the following agreement:

Introduce the following additional capability signalling to existing one for support of 1024QAM in LTE:

- In a band combination, the UE supports 1024QAM in a set of CC provided that:

- The set of CC belong to bands that are indicated to support 1024QAM in that band combination, and

- The following inequality is met: w'⋅NumLayers, 1024QAM+NumLayers, non1024QAM≤y', where

  - NumLayers, 1024QAM is the total number of layers across all configured CCs configured with 1024QAM

  - NumLayers, non1024QAM is the total number of layers across all configured CCs not configured with 1024QAM

  - w'∈1,65,54 and y'∈{10:2:30} are UE capabilities (per UE)

- In case the UE does not report w’ and y’, legacy capability signalling applies (i.e., the inequality does not apply)

- Request RAN2 to introduce a mechanism to ensure backwards compatibility considering both existing and new capability signalling

- Value of y’ can also be signalled in EN-DC band combination. Signalling details are up to RAN2.

  - Note: this agreement applies to only LTE part of EN-DC

  - Note: maximum data rate for LTE part of EN-DC should take y’ into account

# To address the note regarding maximum data rate, RAN1 endorsed the following TP for TS 38.306:

For EUTRA in case of MR-DC, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

Data rate (in Mbps) =

wherein

J is the number of aggregated EUTRA component carriers in MR-DC band combination

is the total maximum number of DL-SCH transport block bits received within a 1ms TTI for j-th CC, as derived from TS36.213 [22] based on the UE supported maximum MIMO layers for the j-th CC, and based on the maximum modulation order for the j-th CC according to indicated UE capabilities and number of PRBs based on the bandwidth of the j-th CC.

# 2. Actions to RAN2:

RAN1 respectfully requests RAN2 to update the UE capability signalling according to the above agreements.

RAN1 respectfully requests RAN2 to update 38.306 according to the above text proposal.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #98	26 - 30 Aug 2019 		      Prague, CZ

TSG RAN WG1 Meeting #98-bis	14 - 18 Oct 2019 		      CN