3GPP TSG RAN WG1 #97		R1-1907624

Reno, USA, May 13th – 17th, 2019

Title:	[DRAFT] LS to RAN on Coexistence of NB-IoT with NR

# Release:	Rel-16

Work Item:	NB_IOTenh3-Core

Attachment: 	None

Source:	Huawei [RAN1]

To:	RAN

# Contact Person:

# Name:	Carmela Cozzo

E-mail Address:	carmela.cozzo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

RAN1 has studied potential specification enhancements for improving performance of NB-IoT coexistence with NR and reached the following conclusions:

It is RAN1 recommendation to introduce DL/UL resource reservation in NB-IoT for at least FDD/TDD NB-IoT non-anchor carriers to avoid time and frequency domain resource overlap between NR and NB-IoT when NB-IoT is deployed within an NR carrier.

Resource reservation in FDD/TDD NB-IoT considers the following aspects:

- subframe/slot/symbol level granularity

- dropped/postponed resources associated to a certain granularity

- semi-static configuration is recommended. FFS: dynamic DCI signaling

- UL valid/invalid subframe configurations for FDD NB-IoT

# 2. Actions:

To RAN

ACTION: RAN1 respectfully requests RAN to revise the WID on Additional enhancements for NB-IoT according to the above.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #98	26-30 August 2019	Prague, Czech Republic

TSG RAN WG1 Meeting #98BIS	14-18 October 2019	China