3GPP TSG RAN WG1 Meeting #97  	R1-1907656

Reno, Nevada, USA, 13th – 17th May, 2019

Agenda Item: 7.2.5.4

Source: MediaTek Inc.

Title: Summary for more delay-tolerant re-transmission mechanisms in NR-NTN

Document for: Discussion and Decision

# Introduction

A RAN3-led Study Item on Rel-16 enhancements for NR-NTN was approved at RAN Plenary #80 [1]. The study item phase has identified range of expected values for the Round Trip Time (RTT) for the considered NR-NTN deployment scenarios [2, 3]. Solutions in the satellite and the UE will be required to compensate and correct the impact of RTT for delay-tolerant re-transmission for HARQ mechanisms.

This contribution aims to summarize proposals from contributing companies for delay-tolerant re-transmission for HARQ mechanisms for NR-NTN.

# Disabling of HARQ in NR-NTN

RAN2#105 made the following agreement:

- Retransmissions at one or several layers shall be supported for NTN and configurable by the network

- The network should be able to configure the UE whether the HARQ is “turned off”.  There is no UL feedback for DL transmission if HARQ is turned off.  FFS the impact on other procedures and how to configure

In Rel.15, MAC retransmission and RLC retransmission are defined.

- MAC HARQ retransmission for each HARQ process ID are soft combined at the receiver to improve reliability of data transmission. UL HARQ ACK/NACK feedback is sent by UE on PUCCH for each TB.

- RLC ARQ retransmission are not soft combined at the receiver. RLC PDUs not received from the MAC layer are reported by the UE or gNB in the RLC status report. The UE transmits RLC ACK (i.e. RLC status report) on PUSCH. UE needs to obtain an UL grant if gNB does not schedule UL grant based RLC poll.

Figure 2. MAC retransmission and RLC retransmission [10]

## Disabling of HARQ

RAN2 agreement allows to configure the UE to turn off HARQ. Dynamic activation/deactivation via DCI allows to disable disable/enable HARQ flexibly. Semi-statically disabling HARQ is simple way and can be done via RRC configuration without new signalling.

The current implementation supports limited functionality for disabling HARQ at the PHY/MAC layers. HARQ is asynchronous on DL and UL with adaptive re-transmissions in NR. The NDI field is toggled in DCI on PDCCH for DL assignment and UL grant to indicate new MAC PDU. The HARQ buffer is flushed when NDI field is toggled. HARQ can be effectively de-activated by gNB if NDI field is always toggled to indicate new packet for DL or UL. Trigger mechanisms used by the gNB to disable HARQ dynamically via physical layer could be studied.

Potential agreement #1: Network semi-Static disabling of HARQ can be considered.

Potential agreement #2: Discuss scope of further studies for trigger mechanisms used by the gNB to disable HARQ dynamically.

## Re-transmission with RLC AM

If there is no UL HARQ Ack/Nack feedback in PUCCH for corresponding DL transmission on PDSCH, retransmission at RLC layer (i.e. RLC ARQ) may be required to meet reliability requirements. In typical network configuration for RLC layer, ARQ re-transmissions in RLC AM can have high latency. This needn’t be the case for NR NTN, but would require configuration of RLC layer to be optimized for latency as well as reliability.

Re-transmission of DL packets at RLC layer requires to transmit RLC UL ACK on PUSCH in RLC AM. The RLC ACK is the RLC status report that is transmitted by the RLC layer. Note that the RLC ACK is not a MAC-layer UL HARQ Ack/Nack feedback. Latency of transmission of the RLC UL ACK depends on RLC configuration and the ways the RLC AKC is transmitted as listed below:

- Piggybacking on an UL TB (configured or dynamic grants)

- By triggering an SR with the feedback sent in the following UL grant

- By triggered CBRA with the feedback included in msg3 or msg5.

The ARQ process of the RLC layer needs to buffer the packets that have been sent until a packet acknowledgment is received or the timer expires to initiate a retransmission. Note that even if HARQ is disabled, MAC layer scheduling latency and reliability of first transmission needs to be taken into account. Overall, impacts on the RLC layer re-transmissions on reliability and latency should be studied if HARQ is disabled. RLC parameters such as the size of Rx/Tx window, sequence number, number of RLC retransmission, retransmission timer, RLC ACK to accommodate larger satellite propagation delays while meeting reliability and latency targets can be considered in the study.

Potential agreement #3: Study enhancements of RLC Acknowledge Mode for re-transmission at RLC layer.

Potential agreement #4: Study impact on reliability and latency of RLC layer if HARQ is disabled semi-statically with at least the following aspects:

- Size of Rx/Tx window

- Sequence number

- Retransmission timer

- Number of RLC retransmission

- RLC ACK

## Compact DCI

HARQ-related DCI fields that can be removed for scheduling PDSCH (DCI format 1_0) or scheduling PUSCH (DCI format 0_0) if HARQ is disabled. The HARQ related fields become redundant if HARQ is disabled semi statically. Compact DCI with reduced DCI fields to reduce L1 overhead and/or increase reliability with lower ECR could be considered. This allow lower effective code rate to improve reliability of PDCCH for reliability of first PDSCH transmission and improve efficiency of PDCCH with lower level of CCE aggregation required.

Observation #1: When HARQ is disabled semi-statically in NR-NTN, HARQ related fields in DCI formats 1_0 and 0_0 are redundant

Potential agreement #5: Removal of HARQ related fields in DCI when HARQ is disabled semi-statically can be considered.

## First transmission reliability when HARQ is disabled

If HARQ is disabled, the BLER target should be improved firstly to guarantee the reliability of the first transmission. This may be done by setting a lower MCS, MCS table with lower target BLER, power increase and repetition with RV cycling or blind / HARQ-less repetitions. Reliability of data packet can be improved with the considered retransmission mechanism with some compromise with resource efficiency.

- Lower MCS and MCS table with lower target BLER are specified solutions in release 15 to improve reliability. There seems to be no need to define new MCS or MCS table for NR NTN as URLLC requirements are already very high. At a minimum, the reliability targets in terms of BLER requirements for PDSCH and PDCCH should be first clarified before considering if new MCS table is needed for NR-NTN.

- Slot aggregation with RV cycling is specified in release 15. It is configured to UE semi-statically. This implies that slot aggregation with RV cycling cannot be changed dynamically, all transmissions use same amount of repetitions. The HARQ A/N is sent at the end of repetitions. There is no HARQ A/N per repetition. The HARQ A/N is for MAC PDU after combining of repetitions.

- Blind/HARQ less repetitions indicated by DCI is being discussed in Rel-16 NR URLLC for UL. It is unlikely that this mechanism will be specified for DL. Note that blind repetition / HARQ-less repetitions was specified in Rel-15 LTE HRLLC with blind back-to-back repetitions indicated by DCI. There is no HARQ A/N per repetition. The HARQ A/N is for MAC PDU after combining of blind repetitions.

Potential agreement #6: Enhancements to improve reliability of first DL or UL transmission when HARQ is disabled is for further study with the following legacy mechanisms re-used as baseline

- Lower MCS

- MCS table with lower target BLER

- Slot aggregation with RV cycling

Potential agreements #7: Study required BLER target for first transmission of PDSCH and BLER target for associated PDCCH.

| Source | Related Proposals & Observations |
| --- | --- |
| Ericsson | - Proposal 1. Study RAN1 impact of disabling HARQ optionally. |
| Nokia | - Proposal 1: For GEO satellites, RRC configuration can be used to support the semi-static HARQ disabling. - Proposal 2: Dynamic HARQ disabling should be supported and how to support dynamic HARQ disabling mechanism in the current protocol should be studied. - The mechanism to guarantee the transmission reliability should be studied when HARQ is disabled: - The mechanism to improve the BLER to guarantee the reliability of the first transmission; - The retransmission scheme without feedback to guarantee the successful transmission. |
| Huawei | - Proposal 2: Both semi-static and dynamic HARQ configuration should be considered. - Proposal 3: Reusing the idle bits in DCI when HARQ disabled should be considered. - Proposal 4: Enhancement for data transmission is required when HARQ disabled. - Proposal 5: The challenges to the RLC should not be neglected if HARQ is disabled in NTN. |
| ZTE | - Proposal 1: Disabling the feedback of HARQ in semi-static way can be considered as the baseline for NTN. - Proposal 3: Following implementation-based methods can be considered as baseline for the transmission performance enhancements once the feeder back of HARQ is disabled. - Scheduling with conservative MCS - MCS table with lower target BLER - Repetition with RV cycling: - Blind scheduling of retransmission |
| CATT | - Proposal 1: HARQ deactivation and HARQ process number should be configurable with UE specific way. - Observation 1: HARQ deactivation can help to reduce HARQ-ACK feedback and data buffering burden. - Proposal 2: Dynamic DCI configuration with specific HARQ process ID to deactivate HARQ retransmission in NTN is supported. - Proposal 3: Support high BLER target in CQI feedback and MCS mapping to reduce the re-transmission latency for NTN. |
| Samsung | - Observation 1: GEO satellite links exhibit very long propagation delays, which make the use of HARQ procedures prohibitive. Other procedures to improve link reliability, like the use of more robust MCS tables are more viable in the GEO satellite links. - Proposal 1: Consider configuring HARQ as an optional feature for NTN, where its use under certain satellite types and propagation conditions can be beneficial. |
| Sony | - Proposal 1: RAN1 should study mechanisms to reduce the signaling overhead indicating HARQ disabling, if dynamic HARQ disabling is supported. - Proposal 2: RAN1 should consider how to support any affected procedures when HARQ-feedback is disabled. - Observation 1: It will bring challenges to the UM RLC layer if and when HARQ re-transmission is disabled in NTN. - Proposal 3: RAN1 should study mechanisms to increase the system reliability and efficiency if and when HARQ re-transmission is deactivated. |
| OPPO | - Poroposal5: Configured grant should be supported for both HARQ and non-HARQ transmission. |
| Intel | - Proposal 2: Support semi-static HARQ disabling - Dynamic HARQ disabling is FFS |
| Qualcomm | - Proposal 1: For NTN, support cell-specific configuration of HARQ on and off. - Proposal 2: For NTN, support configuration of HARQ on and off for configured grants. - For NTN, support dynamic HARQ on and off. |

# HARQ Optimization for NR NTN

## Satellite RTT on data rates with max HARQ processes = 16

In satellite system, larger values of RTT can be typically experienced due to the large propagation delay between the UE and the satellite. The RTT depends on the satellite orbit and elevation angle of the beam spot within the satellite cell as shown in Table 1 and 2.

RTT Satellite propagation delay increases scheduling delay. The longer scheduling delay in NR-NTN significantly reduces max data rates for the UE. Assuming the legacy max number HARQ processes of 16 for DL and UL in NR, we observed the following:

- For bent-pipe GEO=35786 km, the available peak throughput is 3.2% (=16/500*100) of NR terrestrial peak throughput.

- For bent-pipe MEO=10000 km scenario, the available peak throughput is around 8.4% (=16/190*100) of NR terrestrial peak throughput.

- For bent-pipe LEO=600 km scenario, the available peak throughput is around 57% (=16/28*100) of NR terrestrial peak throughput.

Note that due to link budget, availability of bandwidth, UE-satellite link impairments such as Doppler, phase noise and so on, the user experienced data rates can be expected to be significant smaller that the peak data rates in NR-NTN.

Potential agreement #7 Study solutions when HARQ is enabled to mitigate impact of satellite RTT on experienced user data rates.

|  |  | LEO at 600 km |  | LEO at 1500 km |  | MEO at 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| UE: 10° | satellite - UE | 1932.24 | 6,440 | 3647.5 | 12,158 | 14018.16 | 46.727 |
| GW: 5° | satellite - gateway | 2329.01 | 7.763 | 4101.6 | 13.672 | 14539.4 | 48.464 |
| 90° | satellite - UE | 600 | 2 | 1500 | 5 | 10000 | 33.333 |
| Bent pipe satellite |  |  |  |  |  |  |  |
| One way delay | Gateway-satellite_UE | 4261.2 | 14.204 | 7749.2 | 25.83 | 28557.6 | 95.192 |
| Round Trip Delay | Twice | 8522.5 | 28.408 | 15498.4 | 51.661 | 57115.2 | 190.38 |
| Regenerative satellite |  |  |  |  |  |  |  |
| One way delay | Satellite -UE | 1932.24 | 6.44 | 3647.5 | 12.16 | 14018.16 | 46.73 |
| Round Trip Delay | Satellite-UE-Satellite | 3864.48 | 12.88 | 7295 | 24.32 | 28036.32 | 93.45 |

Table 1: Propagation delays for different NGSO satellites (altitude and payload types) [2]

|  | GEO at 35786 km |  |  |
| --- | --- | --- | --- |
| Elevation angle | Path | D (km) | Time (ms) |
| UE :10° | satellite - UE | 40586 | 135.286 |
| GW : 5° | satellite - gateway | 41126.6 | 137.088 |
| 90° | satellite - UE | 35786 | 119.286 |
| Bent Pipe satellite |  |  |  |
| One way delay | Gateway-satellite_UE | 81712.6 | 272.375 |
| Round trip Time | Twice | 163425.3 | 544.751 |
| Regenerative Satellite |  |  |  |
| One way delay | Satellite -UE | 40586 | 135.286 |
| Round Trip Time | Satellite-UE-Satellite | 81172 | 270.572 |

Table 2: Propagation delays for GEO satellite at 35786 km [2]

## Higher number of HARQ processes > 16

Figure 1 illustrates the HARQ timing diagram for THARQ, Tslot, and processing times in the UE and the gNB for the DL. The minimum required number of HARQ processes can be computed directly from the RTT delay of each satellite constellation using simplified formula as

NHARQ, min≥THARQTslot ,

For example, NHARQ-min ≥ 28 with THARQ = 28 ms for satellite deployment with LEO=600 km where RTT can be 28 ms and Tslot = 1ms assuming NR numerology μ=0. NHARQ-min ≥ 50 with LEO=1500 km. Table 1 gives the required number of HARQ processes, NHARQ, min for LEO, MEO, GEO satellite deployments with SCS=15 kHz. With this number of HARQ processes, the impact of satellite RTT on NR data rates is removed. The number of HARQ processes required can be up to 600 in GEO satellite deployment. For larger subcarrier spacing (SCS), i.e., 2μ * 15 kHz, the minimum required number of the HARQ processes is scaled by 2μ. The minimum required number of HARQ processes can be alternatively determined as [8]

NHARQ=Tsf+Tue+Tack+ Tnb+RTTTsf

where RTT, Tsf, Tue, Tack, and Tnb refers to the round trip time, sub-frame duration, UE processing time, ACK/NACK transmission time, and gNB processing time, respectively.

Figure 1: Timing diagram of a single HARQ process for a NTN with a single bent-pipe satellite in the link [2]

| constellation | Max. THARQ | NHARQ, min processes for 1 ms slot operation | UE side feasibility |
| --- | --- | --- | --- |
| Terrestrial | 16ms | 16 | Feasible (Rel. 15) |
| LEO | 50ms | 50 | Feasible (with HARQ extension) |
| MEO | 180ms | 180 | FFS (impact on TBS/MCS) |
| GEO/HEO | 600ms | 600 | FFS (impact on TBS/MCS) |

Table 1: The minimum required number of the HARQ processes, NHARQ, min, assuming a 1ms slot duration for 15 kHz* reference subcarrier-spacing

Issues with >> 16 HARQ processes:

- DCI size increase: The DCI is indicated per HARQ process per slot in a 4-bit field in NR specifications. With > 16 HARQ processes, 10 bits in DCI will be needed to indicate the HARQ process ID for SCS=15 kHz in GEO, and 14 bits with numerology μ=4 (i.e. 2μ *600 = 24 *600 = 9600 HARQ processes)

- HARQ soft buffer size: HARQ combining of re-transmissions requires soft bits of packets of failed (re)-transmissions to be stored in soft buffer. The soft buffer size requirement for the UE and the gNB may increase depending on bandwidth assumption, TBS, number of re-transmissions, number of layers, and BLER target.

- RLC impact: Impact of longer satellite RTT of RLC layer needs to be evaluated. After RLC segmentation and LCP, MAC PDUs may take much longer to be transmitted by MAC layer. This may result in more queuing of RLC packets transmitted by the all the UEs in the satellite cell. It is needed to study the RLC-layer impact at least in gNB.

Potential agreement #8 Study solutions with > 16 HARQ processes when HARQ is enabled at least consider the following:

- DCI size

- HARQ soft buffer size

- RLC-layer  impact

| Source | Related Proposals & Observations |
| --- | --- |
| Huawei | - Observation 1: The long RTT in NTN has a negative influence on HARQ design, such as the increase of UE buffer size, processing complexity, signaling overhead, and overall latency. |
| Nokia | - Proposal 3: The impact on buffer size at both transmitter and receiver on DL and UL when HARQ is enabled should be considered. - Proposal 4: The HARQ process number can be extended and configured. Signaling overhead reduction solutions on HARQ process ID should be studied if the HARQ process number is extended. |
| ZTE | - Observation 1: Requirement on the maximum supported number of HARQ process is different per satellite scenario. - Proposal 2: Necessity on extension of maximum supported number of HARQ should be verified with consideration on the UE capability and impacts on specification. |
| Panasonic | - Proposal 1: support MAC level retransmission regardless of satellite types (i.e. GEO, MEO, LEO) - Proposal 2: UE does not have to make soft buffers available for all processes corresponding to RTT. UE should use soft buffer only when the reception is failure. - Proposal 2: UE does not have to make soft buffers available for all processes corresponding to RTT. UE should use soft buffer only when the reception is failure. - Proposal 3: Solution to allow gNB to control UE buffer usage should be studied. - Proposal 4: HARQ process ID indication to support longer RTT should be studied. Following options should be studied. - Option 1: Increase the number of bits for HARQ process ID indication - Option 2: reuse HARQ process ID within RTT - Option 3: HARQ process is tied to SFN/slot number - Proposal 5: An efficient ACK/NACK transmission method should be studied. |
| Samsung | - Observation 2: LEO satellite links offer far more manageable delays to make the HARQ procedures workable, even in bent-pipe scenarios. In certain propagation scenarios involving block fading, the suitable use of HARQ can be more effective than using more robust MCS tables. - Observation 3: The use of pre-emptive HARQ can increase the transmission efficiency and data rates, while keeping the buffer sizes and signalling overheads down. The number of repetitions in the pre-emptive burst can be determined in relation to the propagation delays, as per the satellite orbit altitude. - Proposal 2: Consider increasing the number of parallel HARQ processes in NTN-NR, as this would enable the use of HARQ to counter a variety of block fading scenarios in LEO and even in MEO satellite links. - Proposal 3: Consider the use of pre-emptive HARQ, with a configurable number of repetitions, to combat lower transmission efficiency without impacting heavily on the buffer sizes or signalling overheads. - Proposal 4: Consider different variants of the pre-emptive HARQ mechanism to suit the synchronous or asynchronous HARQ modes and the prevailing propagation conditions. The flexibility to configure the HARQ mechanism to suit the conditions would bring higher benefits. |
| Sony | - Proposal 4: RAN1 should study whether to support more than 16 HARQ process number in NTN, if HARQ is enabled. |
| OPPO | - Proposal1: NTN should support fixed 16 HARQ processes. The HARQ process ID field size in DCI is kept. - Proposal1: NTN should support fixed 16 HARQ processes. The HARQ process ID field size in DCI is kept. - Proposal3: NTN introduces higher UE capability with more than 16 HARQ processes. The process ID will be jointly decided by process ID and slot number. - Proposal4: NTN supports PDSCH/PUSCH bundling to fully utilize schedulable slots. - Poroposal5: Configured grant should be supported for both HARQ and non-HARQ transmission. |
| Intel | - Proposal 1: Solutions to support increased number of parallel HARQ processes considering soft buffer size limitation and solutions to decrease overhead for signaling of HARQ process ID should be further identified - Whether to increase the number of parallel HARQ processes for NTN is for further study taking into account the identified solution - Proposal 3: Study the reliability of first transmission for NTN considering already specified features (e.g. repetitions and low code rate transmission) |
| Qualcomm | - Proposal 4: For NTN, UE reports the capability on the number of HARQ processes. - Proposal 5: For NTN, more than 16 HARQ processes can be configured. - Proposal 6: For NTN, support slot number based HARQ process identification. |

# References

- RP-181370, Thales,  Study on solutions evaluation for NR to support Non Terrestrial Network, SID, RAN#80, June 2018

- TR38.811 “Study on NR to support non-terrestrial networks”

- TR 38.821 “Solutions for NR to support non-terrestrial networks”

- R1-1906088, Nokia, Consideration on HARQ in NTN, RAN1#97, May 2019

- R1-1907392, Ericsson, On adapting HARQ procedures for NTN, RAN1#97, May 2019

- R1-1905995, Huawei, Discussion on HARQ for NTN, RAN1#97, May 2019

- R1-1907279, Qualcomm, Delay-Tolerant Retransmission Mechanisms for NTN, RAN1#97, May 2019

- R1-1906873, ZTE, Discussion on the HARQ procedure for NTN, RAN1#97, May 2019

- R1-1906466, MediaTek, Delay-tolerant re-transmission mechanisms in NR-NTN, RAN1#97, May 2019

- R1-1906265, Panasonic, Discussion on Retransmission Mechanisms for NTN, RAN1#97, May 2019

- R1-1906326, CATT, HARQ consideration for NTN, RAN1#97, May 2019

- R1-1906594, Samsung, HARQ procedure in NTN, RAN1#97, May 2019

- R1-1906849, Sony, Consideration on delay-tolerant HARQ for NTN, RAN1#97, May 2019