3GPP TSG RAN WG1 #97	R1-1907673

Reno, USA, May 13–17, 2019

Agenda Item:	7.2.1.1

Source:	ZTE

Title:	Summary of 7.2.1.1 Channel Structure for Two-step RACH

Document for:	Discussion and decision

# Introduction

This document summarizes the key issues discussed under agenda 7.2.1.1 Channel Structure for Two-step RACH based on the views expressed in the contributions submitted to this agenda.

# Channel Structure

## msgA PUSCH configuration

The open issues on msgA PUSCH configurations including the following:

- Time/frequency structure of PUSCH occasions

- Configuration method and principles

- Parameters of msgA PUSCH configurations

Based on the contributions submitted to RAN1#97, companies’ views on the above open issues are summarized in Table 1.

Table 1 Companies views on MsgA PUSCH configuration

|  | Configuration method | Parameters or principle of msgA PUSCH configuration | Other remarks |
| --- | --- | --- | --- |
| ZTE | Opt 1 or opt 2 | -Parameters per PO group (opt.2) - Single time/frequency offset between each PRACH slot and the associated PUSCH group - MCS/TBS - Number of TDMed POs - Number of FDMed POs -POs (including guard band or guard period if exist) within a PO group are consecutive in time and frequency domain -Support identical resource size for the POs within a PO group | -Limited MCS levels |
| Huawei | Opt 1 | Resource configuration for PO group -Time domain - Configured grand in principle -Frequency domain - reuse current signaling, or define some default location -POs in a PO group are equally spaced in time/frequency. -Different PO groups can have different resource size or MCS. | -Not support PRACH and PUSCH in the same slot -Frequency resource of msgA PUSCH limited to the bandwidth of PRACH -Limited MCS levels |
| vivo | Opt 1 Type 1 configured grant. | -Configurable number of POs within a PUSCH configuration period -Different POs can have different resource size, configurable. -In a given PO, different PRUs have the same resource size. -Support a single configurable MCS for a PO. | -Support PRACH and PUSCH in the same slot -SSB-PO mapping in association pattern period |
| NTT | Opt 2 |  | Validation rule for PO based on TDD configuration and SSB location, similarly as the rule for valid/invalid RO in Rel.15. |
| Spreadtrum | Opt 1 |  | -Support PRACH and PUSCH in the same slot. -PRACH and associated PUSCH may overlap in frequency domain - PUSCH and other PRACH may neither overlap nor partially overlap in time-frequency domain simultaneously |
| CMCC | Opt 2 | A set of fixed values on relative locations. |  |
| OPPO | Opt 2 | -Parameters per PO pool - Starting frequency/time - Number of PRB/symbols of one PO - Numer of FDMed POs - MCS/TBS - Antenna configuration | -Support both PRACH front-loaded and PUSCH front-loaded. -Whether to support type 0 resource allocation? (non-contiguous) |
| LGE |  | -Parameters Per PO: - Time domain: time gap, number of symbols, starting symbol index within a slot; - Frequency domain: Guard band, number of PRBs, starting PRB index, -Parameters for Multiple POs: - Time domain: number of POs in a slot; slot indication within a duration - Frequency domain: number of POs. | Support PRACH and PUSCH in the same slot -For the case of separate ROs for 2-step and 4-step RACH, allow new configuration that OFDM symbols within a first half RACH slot are used as RO |
| Nokia | Opt 2 | -POs in the same PO group share the same resource size. -A single configured offset between the start of the PRACH slot (or PRACH subframe/60 kHz slot in FR1/FR2) containing the 2-Step RACH ROs and the start of the 2SR PUSCH resource units (PUSCH Occasions). -Another configured offset, in frequency domain, between the start of the first RO in frequency and the start, in frequency, of the 2SR PUSCH resource units (PUSCH Occasions). | Postpone the decision of PRACH and PUSCH in the same slot There should be a possibility to configure 2-step RACH operation such that there is no time-wise gap between the preamble transmission and the associated PUSCH transmission. |
| Intel | Opt 1 with different (longer) configuration period | -Time domain parameters: slot offset, configuration period, starting symbol, and length; -Frequency domain parameters: starting PRB and length of PRBs. -parameters per PO - An MsgA PUSCH occasion ID - Time/frequency resource allocation - MCS - Power control related parameters | - Support PRACH and PUSCH in the same slot - Support repetition of PUSCH |
| Sony | Opt 2 | Configurable time-gap between PRACH and PUSCH |  |
| Panasonic |  | If Option 2 is supported, configurable transmission gap between PRACH preamble in PRACH occasion(s) to PUSCH occasion including zero should be considered |  |
| Samsung | opt 1 or opt 2 | For opt.1: - Value range of PUSCH configuration period is also {10, 20, 40, 80, 160}ms - SSB-PUSCH association should be defined, similar to SSB-PRACH association For opt. 2: - Relative location is based on the last RO of the PRACH slot where the selected RO located - Single semi-statically configured value (alt. 3) | - Not support PRACH and PUSCH in the same slot - It is beneficial for gNB to detect preamble first before reception of PUSCH. -Configurable GB and GT per PO |
| Sierra Wireless | Opt 2 | Multiple 2-step RACH configurations shall be supported. The configuration shall at least include: - Coverage range thresholds (e.g. RSRP/RSRQ range) - PUSCH modulation and coding - PRACH format - FFS: number of 2-Step RACH configurations | -Support PRACH and PUSCH in the same slot -Use preamble for channel estimation (if PRACH and PUSCH are close in time/frequency) |
| Ericsson | Defer opt 1&2 decision until PO configuration is more clear. | -A set of resources containing multiple PUSCH occasions is defined (a ‘msgA PUSCH slot’), where: - A msgA PUSCH slot occurs periodically and has a known length in symbols - Each PUSCH occasion occupies a contiguous set of subcarriers and symbols - Multiple PUSCH occasions in a msgA PUSCH slot can be in one OFDM symbol or in one subcarrier - The PUSCH occasions can have different sizes, and a given PRB can contain PUSCH occasions with different sizes. - UEs frequency division multiplexed in a set of OFDM symbols can correspond to an SSB -Different ROs in one PRACH slot can map to different msgA PUSCH slots |  |
| CAICT | Opt 2 | No matter opt.1 or opt.2, a group of POs is associated with a group of ROs, the position of the PO group is configured | Not support repetition (prefer to enhance the PUSCH coverage by power control) |
| Qualcomm | Opt 2 semi-statically or dynamically. | TxGap between PRACH and PUSCH PO includes GT and GB PO size: {3,6,12} PRBs + 14 symbol (for FR1) Multiple POs per PUSCH configuration period | -Support PRACH and PUSCH in the same slot -Bandwidth can be the same or different as PRACH. - Support slot repetition |
| KDDI |  | MCS should be common for PRUs at least in a PO |  |

Regarding the PUSCH configuration, we have the following proposal:

Proposal 1:

- Both option 1 (separate configuration) and option 2 (relative location) for msgA PUSCH configuration are supported

- The following parameters are defined per msgA PUSCH configuration:

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location)

    - MCS/TBS

    - Number of TDMed POs: # of PUSCH slots, # of POs in each slot

    - Number of FDMed POs

      - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

    - Frequency domain resource allocation per PO

    - Time domain resource allocation per PO

    - Number of DMRS ports/sequences per PO

  - Parameters specific to option 1

    - Periodicity (msgA PUSCH configuration period)

      - Value range can be same as PRACH, i.e. {10, 20, 40, 80, 160}ms

    - Offset(s) or subframe number

    - Frequency starting point

  - Parameters specific to option 2

    - Single time offset respect to the start of each PRACH slot or each PRACH subframe/60 kHz slot in FR1/FR2

    - Single frequency offset respect to the start of the first RO in frequency

- Multiple msgA PUSCH configurations is supported

  - FFS the maximum number of configurations

  - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

- FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH

- FFS validation rule of msgA PUSCH

- FFS whether or not support repetitions for msgA PUSCH

Regarding the PRACH and PUSCH association, we have the following proposal:

Proposal 2:

- Reuse the PRACH configuration tables defined in TS38.211 for the configuration of msgA PRACH

  - 2-step RACH and 4-step RACH share the same PRACH configuration index

- The msgA (PRACH and PUSCH) association period is defined as

  - max{PRACH configuration period, msgA PUSCH configuration period}

  - PO in front of RO association is not supported

- FFS SSB to PO mapping in association pattern period

## Detailed mapping between PRACH preamble and PUSCH resource unit

The following table captures companies' views on detailed mapping between PRACH preamble and PUSCH resource unit. Accordingly, three issues i.e. association period, PUSCH resource unit definition, mapping ratio and mapping order are listed for further discussion.

1) PUSCH Resource Unit (PRU) definition and Mapping Ratio

Regarding the definition of PUSCH resource unit, some companies expressed concerns on supporting both DMRS sequences and DMRS ports. The reason is the user multiplexing capability may not need to be higher than that supported by DMRS ports [8][15]. The channel estimation performance may degrade due to non-orthogonal DMRS sequences [11][15]. The simulation results in [10][17] showed the potential performance degradation with a mere 2-3 UEs multiplexing using DMRS sequences. [1][18] expressed concerns on potential impact on inter-cell interference for certain scrambling sequences. On the other hand, [2][13] mentioned the resource utilization benefit of using DMRS sequence. [10] mentioned UE distinguishing capability with DMRS sequences. The simulation results in [1][2][19] showed similar performance of multiplexed UEs distinguished by DMRS sequences or DMRS ports. Moreover the simulation results in [3] showed better performance with sequence than with multi to 1 mapping at 0.01 BLER.

Multiple companies have provided simulations to compare the performance of multi-to-1 mapping and 1-to-1 mapping considering different definition of PUSCH resource unit, i.e. including DMRS sequences or not. The simulations results in [1] and [2] show that the performance of multi-to-1 mapping is similar to the 1 to 1 mapping given the same total PUSCH resource overhead in case of up to 2 UEs randomly selecting preambles. The simulation in [3] showed some performance difference at the 0.01 BLER operating point. The simulation in full collision cases demonstrate there exists some gaps [2][11], yet the performance for the collided case is still decent [11] since preamble is used for channel estimation. In addition to the performance similarity under randomly selecting preamble case, multi-to-1 mapping could bring benefits of resource utilization and resource overhead reduction [1][8][14]. Some companies mentioned that the mapping ratio could be made flexible catering to receiver implementation and the amount of PUSCH resource or preambles for msgA transmission [4][11][12][16].  In addition, fall-back to 4-step RACH could anyway be performed in case of decoding failure [4][16].

Mapping ratio could be broadcast [1] or implicitly derived [2][7].

Table 2: Companies’ preferences on PUSCH resource unit and mapping ratio.

|  | DMRS port/sequence for PRU | Mapping ratio: Preamble to PRU |
| --- | --- | --- |
| ZTE | DMRS port only | M-to-1(M≥ 1) |
| Huawei | Both DMRS ports and sequences | 1-to-1 |
| vivo | Both DMRS ports and sequences | 1-to-1 |
| NTT | Both DMRS ports and sequences | M-to-1 (M≥1) |
| Spreadtrum | Both DMRS ports and sequences | 1-to-1 |
| Fujitsu | Both DMRS ports and sequences | 1-to-1 |
| CMCC | Both DMRS ports and sequences | M-to-1 (M≥1) |
| OPPO | DMRS port only | M-to-1 (M≥1) |
| LGE | Both DMRS ports and sequences |  |
| Nokia |  | M-to-1 |
| Intel | DMRS port only | M-to-1 |
| Sony |  | M-to-1 (M≥1) |
| InterDigital | Both DMRS ports and sequences | 1-to-1, M-to-1, 1-to-M |
| Panasonic |  | M-to-1 (M≥1) |
| Samsung | DMRS port only | M-to-1 (M≥1) |
| Sierra Wireless |  | M-to-1 (M≥1) |
| Ericsson | Both DMRS ports and sequences | M-to-1 (M≥1) |
| CAICT |  | M-to-1 (M≥1) |
| Qualcomm | Both DMRS ports and sequences | 1-to-1, M-to-1, 1-to-M |
| KDDI | Both DMRS ports and sequences | M-to-1 (M≥1) |

Based on the companies’ preferences, we have the following two alternatives for the compromised proposal on the mapping between PRACH preamble and associated PRUs.

Proposal 3a:

- Support both DMRS port and DMRS sequences for the definition of PRU

- Confirm the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit (PRU) are supported

  - Configurable number of preambles (including one or multiple) mapped to one PRU

Proposal 3b:

- gNB has the flexibility to configure either of the following two cases:

  - The definition of PUSCH resource unit includes both DMRS port and DMRS sequence, 1 to 1 mapping between preamble and PRU is supported only

    - Configurable number of DMRS sequences

  - PUSCH resource unit includes DMRS port only, multi-to-1 mapping is supported

    - Configurable number of preambles (including one or multiple) mapped to one PRU

2) Mapping order

Mapping order may be needed on top of the mapping ratio [1][3][8][10][15][20]. Several companies propose following the order of frequency first and time second ordering [1][8][10][15] for both RO and PO. The preamble and PUSCH resource unit with the same ordering index could therefore be associated [1][8]. On top of this, [3] mentioned an interlace approach for associating preamble the PUSCH resource unit for better spatial multiplexing. [8] proposed mapping preambles with different corresponding beams PUSCH

units in case of multi to 1 mapping to reduce the detection error. [15] mentioned associating the preamble and DMRS with the same SSB.

Proposal 4:

- The PRACH preambles are mapped to PUSCH resource units (PRUs) within an msgA association period in the following order:

  - First, in increasing order of DMRS indexes within a single PUSCH occasion

    - FFS DMRS indexing, e.g. port first or sequence first

  - Second, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

  - Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot

  - Fourth, in increasing order of indexes for PUSCH slots

  - Redundant preambles/PRUs will not be mapped

  - Scrambling RNTI

A simple way to obtain UE-specific scrambling code (considering no preamble collision) is to use the legacy RA-RNTI plus preamble index as the initialization ID. Options proposed in the submitted contributions are summarized in Table 3.

Table 3 RNTI used for msgA PUSCH scrambling

|  | RA-RNTI + preamble index | RA-RNTI + DMRS index (including port/sequences) | RA-RNTI + SSB index | RA-RNTI + configured scrambling IDs |
| --- | --- | --- | --- | --- |
| ZTE |  |  |  |  |
| Huawei |  |  |  |  |
| vivo |  |  |  |  |
| OPPO |  |  |  |  |
| LGE |  |  |  |  |
| Intel |  |  |  |  |
| Sony |  |  |  |  |
| Samsung |  |  |  |  |
| Ericsson |  |  |  |  |
| CAICT |  |  |  |  |
| Qualcomm |  |  |  |  |

It should be noticed that if the mapping between preambles and PRUs is 1-to-1, then the functionality of preamble index is identical to DRMS index. While if the mapping between preambles and PRUs is multiple-to-1, then preamble index should be used to make it more UE-specific.

Therefore we have the following proposal:

Proposal 5:

- The initialization ID for msgA PUSCH scrambling is derived based on RA-RNTI and preamble index.

  - Waveform&Numerology

Companies’ views on the waveform and numerology are listed in Table 4.

For waveform, the remaining issue is whether to follow msg3 or separately configured in SI.

- FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

  - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

  - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

  - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

  - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

For numerology, a remaining issue is whether to support PRACH and PUSCH in the same slot for msgA transmission. And if supported, the numerology of msgA PUSCH should be down-selected from the following three options.

Table 4 Companies’ view on the waveform and numerology for msgA PUSCH

|  | Indication of waveform | PRACH and PUSCH in the same slot (and if supported which option is preferred) |
| --- | --- | --- |
| ZTE |  | Support for NR-U, Opt 3 |
| Huawei | follow msg3 or separately configured | Not support |
| vivo |  | Opt 1 |
| Spreadtrum |  | Opt 2 |
| OPPO | follow msg3 |  |
| LGE |  | Follow UL BWP (a timing gap is needed if different from preamble) |
| Nokia | separately configured | Postpone until NR-U channel access is complete |
| Intel |  | Support, Opt 1 |
| Panasonic | follow msg3 or separately configured |  |
| Samsung | follow msg3 | Not support |
| Sierra |  | Support, either Opt 1 or Opt 2 |
| Ericsson | separately configured | Separately configure numerology for msgA PUSCH |
| Qualcomm |  | Opt 2 |

Proposal 6:

- Clarify the slot in the previous agreement, “Support the PRACH and PUSCH for msgA transmission in different slots”, is defined by min{preamble SCS, UL BWP SCS}.

- The waveform indication of msgA PUSCH is up to RAN2 decision.

FL recommendation:

- FFS whether or not support PRACH and PUSCH in the same slot and the numerology in such case, until the NR-U channel access design is clearer.

  - Frequency hopping

Frequency hopping is supported in [7][8][11]. Base on evaluation result from [7], it can provide better BLER performance. And also in [8], it is considered for better PUSCH coverage.

Since the frequency hopping may have impact on the design of msgA PUSCH configuration.

FL recommendation:

- FFS whether or not support frequency hopping until the msgA PUSCH configuration design is clearer.

# Others

Please feel free to list any missing point in the following table.

| Company | View |
| --- | --- |

# Summary

The following proposals will be treated as the starting point for the discussion in RAN1#97.

Proposal 1:

- Both option 1 (separate configuration) and option 2 (relative location) for msgA PUSCH configuration are supported

- The following parameters are defined per msgA PUSCH configuration:

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location)

    - MCS/TBS

    - Number of TDMed POs: # of PUSCH slots, # of POs in each slot

    - Number of FDMed POs

      - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

    - Frequency domain resource allocation per PO

    - Time domain resource allocation per PO

    - Number of DMRS ports/sequences per PO

  - Parameters specific to option 1

    - Periodicity (msgA PUSCH configuration period)

      - Value range can be same as PRACH, i.e. {10, 20, 40, 80, 160}ms

    - Offset(s) or subframe number

    - Frequency starting point

  - Parameters specific to option 2

    - Single time offset respect to the start of each PRACH slot or each PRACH subframe (for 60 kHz slot in FR1/FR2)

    - Single frequency offset respect to the start of the first RO in frequency

- Multiple msgA PUSCH configurations is supported

  - FFS the maximum number of configurations

  - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

- FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH

- FFS validation rule of msgA PUSCH

- FFS whether or not support repetitions for msgA PUSCH

Proposal 2:

- Reuse the PRACH configuration tables defined in TS38.211 for the configuration of msgA PRACH

  - 2-step RACH and 4-step RACH share the same PRACH configuration index

- The msgA (PRACH and PUSCH) association period is defined as

  - max{PRACH configuration period, msgA PUSCH configuration period}

  - PO in front of RO association is not supported

- FFS SSB to PO mapping in association pattern period

Proposal 3a:

- Support both DMRS port and DMRS sequences for the definition of PRU

- Confirm the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit (PRU) are supported

  - Configurable number of preambles (including one or multiple) mapped to one PRU

Proposal 3b:

- gNB has the flexibility to configure either of the following two cases:

  - The definition of PUSCH resource unit includes both DMRS port and DMRS sequence, 1 to 1 mapping between preamble and PRU is supported only

    - Configurable number of DMRS sequences

  - PUSCH resource unit includes DMRS port only, multi-to-1 mapping is supported

    - Configurable number of preambles (including one or multiple) mapped to one PRU

Proposal 4:

- The PRACH preambles are mapped to PUSCH resource units (PRUs) within an msgA association period in the following order:

  - First, in increasing order of DMRS indexes within a single PUSCH occasion

    - FFS DMRS indexing, e.g. port first or sequence first

  - Second, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

  - Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot

  - Fourth, in increasing order of indexes for PUSCH slots

  - Redundant preambles/PRUs will not be mapped

Proposal 5:

The initialization ID for msgA PUSCH scrambling is derived based on RA-RNTI and preamble index.

Proposal 6:

- Clarify the slot in the previous agreement, “Support the PRACH and PUSCH for msgA transmission in different slots”, is defined by min{preamble SCS, UL BWP SCS}.

- The waveform indication of msgA PUSCH is up to RAN2 decision.

# Reference

- R1-1905989	Further discussions on the channel structure of msgA	ZTE

- R1-1906050	Discussion on channel structure of 2-step RACH	Huawei, HiSilicon

- R1-1906124	Discussion on channel structure for 2-step RACH	vivo

- R1-1906192	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1906366	Considerations on the channel structure on 2-step RACH	Spreadtrum Communications

- R1-1906429	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1906511	Discussion on Channel Structure for Two-step RACH	CMCC

- R1-1906580	On Channel Structure for 2-step RACH	OPPO

- R1-1906717	Discussion on Channel Structure for 2-step RACH	LG Electronics

- R1-1906746	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1906779	Discussion on channel structure for 2-step RACH	Intel Corporation

- R1-1906848	Considerations on Channel Structure for Two-Step RACH	Sony

- R1-1906858	On preamble and PUSCH mapping	InterDigital, Inc.

- R1-1906863	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1906905	Channel Structure for Two-Step RACH	Samsung

- R1-1907127	Channel Structure for Two-Step RACH Considerations	Sierra Wireless, S.A.

- R1-1907180	Channel Structure for Two-step RACH	Ericsson

- R1-1907196	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1907255	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1907446	Discussion on channel structure for two-step RACH	KDDI Corporation

# Appendix A Agreements in previous meetings

In RAN1 #96 and #96bis meetings, some agreements related to the channel structure were achieved as follows.

Agreements:

- PUSCH occasion for 2-step RACH is defined as

    - The time-frequency resource for payload transmissions

- Consider the following methods for PUSCH occasion of msgA transmission:

    - Opt 1: PUSCH occasions are separately configured from PRACH occasions

          - For one PUSCH occasion, it is derived based on:

            - Alt 1: reuse the resource allocation for NR configured grant in principle

            - Alt 2: other potential configurations (i.e., reuse semi-static SFI + BWP,  reuse PRACH RO, etc.)

          - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

    - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

          - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

          - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is single specification fixed value. Different preambles in different PRACH occasions can have different values.

          - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

          - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

          - Note: The time and frequency relation is not required to be the same alternative.

    - FFS detailed mapping between preamble and PUSCH resource + DMRS

Agreements:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt. 2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period

Agreements:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

    - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

    - Alt 1: ​follow the numerology configured for the UL BWP

        - FFS initial vs. active UL BWP

    - Alt 2:  same as msgA preamble numerology at least for some cases

        - i.e., when short preamble is used (L=139)

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

Agreements:

- The beam association rule between SSB and RACH occasion of 4-step RACH is to be used for 2-step RACH

    - FFS beam association for PUSCH

Working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

- Companies are strongly encouraged to perform additional evaluations/analysis

Agreed evaluation assumptions:

| Parameters | Values |
| --- | --- |
| Scenario | - For evaluation of schemes: 200m, UMi, 4 GHz. FFS: 500m, UMa, 4 GHz. - For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz; or 25km, RMa, 700 MHz. Other values can be reported if applicable. Note: this does not restrict preamble format selection. |
| Preamble format | Format 0/[A1]; [32, 64] preambles in each RO. Other preamble formats or number of preambles are not precluded Note: company report number of SSBs per RO |
| Waveform (data part) | CP-OFDM, or DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30/60kHz at 4 GHz, 120kHz at 30GHz |
| The number of PUSCH symbols & PUSCH mapping type | 14, Type A; [6], Type B as optional |
| 1) Total Number of PRBs for msgA PUSCH Or 2) number of PRBs per PUSCH occasion Note: either of them should be aligned for scheme comparison | [6, 12] Or [1,2,3] |
| PUSCH DMRS overhead | [2 or 3] DMRS symbols |
| Frequency hopping for msgA PUSCH | Company report, enabled/disabled |
| TBS | - 56 72 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or can be reported Power modelling for FFS (Note: before the FFS is resolved, companies to report the detailed modelling) FFS: interference from the adjacent PUSCH resource occasion, including how to model relative SINR, timing, etc. |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx. FFS: 2 Tx |
| gNB antenna configuration | 2Rx or 4Rx, 8Rx as optional |
| Propagation channel & UE velocity | TDL/CDL-A 30ns, or TDL/CDL-C 300ns, 3km/h or 30km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| Max number of HARQ transmission | 1 as starting point, other values are not precluded and company should report the details of HARQ |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | Realistic for both channel estimation and TO/FO estimation. Ideal can be considered for calibration, if needed. |
| Target BLER | [10%, 1%] for 1st transmission of msgA as starting points. |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

# Appendix B: Companies' view on configuration of MCS/TBS/resource size

| Company | View |
| --- | --- |
| ZTE | Proposal 2: The number of configurations for PO should be limited for RRC idle/inactive state Support identical resource size for the POs within a PO group Resource size, MCS, and payload size are semi-statically configured FFS different PO sizes within a PO group |
| Huawei, HiSilicon | Proposal 9: Support payload size of 56 and 72 bits for 2-step RACH MsgA, FFS other payload size. In order to let the gNB know the MCS level before decoding the MsgA PUSCH, a mapping between the supported MCS levels and the preamble or PRU could be pre-configured. For example, the preambles are divided into several groups, and each group is mapped to a certain MCS level. Then the gNB can determine the MCS level based on preamble detection before PUSCH decoding. Similarly, the mapping between the supported MCS levels and the DMRS ports of the MsgA PUSCH can also serve the purpose. The gNB can determine the PUSCH resource based on the preamble detection, and further determine the MCS value based on DMRS detection. Proposal 10: The number of supported MCS levels should be limited, FFS the mapping between MCS levels and preamble/DMRS ports. |
| vivo | Proposal 9: For different PUSCH occasions for 2-step RACH, time/frequency resources with the same or different size can be configured. The size of time/frequency resources is fixed for all PUSCH resource units in a given PUSCH occasion. Proposal 13: Support a single configurable MCS for a PUSCH occasion FFS whether and how to support a set of configurable MCSs for a PUSCH occasion. The payload size of PUSCH of msgA should at least support the content of msg3 in 4-step RACH, i.e. 56 bits and 72 bits. As discussed in last meeting, larger payload than the 56 bits or 72 bits can be considered, if small packet UP data transmission in msgA is supported. The payload size of PUSCH of msgA is dependent on the size of time/frequency resource, MCS and received antenna number. |
| CMCC | Proposal 1: The modulation scheme of msgA PUSCH is configured as one modulation scheme, such as QPSK, at least for the idle or inactive state, when the payload size of PUSCH takes 56 bits or 72bits. Proposal 2: Introduce a power offset when UE sending msgA PUSCH with the larger payload size such as 72bits compared with 56 bits. |
| OPPO | There is no UE’s channel states information for gNB to select MCS of PUSCH. If the system assume the MCS and the PUSCH resource based on the worse channel condition (i.e. of the UE at cell edge), for the UE with good channel condition, the transmission efficiency would be too low. Therefore, multiple MCS(s) can be considered to be support PUSCH in MsgA to match different UE’s channel condition. A UE can select the preamble and PUSCH resource based on its DL pathloss estimation. For example, if the RSRP is high, a PUSCH resource with high MCS and its corresponding preamble can be used. Otherwise, a PUSCH resource with lower MCS and its corresponding preamble can be selected. Proposal 9: Multiple MCSs for PUSCH in MsgA should be supported. Proposal 10: Very limited payload sizes (e.g., 56/72) are supported for RRC idle/inactive UEs. The resource allocation (including MCS and TBS) for RRC connected state UE can be configured with RRC signalling. |
| LG Electronics | Supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA etc Proposal 8: For PUSCH in msgA, use very limited number of MCS level (e.g. one or two MCS levels). E.g., only QPSK for CP-OFDM is applied for PUSCH in msgA, and two types of coding rate are used. Proposal 9: If multiple MCS levels are allowed for PUSCH transmission, multiple types of PUSCH resource are defined depending on MCS level. RAPID can be associated with MCS level. If multiple sets of DMRS frequency resource is allowed, it can be defined that each DMRS frequency resource is associated with MCS level. |
| Nokia, Nokia Shanghai Bell | Observation 8: Having a single 2SR PUSCH resource unit size for the MsgA data part with different payload size leads to non-optimal use of the PUSCH resources. Observation 12: Including assisting uplink control information, such as MCS, payload size, amount of PUSCH physical resources simplifies the gNB receiver complexity by avoiding the need to try multiple decode hypothesis. Proposal 12: The resource elements of the PUSCH Occasion are divided into control resource elements carrying the control information for data reception and data resource elements carrying the actual data information. |
| Intel Corporation | Time domain resource allocation of an MsgA PUSCH occasion includes slot offset, configuration period, starting symbol and length. For an MsgA PUSCH occasion, the following parameters are configured. An MsgA PUSCH occasion ID, time/frequency resource allocation, MCS, power control related parameters, and association with PRACH occasion and preamble. It may be desirable to employ repetition for MsgA PUSCH transmission in order to match coverage status of associated PRACH preamble. |
| Sony | Proposal 3: The msgA configuration in 2-Step RACH can be broadcast by the gNB to the UEs (e.g. via system information) or using a dedicated signaling (i.e. RRC signaling). Proposal 4: Support preconfigured MCS for msgA in 2-step RACH and further study whether it is statically configured (e.g. a fixed MCS) or UE can dynamically select from a limited set of MCS and signal via piggy back on msgA. |
| Panasonic Corporation | Proposal 3: In 2-step RACH, allow gNB to configure a set of TBSs, MCS, and time/frequency resource size for PUSCH in Msg.A. The UE may select a set of these parameters based on the configuration and buffer size. Proposal 4: The indication of UE selected parameter set is indicated by the selection of RACH preamble resource set or UCI transmitted together with PUSCH. |
| Sierra Wireless, S.A | Proposal 6: Multiple 2-step RACH configurations shall be supported. The configuration shall at least include: Coverage range thresholds (e.g. RSRP/RSRQ range) PUSCH modulation and coding PRACH format FFS: number of 2-Step RACH configurations |
| Ericsson | A set of resources containing multiple PUSCH occasions is defined (a ‘msgA PUSCH slot’), where: A msgA PUSCH slot occurs periodically and has a known length in symbols Each PUSCH occasion occupies a contiguous set of subcarriers and symbols Multiple PUSCH occasions in a msgA PUSCH slot can be in one OFDM symbol or in one subcarrier The PUSCH occasions can have different sizes, and a given PRB can contain PUSCH occasions with different sizes. |
| CAICT | Observation4: For the gNB, it shall be informed the factors like TBsize and MCS of a PUSCH configuration explicitly through UCI or implicitly for the PO detection in addition to the position of PO and the associated DMRS configuration Proposal4: The factors like TBszie and MCS can be configurable and are initial configured by the gNB through RRC signalling. And for different types of PUSCH configuration, it uses separate ROs or preambles to implicitly inform the gNB of the PUSCH configuration for signalling saving. |
| Qualcomm Incorporated | Observation 3 LLS results for msgA decoding with realistic channel estimation indicate that: MCL at a given BLER target can be used as the performance metric for PO size optimization; PO size optimization depends on the payload size and slot format; PO size of 3 RB and 1 slot (14 symbols, DMRS overhead 1/7) is good choice for 72 bits; PO size of 12 RB and 1 slot (14 symbols, DMRS overhead 1/7) is a good choice for 1000 bits. Proposal 7: For msgA transmission, the following designs should be supported: Low-order linear modulations such as QPSK and pi/2 BPSK should be specified for msgA PUSCH transmission. In particular, QPSK should be specified for CP-OFDM waveform; pi/2 BPSK and QPSK should be specified for DFT-s-OFDM waveform. A finite set of PO size can be specified, to support different payload size (in the range of a few bytes to a few hundred bytes) and different coverage requirements; PO bandwidth can be configured proportional to that of RO bandwidth; In FR1, PO bandwidth of {3, 6, 12} PRBs can be considered for a 14-symbol slot; Slot repetition should be supported. |
| KDDI Corporation | Proposal 2: MCS of msgA PUSCH is configured in common for all PUSCH resource units at least in a PUSCH occasion. |

# Appendix C: Companies' view on mapping between preambles and PRUs

| Company | View |
| --- | --- |
| ZTE | Proposal 3: Considering the following for the mapping ratio between PRACH preamble and PSUCH resource unit Opt 1: by explicit configuration, i.e. a parameter denotes how many preambles can be mapped to single PUSCH resource unit. Opt 2: implicit derived based on the total number of preambles in a PRACH slot and the total number of PUSCH resource units in a PUSCH group Proposal 4: The ordering of PSUCH resource unit in a PO group should be First, in increasing order of DMRS indexes within a single PUSCH occasion Second, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH group FFS the relationship between DMRS indexes and DMRS port/sequence Proposal 5：Conform the working assumption that at least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit. At least support DMRS port as part of the PUSCH resource unit. Further study the necessity of introducing multiple DMRS initialization sequences |
| Huawei, HiSilicon | Observation 6: One-to-one mapping between preamble and PUSCH is sufficient. There is no evidential benefits to support many-to-one mapping. Observation 7: The resource utilization of POs can be imbalanced if the number of preambles is not equal to or divisible by the number of PRUs. Proposal 1: PUSCH occasion groups can be defined for PUSCH resource configuration. Proposal 7: Mapping between PRACH and PUSCH can be defined between PRACH occasion group(s) and PUSCH occasion group(s). Proposal 8: The number of preambles mapped to each PO should be predefined or configured. |
| vivo | Proposal 3: Both of DMRS port and DMRS sequence are supported for the definition of PUSCH resource unit. It is configurable that a PUSCH resource unit is defined by DMRS port or DMRS port/DMRS sequence. From gNB perspective, one-to-one mapping between preamble and PUSCH resource unit can reduce detection effort for msgA. More analysis and evaluation can refer to section 3. One-to-one mapping between preambles and DMRS port + individual DMRS sequence achieves better performance with BLER target 10-2 compared to multiple-to-one mapping between preambles and DMRS port with the same DMRS sequence Proposal 6: Confirm working assumption that one-to-one mapping between PUSCH resource units and preambles.. |
| NTT DOCOMO | Proposal 4: The mapping order for MsgA preamble and MsgA PUSCH is following: First, in increasing order of code domain, i.e., MsgA PUSCH DMRS port/sequence or MsgA preamble index. Second, in increasing order of frequency domain resource Third, in increasing order of time domain resource Proposal 6: Confirm the following working assumption: - At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit. - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit In addition, one-to-multiple mapping is still FFS. One possible use case of one-to-multiple mapping is the scenario that only UE ID is transmitted by MsgA PUSCH and the collision of MsgA preamble does not matter. In that case, the preamble is used only for reference to associated MsgA PUSCH, and one-to-multiple mapping is useful in order to reduce the collision probability of MsgA PUSCH. Such use case should be further considered, and if supported, one-to-multiple mapping should be supported. |
| Spreadtrum Communications | Proposal5: One-to-one mapping could be considered as the starting point. |
| Fujitsu | Proposal 1: Confirm the working assumption on the support of one-to-one mapping between a preamble and a PUSCH resource unit. To save PUSCH overhead, it should be supported that different PUSCH resource units share the same PUSCH occasion but are associated with different DM-RS ports. Proposal 2: Multiple-to-one mapping could be considered only when intending to support a UE number larger than the number of PUSCH resource units. Whether to support it should be subject to the evaluation results including at least a case where the UE number is assumed to be larger than the number of PUSCH resource units. Proposal 3: When the number of preambles available for two-step RACH is limited, e.g. two-step RACH and four-step RACH share preambles in a PRACH occasion, one-to-multiple mapping could be considered to support a UE number larger than the number of preambles. Whether to support it should be subject to further evaluations on potential issues such as gNB blind decoding, TA estimation and so on. |
| CMCC | Within each pair of RO and PO, the association between preambles and PUSCH resource unit can be implicitly determined based the number of preambles configured in the RO and number of DMRS port/DMRS sequence configured in the PO. |
| OPPO | It is still FFS whether DMRS sequence shall be considered when defining a PUSCH resource unit. Since based on the orthogonal PUSCH DMRS ports, 12 PUSCHs could have been supported to be multiplexed in the same PUSCH time/frequency resource. It is noted that in typical channel condition, it would be challenging to support MU-MIMO multiplexing of 12 PUSCHs. Therefore, we don’t see the necessity to improve the PUSCH multiplexing capacity with DMRS scrambling initialization ID. Proposal 6: PUSCH resource unit is defined as the PUSCH occasion and DMRS port used for an msgA payload transmission. As discussed above, with Rel-15 PUSCH and its DMRS design, at most 12 PUSCHs could have been supported to be multiplexed in the same PUSCH time/frequency resource with orthogonal PUSCH DMRS ports. However, in some deployment scenarios such as small cell scenario, it may be impossible to support so many layers multiplexing due to LOS channel condition. Therefore, it would be beneficial to configure fewer DMRS ports than 12 for such scenario to match the channel condition. So, we propose to the DMRS ports for PUSCH in MsgA shall be configurable. Proposal 13: The PUSCH DMRS ports in MsgA shall be configurable. |
| LG Electronics | Even in a certain UL BWP, type 1 DMRS is used for msg3 transmission. Similarly, in 2-step RACH, only type 1 DMRS which is a mandatory feature should be applied for msgA PUSCH since RACH configuration for 2-step RACH procedure can be configured via BWP-UplinkCommon which could be applied for UEs. In this case, these seed valued can be configured via SIB1 and/or RACH-ConfigCommon, and values can be designated depending on RAPID. On the other hand, for the case of DFT-s-OFDM, low-PAPR sequence (a.k.a, Zadoff-chu sequence) is applied, and one sequence is assigned as DMRS sequence. Proposal 4: In case of CP-OFDM, maximum two different initial values are configured via higher layer signal (e.g., SIB1 and/or RACH-ConfigCommon). Sequence is designated depending on the RAPID. For case of DFT-s-OFDM, one root index of ZC-sequence is configured via higher layer signal. |
| Nokia, Nokia Shanghai Bell | With multiple-to-one mapping, the number of preambles mapped to the same PUSCH resource unit, P, can be a configurable value, with P=1 being a special case corresponding to one-to-one mapping of preambles to PUSCH resource units. To mitigate the interference between PUSCH transmissions corresponding to preambles mapped to the same PUSCH resource unit, spatial separation between the PUSCH transmission can be utilized, this minimizes the interference each PUSCH transmission causes to other PUSCH transmissions on the same resource. Observation 6: PUSCH transmissions mapped to the same PUSCH resource unit that correspond to different spatial domain filter cause less interference to each other. To illustrate the mapping with a simple example, consider 2 PRACH Occasions, and 4 PUSCH Occasions, as shown in Figure 6. Each PRACH Occasion has 64 preambles, and 2 SSBs are mapped to a PRACH Occasion. Each PUSCH Occasion has 8 PUSCH Resource units. In total there are 4 SSBs across the 2 ROs, each SSB has 32 preambles. The preambles of each SSB are divided into 4 groups with 8 preambles per group. Each group is mapped to a different PUSCH Occasion, where each preamble within a group is mapped to PUSCH resource unit. In this example, there are a total of 128 preambles, and 32 PUSCH resource units, each PUSCH resource unit as 4 preambles mapped to it associated with different SSBs. Figure 6: Mapping preambles within ROs and associated with SSBs to PUSCH Occasions. |
| Intel Corporation |  It should be noted that DMRS sequence generation mechanism for MsgA PUSCH should follow Rel-15. For CP-OFDM waveform, PN sequence is employed for the DMRS sequence generation for PUSCH transmission, which leads to quasi-orthogonal property if distinct initialization seeds are applied for the generation of DMRS sequence, e.g., based on preamble index. From reception perspective, this may result in channel estimation performance degradation, and thus overall decoding performance loss of MsgA when multiple users transmit the MsgA PUSCH in the same physical resource. Similar issue can be expected when DFT-s-OFDM waveform is configured for MsgA PUSCH transmission, where different root sequence for DMRS sequence may not be desirable in term of overall decoding performance. In our view, DMRS sequence should not be considered as a part of MsgA PUSCH resource. Hence, MsgA PUSCH resource unit is defined as the PUSCH occasion and DMRS AP. Proposal 7 PUSCH resource unit is defined as the PUSCH occasion and DMRS AP. The number of DMRS APs for MsgA PUSCH can be configured. When MsgA PRACH and PUSCH are transmitted in a slot and PUSCH is located within PRACH transmission bandwidth, for many to one mapping, even in case of DMRS AP collision, decent MsgA PUSCH decoding performance can be achieved if PRACH based channel estimation is employed. Proposal 1 Confirm the working assumption to support many to one mapping between PRACH preambles in each RO and associated PUSCH resource unit. |
| Sony | The availability and size of the dedicated PUSCH resources for 2-step RACH may be limited. In addition, all of the preambles for 2-step RACH may not be used depending on the number of UEs. Hence, it would be beneficial to support multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit. One-to-one mapping can also be supported in case the PUSCH resources can be provided by the gNB and/or a need to support collision free PUSCH transmission. Both options shall be supported and the gNB has the flexibility to configure both options. |
| InterDigital, Inc. | Proposal 1: DMRS port and sequence per PUSCH occasion are supported. Proposal 2: Multiplexing capacity of many-to-one mapping is evaluated using link level simulations. Availability of multiple PUSCH resources can be used in several ways. For example, it could reduce PUSCH collisions when multiple UEs select the same preamble. Alternatively, a UE can take advantage of utilizing more than one resource to increase the reliability of its transmission through repetition transmissions, e.g. using different uplink transmit beams. Proposal 3: One-to-multiple mapping is supported. |
| Panasonic Corporation | For the mapping between preambles in each RACH occasion and associated PUSCH resource unit, the design should consider the resource utilization efficiency and decoding complexity at gNB. The gNB will configure the RACH preamble resource configuration considering the resource utilization and collision probability. As the mapping between RACH preamble and the PUSCH resource unit, at least support one-to-one and multiple(N)-to-one was working assumption. For one-to-one mapping, a large amount of PUSCH resources needs to be reserved for the PUSCH in Msg.A. On the other hand, multiple(N)-to-one mapping would cause high collision probability of PUSCH depending on the N value. Such high collision case should be standardized in NOMA. Multiple(N)-to-one mapping with low collision probability which does not require NOMA such as advanced receiver could be considered. Proposal 2: Working assumption of supporting one-to-one and multiple-to-one mapping between preambles in each RACH occasion and associated PUSCH resource unit should be confirmed. |
| Samsung | While the different DMRS sequence will require gNB to reserve a group of scrambling ID, which currently is using Cell ID for scrambling ID before RRC connected. The benefit of using scrambling ID is that it might have larger capacity than that of using the DMRS port. However, we think only the DMRS port to be supported is enough for the following reasons. First, for UEs transmitting in two different PUSCH time/frequency resources, it won’t matter much if they are using the same DMRS sequence or not since gNB anyway are decode them separately. For a given PUSCH time/frequency resource, it’s also not desirable to multiplex too many UEs (PUSCHs) since the interference level will be too high and limit the overall decoding performance. Besides, the orthogonality between the DMRS sequence with different scrambling ID are not good (e.g., the Gold sequence for CP-OFDM case) thus will seriously jeopardize the channel estimation outcome. 12 different DMRS ports could already provide enough multiplexing capacity for one PUSCH occasion without needing much effort. Proposal 6: For the definition of a PUSCH resource unit, only DMRS port is supported. |
| Sierra Wireless, S.A | With multiple-to-one, the probably of collision of the PRACH would be lower than PUSCH, hence the gNB could detect PUSCH resource collisions based on the detected PRACH preambles. When the gNB is able to detect/decode the PRACH preamble but is unable to decode the PUSCH, it would be able to perform HARQ or 4-step fallback for the UE to resend the PUSCH data. This would have lower latency than a full retry and the PRACH transmissions would not have been wasted. Proposal 3: Confirm the working assumption to allow support for both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit. |
| Ericsson | One possible motivation for this is to save PUSCH resource overhead, since there can be 64 preambles in a RACH occasion, and reserving a PO for each preamble would require 64 PRBs, which is a substantial amount of overhead. However, the number of preambles could instead be set by the number of POs or PRUs, and only the subset of preambles could be used. |
| CAICT | A PUSCH resource unit could be defined to contain one PO and one DMRS port, where the DMRS sequence generation ID like scrambling ID can be set default (Alt1), or to contain one PO, one DMRS port and one DMRS sequence generation ID (Alt2). Obviously, Alt2 will have more PUSCH resource unit than Alt1 and enhance the channel estimation performance for the UEs on the same PO and DMRS port. The necessity for Alt2 would depend on the number of PRACH resource units associated with one PO. Here we refer one preamble on a RO as one PRACH resource unit. If the number of PRACH resource units associated with one PO is not greater than the number of DMRS ports, then it would be better to apply Alt1 to avoid selecting the same DMRS port. Considering that the number of POs depends on the gNB’s configuration and it might happen that the number of PRACH resource units associated with one PO is greater than the number of DMRS ports, then one to one mapping with Alt2 might perform better than many to one mapping with Alt1. When to select Alt2, if it happens that the number of PRACH resource units associated with one PO is not greater than the number of DMRS ports, a method is needed to avoid any two preambles on a RO mapping to the same DMRS port of one PO. In addition, the inter-cell interference might also have a negative effect on the performance of Alt2 since a UE from adjacent cell might use the same DMRS scrambling ID on the same PO. Observation2: The selection between Alt1 and Alt2 depends on whether the number of PRACH resource units associated with one PO is greater than the number of DMRS port or not. One to one mapping with Alt2 might perform better than many to one mapping with Alt1. And Alt2 might has the inter-cell interference problem. Proposal2: Down select between Alt1 and Alt2. When to select Alt2, a method is needed to avoid any two preambles on a RO mapping to the same DMRS port of one PO when the number of PRACH resource units associated with one PO is not greater than the number of DMRS ports. |
| Qualcomm Incorporated | Proposal 5: The mapping rules between msgA preamble and msgA PUSCH resource unit should be specified for two-step RACH. To support the mapping rules specified for msgA channel structure, NR Rel-15 design mechanisms for DMRS and PUSCH can be re-used, with extensions in the scrambling ID sets generation. |
| KDDI Corporation | For UEs to identify the relation between preamble IDs to PUSCH resource units, it is not efficient that the explicit table which shows the mappings from all preamble IDs to the associated PUSCH resource units one-by-one is broadcasted from gNB. A preferable alternative is to specify a mapping order by which UEs identify the relation between preamble IDs to PUSCH resource units, which leads to less broadcast information. Figure 1 shows an example of the mapping order. In this example, the number of preambles mapped to one PUSCH resource unit, the number of DMRS ports assigned for a PUSCH occasion, and the number of PUSCH occasion are 2, 4, and 8, respectively. It can be assumed that the numbers are configured by system information and the positions of PUSCH occasions are also configured with indices for a PRACH occasion. If the mapping order is specified, the UE can identify the mapping relation by using it. Figure 1 is one example such that pairs of two consecutive preamble IDs are mapped to PUSCH resource units first in increasing order of DMRS ports in a PUSCH occasion, and then in increasing order of indices for the PUSCH occasions. Figure 1: An example of mapping order form preamble IDs to PUSCH resource units. Proposal 1: Mapping order to identify PUSCH resource units from preamble IDs is specified. |

# Appendix D: Simulation about payload size

ZTE [R1-1905991] (a) 1 PRB (b) 3 PRBs (c) 6 PRBs Figure 1 BLER vs. SNR for 200m, UMi, 4 GHz (a) 1 PRB (b) 3 PRBs (c) 6 PRBs Figure 2 BLER vs. SNR for 500m, UMa, 4 GHz (a) 1 PRB (b) 6 PRBs (c) 12 PRBs Figure 3 BLER vs. SNR for 1732m, RMa, 700 MHz Table 1 Link budget calculation with 4G+200m Format A1 10bytes; 40bytes; 80bytes; 120bytes; 160bytes. Transmitter. (1) Tx Power (dBm); 23; 23; 23; 23; 23. Receiver. (2) Thermal noise density (dBm/Hz); -174; -174; -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5; 5; 5. (4) Interference margin (dB); 0; 0; 0; 0; 0. (5) Occupied channel bandwidth (Hz); 2160000; 2160000; 2160000; 2160000; 2160000. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -105.7; -105.7; -105.7; -105.7; -105.7. (7) Required SINR (dB) (Target BLER=0.1); -7.2; -4.3; -0.8; 4.1; 10.6. (8) Receiver sensitivity = (6)+(7) (dBm); -112.9; -110; -106.5; -101.6; -95.1. (9) Receiver processing gain; 0; 0; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 135.9; 133; 129.5; 124.7; 118.1. Figure 4 Coupling loss distribution in different scenarios Maximum supported TBsize - 4GHz + 200m Figure 5 MCL vs TBsize of scenario 4GHz + 200m It can be found that more than 160 bytes can be supported in this scenario. - 4GHz + 500m Figure 6 MCL vs TBsize of scenario 4GHz + 500m It can be found that more than 11 bytes can be supported in this scenario. - 700MHz + 1732m Figure 7 MCL vs TBsize of scenario 700MHz+1732m It can be found that more than 29 bytes can be supported in this scenario. Observation 1: With appropriate settings of resource size and antenna number, the minimum payload size same as msg3 (i.e. 56/72 bits) can be supported in different scenarios (cell size with 200, 500, 1732m). Larger payload sizes or cells sizes can be supported by allocating more resources or adding receiving antennas, e.g. for 200m ISD, up to 160bits can be supported with 4Rx and 6PRB allocated bandwidth.

CMCC [R1-1906511] Figure 1 BLER performance of PUSCH transmission Proposal 1: The modulation scheme of msgA PUSCH is configured as one modulation scheme, such as QPSK, at least for the idle or inactive state, when the payload size of PUSCH takes 56 bits or 72bits. Proposal 2: Introduce a power offset when UE sending msgA PUSCH with the larger payload size such as 72bits compared with 56 bits. Figure 3 Illustration of frequency hopping for msgA PUSCH Figure 4 Comparison of BLER between frequency hopping disabled and enabled Proposal 4: Frequency hopping is configurable for transmission of msgA PUSCH.

Qualcom [R1-1907255] Figure 7: Resource Size Considerations for Different Payload Size Observation 2: Two-step RACH can operate in any RRC state. Different payload size, waveform and MCS can be specified for different RRC states and mapped to different POs. Proposal 6: For msgA transmission, the following designs should be supported: - a minimum payload size of 7 or 9 bytes can be supported by DFT-s-OFDM waveform and low MCS; payload sizes up to a few hundred bytes can be supported by higher MCS; - multiple POs can be configured per PO configuration period, to accommodate different MCS, payload size and waveform; - multiple two-step RACH UEs can share the same PO, if their msgA transmissions use similar payload size/MCS/waveform; - resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain; - the configurations and resource allocations for RO and PO need to be balanced to achieve a good trade-off in resource utilization efficiency and collision probability reduction. - the mapping rules between msgA preamble sequence and msgA PUSCH resource unit can be indicated by SI or RRC signalling. Figure 10: msgA BLER performance for TBS=72 bits with 1, 2 UEs Table 4: Parameters for MCL Computation (1) UE Tx Power (dBm); 23. (2) Thermal noise density (dBm/Hz); -174. (3) gNB Receiver noise figure (dB); 5. (4) Interference margin (dB); 0. (5) Occupied channel bandwidth (Hz). (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm). (7) required SINR (dB) per RxAnt (dB). (8) Receiver sensitivity = (6)+(7) (dBm). (9) Receiver processing gain. (10) MCL = (1)-(8)+(9) (dB). Table 5: MCL for 72 bits msgA Number of PRB per PO; 1; 2; 3; 4; 6; 12. MCL at 10% Target BLER (dB); 142.35; 142.54; 142.58; 142.53; 142.47; 142.16. Figure 11: msgA BLER performance for TBS=72 bits with CP-OFDM and DFT-s-OFDM Figure 13: msgA BLER performance for TBS=1000 bits with 1, 2 UEs Figure 14: msgA BLER performance for TBS=1000bits with 16 QAM modulation Observation 3 LLS results for msgA decoding with realistic channel estimation indicate that: - MCL at a given BLER target can be used as the performance metric for PO size optimization; - PO size optimization depends on the payload size and slot format; - PO size of 3 RB and 1 slot (14 symbols, DMRS overhead 1/7) is good choice for 72 bits; - PO size of 12 RB and 1 slot (14 symbols, DMRS overhead 1/7) is a good choice for 1000 bits. - DFT-s-OFDM waveform should be supported for small payload transmission, especially for UE experiencing large pathloss or in RRC_IDLE/RRC_INACTIVE state; - For both small and large payload size, non-orthogonal UE multiplexing in both DMRS and PUSCH does not cause performance degradation, if the number of RX antennas is greater than the number of UEs; - Higher order modulation does not show performance gain for large payload size. Based on the observation, we propose the following for msgA transmission. Proposal 7: For msgA transmission, the following designs should be supported: - Low-order linear modulations such as QPSK and pi/2 BPSK should be specified for msgA PUSCH transmission. In particular, - QPSK should be specified for CP-OFDM waveform; - pi/2 BPSK and QPSK should be specified for DFT-s-OFDM waveform. - A finite set of PO size can be specified, to support different payload size (in the range of a few bytes to a few hundred bytes) and different coverage requirements; - PO bandwidth can be configured proportional to that of RO bandwidth; - In FR1, PO bandwidth of {3, 6, 12} PRBs can be considered for a 14-symbol slot; - Slot repetition should be supported. - To enlarge the pool size of DMRS sequences without increasing its overhead, NR Rel-15 DMRS signal generation mechanism can be reused with extended scrambling ID sets configuration.

Vivo [R1-1906126] (1) Power in channel bandwidth (dbm); 23dbm is used in typical UE and do not consider CA scenarios.. (2) Thermal noise density (dbm/Hz); -174. (3) Receiver Noise Figure (dB); NF is related to the front end Low Noise Amplifier. A conservative NF=5 for gNB.. (4) Interference margin; Consider an average interference margin in network =10dB. (5) Occupied channel bandwidth (Hz); SCS*12*N_RB. (6) Required SNR (dB); Derive from LLS. (7) MCL; Calculate: (1)-{(2)+(3)+(4)+10*log10((5))}-(6). Figure 3: BLER performance with different #PRBs and #OFDM symbols TBS=72; 2 PRBs; 3 PRBs; 4 PRBs; 6 PRBs; 12 PRBs. PUSCH target BLER=1%; Threshold: SNR; MCL; Threshold: SNR; MCL; Threshold: SNR; MCL; Threshold: SNR; MCL; Threshold: SNR; MCL. Rx: 2; 6-OS; 7.96; 115.5; 5.18; 116.5; 3.07; 117.3; 0.46; 118.2; -3.54; 119.2. 14-OS; 2.57; 120.9; 0.37; 121.3; -1.26; 121.7; -3.31; 122.0; -6.97; 122.6. Rx: 4; 6-OS; 1.8; 121.6; -0.52; 122.2; -2.10; 122.5; -4.06; 122.7; -7.42; 123.1. 14-OS; -2.92; 126.3; -4.59; 126.3; -5.84; 126.3; -7.59; 126.2; -10.56; 126.2. Figure 4: MCL vs. different PRB sizes Observation 3: From MCL perspective, larger PRB size can achieve better performance than smaller PRB size, when assuming the same number of OFDM symbols. Observation 4: From MCL perspective, 14-OS can achieve better performance than 6-OS, with the same PRB size.

Huawei [R1-1906604] (a). Format 0, SCS=1.25 kHz; (b). Format A1, SCS=30 kHz. (c). Format A2, SCS=30 kHz; (d). Format A3, SCS=30 kHz. Figure 1. Miss-detection rate and false-alarm rate of preamble detection Table 1. Minimum SNR values to meet the requirements of Pm and Pf Preamble format; (Pm,Pf); Minimum SNR value (dB). 0; (1%, 1%); -13.2. (1%, 0.1%); -12.0. A1; (1%, 1%); -6.7. (1%, 0.1%); -4.5. A2; (1%, 1%); -9.7. (1%, 0.1%); -7.5. A3; (1%, 1%); -11.5. (1%, 0.1%); -9.1. Table 2. Link budget calculation of different preamble formats Preamble Format; 0; A1; A2; A3. (1) Tx Power (dBm); 23; 23; 23; 23. (2) Thermal noise density (dBm/Hz); -174; -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5; 5. (4) Interference margin (dB); 0; 0; 0; 0. (5) Occupied channel bandwidth (Hz); 1.04875e6; 4.17e6; 4.17e6; 4.17e6. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -108.8; -102.8; -102.8; -102.8. (7) Required SINR (dB) (Pm=1%, Pf=0.1%); -12.0; -4.5; -7.5; -9.1. (8) Receiver sensitivity = (6)+(7) (dBm); -120.8; -107.3; -110.3; -111.9. (9) Receiver processing gain; 0; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 143.8; 130.3; 133.3; 134.9. (a). Performance of TA estimation for ISD=200 m; (b). Performance of FO estimation for ISD=200 m. (c). Performance of TA estimation for ISD=1732 m; (d). Performance of FO estimation for ISD=1732 m. Figure 2. Estimation error of frequency offset for different preamble formats and ISDs Observation 1: The estimation of timing offset can be comparably accurate irrespective ISD. Observation 2: The estimation error of frequency offset is higher than or close to the average frequency offset for minimum SNR values meeting the requirements of miss-detection and false-alarm. Table 3. Simulation cases for payload size Case; # of PRBs; Modulation; TBS. 0; 1, 2, 3; QPSK; 72. 1; 3, 6, 12; QPSK; 200. 2; 3, 6, 12; QPSK; 400. 3; 6, 12; QPSK; 1000. ### Single UE case (a). TBS=72 bits; (b). TBS=200 bits. (c). TBS=400 bits; (d). TBS=1000 bits. Figure 3. BLER performance of different cases for single UE case Table 4. Link budget calculation of different payload sizes Case; 0; 1; 2; 4. TBS; 72; 200; 400; 1000. SCS (kHz); 30; 30; 30; 30. # of PRBs; 2; 12; 12; 12. (1) Tx Power (dBm); 23; 23; 23; 23. (2) Thermal noise density (dBm/Hz); -174; -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5; 5. (4) Interference margin (dB); 0; 0; 0; 0. (5) Occupied channel bandwidth (Hz); 7.2e5; 3.24e6; 3.24e6; 3.24e6. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -110.4; -103.9; -103.9; -103.9. (7) Required SINR (dB); 2.2; -2.3; 0.6; 5.4. (8) Receiver sensitivity = (6)+(7) (dBm); -108.2; -106.2; -103.3; -98.5. (9) Receiver processing gain; 0; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 131.2; 129.2; 126.3; 121.5. Observation 3: When payload size is 72 bits, the MCL is close to that of short preambles (format A1, A2, A3), and more than 10dB worse than that of long preamble (format 0). Observation 4: When payload size is 1000 bits, the MCL is more than 10dB worse than that of short preambles (format A1, A2, A3), and more than 23dB worse than that of long preamble (format 0), which potentially require more repetitions for payload transmission thus likely larger latency compared to Rel-15 4-step RACH. Observation 5: The reserved resource of large payload size (200, 400, 1000 bits) is much larger than that of 72 bit payload size. ### Multiple UEs case (a). TBS=72bits, 3PRB, SCS=30 kHz; (b). TBS=1000bits, 12PRB, SCS=30 kHz. Figure 4. BLER performance for multiple UE cases Observation 6: With small payload size, more UEs can be multiplexed in the same PO, and higher resource utilization can be achieved.

Nokia [R1-1906748] Figure 6: Average latency in slots for 4-step RACH and 2-step RACH. For 2-step RACH M is the number of PUSCH occasions associated with a PRACH occasion. Observation 1: The availability of multiple antenna ports increases the 2-step RACH performance. Proposal 1: The use of multiple antenna ports in the PUSCH resources for MsgA should be considered in 2-step RACH. Figure 7: PUSCH resource reservation and usage for four-step RACH and 2-step RACH. For 2-step RACH M is the number of PUSCH occasions associated with a PRACH occasion. Observation 2: The benefit of reduced latency with 2-step RACH is accompanied by an increase in PUSCH resource reservation and usage.

Nokia [R1-1906749] Figure 1. Miss-detection rate of preamble detection for false alarm rate 1%: (a) Format A1, 30 kHz (b) Format 0, 1.25KHz Figure 2. False alarm rate. Format A1, 30kHz Figure 3. Standard deviation of TA. Format A1, 30kHz Observation 1. Increasing the number of transmitting UEs for a fixed false alarm rate leads to an increase of the miss- detection rate of preamble detection. Table 1. Link budget calculation of preamble formats A1 and 0 Preamble A1; Preamble 0. (1) Tx Power (dBm); 23; 23. (2) Thermal noise density (dBm/Hz); -174; -174. (3) eNB receiver noise figure (dB); 5; 5. (4) Interference margin (dB); 0; 0. (5) Occupied channel bandwidth (Hz); 4.32e6; 1.08e6. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -102.79; -108.66. (7) Required SINR (dB); -5.5; -7.1. (8) Receiver sensitivity = (6)+(7) (dBm); -108.29; -115.79. (9) Receiver processing gain; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 131.29; 138.79. Figure 4. BLER performance for single and multiple UE cases for MCS index 4 Figure 5. BLER performance for single and multiple UE cases for MCS index 2 Observation 2. As the number of users multiplexed in a PUSCH occasion increases, the BLER performance degrades significantly. Observation 3. Increasing the number of DMRS ports/sequences has diminishing returns as the BLER performance with multiple simultaneously transmitted UEs degrades significantly. Table 2. Link budget calculation of MsgA PUSCH MsgA PUSCH 1 PRB; MsgA PUSCH 2 PRB; MsgA PUSCH 6 PRB. TBS; 80; 168; 528. (1) Tx Power (dBm); 23; 23; 23. (2) Thermal noise density (dBm/Hz); -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5. (4) Interference margin (dB); 0; 0; 0. (5) Occupied channel bandwidth (Hz); 0.36e6; 0.72e6; 2.16e6. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -113.43; -110.42; -105.65. (7) Required SINR (dB); 2.2; 0.22; -0.56. (8) Receiver sensitivity = (6)+(7) (dBm); -111.23; -110.20; -106.21. (9) Receiver processing gain; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 134.23; 133.20; 129.21. Observation 4. The MCL for low payload sizes in case of 1,2 or 6 PRBs transmissions is closer to the MCL of short preamble format (A1) than that of long preamble format (0).

Intel [R1-1906781] MsgA PRACH. MsgA PUSCH. Figure 1. Required SNR for MsgA PRACH and PUSCH Table 2. MCL analysis for MsgA PRACH and PUSCH Transmitter; MsgA PRACH (Format 0); MsgA PRACH (Format A1); MsgA PUSCH (56 bits); MsgA PUSCH (480 bits); MsgA PUSCH (1032 bits). (1) Tx power (dBm); 23 dBm; 23 dBm; 23 dBm; 23 dBm; 23 dBm. Receiver. (2) Thermal noise density (dBm/Hz); -174 dBm/Hz. (3) Receiver noise figure (dB); 5 dB. (4) Interference margin (dB); 0 dB. (5) Occupied channel bandwidth (Hz); 1.08MHz; 2.08MHz; 0.36MHz; 2.16MHz; 7.2MHz. (6) Effective noise power = (2) + (3) + (4) + 10log((5)) (dBm); -108.8dBm; -105.9dBm; -113.4dBm; -105.7dBm; -100.4dBm. (7) Required SINR (dB) (1% miss detection for PRACH, 10% BLER for PUSCH, 2 Rx antennas, TDL-A 30ns); -7.9 dB; -3.1 dB; 1.5dB; 1.8dB; 0dB. (8) Receiver sensitivity = (6) + (7) (dBm); -116.7dBm; -109dBm; -111.9dBm; -103.9dB; -100.4dBm. (9) Rx processing gain; 0; 0; 0; 0; 0. (10) MCL = (1) (8) + (9) (dB); 139.7dBm; 131.9dBm; 134.9dBm; 126.9dBm; 123.4 dBm. Table 3. MCL difference between MsgA PRACH and PUSCH TDL-A 30ns; 2 Rx antennas; 4 Rx antennas. Format 0; Format A1; Format 0; Format A1. PUSCH, 56 bits; 5 dB; -3 dB; 6.5 dB; -3 dB. PUSCH, 480 bits; 13 dB; 5 dB; 14.5 dB; 6 dB. PUSCH, 1032 bits; 17 dB; 9 dB; 18.5 dB; 10 dB. TDL-C 300ns; 2 Rx antennas; 4 Rx antennas. Format 0; Format A1; Format 0; Format A1. PUSCH, 56 bits; 6 dB; -1.5 dB; 7 dB; -1 dB. PUSCH, 480 bits; 14 dB; 6.5 dB; 14.5 dB; 6 dB. PUSCH, 1032 bits; 16 dB; 8.5 dB; 17.5 dB; 9 dB. Observation 1 - Relatively large range of MCL difference between MsgA PUSCH and PRACH is observed, which depends on payload size carried by MsgA PUSCH, associated PRACH format, MCS, allocated resource and subcarrier spacing. - For short PRACH format, when same subcarrier spacing is employed for the transmission of PRACH and PUSCH in MsgA, the coverage difference between PRACH and PUSCH can be relatively small. This may indicate that a large payload size, e.g., > 480 bits, can be carried by MsgA PUSCH for cell center UEs with good channel condition. - For certain combinations of MsgA PRACH format and PUSCH, appropriate power offset may be applied to match overage status in MsgA.

Erricson [R1-1907182] Table 2: System level parameters for coverage evaluations Parameters; Values. Layout; Single layer - Macro layer: Hex. Grid, 57 cells. Inter-BS distance; 200m. Carrier frequency; 4 GHz. Channel model; UMi in TR 38.901. UE Tx power; Max 23 dBm. BS antenna configurations; 2 ports: (M, N, P, Mg, Ng) = (10, [1,2,4], 2, 1, 1), +-45 Polarization dH = dV = 0.5λ;. BS antenna downtilt; 102. BS antenna height; 25m. BS antenna element gain + connector loss; 8 dBi, 0dB cable loss. BS receiver noise figure; 5dB. UE antenna configuration; 1. UE antenna height; Follow the modelling of TR 38.901. UE antenna gain; 0dBi. UE distribution; Follow the modelling of TR 38.901. UE attachment; Refer to 36.873. Proposal: - 2-step RACH coverage evaluations use the parameters and values in Table 2 for the 200m ISD UMi scenario

# Appendix E: Simulation about mapping design

ZTE [R1-1905991] For total 6 PRBs, PUSCH resource can be equally divided into 2, 3 and 6 groups, each UE can use 3, 2 and 1 PRBs respectively, denoted as scheme 1, 2, 3; For total 12 PRBs, PUSCH resource can be equally divided into 4, 6 and 12 groups, each UE can use 3, 2 and 1 PRBs respectively, denoted as scheme 4, 5, 6. For the 6 mapping types above, scheme 1 and 2 with total 6 PRBs has a DMRS collision probability of 1/24 and 1/36 respectively. Scheme 4 with total 12 PRBs has a DMRS collision probability of 1/48. The other 3 mapping types have no DMRS collision probability. Note: X label is normalized SNR in order to ensure that each scheme has the same transmit power. (a). 1T2R (b). 1T4R Figure 8 BLER vs. SNR for 2UEs with Preamble Format 0 . (a). 1T2R (b). 1T4R Figure 9 BLER vs. SNR for 2UEs with Preamble Format A1 Observation 2: With the aligned transmit power and total reserved resource overhead, the performance of different mapping schemes is similar. (a)1T2R (b) 1T4R Figure 10 Comparison between different DMRS sequences with Preamble Format 0 (a). 1T2R - . 1T4R Figure 11 Comparison between different DMRS sequences with Preamble Format A1 Observation 3: Performance of multiple DMRS sequences without collision is similar with that of 1 DMRS sequence with potential collision.

Qualcom [R1-1907255] Figure 12: msgA BLER Performance for TBS 72 bits with Orthogonal and Non-Orthogonal DMRS Observation 3 LLS results for msgA decoding with realistic channel estimation indicate that: - For both small and large payload size, non-orthogonal UE multiplexing in both DMRS and PUSCH does not cause performance degradation, if the number of RX antennas is greater than the number of UEs;

Vivo [R1-1906126] Four cases are considered in the simulation. - Case 1: 64 preambles of a RO are mapped to 1 PO with 6 PRBs, wherein 8 preambles are mapped to 1 DMRS port, as shown in the figure of case 1. In this case, single scrambling ID is assumed for a DMRS port. - Case 2: 64 preambles of a RO are mapped to 2 POs each of which contains 3 PRBs. Preamble index 0~31 maps to 1st PO and preamble index 31~63 maps to 2nd PO, wherein 4 preambles are mapped to 1 DMRS port, as shown in the figure of case 2. In this case, single scrambling ID is assumed for a DMRS port. Compared to case 1, the code-rate and also the transmit power per RE are doubled in case 2 for fair comparion. - Case 3: 64 preambles of a RO are mapped to 1 PO with 6 PRBs, wherein 8 preambles are mapped to 1 DMRS port. For each DMRS port, 8 different scrambling IDs are assumed for DMRS sequence generation, as shown in the figure of case 3. In such case, there is one-to-one mapping between preambles and DMRS port + DMRS sequence, which are pseudo-orthogonal and have lower correlation. - Case 4: 64 preambles of a RO are mapped to 2 POs each of which contains 3 PRBs. Preamble index 0~31 maps to 1st PO and preamble index 31~63 maps to 2nd PO, wherein 4 preambles are mapped to 1 DMRS port. For each DMRS port, 4 different scrambling IDs are assumed for DMRS sequence generation, as shown in the figure of case 4. In such case, there is one-to-one mapping between preambles and DMRS port + DMRS sequence. Case 1: 1 PO with 6 PRBs per PO, 8 DMRS ports with single scrambling ID per DMRS port; Case 1: 2 POs with 3 PRBs per PO, 8 DMRS ports with single scrambling ID per DMRS port. Case 3: 1 PO with 6 PRBs per PO, 8 DMRS ports with 8 scrambling ID per DMRS port; Case 4: 2 POs with 3 PRBs per PO, 8 DMRS ports with 4 scrambling ID per DMRS port. Figure 1: Mapping relationship between preambles and PUSCH resource units in simulation In the evaluation two UEs are assumed in every slots, and UE randomly selects a preamble index and the corresponding DMRS port and scrambling ID according to mapping relationship as the above diagram. Figure 2: BLER performance assuming two UEs transmitting simultaneously Observation 1: One-to-one mapping between preambles and DMRS port + individual DMRS sequence achieves better performance with BLER target 10-2 compared to multiple-to-one mapping between preambles and DMRS port with the same DMRS sequence. Observation 2: In case of one-to-one mapping, case 3 with PO of 6 PRBs/PO and 8 scrambling IDs per DMRS port has better performance than that of case 4 with 2 POs of 3 PRBs/PO and 4 scrambling IDs per DMRS port.

Huawei [R1-1906605] Observation 2: With more DMRS sequences, the collision probability of PUSCH resource units can be reduced, and the decoding performance of PUSCH can be improved. Proposal 2: Multiple DMRS sequences in one PUSCH occasion should be supported. (a). TDLA-30ns, 1T2R, 2 preambles, 72 bits; (b). TDLC-300ns, 1T2R, 2 preambles, 72 bits. (c). TDLA-30ns, 1T2R, 2 preambles, 200 bits; (b). TDLC-300ns, 1T2R, 2 preambles, 200 bits. Figure 4. PUSCH decoding performance with and without PRU collision Observation 3: In the case that PRU collision may happen, the PUSCH decoding performance of 1-to-1 mapping is much better than that of the multi-to-one mapping. Proposal 3: The necessity to support multiple-to-one mapping should be further verified. (a). TDLA-30ns, 1T2R, 4 preambles, 72 bits; (b). TDLC-300ns, 1T2R, 4 preambles, 72 bits. (c). TDLA-30ns, 1T2R, 4 preambles, 200 bits; (d). TDLC-300ns, 1T2R, 4 preambles, 200 bits. Figure 5. PUSCH decoding performance with 4 random active preambles (a). TDLA-30ns, 1T2R, 2 preambles, 72 bits; (b). TDLC-300ns, 1T2R, 2 preambles, 72 bits. (c). TDLA-30ns, 1T2R, 2 preambles, 200 bits; (d). TDLC-300ns, 1T2R, 2 preambles, 200 bits. Figure 6. PUSCH decoding performance with 2 random active preambles

Nokia [R1-1906748] Figure 8: Average latency in slots with M PUSCH resource units (M=4, 8 or 16). P is the number of preambles mapped to one PUSCH resource unit. The number of preambles is M*P. Proposal 2: Support multiple-to-one mapping of preambles to PUSCH resource units.

Intel [R1-1906781] Figure 2. Link level simulation results for one to one and many to one mapping between MsgA PRACH preamble and PUSCH resource unit Observation 2 - When MsgA PRACH and PUSCH are transmitted in a slot and PUSCH is located within PRACH transmission bandwidth, for many to one mapping, even in case of DMRS AP collision, decent MsgA PUSCH decoding performance can be achieved if PRACH based channel estimation is employed. Proposal 1 - Confirm the working assumption to support many to one mapping between PRACH preambles in each RO and associated PUSCH resource unit.

Erricson [R1-1907185] Table 1: Coverage of 72 bits msgA vs. outage and BLER (1) Tx Power (dBm); 23. (2) Receiver thermal noise density (dBm/Hz); -174. (3) eNB receiver noise figure (dB); 5. (4) Interference margin (dB); 0. (5) Occupied channel bandwidth (Hz); 30kHz*#PRBs. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm). (7) Required SINR (dB). (8) Receiver sensitivity = (6)+(7) (dBm). (9) Receiver processing gain. (10) MCL = (1)-(8)+(9) (dB). Figure 1: Maximum coupling loss vs. #PRBs for 2 colliding UEs at 10% PUSCH BLER with different DMRS antenna ports, scrambling initialization and power offset Figure 2: Maximum coupling loss vs. maximum delay for 2 colliding UEs with different DMRS antenna ports and scrambling initialization Observations: - Different scrambling initializations have some potential as a means to increase the number of distinct DMRS for msgA PUSCH. - When two UEs collide in a PUSCH resource occasion with 4 gNB Rx antennas and 72 bit payloads the performance of reception using different DMRS scrambling initialization degrades somewhat relative to different DMRS antenna ports. - When UEs arrive at the same time, and with equal power, the loss is about 1.0 and 2.4 dB for 1% and 10% BLER, respectively. - msgA PUSCH reception does not appear to be too sensitive to timing offsets. For up to 2 CPs maximum delay: - at 10% BLER, the difference in maximum coupling loss is about 0.5 dB - at 1% BLER, the difference is about 0.5 to 1 dB depending on the number of PRBs Proposal: - Further investigate the relative performance of msgA PUSCH using DMRS with different scrambling initializations and using different DMRS antenna ports, e.g. considering relative path loss.

Huawei [R1-1907541] (a). N=2; (a). N=4. Figure 1. Collision probability for multiple preamble groups Observation 3: For the case with multiple preamble groups, the collision probability increases if the traffic load with respect to the available PRACH resource is not matched. Observation 4: The number of UEs performing contention-based 2-step RACH on the same RO needs to be small to avoid large collision probability and error floor in BLER performance.