3GPP TSG RAN WG1 meeting #97	 R1-1907710

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.5

Source: 	Samsung

Title: 	On strongest coefficient indicator

Document for:	Discussion and Decision

# Introduction

The following agreement was made in RAN1#96bis [1].

Agreement For RI=1, strongest coefficient indicator (SCI) is a log2KNZ-bit indicator. For RI>1, SCI design will be chosen from the following alternatives in RAN1#97 (Reno): - Alt3.1 (applicable to Alt1.2): Per-layer SCI, where SCIi is a –bit indicator (i=0, 1, …, (RI-1)) - Alt3.2 (applicable to Alt1.1): Per-layer SCI, where SCIi is a log2mini=0RI-1KNZ,i,2LMi–bit indicator - Alt3.3: Per-layer SCI, where SCIi is a log22K0–bit or log2K0 indicator (i=0, 1, …, (RI-1)) - Alt3.4: Per-layer SCI, where SCIi is a log22L–bit (i=0, 1, …, (RI-1))

This contribution provides simulation results and analysis to compare Alt3.3 and 3.4 in support of proposals made in the companion contribution [2].

# Overhead analysis for Alt3.3 and Alt3.4

First, to compare Alt3.3 and 3.4, we perform the following analysis of the absolute overhead saving (Alt 3.3 – Alt 3.4), and the normalized saving (overhead saving normalized by the total payload) for the following parameters: L=4, NSB={3,7,10,19}, p=14,12, and β=14,12,34. The overhead saving plots are shown in Figure 1. We can observe the following:

- For small number of SBs (e.g. 3, 7), there could be loss with Alt3.4

- For large number of SBs (e.g. 13,19), there is some saving (within 0-2% of the total payload)

- The maximum absolute saving is as follows.

  - For p=14, the saving is 4, 6, and 8 bits for rank 2, 3, 4, respectively.

  - For p=12, the saving is 6, 9, and 12 bits for rank 2, 3, 4, respectively.

Observation 1:

- Alt3.4 can be worse in overhead than Alt3.3 if NSB is small

- The overhead saving with Alt3.4 is small (0-2% of the total payload) only if NSB is large

Figure 1: overhead analysis (Alt3.3 – Alt3.4)

# Simulation results for strongest coefficient indicator

Next, we provide simulation results comparing Alt3.3 and 3.4. For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix. As reference scheme, for dynamic rank 1-2, Rel. 15 Type II, and for dynamic rank 1-4, Rel. 15 Type II for rank 1-2 and Rel. 15 Type I for rank 3-4 are considered in this evaluation.

The performance-overhead trade-offs of Alt3.3 and Alt3.4 are compared. The results are provided in Figure 2 (for dynamic rank 1-2), and in Figure 3 (for dynamic rank 1-4) for the following parameters.

- Spatial compression: L = 4

- Frequency compression: M = 7 (i.e., p=12)

- Rank 3-4: Alt3C with y0,v0=12,14

- β∈{14,12,34}

We can observe the following.

Observation 2: There is no noticeable difference in performance-overhead tradeoffs achieved by Alt3.3 and Alt3.4

Figure 2: MU, dynamic rank 1-2

Figure 3: SU, dynamic rank 1-4

# Spec impact

Finally, it is evident that there is additional spec impact (e.g. in terms of UCI encoding/multiplexing rule) with Alt3.4 due to necessary phase rotation or modulo operation in order to rotate the FD basis vectors and coefficients such that the strongest coefficient lies in the first FD component. Without such spec support, the performance of Alt3.4 will be much worse than Alt3.3.

Observation 3: Alt3.4 has additional spec impact such as UCI encoding/multiplexing rules.

# Conclusions

In this contribution, simulation results are provided for Alt3.3 and Alt3.4 for the strongest coefficient indicator. The observations made are summarized as follows.

Observation 1:

- Alt3.4 can be worse in overhead than Alt3.3 if NSB is small

- The overhead saving with Alt3.4 is small (0-2% of the total payload) only if NSB is large

Observation 2: There is no noticeable difference in performance-overhead tradeoffs achieved by Alt3.3 and Alt3.4

Observation 3: Alt3.4 has additional spec impact such as UCI encoding/multiplexing rules.

# References

- Chairman’s notes, RAN1#96bis

- R1-1907708, “CSI enhancement for MU-MIMO,” Samsung

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz with 13 SBs, 10 MHz BW |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for MU 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for SU |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU for dynamic rank 1-2, and SU for dynamic rank 1-4 |
| MIMO layers |  | Up to 4 layers for MU |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% RU for MU, 20% for SU |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |