3GPP TSG RAN WG1 Meeting #97															        R1- 1907714

Reno, USA, 13th-17th, May 2019

Source:	InterDigital, Inc.

Title:	Feature Lead Summary #1 for NR-V2X AI - 7.2.4.8 Others

Agenda item:	7.2.4.8

Document for:	Discussion and Decision

Introduction

This is a summary documents for AI 7.2.4.8 Others. The summary includes following topics:

- Sidelink RLM/RLF in NR V2X for unicast

- Simulation assumption update

Contributions not related to abovementioned topics are not summarized in this document.

Sidelink RLM/RLF

In this section, we provide summary of contributions related sidelink RLM/RLF which is RAN2 led aspect (RAN1 as secondary).

Issue #1: stand-alone RS for SL RLM/RLF

- Background:

  - In RAN1 #96, It has been agreed that no stand-alone RS dedicated to CSI is introduced in Rel-16

  - In RAN1 #96bis, the following has been agreed

    - No new sidelink reference signal is introduced for sidelink RLM/RLF

    - CSI-RS for CQI/RI is supported and confined within PSSCH

  - No clear agreement yet whether stand-alone RS is supported for sidelink RLM/RLF

- No stand-alone RS supported for SL RLM/RLF: [Vivo], [Oppo], [ZTE], [Samsung], [FirstNet], [Ericsson], [Huawei]

  - Increased implementation complexity due to resource allocation [Vivo]

  - Throughput loss due to half-duplex issues [Vivo]

  - Overhead due to system-wide standalone RS [Vivo]

Proposal for agreement:

- No standalone RS dedicated to SL RLM/RLF in Rel-16

Issue #2: both Tx UE and Rx UE in a unicast link perform sidelink RLF declaration

- Yes: [MTK], [LG], [Samsung], [FirstNet], [IDC]

  - To avoid unnecessary congestion when Rx UE has no link coverage

Proposal for agreement:

- Both Tx UE and Rx UE in a unicast link perform sidelink RLF declaration

Issue #3: which RS to use for SL RLM/RLF

- Background:

  - Since no new RS will be introduced for SL RLM/RLF, at least one of existing sidelink reference signals should be selected for SL RLM/RLF

    - SL-SSB, PTRS, S-CSI-RS, PSCCH DM-RS, and PSSCH DM-RS

- SL-SSB

  - No: [Vivo],[Oppo],[ZTE], [Samsung], [FirstNet], [Ericsson]

    - Not possible to identify Tx UE

    - UEs in proximity will likely send S-SSB at the same time and they cannot communicate each other

- PTRS

  - No: [Vivo], [ZTE], [Samsung], [FirstNet], [Ericsson]

    - Only for FR2 and low density

- CSI-RS

  - Yes: [Oppo], [IDC], [Samsung], [FirstNet], [Huawei]

  - No: [Vivo], [MTK]

    - S-CSI-RS is not available when there is no data

    - SCI decoding failure will miss S-CSI-RS samples

    - No clear CSI-RS configuration yet and the accuracy of RLM measurement is unknown

  - FFS: [ZTE], [Ericsson]

    - It depends on whether CSI-RS is always available in PSSCH transmission

    - Additional RS overhead if CSI-RS is transmitted always for RLM purposes

- DM-RS:

  - Yes: [Vivo], [ZTE], [MTK], [Samsung], [FirstNet], [Ericsson]

    - PSCCH DM-RS only [Vivo], [ZTE], [MTK]

      - SCI decoding failure will miss S-CSI-RS samples

      - UE-specific precoding and power control applies for PSSCH DM-RS

      - Precoded DM-RS for PSSCH is not suitable for RLM

Proposal for agreement:

- Down-select from the following two options as the sidelink RS for SL RLM/RLF in Rel-16

  - Option-1: PSCCH DM-RS

  - Option-2: Sidelink CSI-RS

  - FFS: additional sidelink RS is used as a supplementary RS for SL RLM/RLF

Issue #4: Periodic IS/OOS indication support for SL RLM/RLF

- Background:

  - Without supporting standalone RS for SL RLM/RLF, using an RS in a periodic traffic for SL RLM/RLF has been proposed.

  - When a periodic traffic is available, periodic measurement of sidelink RS may be available. Otherwise, no periodic measurement may be available.

- Yes: [Vivo], [LG]

  - Using PSCCH DM-RS for broadcast/groupcast could be used

- No: [IDC]

  - A unified solution for sidelink RLM/RLF should be used for all traffic types

Proposal for discussion:

- Support periodic IS/OOS indication as in NR Uu for SL RLM/RLF

Issue #5: Metric for sidelink RLF declaration

- CBR

  - No: [Vivo], [Oppo], [ZTE], [Samsung], [FirstNet]

    - CBR cannot represent the link quality

- HARQ feedback

  - Yes: [MTK], [Samsung], [FirstNet], [LG]

    - Use A/N ratio [MTK]

    - Tx UE could use HARQ feedback to declare sidelink RLF [LG]

  - No: [Vivo], [Oppo], [ZTE]

    - HARQ feedback is only available when enabled

- PSCCH decoding success/failure

  - Yes: [CATT], [LG], [IDC]

    - Failure of PSCCH decoding represent link failure [CATT]

- CSI feedback

  - Yes: [Samsung], [FirstNet]

    - It can be used when it is enabled

- IS/OOS as in NR Uu (i.e., hypothetical BLER)

  - Yes: [Oppo], [ZTE], [Samsung], [FirstNet], [IDC], [Ericsson]

  - No: [LG]

Proposal for discussion:

- Wait until RAN2 provides input for the metric for sidelink RLF declaration

Others:

- CSI-RS can be transmitted with PSSCH padded with dummy bits [Oppo]

- For periodic data, the window size is determined based on the periodicity of UE’s traffic data [ZTE]

- When and how to trigger/stop SL RLM/RLF procedure considering the overhead of RLM/RLF should be studied [Huawei]

Simulation profile update

- In [11], following simulation profile update is proposed to include high congestion scenario:

- For the high congestion scenario, UE dropping option A with vehicle speed 10 km/h for Highway is taken as an additional simulation assumption.

  - 100% vehicles generate packets, and companies can select the traffic model(s) from the following two models in TR 37.885:

    - Periodic traffic model 1 (low traffic intensity)

    - Aperiodic traffic model 1 (medium traffic intensity)

  - Company to provide details of congestion control operation, if it is modelled

- Proposal for agreement

- Agree on the following proposal for high congestion scenario:

  - For the high congestion scenario, UE dropping option A with vehicle speed 10 km/h for Highway is taken as an additional simulation assumption.

    - 100% vehicles generate packets, and companies can select the traffic model(s) from the following two models in TR 37.885:

      - Periodic traffic model 1 (low traffic intensity)

      - Aperiodic traffic model 1 (medium traffic intensity)

    - Company to provide details of congestion control operation, if it is modelled

- Conclusions

In this contribution, we provided initial input on topics related to RAN2 led aspects and simulation assumption update in this sub-agenda.

References

- R1-1906145	Discussion on RLM in NR sidelink	vivo

- R1-1906322	Radio link monitoring of sidelink unicast communication	CATT

- R1-1906476	Discussion of RLM for Unicast	OPPO

- R1-1906481	Discussion on AS level link management for unicast	ZTE, Sanechips

- R1-1906560	Discussion on SL RLM	MediaTek Inc.

- R1-1906945	On Sidelink RLM/RLF	Samsung

- R1-1907021	Discussion on AS level link management for unicast	LG Electronics

- R1-1907100	On Sidelink RLM/RLF for NR V2X	InterDigital, Inc.

- R1-1907146	On SL RLM / RLF in NR V2X for unicast	Ericsson

- R1-1907494	On RLM and RLF in NR V2X	Huawei, HiSilicon

- R1-1907022	Proposal for simulation assumption in the congested scenario	LG Electronics

Previous RAN1 agreements

RAN1 #96bis

Agreements:

- No new reference signal dedicated to SL RLM is introduced.

  - Existing SL RS is reused for SL RLM/RLF

    - Note: CSI-RS is not precluded

  - RAN1 has no intention to introduce RS transmitted in a periodic manner only for SL RLM purposes

  - FFS:

    - Whether SL RS is transmitted in a stand-alone manner for SL RLM/RLF

Agreements:

- Regarding metric for SL RLM/RLF declaration, RAN1 discussed the following (to be further studied):

  - Reuse IS/OOS metric in Uu RLM as much as possible but considering the condition that RAN1 has no intention to introduce RS transmitted in a periodic manner only for SL RLM purposes

  - Other metrics, e.g., congestion control metric (similar to CBR in LTE), consecutive HARQ-NACKs, etc.

  - Note: RAN1 expects further input from RAN2 to further progress on this topic

To prepare reply LS using the above two agreements – Moonil (IDC), R1-1905788, which is approved with the following update:

RAN1 has agreed that no new reference signal dedicated to SL RLM is introduced and existing SL RS is reused for SL RLM/RLF (i.e., a SL RS introduced for other purpose(s) is reused for SL RLM/RLF. The details are still being discussed in RAN1).

Final LS in R1-1905863

Agreements:

- For further evaluation of NR V2X,

  - 2Tx/2Rx is taken as an additional simulation assumption for FR1.