3GPP TSG-RAN WG1 Meeting #97	R1- 1907753

# May 13th  – 17th,  2019

Reno, USA

Agenda item:	7.1.3

Source: 	Qualcomm Incorporated

Title: 	Summary#2 of CRs on UCI multiplexing on PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, a summary of CRs on UCI multiplexing on PUSCH is provided in Section 2.

# Summary of CRs

There are in total 6 CRs submitted to 7.1.3 which related to UCI multiplexing on PUSCH. 5 CRs are addressing scenarios with different numerologies between different UL channels. One CR proposes to fix a typo in 38.213. The 6 CRs are categorized in below sections.

## Timeline check for multiplexing PUCCH on PUSCH with different numerologies

Two CRs both from CATT are submitted regarding the timeline check for multiplexing UCI on PUSCH with different numerologies used for PUCCH/PUSCHs. These two CRs are related but still addressing different aspects on timeline check.

R1-1906297	Correction on timeline check for uplink physical channel overlapping	CATT

Recommendation: The CR is not needed.

Reason: Current spec is clear. Timeline check is needed if at least one channel in the group of overlapping PUCCHs/PUSCHs is in response to a DCI. While timeline check is not needed if none of the channel in the group of overlapping PUCCHs/PUSCHs is in response to a DCI. If adopting this CR, UE needs to check timeline even if none of the channel in the group of overlapping channels is in response to a DCI. This is an unnecessary NBC change which is against previous agreements:

Agreements in RAN1 94:

- When PUSCH without corresponding DCI overlaps with configured PUCCH resource(s) for CSI and/or SR in a slot,

  - The timeline requirement for UCI multiplexing is not applicable

  - The pseudo-code in 9.2.5 of TS38.213 is applicable to resolve overlapping among PUCCH resources (if any)

  - After resolving overlapping among PUCCH resources (if any), the PUCCH UCI is multiplexed on the PUSCH without corresponding DCI if overlapping occurs.

  - Note: this does not apply to PUSCH as part of the activation

Offline conclusion: the CR will be rejected if no consensus can be made in this week to accept the CR.

R1-1906298	Correction on timeline definition for uplink physical channel overlapping	CATT

Offline outcome: Further discuss and conclude in RAN1 97 (this meeting).

## PUSCH selection for multiplexing PUCCH on PUSCH with different numerologies

There was a post RAN1 96bis email discussion on the topic of which PUSCH to be selected to multiplex UCI in case PUCCH overlap with multiple PUSCHs where the numerologies are different among PUCCH and PUSCHs.

Email discussion to decide potential solution to the issues identified in CR R1-1904538 – until 04/19, Yi (Qualcomm)

Unfortunately, during this email discussion no consensus was reached, and it was concluded that this issue will be reconsidered in RAN1#97.

Two CRs/contributions are submitted to this meeting to discuss this issue, as listed below. As these two CR/contribution are address the same issue, they should be discussed and treat together.

R1-1906302	Correction on PUSCH selection for overlapping of PUCCH and PUSCH	CATT

R1-1907441	Multiplexing of overlapping PUCCH and PUSCH with different numerologies		Nokia

Recommendation: The CR is not needed. No Spec update is needed.

Reason: Current spec already defined UE behavior in this scenario. If some companies think spec is not clear enough, a clarification can be captured in chairman’s notes.

Offline Proposal: CR R1-1906302 is not needed. The following is captured in Chairman’s notes to clarify specification.

- For UCI multiplexing, within a PUCCH group, on PUSCH, the following two steps are performed with step 1 first, then followed by step 2:

  - Step 1: UCI in overlapped PUCCH transmissions is multiplexed into one PUCCH resource (resource Z) on PCC. This step is done per PUCCH slot.

  - Step 2: UCI, that doesn’t include SR, in Z is multiplexed into one PUSCH, if Z overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below.

    - First priority: PUSCH with A-CSI as long as it overlaps with Z

    - Second priority: earliest PUSCH slot(s)

    - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low)

      - Third priority: Dynamic grant PUSCHs > configured grant PUSCHs

      - Fourth priority: PUSCHs on CC with smaller CC index > PUSCHs on CC with larger CC index

      - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission

Note: UE is not expected to be scheduled with more than one PUSCHs with A-CSI which overlap with Z.

## PRACH overlap with PUCCH/PUSCH/SRS

One CR is submitted to address the ambiguity of slot definition in the scenario PRACH and PUCCH/PUSCH/SRS overlapping or back-to-back transition with different numerologies used between PRACH and PUCCH/PUSCH/SRS.

R1-1907502	Correction on simultaneous PRACH and other UL transmissions		Huawei, HiSilicon

Recommendation: Further discuss and conclude in RAN1 97 (this meeting).

Reason: The spirit of the CR seems reasonable. But how to deal with different numerologies among PUCCH/PUSCH/SRS needs further discussion.

Offline proposal: for PRACH overlapping with PUCCH/PUSCH/SRS in the same slot or the definition of gap of N symbols between PRACH and PUCCH/PUSCH/SRS, down selection from the following two options to resolve the issue with different numerologies among PUCCH/PUSCH/SRS,

Option 1: u is referenced to a slot with smallest numerology among PUSCH/PUCCH/SRS, with the following TP adopted for Subclause 8.1 in 38.213

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than  symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where for  or ,  for  or , and  is the smallest SCS configuration of the PUSCH/PUCCH/SRS transmissions .

Option 2: u is referenced to a slot with numerology of PUSCH/PUCCH/SRS respectively, with the following TP adopted for Subclause 8.1 in 38.213

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than  symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where for  or ,  for  or , and  is the SCS configuration of the PUSCH/PUCCH/SRS transmission .

## CR to fix a typo

There is a CR submitted under 7.1.3 to fix a typo in Section 9.3 in 38.213.

R1-1906123	Draft TS 38.213 CR on UCI reporting in PUSCH		VIVO

Offline Proposal: Reject CR R1-1906123 and adopt CR1 included in feature lead summary R1-1907753.