3GPP TSG RAN WG1 #97	           	R1- 1907772

Reno, China, 13th – 17th May, 2019

Title:	Summary of PDCCH-based Power Saving Signal/Channel

Agenda item:	7.2.9.1

Source:	CATT

Document for:	Discussion

- Power Saving Signal/Channel Design

RAN1#96bis agreements on PDCCH-based power saving signal/channel are as follows,

Agreements: - The PDCCH-based power saving signal/channel is UE-specifically configured. - The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - Alt 1: triggering a single UE only - Alt 2: triggering UE(s) within a group - FFS whether to always trigger all UEs in a group or a subset of it - Alt 3: Alt 1 & Alt 2 Agreements: The assumptions of the DCI design of the PDCCH-based power saving signal/channel include: - No increase of DCI format size budget - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time - Working assumption: no increase of UE BD/non-overlapping CCE limit Agreements: Possible candidates of DCI format design for the PDCCH-based power saving signal/channel (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - New DCI format(s) - The size of new DCI format may or may not be the same size as the existing DCI size - Enhancement of existing DCI. E.g.,: - Additional new field(s) - Using the existing DCI format for the power saving purpose - Re-purpose field(s) in the DCI - The detection of existing DCI format as the indication for the power saving technique Agreements: The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide: - Indication of one or more power saving associated operations. - Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format Agreements: Potential DCI contents in DCI format(s), to be further investigated: - Power saving technique associated with C-DRX– - Essential for UE function for the C-DRX - Wakeup – - UE is indicated to transition from outside Active Time to Active Time - UE is indicated to stay at Active Time - Go to sleep– - UE is indicated to transition from Active Time to outside Active Time. - UE is indicated to stay outside Active Time - FFS: The time of receiving the wakeup and go-to-sleep indication inside or outside Active Time. - Cross-slot scheduling - Triggering RS transmission - CSI report - Single vs. multi-cell operation - BWP /SCell - BWP & SCell together - BWP and SCell have separated fields - MIMO layer adaptation/number of Anenna adaptation - May further depend on RAN4’s work - Indication of CORESET/search space/candidate of subsequent PDCCH decoding - PDCCH monitoring periodicity - PDCCH skipping (skipping duration)- - PDCCH skipping – UE is indicated to skip number of the PDCCH monitoring occasions and stays in the Active Time # Note that - For the bullets in italic, there are concerns that some of which may have dependence on the ongoing SI in RAN2. - For the last two bullets, there are additional concerns that these are deemded by some companies to be not in the scope of the power saving WI approved so far The following candidates may be discussed after RAN2’s SI is completed: - Skipping number of DRX monitoring - SPS activation - DRX configuration Agreements: - The design of the DCI format(s) and size needs to account for one or more of the following aspects - Within or outside Active Time - DCI format size for the power saving signal/channel to fit the DCI format size budget - Including aspects whether or not it is necessary to align it with existing DCI format size - Other aspects are not precluded Agreements: - Support at least one CORESET with configured search space(s) for the power saving signal/channel - FFS separate vs. shared with a CORESET (and/or search space(s)) configured for other purposes (when applicable) Agreements: - For PDCCH-based power saving signal/channel, - The set of AL(s) is configured - The number of PDCCH candidate(s) for each AL is configured

The design of the power saving signal/channel for the power saving techniques had been discussed.   The discussions of the power saving signal/channel design are summarized with category of outside and within Active Time

Outside Active Time

The power saving signal/channel is used for triggering UE wakeup or go-back-to-sleep outside Active Time when DRX is configured.  RAN2 agreed that the power saving signal/channel is configured offset before the starting of DRX ON.  The configuration of the power saving signal/channel outside the Active Time is configured by gNB with UE-specific offset before the DRX ON.

Proposal:

The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON

- “Indicated” implies the explicit signalling by higher layer signalling or implicit through the CORESET/search space

- FFS: The value and the range of offset

Agreements:

The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON

- “Indicated” implies the explicit signalling by higher layer signalling or implicit through the CORESET/search space

- FFS: The value and the range of offset

DCI format –

- New DCI format –

  - UE-Specific DCI – NEC, Huawei, vivo, CATT, Spreadtrum, MediaTek, OPPO, ZTE, LGE, Samsung, InterDigital, Qualcomm, Ericsson, Apple, Nokia

    - DCI size up to 12 bits – CATT, Spreadtrum, MediaTek, Ericsson

    - DCI size 10-20 bits - CMCC

  - Group Common DCI – Huawei, vivo, CMCC, MediaTek, OPPO, ZTE, Intel, Sony, InterDigital, Qualcomm, Apple, Panasonic

    - Aligned with DCI format 0_0, 1_0 – MediaTek

    - Joint coding - ZTE

- Enhancement of existing DCI format -

  - Additional fields –Convida

  - Repurpose existing fields – Convida

Proposal:

For power saving signal/channel configured outside Active Time, new DCI format(s) is supported.

- The power saving information is only UE-specific from UE perspective and included in a UE-specific DCI or a field in the group DCI

- FFS:

  - Whether UE-specific DCI and the group DCI is one or two DCI formats.

  - Whether UE can be configured to monitor both UE-specific and group DCI

    - Whether UE monitoring is in the same or different time.

Agreements:

For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format, with the power saving information for the UE in the DCI configurable by RRC

- FFS whether the DCI is in UESS or CSS or both

- FFS detailed configuration of the power saving information

- FFS the new DCI format

- Note: the same DCI may carry power saving information for one or more UEs

CORESET/Search Space configuration –

- Dedicated CORESET/Search Space for the power saving signal/channel – vivo, CATT, CMCC, ZTE, LGE, Qualcomm, Ericsson, Apple, Panosonic, Nokia, OPPO (search space), Panasonic

  - DMRS symbols mapped to all contiguous RBs in the CORESET – Qualcomm

  - Wakeup BWP - Qualcomm

- CORESET/search space configured for PDCCH – Huawei, vivo, DoCoMo, Spreadtrum, CMCC, MediaTek, Intel, Samsung Ericsson, Convida, OPPO (CORESET)

  - Dedicated DM RS initialization - Huawei

  - Reduced number of  aggregation level/candidates – vivo, CATT, Qualcomm

Proposal:

The dedicated configuration of the search space set for the power saving signal/channel outside Active Time is supported.

- Following the principle of Rel-15 search space

- FFS: UE behaviour in monitoring the power saving signal/channel outsidet Active Time

Proposal:

The CORESET for the power saving signal/channel can be same or different as that  for other PDCCH monitoring.

RNTI –

- Use same RNTI as that within the Active Time – CATT, , InterDigital

- New RNTI for power saving signal/channel– NEC, CATT, Spreadtrum, CMCC, MediaTek, Samsung, InterDigital, Qualcomm, Ericsson, Apple, Nokia, Panasonic

Proposal:

New PS-RNTI is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time

- FFS: Scrambling of the PDCCH-based power saving signal/channel

Multi-beam configuration –

- Up to 3 CORESETs per BWP with each CORESET associated with its TCI state and QCL assumption -  Huawei, vivo, CATT, OPPO, Qualcomm

- Wake-up CORESETs in a single slot - Qualcomm

- Dedicated TCI states and QCL assumptions for Power saving signal/channel – vivo, CATT, ZTE, Samsung

Proposal:  Up to 3 CORESETs per BWP is supported for the power saving signal/channel outside Active Time with each CORESET associated with its TCI state and QCL assumption

- FFS: whether 3 CORESETs are same of different to those within Active Time

Indication of power saving techniques

- Wakeup –  NEC, Huawei, vivo, DoCoMo, CATT, Spreadtrum, CMCC, MediaTek, OPPO, ZTE, LGE, Intel, Sony, Samsung, InterDigital, Qualcomm, Ericsson, Apple, Panasonic, Nokia

- Go-to-sleep – vivo, CATT, Sony, Samsung, Convida, Panasonic, Apple

- Cross-slot scheduling – vivo, CATT, Spreadtrum, Intel, Sony, InterDigital, Panasonic, Nokia

- Triggering RS transmission – vivo, CATT, Spreadtrum, CMCC, MediaTek, ZTE, Sony, Qualcomm, Panasonic, Nokia

- CSI report -  Spreadtrum, CMCC, MediaTek, Intel, Panasonic, Nokia

- multi-cell operation - CATT, Spreadtrum, Intel, Samsung, Qualcomm, Nokia

- BWP /SCell – vivo, CATT, Spreadtrum, CMCC, MediaTek, OPPO, ZTE, Samsung, Qualcomm

  - BWP & SCell together –CATT, Spreadtrum, Intel, Panasonic

  - BWP and SCell have separated fields – vivo, CMCC, MediaTek, OPPO, ZTE, Samsung, Qualcomm, Nokia

- MIMO layer adaptation/number of Anenna adaptation –Huawei, vivo, CATT, Spreadtrum, Intel, InterDigital

- Indication of CORESET/search space/candidate of subsequent PDCCH decoding,- vivo, Spreadtrum, LGE, Samsung, Qualcomm, InterDigital

- PDCCH monitoring periodicity – vivo, Spreadtrum, MediaTek, Sony

- PDCCH Skipping – Apple, Panasonic

- Others –

  - SPS Activation - Sony

  - Skipping number of DRX - Sony, Samsung

  - Dynamic DRX configurations – Sony

  - Short or long DRX cycle – Samsung

Proposal:  The power saving signal/channel is configured for triggering UE wakeup/go-to-sleep when it is configured outside Active time.  Additional power saving techniques to be included in the power saving signal/channel includes

- RS transmission (Aperiodic CSI-RS, TRS)

- CSI report

- BWP

- SCells

- Cross-slot/same-slot scheduling

- MIMO layers

- Indication of CORESET/search space/candidate and monitoring periodicity of subsequent PDCCH decoding

Within Active Time

When UE is at the Active Time, UE needs to monitor both PDCCH for scheduling DCI and  the power saving signal/channel for triggering adaptation to the power saving techniques.

DCI format –

- New DCI format –  CATT, Samsung, Qualcomm, Apple, Nokia

  - PDCCH skipping only – CATT, Apple

- Enhancement of existing DCI format

  - Additional fields – NEC, CATT, Spreadtrum, ZTE, Sony, Ericsson, Apple, Nokia, Convida

  - Repurpose the existing fields – CMCC, OPPO, Intel, Convida

    - Repurpose DCI 0_0 – CMCC

    - Reuse DCI 0_1 – OPPO

Proposal:

The explicit indication of the power saving technique(s) is supported at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time.

- It applies to UE-specific search space.

  - It is FFS for the common search space.

- FFS: Which existing DCI formats includes the power saving information

  - Whether power saving information is not included in the fallback DCI(s) (e.g., DCI format 0_0, DCI format 1_0)

  - Use of non-scheduling DCI formats.

- It is FFS which field(s) is used to be repurposed for the indication of the power saving technique if the repurpose of existing field(s) is used.

- FFS: Implicit indication

- FFS: New DCI format with size aligned with existing DCI format

CORESET/Search Space configuration –

- Dedicated CORESET/Search Space for the power saving signal/channel – Qualcomm

- CORESET/Search Space configured for PDCCH – NEC, Huawei, vivo,  CATT, Spreadtrum, CMCC, OPPO, ZTE, Intel, Sony, AsusTek, Samsung, Ericsson, Panasonic, Convida

Proposal:

The CORESET/Search Space configured for PDCCH is used for the power saving signal/channel when UE is in the Active Time.

Indication of power saving techniques

- Go-to-sleep – vivo, CATT, Spreadtrum, Intel, Sony, Convida, Panasonic, Apple

- Cross-slot scheduling – vivo, CATT, Spreadtrum, CMCC, MediaTek, OPPO, ZTE, Sony, InterDigital, Nokia, Panasonic

- Multi-cell operation – CATT, CMCC, MediaTek, Intel, Sony, Qualcomm

  - SCell PDCCH monitoring adaptation – MediaTek

- BWP – AsusTek, Samsung, Qualcomm, OPPO, Panasonic

  - HARQ-feedback - Qualcomm

- MIMO layer adaptation/number of Anenna adaptation  - Huawei, CATT, Spreadtrum, CMCC, Sony, InterDigital

- Indication of CORESET/search space/candidate of subsequent PDCCH decoding,- vivo, Spreadtrum, LGE, Samsung, Qualcomm, InterDigital, Panasonic

- PDCCH monitoring periodicity – MediaTek, Sony, Samsung

- PDCCH skipping (skipping duration)-  Huawei,  CATT, Spreadtrum, CMCC, OPPO, ZTE, Apple, Intel, Sony, Qualcomm, Panasonic

Proposal:  For UE in the Active Time, the power saving signal/channel supports the indication of

- PDCCH skipping for active cell(s) and associated BWP

- Cross-slot/same-slot scheduling

- MIMO layer adaptation

- Subsequent CORESET/search space/candidate

- PDCCH monitoring periodicity

Other Aspects in the Design of Power Saving Signal/Channel

- UE monitoring BWP/Cells -

- Power saving signal/channel multiplexing with other signals/channel

- Erros handling

- Conditions to deactivate WUS monitoring (e.g. . if a measurement is below a threshold)

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- 3GPP RP-190727: “WID: UE Power Saving in NR”, CATT, CAITC

- R1-1905988	DCI format design for PDCCH-based Power Saving Signal/Channel		NEC

- R1-1906005	PDCCH-based power saving signal/channel	Huawei, HiSilicon

- R1-1906170	PDCCH-based power saving signal/channel	vivo

- R1-1906227	Discussion on PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

- R1-1906350	Power saving signal/channel  design and performance		CATT

- R1-1906373	Discussion on PDCCH-based power saving channel	Spreadtrum Communications

- R1-1906524	Discussion on PDCCH-based power saving signal/channel design	CMCC

- R1-1906548	Design of PDCCH-based power saving signal/channel for NR	MediaTek Inc.

- R1-1906579	Discussion on PDCCH-based power saving signal/channel	OPPO

- R1-1906639	Discussion on PDCCH-based power saving signal		ZTE

- R1-1906695	Discussion on PDCCH-based power saving signal/channel	LG Electronics

- R1-1906819	Considerations on PDCCH-based power saving signal	Intel Corporation

- R1-1906856	Issues related to PDCCH-based power saving channel	Sony

- R1-1906877	PDCCH-based power saving signal/channel for UE adaptation	ASUSTEK

- R1-1906980	On PDCCH-based power saving signal/channel	Samsung

- R1-1907104	PDCCH-based Power Saving Signal Design	InterDigital, Inc.

- R1-1907294	PDCCH-based power saving channel design	Qualcomm Incorporated

- R1-1907322	Design of PDCCH-based power saving signal/channel	Ericsson

- R1-1907345	Design of PDCCH-based power saving signal and channel	Apple Inc.

- R1-1907363	Discussion on PDCCH-based power saving signal/channel (PoSS)	Panasonic

- R1-1907375	PDCCH-based power saving signal/channel	Nokia, Nokia Shanghai Bell

- R1-1907467	Go-To-Sleep Signaling for Power Savings	Convida Wireless