3GPP TSG-RAN WG1 Meeting #97	R1-1907781

Reno, USA, May 13th – 17th 2019

Agenda Item:	7.2.5.1

Source:	Thales

Title:	Summary#2 on simulation assumptions for NTN

Document for:	Discussion

# Introduction

This document aims to capture the potential agreements based on the companies’ contributions and the offline discussions. The proposals are focused on :

- SLS parameters and assumptions for calibration

- SLS parameters and assumptions for performance evaluation

- LLS parameters and assumptions

- Link budget assumptions and methodology

# Potential agreements :

Proposal #1: The updated Tables X.1 and X.2 defining the 2 sets of satellite parameters are proposed for agreement:

Table X.1 : Set-1 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 62 dBi | 42 dBi | 42 dBi |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table X.2 : Set-2 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 | 24 | 24 |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 | 30.5 | 30.5 |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 | 24 | 24 |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 2 m | 0.2 m | 0.2 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 54 | 34 | 34 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Proposal #3: The updated Tables X.3 defining the UE characteristics is proposed for agreement:

Table X.3 : UE characteristics for System Level Simulations

| Characteristics | VSAT | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 20 GHz UL and 30 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 20 GHz UL and 30 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (4,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (3dB-BW=65 deg) |
| Polarisation | circular | Linear : +/-45°X-pol | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | 8dBi per element |
| Antenna temperature | 150 K | 290 K | 290 K |
| Noise figure | 1.2 dB | 7 dB | 9 dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [2 W (33 dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | 8dBi per element |
| Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. |  |  |  |

Proposal #4: Based on the two tables already agreed last meeting , the Table X.5 capturing the simulation assumptions for calibration is proposed for agreement

Table X.5 : SLS assumptions for calibration

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) / Ka- Band (i.e. 20 GHz DL, 30 GHz UL) |
| Maximum Bandwidth per beam (DL + UL) | S-band : DL 30 MHz and UL 30 MHz Ka-band : DL 400 MHz and UL 400 MHz The bandwidth per beam must be adapted based on the frequency factor and the polarization re-use option considered. |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table X.1 and Table X.2 Note : Same satellite characteristics should be considered for both single and multi-satellite simulations |
| Satellite antenna pattern | TR 38.811v15.0.0 section 6.4.1 Bessel function |
| Satellite polarization configuration | Circular |
| Beam layout definition | For singles satellite simulation : See Table X.4 For multi satellites simulation : FFS |
| Frequency re-use factor | Option 1 : 1 Option 2 : 3 Option 3 : 2 if polarization re-use is enabled |
| Polarization re-use | Option 1 : Disable Option 2 : Enable Note : Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| Deployment scenarios | Base-line : Rural Additional deployment scenario results can be provided |
| Propagation conditions | Base-line : Clear Sky |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | Base-line : at least X=10 UEs per beam with uniform distribution in all the beams |
| UE configuration | S-band : - Handheld (optional for scenario A) Ka-band : - VSAT - Others (optional for scenario A) See Table X.3 |
| UE orientation | VSAT and Others: Ideal Tracking serving beam; Handheld: Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base-line : Coupling loss, Geometry Note : Coupling loss is defined as the signal loss from the antenna port to the antenna port |
| Note 1 : Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. |  |

Proposal #5 : The beam layout parameters captured in the following table should be adopted as a starting point for single satellite simulation.

Table X.6 : Beam layout parameters for single satellite simulation

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band : 2 GHz Ka-band : 20 GHz for DL | S-band : 2 GHz Ka-band : 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band : ABS = 0.0061 Ka-band : ABS = 0.0027 | S-band : ABS = 0.0668 Ka-band : ABS = 0.0267 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline : - Case 1 : Not considered - Case 2 : 45 degrees | Baseline : - Case 1 : 90 degree - Case 2 : FFS |
| Central beam bore sight direction coordinates in UV plane | Baseline : - Case 1 : Not considered - Case 2 : (0.107,0) | Baseline : - Case 1 : (0,0) - Case 2 : FFS |
| Gateway direction coordinates in UV plane | Baseline : Same as central beam bore sight direction coordinates in UV plane Note : Not needed for calibration |  |

Proposal #6 : Indication about the wrap around methodology used should be provided by the companies together with their simulation results.

Proposal #7 : The value of amplitude index S_4 used in ionospheric scintillation loss model should be provided in the simulation assumptions if the UE latitude is comprised within ±20° and the carrier frequency is below 6GHz.

Proposal #8: The following table is proposed for agreement on SLS parameters for performance evaluation.

Table X.7 : SLS parameters for performance evaluation

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | Same as in Table X.5 |
| Maximum Bandwidth per beam (DL + UL) | Same as in Table X.5 |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | Same as in Table X.5 |
| Satellite antenna pattern | Same as in Table X.5 |
| Satellite polarization configuration | Same as in Table X.5 |
| Beam layout | Same as in Table X.5 |
| Number of beams | Same as in Table X.5 |
| Frequency re-use factor | Same as in Table X.5 |
| Polarization re-use | Same as in Table X.5 |
| Deployment scenarios | Same as in Table X.5 |
| Fast fading model | Frequency selective channel model listed in TR 38.811 |
| Propagation conditions | Same as in Table X.5 |
| UEs outdoor/indoor distribution | Same as in Table X.5 |
| UEs coverage distribution | Same as in Table X.5 |
| UE configuration | Same as in Table X.5 |
| UE orientation | Same as in Table X.5 |
| Handover Margin | To be reported by the companies |
| UE attachment | RSRP |
| Receiver type | MMSE-IRC |
| Scheduler | To be reported by the companies |
| Traffic model | FTP 3 (see TR 38.802) packet size = 0.5 Mbyte |
| Channel estimation | Realistic |
| CSI feedback | Release 15 (Note 2) |
| Metrics for performance | Baseline: UE throughput (5%, 50%, 95%) at 20% and 50% RU (Note 4) |
| Note 1 : Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. Note 2 : The impact of RTT should be considered for evaluation Note 3 : The overhead due to control channel and RS can be reported by the companies [e.g. 4/14] Note 4 : The bandwidth used for RU calculation is based on the allocated bandwidth for each beam |  |

Proposal #9 : The following skeleton table is proposed for agreement to capture the impairments introduced on the RF signal due to the satellite payload and movement.

Table X.9 : Impairments due to satellite payload and satellite movement

|  | FR1 – Below 6 GHz | FR2 – Above 6 GHz |
| --- | --- | --- |
| Satellite HPA model | Table 9 in R1-1907481 (ESA) | Table 8 in R1-1907481 (ESA) |
| Phase noise model (Note 2) | Table 6 in R1-1907481 (ESA) | Table 7 in R1-1907481 (ESA) |
| Local oscillator drift | FFS | FFS |
| Max Doppler shift (Note 1) | Scenario A : 0.15 ppm Scenario C2/D2 : - 1200 km : 20 ppm - 600 km : 24 ppm |  |
| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Max Doppler rate | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Note 1 : Min. Elevation angle for both sat-gateway and user equipment is equal to 10 degree. Note 2 : For regenerative scenario, this can be considered as phase noise model for the gnB. For transparent scenarios, it should be considered as an additional phase noise w.r.t the phase noise generated by the gnB and the UE. |  |  |

Proposal #10 : The following skeleton table is proposed for agreement on LLS parameters for DL synchronization performance evaluation. Companies are encouraged to provide values to fill the table for the next meeting.

Table X.10 : LLS parameters for DL synchronization evaluation

|  | FR1 - Below 6GHz | FR2 - Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| Subcarrier Spacing(s) | FFS | FFS |
| SNR range | FFS | FFS |
| Search window | FFS |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | FFS | FFS |
| Frequency Offset | UE: uniform distribution [+/- FFS ppm] BS: uniform distribution [+/- FFS ppm] Satellite : uniform distribution [+/- FFS ppm] (see Table X.7) Doppler in channel due to satellite movement: max. [+/- FFS] ppm (see Table X.7) Doppler in channel due to UE movement: max. [+/- FFS] ppm |  |
| Number of interfering beams | 0 as start point FFS interference from other beams, based on the frequency reuse pattern or constellation |  |
| Phase noise model | Table X.9 | Table X.9 |
| Satellite HPA model | Table X.9 | Table X.9 |
| Metrics | FFS |  |

Proposal #11 : The following skeleton tables are proposed for agreement on LLS parameters for PRACH performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.11 : LLS parameters for PRACH performance evaluation

| Configurations | FR-1 | FR-2 |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| Antenna Configuration at the TRP (satellite) | 1Rx | 1Rx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | UE: value FFS, based on the residual frequency offset of DL synchronization BS : uniform distribution +/- 0.05 ppm |  |
| UE speed | FFS | FFS |
| Initial timing Offset (Note 1) | FFS |  |
| Phase noise model | Table X.9 | Table X.9 |
| PRACH design | FFS | FFS |
| Metric | FFS |  |
| Note1 : Ideal common delay compensation is assumed. |  |  |

Proposal #12 : The following skeleton tables are proposed for agreement on LLS parameters for data transmission performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.12 : LLS parameters for data transmission performance evaluation

| Parameters | FR 1 | FR 2 |
| --- | --- | --- |
| Carrier frequency | 2GHz | DL 20GHz |
| Duplex Mode | FDD |  |
| Subcarrier spacing | 15kHz, 30kHz | 60kHz, 120kHz |
| CSI feedback | FFS |  |
| Modulation order, Coding rate | FFS |  |
| Channel coding scheme | NR channel coding |  |
| Link adaptation | FFS |  |
| HARQ | FFS |  |
| Channel estimation | Realistic estimation |  |
| Frequency offset model | Frequency offset in case of non-initial acquisition TRP: uniform distribution +/- FFS ppm UE: uniform distribution +/- FFS ppm |  |
| UE speed | FFS |  |
| Channel model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| TRP antenna configuration | 1Tx | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | Table X.9 | Table X.9 |
| Satellite HPA Model | Table X.9 | Table X.9 |
| Metrics | FFS |  |

Proposal #13 : Link budget methodology and analysis should be captured in TR 38.821

Proposal #14: Endorse section X.Y.1 from Nokia TP in R1-1906090 to capture link budget methodology in the TR.

Proposal #15: Endorse skeleton tables from section X.Y.2 from Nokia TP in R1-1906090 to capture the simulation assumptions for link budget analysis. Parameters values can be further discussed by the companies.

Table X.Y.2-1 Simulation assumptions for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| Bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Satellite altitude | 600km, 1200km, 35786km |
| SCS | 30kHz (S band), 120kHz (Ka band) |
| Elevation angle | 10, 30, 90 |
| Atmospheric loss | 0.1 dB (S band), 1 dB (Ka band) |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Average C/I within a satellite beam | 16 dB |
| Frequency reuse factor (Note 1) | 4 |
| Terminal type | - Ka band: VSAT, (M, N, P) = (4, 8, 2) - S band: (M, N, P) = (1,1,1), (M, N, P) = (1,1,2), and (M, N, P) = (1, 2, 2) |
| Terminal speed | 0 km/h |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Fast fading channel model | AWGN, NTN-CDL-D in TR 38.811 |
| Target BLER | 10%, 0.1%, 0.001% |
| Note 1: Frequency reuse factor of 1 is not precluded from simulation studies. |  |

Table X.Y.2-2 Terminal parameters

| Terminal Type | VSAT | (M, N, P) = (1,1,1) | (M, N, P) = (1,1,2) | (M, N, P) = (1, 2, 2) | (M, N, P) = (8, 4, 2) |
| --- | --- | --- | --- | --- | --- |
| Terminal receive antenna element gain (dBi) | 39.7 | 0 | 0 | 0 | 5 |
| Terminal noise figure (dB) | 1.2 | 7 | 7 | 7 | 10 |
| Terminal ambient temperature (K) | 290 | 290 | 290 | 290 | 290 |
| Terminal antenna temperature (K) | 150 | 290 | 290 | 290 | 290 |
| Terminal maximal EIRP (dBW) | 45.75 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal EIRP (dBW) | 44 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal output back off (dB) | 1.75 | 0 | 0 | 0 | 0 |
| Terminal transmit power (dBW) | 3 | -7 | -7 | -7 | -7 |
| Terminal transmit antenna element gain (dBi) | 43.2 | 0 | 0 | 0 | 5 |
| Terminal cable loss (dB) | 0.45 | 0 | 0 | 0 | 0 |

Table X.Y.2-3 Satellite parameters

|  |  | Configuration 1 |  |  | Configuration 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 |
| S band | Satellite EIRP density (dBW/MHz) | 59 | 40 | 34 |  |  |  |
|  | Satellite G/T (dB/K) | 19 | 11.2 | 11.2 |  |  |  |
| Ka band | Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |  |  |  |
|  | Satellite G/T (dB/K) | 22 | 13 | 13 |  |  |  |

Proposal #14 : Endorse skeleton tables from section X.Y.3 from Nokia TP in R1-1906090 to capture the link budget analysis.

|  |  |  |  | Ka band |  |  |  |  |  | S band |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE type |  |  |  | VSAT |  |  | (M, N, P) = (8, 4, 2) |  |  | (M, N, P) = (1,1,1) |  |  | (M, N, P) = (1,1,2) |  |  | (M, N, P) = (1,2,2) |  |  |
| Elevation angle () |  |  |  | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 |
| Satellite configuration 1 | DL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite configuration 2 | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Reference

[1] R1-1907087, On beam layout definition for NTN system level simulations, Thales
[2] R1-1907687, On beam layout definition for NTN system level simulations, Thales
[3] R1-1906463, Link budget and Doppler Aspects for evaluation of NR NTN, MediaTek Inc.
[4] R1-1906870, Discussion on the simulation assumption for NTN, ZTE
[5] R1-1906323, Simulation assumption for NTN evaluation, CATT
[6] R1-1906802, Considerations on performance evaluation for NTN, Intel Corporation
[7] R1-1906085, Considerations on Performance Evaluation in NTN, Nokia
[8] R1-1905992, Discussion on performance evaluation for NTN, Huawei
[9] R1-1906951, Remaining issues on NTN link-level and system-level simulation assumptions, Samsung
[10] R1-1907369, On Dowlink Performance in NTN, Fraunhofer IIS
[11] R1-1907481, Beam size computation and alternative satellite specifications, ESA
[12] R1-1907395, On NTN link level and system level evaluations, Ericsson
[13] R1-1906090, TP for Link Budget Analysis in TR 38.821, Nokia
[14] R1-1905997, Discussion on link budget for NTN, Huawei
[15] R1-1906876, Preliminary link budget for NTN, ZTE
[16] Radar System Engineering, Lecture 9, Antennas, Robert M.O’Donnel, IEEE New Hampshire