3GPP TSG RAN WG1 Meeting #97    	               R1-1907792

Reno, USA, May 13th – 17th, 2019

Source:	NTT DOCOMO, INC.

Title:	Offline discussion for Rel-15 DL SPS and UL CG

Agenda Item:	7.1.3

Document for: 	Discussion and Decision

- Background

Following draft CRs need to be further discussed.

R1-1907349	Correction to DL SPS on the number of codewords supported	Nokia, Nokia Shanghai Bell

Continue discussion offline

R1-1907045	Clarification on aggregationFactor with CG retransmission	Ericsson

R1-1907504	Correction on slot aggregation with fallback DCI	Huawei, HiSilicon

Continue discussion offline

- Discussions

R1-1907349 (number of codewords for DL SPS, Nokia, Nokia Shanghai Bell)

Reason for change:

- In current specifications, there is no restriction on the number of codewords that can be supported for SPS PDSCH. However, only 1 HARQ-ACK bit is generated for SPS PDSCH. The behavior when SPS is activated with two codewords is unclear.

Summary of change:

- Add the restriction that DL SPS can only be activated with one codeword.

#### 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and - the new data indicator field for the enabled transport block is set to '0'. Validation of the DCI format is achieved if - all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2, and - if DCI format 1_1 is used to activate DL SPS, only one codeword is enabled via “Antenna port(s)” field as described in Subclause 7.3.1.2.2 of [5, TS 38.212]. If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format. Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation DCI format 0_0/0_1; DCI format 1_0; DCI format 1_1. HARQ process number; set to all '0's; set to all '0's; set to all '0's. Redundancy version; set to '00'; set to '00'; For the enabled transport block: set to '00'. Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation DCI format 0_0; DCI format 1_0. HARQ process number; set to all '0's; set to all '0's. Redundancy version; set to '00'; set to '00'. Modulation and coding scheme; set to all '1's; set to all '1's. Frequency domain resource assignment; set to all '1's; set to all '1's.

Based on offline discussions, the correction changes the UE behavior for validating the DCI for SPS, it is NBC. Therefore, following suggestion is proposed:

Suggestion:

- Capture following conclusion in chairman’s notes.

  - Conclusion: if DCI format 1_1 is used to activate DL SPS with two codewords, UE behavior is not defined.

| Company | View |
| --- | --- |

R1-1907045 (aggregationFactor for CG retransmission, Ericsson)

R1-1907504 (slot aggregation for SPS/CG with fallback DCI, Huawei, HiSilicon)

Reason for change:

- For a UE configured with pusch-AggregationFactor, the limitation on transmitted PUSCH and expected UE behaviour is described in 38.214 6.1.2.1, which applies currently only on C-RNTI and MCS-C-RNTI. From the description of 38.214 chapter 6.1, it is understood that the retransmission of configured grant (CG) shall also apply the pusch-AggregationFactor if configured in PUSCH-Config however the related procedure is undefined. To maintain a unified UE behaviour, the CG retransmission procedure shall get clarified when pusch-AggregationFactor is configured in PUSCH-Config.

- In RAN1#96bis meeting, it was concluded that only 0_1/1_1 can trigger PUSCH/PDSCH with slot aggregation, and corresponding CR is approved in R1-1905907 (38.214, CR0031, Rev. 2), see appendix. Therefore, UE bahvior on DL SPS PDSCH and UL configured grant transmission with slot aggregation should be corrected to be aligned with the latest RAN1 agreements.

Summary of change:

- Add activation DCI format 1_1 for DL SPS and add CS-RNTI with NDI=1 for DCI format 0_1 scheduling UL PUSCH retransmission when slot aggregation is configured, respectively.

- A minor correction of typo in 6.2.2, “MCS-RNTI” is not defined in specification, shall be “MCS-C-RNTI”.

#### 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in sub-clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is , where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PDSCH are determined from the start and length indicator SLIV: if then else where, and - The PDSCH mapping type is set to Type A or Type B as defined in sub-clause 7.4.1.1.2 of [4, TS 38.211]. The UE shall consider the S and L combinations defined in table 5.1.2.1-1 as valid PDSCH allocations: Table 5.1.2.1-1: Valid S and L combinations PDSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; {0,1,2,3} (Note 1); {3,…,14}; {3,…,14}; {0,1,2,3} (Note 1); {3,…,12}; {3,…,12}. Type B; {0,…,12}; {2,4,7}; {2,…,14}; {0,…,10}; {2,4,6}; {2,…,12}. Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3. When receiving PDSCH scheduled by DCI format 1_1 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1, if the UE is configured with pdsch-AggregationFactor, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the pdsch-AggregationFactor consecutive slots and the PDSCH is limited to a single transmission layer. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 5.1.2.1-2. <Unchanged parts omitted> #### 6.1.2.1 Resource allocation in time domain When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission. When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table which is defined by the higher layer configured pusch-TimeDomainAllocationList in pusch-Config. The indexed row defines the start and length indicator SLIV, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter reportSlotOffsetList in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of . - The slot where the UE shall transmit the PUSCH is determined by K2 as where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and and are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and - The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row: if then else where, and - The PUSCH mapping type is set to Type A or Type B as defined in Subclause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row. The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations Table 6.1.2.1-1: Valid S and L combinations PUSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; 0; {4,…,14}; {4,…,14}; 0; {4,…,12}; {4,…,12}. Type B; {0,…,13}; {1,…,14}; {1,…,14}; {0,…, 11}; {1,…,12}; {1,…,12}. When transmitting PUSCH scheduled by DCI format 0_1 in PDCCH with CRC scrambled with CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI, if the UE is configured with pusch-AggregationFactor, the same symbol allocation is applied across the pusch-AggregationFactor consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the pusch-AggregationFactor consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2. Table 6.1.2.1-2: Redundancy version when pusch-AggregationFactor is present rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. If the UE procedure for determining slot configuration, as defined in subclause 11.1 of [6, TS 38.213], determines symbols of a slot allocated for PUSCH as downlink symbols, the transmission on that slot is omitted for multi-slot PUSCH transmission. <Unchanged parts omitted> ### 6.2.2 UE DM-RS transmission procedure When transmitted PUSCH is neither scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration. When transmitted PUSCH is scheduled by activation DCI format 0_0 with CRC scrambled by CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type provided by higher layer parameter dmrs-Type in configuredGrantConfig on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, and additional DM-RS with dmrs-AdditionalPosition from configuredGrantConfig can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled. For the UE-specific reference signals generation as defined in Subclause 6.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PUSCH mapping Type A and Type B. When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, or corresponding to a configured grant, - the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Subclause 6.4.1.1 of [4, TS 38.211]. - the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig. - if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3'. - if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'. - and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Subclause 6.4.1.1.3 of [4, TS 38.211]. If a UE transmitting PUSCH is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH - any DM-RS ports among 4-7 or 6-11 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE. For PUSCH scheduled by DCI format 0_1, by activation DCI format 0_1 with CRC scrambled by CS-RNTI, or configured by configured grant Type 1 configuration, the UE shall assume the DM-RS CDM groups indicated in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of Subclause 7.3.1.1 of [5, TS38.212] are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) correspond to CDM group 0, {0,1}, {0,1,2}, respectively. For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0 with CRC scrambled by CS-RNTI, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, the UE shall assume that the number of DM-RS CDM groups without data is 3 which corresponds to CDM group {0,1,2} for the case of PUSCH scheduled by activation DCI format 0_0 and the dmrs-Type in cg-DMRS-Configuration equal to 'type2' and the PUSCH allocation duration being more than 2 OFDM symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases. For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by . Table 6.2.2-1: The ratio of PUSCH EPRE to DM-RS EPRE Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; -3 dB; -3 dB. 3; -; -4.77 dB.

Suggestion:

Discuss above CRs after decision made for the CR

- R1-1906382 in CA session.

| Company | View |
| --- | --- |

- Appendix

Approved CR in R1-1905907 (38.214, CR0031, Rev. 2)

“When receiving PDSCH scheduled by DCI format 1_1 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config, if the UE is configured with pdsch-AggregationFactor, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the pdsch-AggregationFactor consecutive slots and the PDSCH is limited to a single transmission layer. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 5.1.2.1-2. ”

“When transmitting PUSCH scheduled by DCI format 0_1 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, if the UE is configured with pusch-AggregationFactor, the same symbol allocation is applied across the pusch-AggregationFactor consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the pusch-AggregationFactor consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2. ”