3GPP TSG RAN WG1 Meeting #97 R1-1907813 Reno, USA, May 13th - 17th, 2019 CR-Form-v12.0. CHANGE REQUEST. 38.212; CR; DRAFT; rev; -; Current version:; 15.5.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PMI bitwidth for Type I CSI. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-5-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; For Type I CSI reporting, the bitwidth of (i1,1,i1,2) is defined by implying a joint coding of i1,1 and i1,2 in Table 6.3.1.1.2-1 and Table 6.3.1.1.2-2, TS 38.212. However, in TS 38.214, there is no description of how the index of this joint indicator is mapped to the values of i1,1 and i1,2. In fact, in all the possible cases, the joint indication provides same overhead as separate coding. Hence the simplest way is to change the bitwidth in TS 38.212.. Summary of change:; The bitwidth for (i1,1,i1,2) is given for i1,1 and i1,2 respectively instead of for both fields jointly.. Consequences if not approved:; Mapping of wideband PMI indices can be misinterpreted, leading to that Type I CSI reporting cannot be implemented.. Clauses affected:; 6.3.1.1.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: Since the resulting bitwidth is not changed, the change is backwards compatible.. This CR's revision history:; This is the first version for this CR.. ##### 6.3.1.1.2 CSI only The bitwidth for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports is 2 for Rank=1 and 1 for Rank=2, according to Subclause 5.2.2.2.1 in [6, TS 38.214]. The bitwidth for PMI of codebookType=typeI-SinglePanel with more than 2 CSI-RS ports is provided in Tables 6.3.1.1.2-1, where the values of and are given by Subclause 5.2.2.2.1 in [6, TS 38.214]. Table 6.3.1.1.2-1: PMI of codebookType=typeI-SinglePanel Information field for wideband PMI; Information field for wideband PMI or per subband PMI. (,). codebookMode=1; codebookMode=2; codebookMode=1; codebookMode=2. Rank = 1 with >2 CSI-RS ports,; (,); (,); N/A; 2; 4. Rank = 1 with >2 CSI-RS ports,; (,); (, 0); N/A; 2; 4. Rank=2 with 4 CSI-RS ports,; (,); (, 0); 1; 1; 3. Rank=2 with >4 CSI-RS ports,; (,); (,); 2; 1; 3. Rank=2 with >4 CSI-RS ports,; (,); (, 0); 2; 1; 3. Rank=3 or 4, with 4 CSI-RS ports; (,); 0; 1. Rank=3 or 4, with 8 or 12 CSI-RS ports; (,); 2; 1. Rank=3 or 4 , with >=16 CSI-RS ports; (, ); 2; 1. Rank=5 or 6; (,); N/A; 1. Rank=7 or 8,; (, ); N/A; 1. Rank=7 or 8,; (,); N/A; 1. Rank=7 or 8, with or or; (,); N/A; 1.

The bitwidth for PMI of codebookType= typeI-MultiPanel is provided in Tables 6.3.1.1.2-2, where the values of and  are given by Subclause 5.2.2.2.2 in [6, TS 38.214].

Table 6.3.1.1.2-2: PMI of codebookType= typeI-MultiPanel

|  | Information fields for wideband |  |  |  |  | Information fields for wideband or per subband |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |  |  |  |  |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | N/A | N/A | 2 | N/A | N/A | N/A |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | 2 | 2 | 2 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=1 with codebookMode=2 | (,) | N/A | 2 | 2 | N/A | N/A | 2 | 1 | 1 |
| Rank=2 with , codebookMode=2 | (,) | 1 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=3 or 4 with , codebookMode=2 | (,) | 0 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=2 or 3 or 4 with , codebookMode=2 | (,) | 2 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |

<Unchanged parts are omitted>