3GPP TSG RAN WG1 97			R1-1907894

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary on offline discussion for SCI-related issues

Document for:	Discussion and Decision

Summary

Based on the online and additional offline discussions, the offline proposal in R1-1907833 on SCI-related issues is revised as the following offline agreement. The revision is highlighted in red.

Offline agreement: On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table.

- FFS: details on bitwidth and possible values for Minitial reporting in UCI part 2

- FFS: whether the possible value(s) for Minitial can depend on configured FD compression parameters

- Up to the editor to capture this agreement

|  | Alt B |
| --- | --- |
| SCI for RI>1 | Alt3.4: Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1)). The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIi=li*, and mi* is not reported |
| Index remapping | For layer i, the index mi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmi associated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3. |
| Combinatorial indicator for N3≤19 | log2N3-1Mi-1 bits |
| Combinatorial indicator for N3>19 | log2N3'-1Mi-1 bits |
| Minitial | Reported in UCI part 2, details on bitwidth and possible values are FFS |
| Support (17) | Huawei/HiSi, Ericsson, Samsung, Nokia/NSB, MotM/Lenovo, vivo, MediaTek, Intel, ZTE, OPPO, CATT, NEC, Spreadtrum, APT |