3GPP TSG RAN WG1 #97		R1-1906403

Reno, USA, May 13th – 17th, 2019

Source:	Panasonic

Title: 	Discussion on resource allocation mechanism of mode 2 in NR V2X

Agenda Item:		7.2.4.2.2

Document for:	Discussion, Decision

Introduction

This contribution is modification of R1-1905381 and to continue the discussion on resource allocation mechanism of mode 2 in NR V2X.

Discussion

For a UE, it may simultaneously support multiple traffics, for example uncast, groupcast and broadcast.  Each of them may have periodic and aperiodic traffics respectively. One resource pool is used for each type of traffic can simplify the sensing procedure but it is not efficient resource utilization because separate multiple resource pools may not fit the change of traffic ratio although sensing/resource allocation can be optimized for each type of traffic. Therefore, to allow the coexistence of different traffics in the same Tx resource pool is necessary. For Rx resource pool, there is no issue on coexistence in our view. Regardless of outcome of coexistence among unicast, groupcast and broadcast in the same Tx resource pool, one Rx resource pool may cover same or different traffics based on (pre)configuration.

Proposal 1: Multiple traffics are allowed to coexist in the same Tx resource pool and Rx resource pool.

In the following, we discuss how/if to allow the coexistence of different traffics in the same Tx resource pool.

The coexistence among unicast, groupcast and broadcast in the same resource pool

For the sensing procedure for PSSCH/PSCCH, similar to LTE-V2X, we see the need of decoding PSCCH and energy based sensing of PSSCH. When unicast/groupcast share the resource pool with broadcast traffic, if one UE using a resource pool is not able to decode PSCCH from other UEs in the same resource pool due to some reasons (i.e. not capable of unicast/groupcast, not in the group of groupcast or not target of unicast), such UE cannot well sense the interference from unicast/groupcast. In order to allow all UEs to use the same resource pool by sensing the all other UEs’ interference well, it is required that all UEs sharing the same resource pool are able to decode all PSCCHs. Therefore, CRC of PSCCH of unicast/groupcast should not be scrambled by UE ID/group ID.

Proposal 2: All UEs sharing the same Tx resource pool should be decodable of PSCCH, i.e. CRC of PSCCH of unicast/groupcast should not be scrambled by UE ID/group ID.

The coexistence between periodic and aperiodic traffic in the same resource pool

Based on above discussion of the resource pool sharing among unicast, groupcast and broadcast traffic, the remaining issue would be coexistence between periodic and aperiodic traffic regardless of the traffic is unicast, groupcast or broadcast. As the sensing procedure, we envisage two types of sensing/resource selection mechanism. One is LTE V2X like long-term sensing/resource selection mechanism, where it is more optimized for periodic traffic. The sensing window could be like 1s and resource selection window size is within 100ms similar to LTE V2X. The other is short-term sensing/resource selection mechanism. The required sensing time could be in granularity of a few 10 of usec and depending on latency requirement. This short-term sensing/resource selection mechanism would be more optimized for aperiodic traffic.

Using long-term sensing/resource selection mechanism and short-term sensing/resource selection mechanism, we see following three approaches are possibilities.

Approach 1: Periodic traffic and aperiodic traffic share the same resource pool. Long-term sensing/resource selection mechanism is used in the resource pool

- For periodic traffic transmission, it could be based on long-term sensing/resource selection mechanism.

- For aperiodic traffic transmission, it is also based on long-term sensing/resource selection mechanism but how to report the set B (e.g., 20% of total resources) to MAC layer needs to modified as there is no periodicity.

- This approach has less standardization impact as LTE V2X behaviours are largely reused but the performance of aperiodic traffic transmission may not be optimized.

Approach 2: Periodic traffic and aperiodic traffic do not share the same resource pool. The resource pool with periodic traffic uses long-term sensing/resource selection mechanism. The resource pool with aperiodic traffic uses short-term sensing/resource selection mechanism.

For aperiodic traffic, to use short-term sensing/resource selection mechanism would optimize aperiodic traffic. As periodic traffic and aperiodic traffic do not coexist in the same resource pool, there is no mutual impact and each traffic’s performance can be optimized. The demerit is resource pool size for aperiodic and periodic need to be determined semi-static manner and the total resource utilization may not be optimized when the ratio of periodic or aperiodic traffic is variable in time.

Approach 3: Periodic traffic and aperiodic traffic share the same resource pool. Long-term sensing/resource selection mechanism is used for periodic traffic and short-term sensing/resource selection mechanism is used for aperiodic traffic in the same resource pool.

In this approach, there are some mutual impact between periodic traffic and aperiodic traffic. In high level, long-term sensing/resource selection mechanism is more like FDMed channel access mechanism but short-term sensing/resource selection mechanism is more TDMed channel access mechanism. In order to co-exist well in the same resource pool, short-term sensing/resource selection mechanism to select the resource from FDMed utilized resource is necessary.

We see some more studies are necessary on the above approaches. It would not be required to choose only one approach. Therefore, we propose following.

Proposal 3: Following three approaches are supported for coexistence of periodic traffic and aperiodic traffic:

- Approach 1: Periodic traffic and aperiodic traffic share the Tx same resource pool. Long-term sensing/resource selection mechanism is used in the Tx resource pool

- Approach 2: Periodic traffic and aperiodic traffic do not share the same Tx resource pool. The Tx resource pool with periodic traffic uses long-term sensing/resource selection mechanism. The Tx resource pool with aperiodic traffic uses short-term sensing/resource selection mechanism.

- Approach 3: Periodic traffic and aperiodic traffic share the same Tx resource pool. Long-term sensing/resource selection mechanism is used for periodic traffic and short-term sensing/resource selection mechanism is used for aperiodic traffic in the same Tx resource pool.

Conclusion

In this contribution we discussed resource allocation mechanism in NR V2X and followings are proposed,

Proposal 1: Multiple traffics are allowed to coexist in the same Tx resource pool and Rx resource pool..

Proposal 2: All UEs sharing the same Tx resource pool should be decodable of PSCCH, i.e. CRC of PSCCH of unicast/groupcast should not be scrambled by UE ID/group ID.

Proposal 3: Following three approaches are considered for coexistence of periodic traffic and aperiodic traffic:

- Approach 1: Periodic traffic and aperiodic traffic share the same resource pool. Long-term sensing/resource selection mechanism is used in the resource pool

- Approach 2: Periodic traffic and aperiodic traffic do not share the same Tx resource pool. The Tx resource pool with periodic traffic uses long-term sensing/resource selection mechanism. The Tx resource pool with aperiodic traffic uses short-term sensing/resource selection mechanism.

- Approach 3: Periodic traffic and aperiodic traffic share the same Tx resource pool. Long-term sensing/resource selection mechanism is used for periodic traffic and short-term sensing/resource selection mechanism is used for aperiodic traffic in the same Tx resource pool.

Reference

[1] RP-190766, New WID on 5G V2X with NR sidelink, LG Electronics, Huawei