3GPP TSG RAN WG1 Meeting #97    	            R1-1907696

Reno, USA, May 13th – 17th, 2019

Source:	Qualcomm Incorporated

Title:	Summary of Rel.15 maintenance for PDCCH and search space

Agenda Item:	7.1.3

Document for: 	Discussion and Decision

# Background

This contribution summarizes following contributions.

- R1-1906408	Draft CR on PDCCH-ConfigCommon and PDCCH-Config configuration for Scell	ZTE

- R1-1907490	Discussion on PDCCH monitoring for NR-DC	Huawei, HiSilicon

# R1-1906408

This draft CR proposes following change in TS38.213 Section 10.1:

If a UE is configured with one or more downlink bandwidth parts (BWPs), as described in Subclause 12, the UE can be configured with PDCCH-ConfigCommon and PDCCH-Config for each configured DL BWP on the serving cell, other than the initial DL BWP, as described in Subclause 12.

According to TS38.331, BWP-DownlinkCommon which can contain PDCCH-ConfigCommon, as well as BWP-DownlinkDedicated which can contain PDCCH-Config, can be configured for any serving cells. Therefore, the spirit of the draft CR is correct.

On the other hand, the text is still incorrect; according to TS38.331 Section B.2, in case of BWP option 1, PDCCH-ConfigCommon can only be configured for the initial DL BWP, and in case of BWP option 2, PDCCH-ConfigCommon and PDCCH-Config can be configured even for the initial DL BWP. See copied below:

# B.2 Description of BWP configuration options There are two possible ways to configure BWP#0 (i.e. the initial BWP) for a UE: 1) Configure BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon, but do not configure BWP-DownlinkDedicated or BWP-UplinkDedicated in ServingCellConfig. 2) Configure both BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon and configure at least one of BWP-DownlinkDedicated or BWP-UplinkDedicated in ServingCellConfig. With the first option (illustrated by figure B2-1 below), the BWP#0 is not considered to be an RRC-configured BWP, i.e. UE only supporting one BWP can still be configured with BWP#1 in addition to BWP#0 when using this configuration. The BWP#0 can still be used even if it does not have the dedicated configuration, albeit in a more limited manner since only the SIB1-defined configurations are available. For example, only DCI format 1_0 can be used with BWP#0 without dedicated configuration, so changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching. Figure B2-1: BWP#0 configuration without dedicated configuration With the second option (illustrated by figure B2-2 below), the BWP#0 is considered to be an RRC-configured BWP, i.e. UE only supporting one BWP cannot be configured with BWP#1 in addition to BWP#0 when using this configuration. However, UE supporting more than one BWP can still switch to and from BWP#0 e.g. via DCI normally, and there are no explicit limitations to using the BWP#0 (compared to the first option). Figure B2-2: BWP#0 configuration with dedicated configuration For BWP#0, the BWP-DownlinkCommon and BWP-UplinkCommon in ServingCellConfigCommon should match the parameters configured by MIB and SIB1 (if provided) in the corresponding serving cell.

Regarding the concerned text in TS38.213 Section 10.1, the information available from this text is also available by the other specs including TS38.331 B.2. Therefore, here our suggestion is to delete the whole sentence from the TS38.213 Section 10.1.

Proposal:

- Adopt following TP for TS38.213.

# R1-1907490

RAN1 agreed to introduce new UE capability signaling, pdcch-BlindDetectionMCG-UE and PDCCH-BlindDetectionSCG-UE, and new RRC parameters, pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, for Rel.15 NR-DC (including late drop) UEs. However, Alt.1 vs Alt.2 described in R1-1905914 is still an open issue that needs to be concluded to finalize the signaling details.

This contribution proposing following two aspects:

Proposal 1: A UE supporting NR-DC without reporting pdcch-BlindDetectionCA is configured with neither pdcch-BlindDetectionMCG-UE nor pdcch-BlindDetectionSCG-UE. Proposal 2: RAN1 confirms that for a NR-CA operation with only the fallback NR-CA band combination derived from a NR-DC band combination, the determination of PDCCH blind decoding capability follows the existing mechanism for NR-CA without involvement of any pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE

Regarding the above proposal 1, the discussion has been continued from the last RAN1 meeting. Companies’ views and discussions are summarized in R1-1905914, which are also copied below.

Status: - Alt.1 (a UE supporting NR-DC with not more than 4 CCs can report the agreed UE capability signalling, pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE): DOCOMO, Ericsson, Qualcomm, vivo, Nokia (OK), Samsung (OK) - Alt.2 (a UE supporting NR-DC with not more than 4 CCs shall not report the agreed UE capability signalling, pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE): Huawei, Nokia (OK), Samsung (OK) - One company (Huawei) still prefers Alt.2, and do not see the benefit of Alt.1 for the case with UE not reporting pdcch-BlindDetectionCA (i.e., not more than 4 CCs). Huawei's points: - The need of UE capability signalling, pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, is unclear, at least for a UE not reporting pdcch-BlindDetectionCA (i.e., not more than 4 CCs), while can accept these UE capability signalling for a UE reporting pdcch-BlindDetectionCA (i.e., more than 4 CCs). - Only async NR-DC could be the use-case of the UE capability signalling. Therefore, suggest to add one bullet in Alt.1 saying that network may ignore the UE capability signalling pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, in which case the UE shall be able to perform PDCCH BD same as for NR-CA with the same band combinations. - Network flexibility is lost, if the UE is allowed to report pdcch-BlindDetectionMCG-UE < max no. of CCs supported in MCG. Other companies' points: - It was already agreed that UE can report pdcch-BlindDetectionCA, pdcch-BlindDetectionMCG-UE, and pdcch-BlindDetectionSCG-UE, which is acceptable also for Huawei. Based on this agreements, a UE can report pdcch-BlindDetectionMCG-UE=2 and pdcch-BlindDetectionSCG-UE=2 for NR-DC with more than 4 CCs. It is quite natural to allow UE to report pdcch-BlindDetectionMCG-UE=2 and pdcch-BlindDetectionSCG-UE=2 also for NR-DC with not more than 4 CCs. - It was already agreed that for Rel.15 sync NR-DC, pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are supported. Rel.15 NR-DC is only for synchronous case. Therefore, sync vs async should not be the matter of this discussion. - Alt.1 does not lose network flexibility, while just increase UE flexibility. It is network's choice whether to configure NR-DC if a UE reports pdcch-BlindDetectionMCG-UE < max no. of CCs supported in MCG.

Proposal:

- Select either Alt.1 or Alt.2 from R1-1905914.

  - Send a LS to inform the decision to RAN2 (CC: RAN3).

  - Corresponding RAN1 specification update will be done by an email discussion.

# Appendix: Alt.1 and Alt.2 in R1-1905914

Alt.1:

The agreement is updated and agreed as follows:

- For NR-DC (including late drop UEs), ask RAN2 if it is possible to introduce RRC parameters pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, which configures the reference number of cells for blind detection for MCG and SCG, respectively, and RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively. If yes,

  - The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is

    - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and

    - FFS: [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA.

  - If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE,

    - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and

    - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA.

  - If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them).

  - pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling.

  - FFS: If the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE. the UE must support at least one NR-DC band combination where the UE behaviour of blind decoding operation is the same as in Rel.15 NR-CA.

  - For MCG and SCG, the UE calculates BD/CCE limit for the CGs by using pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively.

  - The value range of pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG is [1, 2, …, 15].

  - pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= the number of configured DL serving cells over CGs if the UE does not report pdcch-BlindDetectionCA.

  - For a UE configured with NR-DC, if the UE reports pdcch-BlindDetectionCA, the UE is always configured with pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG.

    - FFS: if the UE does not report pdcch-BlindDetectionCA.

  - The information exchange between MCG and SCG (if any) is up to RAN2/RAN3 to discuss.

- For a NR-CA operation with only the fallback NR-CA band combination derived from a NR-DC band combination, the determination of PDCCH blind decoding capability follows the existing mechanism for NR-CA without involvement of any pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE.

Alt.2:

The agreement is updated and agreed as follows:

- For NR-DC (including late drop UEs), ask RAN2 if it is possible to introduce RRC parameters pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, which configures the reference number of cells for blind detection for MCG and SCG, respectively, and RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively. If yes,

  - The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is

    - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and

    - FFS: [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA.

  - If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE,

    - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and

    - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA.

  - If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them).

  - pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling.

  - FFS: If the UE does not report pdcch-BlindDetectionCA, the UE must support at least one NR-DC band combination where the UE behaviour of blind decoding operation is the same as in Rel.15 NR-CA.

  - For MCG and SCG, the UE calculates BD/CCE limit for the CGs by using pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively.

  - The value range of pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG is [1, 2, …, 15].

  - pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= the number of configured DL serving cells over CGs if the UE does not report pdcch-BlindDetectionCA.

  - For a UE configured with NR-DC, if the UE reports pdcch-BlindDetectionCA, the UE is always configured with pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG.

    - FFS: if the UE does not report pdcch-BlindDetectionCA, the UE is not configured with pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG.

  - The information exchange between MCG and SCG (if any) is up to RAN2/RAN3 to discuss.