3GPP TSG-RAN WG1 Meeting #98	R1-1909689

Prague, Czech Republic, 26th – 30th August 2019

Source:	WI rapporteur (Futurewei)

Title:	RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT

Agenda Item:	6.2.2

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made for the Rel-16 WI on ‘Additional Enhancements for NB-IoT’ (WI code NBIOTenh3-Core; WID in RP-191576), until and including RAN1#98.

Notes:

- The WI objective from the WID in RP-191576 is inserted below for convenience.

- The list of submitted feature lead summaries is included.

- RAN2 agreements are summarised in R2-1911575.

# WI objective

Core part WI objective:

The objective is to specify the following set of improvements for machine-type communications for NB-IoT FDD. Improved DL transmission efficiency and/or UE power consumption: - Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3] - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4] Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4] - Both shared resources and dedicated resources can be discussed - Note: This is limited to orthogonal (multi) access schemes Scheduling enhancement: - Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2] - Enhancement of SPS can be discussed. Network management tool enhancement: - SON support for reporting of [RAN2, RAN3] - Cell Global Identity and strongest measured cell(s) (ANR) - Random access performance - Radio link failure (RLF), if needed Improved multi-carrier operation: - Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2, RAN4] - Specify support for quality reporting in connected mode for anchor and non-anchor carriers. The quality report is not carried in the physical layer. [RAN1, RAN2, RAN4]. - Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4] Mobility enhancement: - Specify power efficient NB-IoT mechanism which would assist idle mode inter-RAT cell selection for NB-IoT to and from LTE, LTE-MTC and GERAN [RAN2] Coexistence with NR - Specify NB-IoT resource reservation for at least FDD/TDD NB-IoT non-anchor carriers, at least in DL, to avoid resource overlap between NR and NB-IoT when NB-IoT is deployed within an NR carrier [RAN1, RAN2]. - For NB-IoT in-band, guard band and standalone operation co-existence with NR, including the case of NR configured in 15kHz SS Block SCS and the case of 30kHz SS Block SCS as specified in 38.101-1, investigate the following: [RAN4] - 15KHz, 30KHz, and 60KHz numerologies for NR FR1 concerned bands, with higher priority given first to 15kHz and then to 30kHz - Channel raster, PRB and subcarrier grid alignment between NB-IoT and NR - Study feasible NB-IoT carrier(s) placement allocation without RF backward compatibility impact and compatible with Rel’13 NB-IoT and Rel’15 NR, to operate simultaneously within various NR channel bandwidths - Study if the +6dB downlink RE power boosting can still be allowed for both in-band and guard band operation modes when co-existing with NR - Synchronization issue between NR and NB-IoT, including timing advance - Frequency band support in NB-IoT and NR - Testability applicable to RF - Coexistence between R15 NR and R13/R14/R15 NB-IoT. Note: After RAN1 concludes the objective on R16 NB-IoT coexistence aspects, evaluate coexistence between R15 NR and R16 NB-IoT. Connection to 5GC: - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of User Plane CIoT optimization - Support of EDT for Data over NAS and User Plane CIoT optimization - Support of Inter-UE QoS for data over NAS (resource prioritisation between different NB-IoT UEs) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC

Performance part WI objective:

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements.

# UE-group wake-up signal

RAN1#94:

R1-1809532 Feature summary of 6.2.2.1 UE-group wake-up signal Ericsson Agreement: UE-group WUSs are only multiplexed in the same carrier as associated PO - FFS TDM/CDM for UE-group NWUS multiplexing Agreement Rel-16 UE-group NWUS sequence should consider at least - Fallback to legacy UE behavior - Inter-cell interference randomization - UE group ID for different UE-group NWUS - Reuse of Rel-15 sequences is not precluded - Effect of sequence detection on UE complexity Agreement Study the RAN1 consequence of UE-grouping on the following basis: - UE ID - Coverage - DRX/eDRX - Gap configuration - Services

RAN1#94bis:

R1-1811664 Feature summary of UE-group wake-up signal for NB-IoT Ericsson R1-1811679 Further topics for UE-group wake-up signal for NB-IoT Ericsson Agreement UE-group NWUS is supported based on eNB’s and UE’s capability. Whether the network supports UE-group NWUS is done by higher layer signaling. FFS: The number of UE groups is configured by SIB. Note that the UE-group NWUS is UE optional Agreement From a RAN1, perspective, UE grouping is based on at least UE ID or some function of UE ID. Agreement The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled - Performance impact on legacy WUS should be carefully considered Agreement Group WUS is based on at least legacy WUS and UE-group ID. Agreement Configuration of group WUS is at least signaled in SI Agreement A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS Agreement Rel-16 UE-groups WUS with same DRX/eDRX gap configuration are multiplexed with TDM and/or single sequence CDM - FFS: Whether single sequence CDM can include legacy WUS - FFS: Multiplexing between Rel-15 and Rel-16 UE groups - Note: Single sequence CDM is where different sequences share the same resource and only one sequence is transmitted at a given time

RAN1#95:

R1-1813714 Summary of 6.2.2.1 Group WUS for NB-IoT Ericsson R1-1813756 Further topics on 6.2.2.1 Group WUS for NB-IoT Ericsson Agreement UE group ID is used as a parameter to generate WUS UE group sequence(s) Agreement One group WUS is designed as a single sequence Agreement Further study false detection (cross/auto correlation) performance properties for the following designs: - legacy WUS + cover codes, - legacy WUS + shifted scrambling codes, - legacy WUS + phase shift + cover code + scrambling bits - Including combinations of phase shift, cover code, and/or scrambling bits Other designs are not precluded. Agreement The number of UE groups is configurable and broadcasted in SIB. - FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO. Agreement Multiplexing of Rel-16 UE groups is down-selected among - Single sequence CDM is supported - Single sequence CDM is supported and TDM can be additionally applied - FFS: Configurability of CDM and/or TDM Agreement A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource. - FFS: Whether the above is also applicable for Rel-15 WUS UEs - FFS: Whether to support waking up a subset of all WUS UE groups Agreement For multiplexing between Rel-16 UE-group WUS and Rel-15 legacy WUS within the same carrier, further evaluate and down select among the following options - TDM - single-seq. CDM - single-seq. CDM+TDM FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs. Agreement Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM. - No new gap higher layer signaling will be introduced for TDM

RAN1#96:

R1-1903247 Feature lead summary for Group wake-up signal for NB-IoT Ericsson R1-1903508 Further topics for 6.2.2.1 Group WUS for NB-IoT Ericsson For further evaluation and study (especially from UE implementation perspective): Benefits and costs with more grouping levels, i.e., subgroups of subgroups. Agreement A UE is required to monitor WUS(s) in only one WUS (T/F) resource location Agreement Up to 2 time-multiplexed WUS resources may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not. For further study until next meeting Companies are encouraged to provide results for the different sequence candidates, taking into account, e.g., - Detection window lengths - Frequency errors - Channel model where a starting point for evaluations is the Rel-15 WUS assumptions in R1-1714993 and Rel-15 WUS functionality (w.r.t. frequency error and detection window lengths).

RAN1#96bis:

R1-1905536 Feature lead summary of UE-group wake-up signal in NB-IoT Ericsson R1-1905583 Further topics for UE-group wake-up signal in NB-IoT Ericsson Agreement Up to 2 time-multiplexed WUS resources, for both legacy WUS and group WUS, may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not. Agreement Group WUS location in relation to legacy WUS may be configured such that: - If one group WUS resource is configured, that group WUS resource may be configured to coincide with the legacy WUS resource or to occur immediately before the legacy WUS resource, and, - If two group WUS resources are configured, the first group WUS resource coincides with the legacy WUS resource and the second group WUS resource occurs immediately before the first group WUS resource. Agreement Group WUS sequence design is down selected between the following in RAN1#97 - Legacy WUS with shifted scrambling codes - Legacy WUS with time-frequency short OCCs - Phase shifted legacy WUS using a suitable subset of the phase increments. - Legacy WUS with Gold codes Companies are encouraged to provide evaluation results Agreement - Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation. - Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

RAN1#97:

R1-1907578 Feature lead summary of UE-group wake-up signal in NB-IoT Ericsson Working Assumption At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to wgroup(m’) = w(m’) exp(j2πgm/G) - FFS: Details of g and G Agreement If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, the common WUS sequence for all the group WUS UEs in the same WUS resource can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence. Working Assumption The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence. Working Assumption The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence. Agreement Each UE monitors up to X WUS sequences. - Value of X will be selected between 2 and 3 - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

RAN1#98:

R1-1909489 Feature lead summary of UE-group wake-up signal in NB-IoT Ericsson R1-1909716 Remaining topics for UE-group wake-up signal in NB-IoT Ericsson Agreement The maximum number of UE groups per WUS resource is 8. Agreement The following WA is confirmed: At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to w_(group)(m’) = w(m’) exp(j2πgm/G) Agreement The specification supports configurability to enable UE group to alternate between WUS resources Agreement - Different WUS resources use different scrambling initialization, c_init Agreement A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs Agreement G = 132 and g = 14*(UE_group_index+1), 0 ≤ UE_group_index ≤ 7 Working Assumption The sequence resulting from g = 126 is the common WUS unless common WUS is configured to be legacy WUS Agreement The following working assumption is confirmed with the modification and under the condition that the eNB can set the power offset between Rel-15 and Rel-16 sequences (UE does not need to know the power offset) - UE may assume the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence. - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

# Support for transmission in preconfigured UL resources

RAN1#94:

R1-1809571 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon Agreement Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA - FFS: Validation mechanism for TA - FFS: How the pre-configured UL resources is acquired Agreement For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed Agreement Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types. Agreement HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered: - Whether to support HARQ; - If supported, details of HARQ design including the number of HARQ processes; - Whether ACK/NACK is necessary Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

RAN1#94bis:

R1-1811697 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1811678 Summary of offline discussions on PUR for eMTC and NB-IoT Huawei, HiSilicon Agreement In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed): - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on) - FFS Other attributes: - Neighbour cell NRSRP change - TDOA of >=2 eNBs - TA History - Subscription based UE differentiation - Others not precluded (for example, attributes that need to be considered for high mobility UEs) Note that UE power consumption should be taken into account for the FFS attributes Agreement For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria Agreement Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported. Agreement The resource configuration includes at least the following - Time domain resources including periodicity(s) - Frequency domain resources - TBS(s)/MCS(s) Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement Dedicated preconfigured UL resource is defined as an NPUSCH resource used by a single UE - NPUSCH resource is time-frequency resource - Dedicated PUR is contention-free Contention-free shared preconfigured UL resource (CFS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE - NPUSCH resource is at least time-frequency resource - CFS PUR is contention-free Contention-based shared preconfigured UL resource (CBS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE - NPUSCH resource is at least time-frequency resource - CBS PUR is contention-based (CBS PUR may require contention resolution) Agreement In IDLE mode, HARQ is supported for transmission in dedicated PUR - A single HARQ process is supported, - FFS whether two HARQ processes are supported - FFS: The design of the corresponding NPDCCH search space Agreement In idle mode, dedicated PUR is supported. - Support for CFS PUR is FFS. - Support for CBS PUR is FFS.

RAN1#95:

R1-1813717 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1813762 Further topics for transmission in preconfigured UL resources Huawei, HiSilicon - Agreement - In idle mode, at least the following TA validation attributes are supported: - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on) - Based on NRSRP measurement definition in existing Rel-15 TS36.214 Send LS to RAN2, RAN4 to consider in their work. All the agreements with ‘include in LS to RAN2, RAN4’ for NB-IoT and eMTC should be captured in this LS. The LS is endorsed in R1-1813778 - Agreement - The UE can be configured to use at least these TA validation attributes: - Time Alignment Timer for idle mode - Serving cell NRSRP changes - Note: the configuration shall support disabling of the TA validation attributes Include in LS to RAN2, RAN4. For further study: TA validation attributes: - Subscription based UE differentiation (or Stationary indication in held in subscription) - Cell specific indication where TA is valid within that cell Agreement Include in LS to RAN2, RAN4: RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode. Agreement For dedicated PUR in idle mode, the UE may skip UL transmissions. - FFS: Resource release mechanism - FFS: Whether or not to support mechanism to disable skipping by eNB Agreement In idle mode, only one HARQ process is supported for dedicated PUR Agreement For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in search space - FFS: Details on the search space (for example USS, CSS)

RAN1#96:

R1-1903254 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1903513 Updated feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon Agreement When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied. Agreement For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuratn. - PUR search space is the search space where UE monitors for NPDCCH - FFS: Whether PUR search space is common or UE specific Agreement In idle mode, the TA validation configuration can include “PUR Time Alignment Timer” - Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer - Details on how to specify the “PUR Time Alignment Timer” is up to RAN2 Agreement In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes - Above applies for the case where the UE is configured to use the serving cell change attribute Agreement For dedicated PUR in idle mode, the dedicated PUR ACK is at least sent on NPDCCH - FFS: Whether to introduce new field in DCI or reuse existing field - RAN2 can decide if a higher layer PUR ACK is also supported Agreement When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures - FFS whether only TA is acquired and then data sent on PUR is supported - FFS other approaches to obtain a valid TA Agreement In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission: - Timing advance adjustment - UE TX power adjustment - FFS: Repetition adjustment for NPUSCH FFS: Whether the above update is done in L1 and/or higher layer Agreement In idle mode, the PUR search space configuration includes at least the following: - NPDCCH repetitions and aggregation levels - NPDCCH starting subframe periodicity (variable G) - Starting subframe position (alpha_offset) Agreement For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following - Time domain resources including periodicity(s) - Note: also includes number of repetitions, number of RUs, starting position - Frequency domain resources - TBS(s)/MCS(s) - Power control parameters - Legacy DMRS pattern

RAN1#96bis:

R1-1905538 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1905585 Updated feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon Agreement In idle mode, a UE can be configured such that TA is always valid within a given cell. - Up to RAN2 how to implement - e.g. PUR Time Alignment Timer or NRSRP Threshold = infinity Agreement The value(s) of NRSRP threshold(s) is UE specific Agreement The UE monitors the NPDCCH for at least a time period after a PUR transmission. - FFS: Details of the time period - FFS: UE behaviour if nothing is received in that time period. - FFS: If and how often UE monitors NPDCCH after a PUR allocation in which it has not transmitted Agreement Reuse existing field(s) of DCI format N0 to convey the dedicated PUR ACK Agreement After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH. Other behaviors are FFS. - Working Assumption#1 - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters - The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted. - Working Assumption#2 - For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources - Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. (LS is endorsed in eMTC agenda item) Agreement For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

RAN1#97:

R1-1907573 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1907627 Further topics for transmission in preconfigured UL resources Huawei, HiSilicon Agreement For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission - FFS: Value of x, and if x is fixed or signaled - FFS: Support for monitoring of PUR SS Window before PUR transmission Note: The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission Agreement NPDCCH candidates are determined by USS like search space - FFS: Other details on the USS like search space - Type2-CSS can also be discussed as part of the FFS Conclusion CBS PUR is not supported in Rel-16 For further discussion - Aspects related to notifying eNB of unused PUR resources. - Potential enhancements of power control mechanisms for PUR. (The baseline is the existing NB-IoT open loop power control.)

RAN1#98:

R1-1909556 Feature lead summary of Support for transmission in preconfigured UL resources Futurewei R1-1909688 Summary of offline discussion on PUR for NB-IoT and eMTC Futurewei, Sierra Wireless Agreement Confirm the Working Assumption#1 in RAN1#96bis - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters Agreement Confirm the Working Assumption#2 in RAN1#96bis - For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources Agreement For dedicated PUR in idle mode, the maximum NPDCCH repetitions, X, is included in the PUR configuration and the PUR NPDCCH candidates are derived using legacy USS rules where rmax= X. Agreement RAN1 will downselect among the following indications for PUR L1 ACK in RAN1#98bis Timing advance adjustment (including TA adjustment of 0) UE TX power adjustment NPUSCH repetition adjustment Indication of PUR SS monitoring termination Flag to indicate for L1 ACK None of the above Note: No new indications will be considered. Agreement After data transmission on PUR, if nothing is received by the UE in a time period, the UE shall fallback to legacy RACH/EDT procedure. FFS: Details on time period Agreement Open loop power control is used as power control mechanism for PUR. - The power for PUR transmission is calculated based on pathloss regardless of the number of repetitions. Agreement For dedicated PUR in idle mode and for FDD NB-IoT UEs, the start of the PUR SS Window is 4 subframes after the end of the PUR transmission. Agreement The dedicated PUR ACK DCI at least allows for indicating implicitly or explicitly that the current PUR monitoring is terminated - FFS: The dedicated PUR ACK DCI at least allows for indicating whether the UE continues monitoring the PUR search space after a gap (whether the gap is always zero or it can be zero) Agreement After data transmission on PUR, the UE may expect an explicit indication on NPDCCH for fallback to EDT or RACH Working assumption For NB-IoT allocation with 12 tones, for PUR with R>= [64 or 128 repetitions or ms]: - Allow for UE-specific cyclic shift for DMRS: - For 12 tone allocation, [2 or 4 or 8] cyclic shifts. - For NPUSCH scrambling, the baseline is the current c_init equation by using the configured PUR_RNTI. - Companies are welcome to evaluate baseline c_init, alternative c_init (e.g. c_init for NPDSCH rotation sequence) equations, and other proposals. Note 1: The Working Assumption is also subject to the potential RAN2 and RAN4 specification impacts. Note 2: It is transparent to a given UE whether the eNB is allocating other UEs in the same resource.

# Scheduling of multiple DL/UL transport blocks

RAN1#94:

R1-1809523 Feature lead summary of scheduling enhancement for NB-IoT ZTE Agreement For unicast, scheduling multiple DL/UL transport blocks with single DCI is supported. Agreement - One DCI to schedule multiple TBs for SC-MCCH is not supported Agreement For Unicast, the possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD Agreement For unicast, the number of TBs scheduled should be dynamically indicated in the DCI, the maximum number of TBs is FFS

RAN1#94bis:

R1-1811660 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement The UE should only monitor one DCI size in the UE specific search space. Working Assumption For UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE Agreement Individual feedback for each HARQ process is supported. FFS if HARQ bundling/multiplexing can be optionally supported. Agreement Using one DCI to schedule multiple TBs for SC-MTCH is supported, and it is configured and enabled per SC-MTCH via SC-PTM configuration message in SC-MCCH.

RAN1#95:

R1-1813740 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE R1-1813757 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement For multi-TBs scheduling - UL: I_sc for each TB is same Agreement Confirm the working assumption that for UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE. Agreement For UL/DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI. - ‘consecutive resource allocation in time’ means no new scheduling gap between the end of previous TB and the start of the next TB FFS: Whether scheduling gaps is also supported FFS: How to schedule repetitions within the consecutive resource allocation Agreement For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, the relationship(s) between HARQ process and TB is/are selected from the following two candidates(multiple choices are allowed) - Relationship 1: 1 HARQ process corresponds to 1 TB - Relationship 2: 1 HARQ process corresponds up to 2 TBs Agreement Maximum UL HARQ process supported is 2. Agreement Maximum DL HARQ process supported is 2. Agreement The maximum number of TBs for multicast is one of [4, 8] - FFS: Whether the TBs are back to back without gap

RAN1#96:

R1-1903257 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement One DCI can be used to schedule both initial and retransmission of different HARQ processes. Agreement For unicast, when all the TBs are scheduled by one DCI - MCS, repetition number, resource allocation, are common across all UL transport blocks - There is a single field for each of the following as in Rel-15: Scheduling delay, DCI subframe repetition number, Flag for differentiation - MCS, repetition number, resource assignment are common across all DL transport blocks - There is a single field for each of the following as in Rel-15: Scheduling delay, DCI subframe repetition number, NPDCCH order indicator, Flag for differentiation - FFS: HARQ-ACK resource Agreement For unicast, relationship 1 is supported: 1 HARQ process corresponds to 1 TB - FFS: Whether to support relationship 2 (1 HARQ process corresponds up to 2 TBs) in addition to relationship 1 - RAN1 will make decision on the support for the FFS part in RAN1#96bis Agreement For unicast, scheduling gaps between TBs scheduled by one single DCI are not supported for relationship 1 Agreement For TBs scheduled by one DCI that are contiguous, the ACK/NACK resources are back-to-back. FFS details. Agreement For SC-MTCH, the maximum number of TBs scheduled is 8. Agreement For SC-MTCH, all the TBs scheduled by one DCI use the same resource assignment, MCS and repetition number. Agreement For SC-MTCH multiple TBs scheduling, down-select from the following options: - Modify existing DCI to indicate the number of scheduled TBs (e.g. by adding new field) - Reuse Rel-15 DCI and use SC-MCCH to indicate TB numbers. - Support both a) and b)

RAN1#96bis:

R1-1905573 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement - 1 bit for RV indication in UL transmission is used regardless of the number of TBs - Common RV indication is mapped to both TBs Working Assumption 3 bits are used to indicate scheduled TB number, HARQ process index, NDI for HARQ operation - FFS: Details coding scheme of these 3 bits Conclusion Relationship 2 is not supported in Rel-16. Agreement In case 2 TBs are scheduled in the downlink, the timing of the ACK/NACKs for the scheduled TBs is with respect to the last TB scheduled by the DCI, detailed value FFS. - For the case of 1 TB scheduling, legacy UE behavior is maintained Agreement For SC-MTCH multiple TBs scheduling, select one from the three options in RAN1#97 - Modify existing DCI to indicate the number of scheduled TBs (e.g. by adding new field) - Reuse Rel-15 DCI and use SC-MCCH to indicate TB numbers. - Support both a) and b)

RAN1#97:

R1-1907567 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement Confirm the working assumption last meeting: 3 bits are used to indicate scheduled TB number, HARQ process index, NDI for HARQ operation - FFS: Details coding scheme of these 3 bits Agreement The coding scheme of the three bits (scheduled TB number, HARQ process index, NDI for HARQ operation) is as follows: Codepoint; Description. 000; Single TB scheduling, HARQ ID=0, NDI=0. 001; Single TB scheduling, HARQ ID=0, NDI=1. 010; Single TB scheduling, HARQ ID=1, NDI=0. 011; Single TB scheduling, HARQ ID=1, NDI=1. 100; Multi-TB scheduling, NDI=00. 101; Multi-TB scheduling, NDI=01. 110; Multi-TB scheduling, NDI=10. 111; Multi-TB scheduling, NDI=11. Table 1: Joint coding scheme of scheduled TB number, HARQ process index, NDI for HARQ operation - Only one additional bit is added in the DCI format to support the above indication of 8 states Note: How to capture the above agreement is up to the editor Agreement In case of non-interleaved transmission, for individual feedback of 2 TBs case, continuous uplink feedback starts, after the end of DL subframe for FDD, where n is the ending subframe of last scheduled TB and k0' is down-selected from the following two choices: - The same value as the one for legacy one TB case (i.e. reuse the existing specification without change) - New values are introduced which depends on the length of last TB and ACK/NACK resources - Existing values can also be used Agreement HARQ-ACK resource field is common across the HARQ-ACK feedback for all the DL TBs scheduled. - ACK/NACK subcarriers are the same across all the TBs

RAN1#98:

R1-1909535 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement Legacy HARQ delay timing constraint is used, i.e. the interval between the end of the corresponding DL TB and the start of ACK/NACK transmission is >= 12ms For next meeting On the issue of new values for companies are encouraged to submit detailed proposals for decision on whether to support such values in RAN1#98bis Conclusion In Rel-16, HARQ multiplexing for multiple TB scheduling is not supported. - FFS if HARQ bundling can be optionally supported. Agreement For unicast, for a Rel-16 UE configured with multiple TB scheduling, after receiving NPDCCH with a DL (UL) grant ending in subframe n, and if the corresponding NPDSCH(NPUSCH format 1) transmission starts from n+k, the UE is not required to monitor NPDCCH starting from subframe n+k-2 to subframe n+k-1. - If two TBs are scheduled by the DCI, the UE is not required to monitor another NPDCCH from subframe n+1 to subframe n+k-1 Agreement Non-continuous transmission between SC-MTCH TBs is supported - Details FFS (including UE capability and continuous transmission) Agreement For unicast, for a Rel-16 UE configured with multiple TB scheduling: - When one TB is scheduled by the DCI, the repetitions for one transport block are contiguously transmitted - When multiple TBs are scheduled by the DCI - The repetitions for one transport block can be either contiguously transmitted or interleaved. - Interleaving is an eNB configured feature - FFS: Interleaving feature is a UE optional feature

# Coexistence of NB-IoT with NR

RAN1#94:

R1-1809572 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Observation From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and NB-IoT. Agreement RAN1 studies additional specification enhancement for improving the performance of coexistence of NB-IoT with NR.

RAN1#94bis:

R1-1811698 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Agreement The following aspects are studied to improve the performance of coexistence of NB-IoT with NR - Resource reservation in NB-IoT - Overlap of NR SSB with NB-IoT

RAN1#95:

R1-1813718 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Agreement RAN1 continues to study the following techniques for performance improvements of NB-IoT resource allocation until the next meeting: - Resource reservation at symbol level/slot level/subframe level/subcarrier level - Whether the resource reservation for NB-IoT is dynamic or semi-static (if supported) - Whether and how to support NB-IoT transmission in a portion of the subframe - Impact of resource reservation to legacy UEs - Whether NB-IoT transmission is postponed or dropped in reserved resources - Whether resource reservation is used for anchor/non-anchors Conclusion Overlap of NR SSB with NB-IoT anchor carrier can be avoided.

RAN1#96:

R1-1903255 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon For further study in future meeting (including): - NB-IoT resource reservation on non-anchor carrier to improve the performance of coexistence of FDD/TDD NB-IoT with NR - Subframe-level resource reservation in NB-IoT - Symbol-level resource reservation in NB-IoT - Slot-level resource reservation in NB-IoT For further study in future meeting (including):: NB-IoT transmission is dropped/postponed in reserved resources. Conclusion From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and FDD/TDD NB-IoT.

RAN1#96bis:

R1-1905539 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon R1-1905586 Updated feature lead summary of coexistence of NB-IoT with NR Huawei, HiSilicon For further study until RAN1#97: - RAN1 continues to study finer-granularity NB-IoT resource reservation until RAN1#97. - Assume that the NB-IoT resource reservation should accommodate at least part of NR initial CORESET, NR CSI-RS and NR TRS. - The reference case is the existing DL/UL subframe-level NB-IoT resource reservation for TDD NB-IoT. For FDD NB-IoT, the reference case is the existing DL subframe-level NB-IoT resource reservation. - Assume that the resource reservation is semi-statically configured. It is FFS whether the eNB can furthermore dynamically override or indicate additional reserved resource via DCI. - Companies are encouraged to consider the impact on UE complexity for DCI based indication - Consider whether the time-domain granularity of the NB-IoT valid subframe configuration should be scaled dependent on the NR subcarrier spacing. - Consider both DL and UL aspects. - NPDSCH, NPUSCH, NPDCCH - It is FFS whether and when to drop or postpone NB-IoT transmissions that would fall into NB-IoT reserved resources (if introduced). Conclusion RAN1 concludes that overlap between NR SSB and NB-IoT non-anchor carriers can be avoided using the NB-IoT DL valid subframe bitmap. For further study until RAN1#97: RAN1 continues to study the aspects of coexistence of NB-IoT and NR using different SCSs.

RAN1#97:

R1-1907574 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Conclusion Based on the study of aspects of FDD/TDD NB-IoT coexistence with NR, it is RAN1 recommendation to introduce DL/UL resource reservation in NB-IoT for at least FDD/TDD NB-IoT non-anchor carriers to avoid time and frequency domain resource overlap between NR and NB-IoT when NB-IoT is deployed within an NR carrier. Resource reservation in FDD/TDD NB-IoT considers the following aspects: - sub-frame/slot/symbol level granularity - dropped/postponed resources associated to a certain granularity - semi-static configuration is recommended. FFS: dynamic DCI signaling - UL valid/invalid subframe configurations for FDD NB-IoT Agreement Include above conclusion as part of LS to RAN The LS is endorsed in R1-1907635

RAN1#98:

R1-1909557 Feature lead summary of Coexistence of NB-IoT with NR Futurewei Agreement Specify NB-IoT resource reservation only for FDD/TDD NB-IoT non-anchor carriers. Agreement UL resource reservation for NB-IoT is supported. Agreement - FDD UL resource reservation is supported with subframe-level granularity - FFS: Slot-level, symbol-level - DL resource reservation is supported with slot-level and symbol(s)-level granularity. Agreement The reserved resource in NB-IoT non-anchor carrier is semi-statically configured by higher layer signalling. - FFS whether the resource in an invalid and/or valid subframe indicated by legacy downlink bitmap can be configured as the reserved resource - FFS signalling Agreement NB-IoT transmission can be postponed or dropped depending on the granularity of the reserved resources. - NB-IoT transmission is dropped for symbol-level reserved resources. - NB-IoT transmission is dropped for slot-level reserved resources. - NB-IoT transmission is postponed for subframe-level reserved resources.

# Support of Quality report in Msg3 for non-anchor access

RAN1#94:

R1-1809600 Feature lead summary on support of quality report in Msg3 for non-anchor access Huawei, HiSilicon Agreement For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1% - FFS: Whether the details on the hypothetical NPDCCH are specified or not Working Assumption For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR) - FFS: Whether the UE performs measurement on other carriers Agreement For non-anchor access, RAN1 further studies how UEs report the measured channel quality.

RAN1#94bis:

R1-1811699 Feature lead summary of support of quality report in Msg3 for non-anchor access Huawei, HiSilicon Agreement RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access. Agreement From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed Agreement RAN1 does not define measurement reference resource for non-anchor access. For further study: The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access. - For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE - PUR

RAN1#95:

R1-1813719 Feature lead summary of support of quality report in Msg3 for non-anchor access Huawei, HiSilicon Agreement In case 4 bits is used for a non-anchor carrier, all repetition i.e. 12 candidate values {1,2,4,8,16,32,64,128,256,512,1024,2048} can be reported in Msg3. Agreement In case of 2 bits is used for a non-anchor carrier, 3 candidate values can be reported in Msg3. Select one of the following alternatives for determining the 3 values: - Depending on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. - Depending on R, "DCI subframe repetition number" indicated in DCI format N1 for Msg2 scheduling. - Depending on Rdecoded, based on the number of repetitions for NPDCCH scheduling Msg2 where UE decodes successfully.

RAN1#96:

R1-1903256 Feature lead summary of Support of Quality report in Msg3 for non-anchor access Huawei, HiSilicon For further study in future meetings: For channel quality report in Msg3 on non-anchor access, RAN1 identified the following scenarios in which UE may monitor a carrier that is different from the carrier in which the UE receives Msg2 (i.e. RAR): - Idle mode - Non-EDT random access procedure - EDT procedure - Connected mode - NPDCCH ordered NPRACH procedure - Random access procedure to apply for UL-SCH resource Agreement For the measurement on carrier(s) other than the one UE is receiving RAR for non-anchor access, if supported, RAN1 to select one or more among the following candidates: - Paging carrier - Anchor carrier - Carrier(s) configured by CarrierConfigDedicated-NB in connected mode - Other carriers configured by network with implicit or explicit signaling The following issues are identified for RAN1 further study - Whether amount of time or gap is needed for the measurement - Which carrier(s) UE reported if more than one carrier is measured - What kind of NRS UE can use for the measurement and whether needs indication Agreement In case of 2 bits are used for a non-anchor carrier, one of 3 candidate values can be reported in Msg3 - Which depends on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. For further study in future meetings: If supported, RAN1 further discuss how to derive the candidate values reported in Msg3 if no NPDCCH Type2 CSS on the carrier.

RAN1#96bis:

R1-1905540 Feature lead summary of quality report in Msg3 and connected mode Huawei, HiSilicon R1-1905587 Updated Feature lead summary of Support of Quality report in Msg3 for non-anchor access and connected mode Huawei, HiSilicon Agreement For channel quality report in connected mode, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%. Agreement For channel quality report in connected mode other than Msg3, UE performs measurement on the carrier it is assigned to monitor in USS for NPDCCH and the associated NPDSCH. - FFS other carrier(s) Agreement For channel quality report in Msg3 for non-anchor access, RAN1 send LS to RAN2 requesting the following information: - Number(s) of available bits for the quality reporting in Msg3 - Whether the quality report in Msg3 is transmitted in RRC or MAC CE Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases R1-1905592 [DRAFT] Draft LS on channel quality report in Msg3 for NB-IoT and LTE-M Huawei, HiSilicon The LS is endorsed in R1-1905593 Agreement For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is no larger than 2. - FFS detail of the maximum number (1 or 2) Agreement eNB can enable/disable channel quality report in Msg3 for non-anchor access via SIB - FFS details - FFS whether the field in Rel-14 for anchor access is reused

RAN1#97:

R1-1907575 Feature lead summary of support of quality report in Msg3 and connected mode Huawei, HiSilicon R1-1907633 Further topics for quality report in Msg3 and connected mode Huawei, HiSilicon Agreement For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1. Agreement For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported. - FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI) Agreement For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4. - number of candidate values for channel quality reports - mapping value of reported channel quality - report in RRC message or MAC CE LS to RAN2 and RAN4 to convey this agreement is endorsed in R1-1907624. For further discussion For channel quality report in connected mode other than Msg3, RAN1 further discuss whether physical layer triggering mechanism is needed. For further discussion RAN1 further discuss the followings for channel quality report in Msg3 on non-anchor access in connected mode - The support of channel quality report in Msg3 in connected mode - If supported, which carrier can be measured for the channel quality report in Msg3 - RAR carrier - unicast carrier - If supported, how to trigger the report in Msg3

RAN1#98:

R1-1909588 Feature lead summary for support of quality report in Msg3 and connected mode Huawei, HiSilicon Conclusion For channel quality report in Msg3 on non-anchor access in idle mode, RAN1 does not further discuss any other carrier than the one UE monitors to receive Msg2 (i.e. RAR) in Rel-16. For next RAN1 meeting On the issue of supporting channel quality report in Msg3 on non-anchor access in connected mode for Rel-16, companies are encouraged to consider whether to support this feature considering recent RAN2 decision, details on triggering mechanism, the measurement carrier, etc Conclusion No consensus on the support of physical layer triggering for channel quality report in connected mode (other than Msg3) in Rel-16.

# Presence of NRS on a non-anchor carrier for paging

RAN1#94:

R1-1809522 Feature lead summary of presence of NRS on a non-anchor carrier for paging Qualcomm Agreement (confirmed by Friday) The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by broadcast signalling. - Above applies only for non-anchor carriers Agreement (confirmed by Friday) For the objective on presence of NRS on a non-anchor carrier for paging, consider the following use case: - Early termination of NPDCCH / WUS Above is not intended to restrict the study of other use cases in other WGs which may or may not have RAN1 specification impact.

RAN1#94bis:

R1-1811326 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Europe Inc. (Spain) Agreement The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by SIB. Agreement Subframes which will contain NRS even when no paging NPDCCH is transmitted are associated to a PO. - FFS: Whether PO is from UE perspective or NW perspective Agreement A subset of the POs have associated subframes which will contain NRS even when no paging NPDCCH is transmitted. FFS: Details of the subset, whether it depends on some existing parameters such as DRX cycle or new parameters by configuration FFS: Whether the subset can be the whole set of POs by configuration.

RAN1#95:

R1-1813716 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Agreement RAN1 to discuss the issue of NRS presence for NPDCCH early termination when NWUS is enabled. Down-select among: - Alt1: If NWUS is enabled (from network perspective), there is no NRS for NPDCCH early termination, but there may be NRS for NWUS early termination. - Alt2: If NWUS is enabled (from UE perspective, i.e., enabled by network and supported by UE), there is no NRS for NPDCCH early termination, but there may be NRS for NWUS early termination. - Alt3: Decouple configuration of NRS for NPDCCH early termination and NWUS early termination - e.g. eNB can enable NRS for NWUS and disable for NPDCCH, or vice versa. - Alt4: NRS is always associated with NPDCCH for paging Agreement In the POs that have associated subframes which will contain NRS even when no paging NPDCCH is transmitted for NPDCCH early termination: - NRS is present in the first M subframes out of the 10 NB-IoT DL subframes before the PO, and the N first NB-IoT DL subframes of NPDCCH search space - FFS value(s) of N, M Agreement The subset of POs that have associated subframes which will contain NRS even when no paging NPDCCH is transmitted can be the whole set of POs.

RAN1#96:

R1-1903251 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Incorporated R1-1903515 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Incorporated Agreement For the issue of NRS presence for NPDCCH early termination when NWUS is enabled, down-select among: - Alt3: Decouple configuration of NRS for NPDCCH early termination and NWUS early termination - e.g. eNB can enable NRS for NWUS and disable for NPDCCH, or vice versa. - Alt4: NRS is always associated with NPDCCH for paging For further study in future meetings: RAN1 to consider enabling presence of CRS in non-anchor carriers in subframes not containing NRS. Agreement The decimation pattern (i.e., the pattern that determines which POs have subframes with NRS even when no NPDCCH is transmitted) is designed at least with the following principles: - P1: The decimation pattern shall be fair across UEs, i.e., all UEs see the same/similar percentage of POs with NRS. - FFS: the case for eDRX - P2: A UE belonging to a given UE group (being a UE group the group of UEs that monitor paging in the same PO) can use NRS belonging to a PO of a different group in addition to the NRS of its own UE group. - The maximum gap between the PO with NRS and the PO the UE monitors is not larger than X (to ensure that the UE can reliably estimate the SNR for NPDCCH early termination) - P3: The POs with NRS are quasi-uniformly/uniformly distributed from UE perspective. - P4: The POs with NRS are quasi-uniformly/uniformly distributed from network perspective. NOTE: It was already agreed in previous meetings that a subset of the POs have associated subframes which will contain NRS even when no paging NPDCCH is transmitted. Further study details of decimation pattern until RAN1#96bis.

RAN1#96bis:

R1-1905531 Presence of NRS on a non-anchor carrier for paging (feature lead summary) Qualcomm Conclusion There is no consensus in RAN1 to support configuration of NRS for WUS early termination in Rel-16. Agreement The subset of POs that have subframes with NRS even when no NPDCCH is transmitted is determined based at least on nB-r13/nB - FFS: defaultPagingCycle-r13/T - Note: The paging frame and the number of UE groups can be derived from T and nB Agreement For the NRS pattern: Define detailed pattern of (M,N) depending on the value of nB: - If nB >= X, specify the decimation pattern/M/N for each nB. - FFS if the value of M/N is different for different POs within a DRX cycle - For nB < X, all POs have NRS. FFS value of M/N. where X is a fixed value which will be determined in RAN1#97.

RAN1#97:

R1-1907571 Presence of NRS on a non-anchor carrier for paging (feature lead summary) Qualcomm For further discussion Companies are encouraged to provide detailed decimation patterns to RAN1#98 Agreement With regards to the RAN1#96bis agreement: - If nB >= X, specify the decimation pattern/M/N for each nB. - FFS if the value of M/N is different for different POs within a DRX cycle - For nB < X, all POs have NRS. FFS value of M/N. It is agreed that the value of X is T/2. Agreement For nB < X, the value of M+N is 10 - FFS: Whether the values of M and N are configurable Agreement For nB>=X, the decimation/N/M pattern guarantees that there are [10] subframes with NRS nearby every PO (after decimation pattern, and from network perspective) Agreement For the decimation pattern for nB>=X, RAN1 to decide for each nB value: - M/N - Decimation pattern - FFS: Offset (if used)

RAN1#98:

R1-1909395 Presence of NRS on a non-anchor carrier for paging (feature lead summary) Qualcomm Incorporated R1-1909789 Presence of NRS on a non-anchor carrier for paging (feature lead summary) Qualcomm Incorporated Agreement - For nB<T/2, M=10, N=0 Agreement The decimation factor (fraction of POs that have NRS) is: - T/2: Decimation factor of 1/2 - T: Decimation factor of 1/2 - 2T: Decimation factor of 1/2 - 4T: Decimation factor of 1/2 For discussion in RAN1#98bis For the cases where decimation is applied, the decimation pattern is based on the following formula: - R = (PO_Index+(X div T)) mod 2, where: - PO_Index is the index of the PO within one DRX cycle: PO_Index = (SFN/Ns * N + i_s) mod T - X = SFN + 1024 * H-SFN - T is the DRX cycle measured in radio frames. - If R=1, the PO has NRS associated with it. If R=0 the PO does not have NRS associated with it. - FFS: How to capture the above in the specification Conclusion Presence of CRS in non-anchor carriers in subframes not containing NRS is not supported in Rel-16. Assumption of presence of CRS is the same as in Rel-15 - (from 36.211) The cell-specific reference signals are available in all subframes where the narrowband reference signals are available.