3GPP TSG-RAN WG1 #98bis		R1-1910956

Chongqing, China, 14th-20th October, 2019

Source: 	Apple Inc.

Title:                     UL signals and channels for NR-U

Agenda item:	7.2.2.1.3

Document for:	Discussion and Decision

# 1 Introduction

At the RAN1 #98 meeting, good progress was made on UL signals and channels for NR-U operation and the following agreements were made [1]:

Agreement: The working assumption from RAN1 AH1901 is converted to an agreement with the following modifications: - For a given SCS, the following PRB-based interlace design is supported at least for PUSCH and PUCCH: - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth - Point A is the reference for the interlace definition - For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths - FFS: Whether and how partial interlace allocation is supported considering mechanisms specific to PUSCH and PUCCH - FFS: PUCCH bandwidth - FFS: Whether or how an interlace design for PUSCH and/or PUCCH is supported on 10 MHz according to the revised WID objective Agreement: Alt-1a (Cycling of cyclic shifts across PRBs of the interlace) is selected from the four alternatives in the RAN1#97 agreement on enhanced Rel-15 PUCCH formats PF0 and PF1 - FFS: Cyclic shift ordering Agreement: A bandwidth occupied by a PUCCH resource does not exceed the bandwidth corresponding to a 20 MHz carrier/LBT bandwidth Agreement: - For interlaced PUSCH transmission in a BWP, X bits of the PUSCH frequency domain resource allocation field are used for indicating which combination of M interlaces is allocated to the UE. - This applies to PUSCH of the following types: - Msg3 PUSCH - PUSCH Scheduled by fallback and non-fallback DCI - Type 1 and Type 2 Configured Grant PUSCH - For 30 kHz SCS - Support X = 5 (5-bit bitmap to indicate all possible interlace combinations) - For 15 kHz SCS - Down-select between the following two alternatives: - Alt-1: Support X = 10 (10-bit bitmap to indicate all possible interlace combinations) - Alt-2: Support X = 6 bits to indicate start interlace index and number of contiguous interlace indices (RIV) and using remaining up to 9 RIV values to indicate specific pre-defined interlace combinations Conclusion: For 10 MHz carrier bandwidth, enhancements to Rel-15 UL signals and channels are not necessary. Agreement: A PUCCH resource configured with interleaved mapping occupies consecutive PRBs within at least one interlace within a BWP. The PUCCH resource configuration includes the following: - An indication of the allocated interlace - An indication of the location of the PUCCH resource within the allocated interlace - Note: This may not be needed for a bandwidth part of 20 MHz or less - The number of PRBs NPUCCH within the allocated interlace given by the following: - For Interlaced PF0/1/2: - NPUCCH = 10 or 11 depending on the allocated interlace - For Interlaced PF3: - NPUCCH = 10 - FFS: Whether/how an interlaced PF2/3 resource can be configured on 2 interlaces to increase the number of allocated PRBs to 20, 21, or 22 depending on the allocated interlaces - FFS: Whether or not the BWP can be configured such that NPUCCH is less than 10 or 11 - FFS: Potential impact due to in-carrier guard bands - Note: The UE is not expected to be configured with PUCCH transmissions spanning multiple LBT bandwidths

This contribution studies different remaining open issues for PUCCH design of NR-U and presents our views. In addition, we discuss the resource configuration for interlaced PUCCH transmission especially for the case that one UL BWP spans multiple LBT sub-bands.

# 2. Discussion

In the RAN1 #98 meeting, cycling of cyclic shifts across PRBs of the interlace (i.e. Alt.1a) was selected from the four alternatives in the RAN1#97 agreement on enhanced Rel-15 PUCCH formats PF0 and PF1. However, it remains open regarding the CS ordering for each PRB within a interlace.

In Rel-15, the phase rotations representing the different information bits are separated with π and π2 for one and two bits HARQ-ACK for PUCCH format 0 (PF0), respectively. The Rel-15 CS computation for HARQ-ACK bits only can be described using the following equation:

CSb0,b1=mod(CS0+6b0+3b1,12)

Wherein CS0 is the initial CS assigned by RRC signalling and b0 denotes the 1-bit HARQ-ACK or one of two bits in case of 2-bits HARQ-ACK and b0 represents the other HARQ-ACK bit. In the case of a simultaneous SR and HARQ-ACK bits transmission, the phase rotation is increased π2 for 1-bit HARQ-ACK and π6 for 2-bits HARQ-ACK information.

The Rel-15 PUCCH resource determination framework can be extended to enable CS cycling across RBs in one interlace for the case of only HARQ-ACK bits transmission. In more details, the CS for RB i in an ePF0 interlace, denoting as CS0i, can be determined as follows:

CS0i=mod(CS00+∆∙i+6b0+3b1,12)                               (1)

Where ∆ is RRC-configured parameter or predefined in spec e.g. ∆=1.

For the case of simultaneous HARQ-ACK and SR transmission, some solutions can be considered to improve the UE multiplexing capacity. We think it is important for interlaced ePF0 in NR-U system considering the fact that one ePF0 occupies 10 or 11 PRBs. In Rel-15, separate CS values are reserved for potential concurrent transmission of HARQ-ACK and SR bits i.e. CS = 3/9 for 1-bit HARQ-ACK and CS =1/4/7/9 for 2-bits HARQ-ACK information. This can be avoided for ePF0 to increase the multiplexing capability by using different permutation patterns of two basic CS sequences associated with two HARQ-ACK states (i.e. <ACK>, <NACK> for 1-bit HARQ-ACK, or <ACK, ACK> and <NACK, NACK> for 2-bits HARQ-ACK information). With this approach, the UE multiplexing capability can be at least doubled compared to simply reusing the Rel-15 method.

Figure 1 provides an example of UCI feedback mechanism, assuming CS00=0,∆=1 and each ePF0 interlace consists of 10 RBs. In case of negative SR, the CS sequence is generated based on 2-bits HARQ-ACK only using equation (1). As shown in FIG.1, instead of reserving different initial CS CS00 values for the other six states as in Rel-15, i.e. <ACK, NACK, N-SR/P-SR>, <NACK, ACK, N-SR/P-SR>, <ACK, ACK, P-SR>, <NACK,NACK, P-SR>, different states can be signaled using different permutation patterns of two basic CS sequences that are generated for the <ACK,ACK> and <NACK,NACK> state. Consequently, the multiplexing capability per interlace is increased to two times in case of 1-bit HARQ-ACK information and three times for 2-bit HARQ-ACK information case.

Figure 1: Example of 2-bit HARQ-ACK and SR feedback

Proposal 1:

- For 1 or 2 HARQ-ACK bits <b0>, <b0,b1> on ePF0, the cyclic shift for PRB i in a interlace is determined as follows:  CS0i=mod(CS00+∆∙i+6b0+3b1,12) where CS00 is initial CS configured by RRC signalling.

- For the case of simultaneous HARQ-ACK and SR on ePF0, the CS for each PRB i is determined based on a permutation pattern of CS sequences associated with 1-bit or 2-bits HARQ-ACK information.

For PUCCH format 1, up to two bits to be transmitted are BPSK or QPSK modulated, respectively and multiplied by the same sequence with an RRC-configured CS. The equation (1) above can be reused to determine the CS ordering for each PRB. In the case of simultaneous SR and HARQ-ACK transmission, the modulated symbol is transmitted using either HARQ-ACK resource or separated SR resource depending on the positive or negative SR. Similar to the consideration on ePF0, we are supportive to adopt some enhanced scheme to reduce the UL signaling overhead by avoid reserving dedicated resource for SR information. One possible solution is to phase rotate the modulated BPSK or QPSK symbols with π2 on odd or even PRBs within the interlace depending on positive or negative SR.

Proposal 2:

- Support to introduce mechanism to reduce signaling overhead for concurrent transmission of HARQ-ACK and SR using enhanced PUCCH format 1.

Another FFS aspect relates to how to indicate the ePF0/ePF1 interlace index prior to RRC connection setup. Our view is that the PRB offset column value in the Rel-15 default table can be simply re-interpreted as interlace index. In addition, the value of NBWPsize4 in row 15 can be simply reserved as of now.

Proposal 3:

- For the PUCCH resource allocation prior to dedicated RRC connection setup, support re-interpreting the “PRB offset” column in Rel-15 default Table as interlace index of ePF0 and ePF1 for HARQ-ACK transmission.

# 3. Conclusion

In this contribution, we discussed several FFS aspects related to UL signals and channels for NR-U o. Based on the discussions above, we made the following proposals:

Proposal 1:

- For 1 or 2 HARQ-ACK bits <b0>, <b0,b1> on ePF0, the cyclic shift for PRB i in a interlace is determined as follows:  CS0i=mod(CS00+∆∙i+6b0+3b1,12) where CS00 is initial CS configured by RRC signalling.

- For the case of simultaneous HARQ-ACK and SR on ePF0, the CS for each PRB i is determined based on a permutation pattern of CS sequences associated with 1-bit or 2-bits HARQ-ACK information.

Proposal 2:

- Support to introduce mechanism to reduce signaling overhead for concurrent transmission of HARQ-ACK and SR using enhanced PUCCH format 1.

Proposal 3:

- For the PUCCH resource allocation prior to dedicated RRC connection setup, support re-interpreting the “PRB offset” column in Rel-15 default Table as interlace index of ePF0 and ePF1 for HARQ-ACK transmission.

# References

- RAN1 Chairman’s Notes, RAN1 #98, May 2019