3GPP TSG RAN WG1 #98                  		                                   R1-1911133

Chongqing, China, 14 – 18 October, 2019

Agenda item:	7.2.10.2

Source: 	Qualcomm Incorporated

Title: 	UL Reference Signals for NR Positioning

Document for:		Discussion and Decision

# Introduction

In the last RAN1 meeting (RAN1#98) the following agreements were made on UL reference signals:

Agreement: - The RE pattern of an SRS resource for positioning is configured with a comb offset for the first symbol in an SRS resource. - The relative RE offsets of following symbols are defined relative to the comb offset of the first symbol in the SRS resource. - A relative RE offset of each of the following symbols is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource. - FFS: The use of the following for deriving the relative offset in addition to the above - A configurable number of consecutive symbols with the same offset - Repetition factor - No additional parameters will be considered Conclusion: No additional comb sizes for SRS for positioning purposes will be introduced in Rel-16 beyond those already agreed. Conclusion: No additional number of symbols per SRS resource other than the values already agreed is supported

In this paper, we present our views on UL Reference Signals for positioning with a focus on finalizing the high layer parameters that are needed for the SRS for positioning configuration ([1]):

- Relation to BWP

- Cyclic-shifts

- Repetition Factor

- Sequence hopping

- Number of bits for SequenceId

- Time-behavior

- Power control parameters

- Number of ports

# SRS for positioning & Relation to BWP

In NR Rel-15, an SRS resource is defined with a BWP of a component carrier.

The SRS resource mapping in the frequency domain is defined with the following approach: Each SRS resource is configured within a BWP, and it has also a high layer parameter nshift∈{0,…,268} configured. Then,

- if NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP.

Or in other words, the frequency domain resource mapping is not always with respect to an absolute point-A, but it depends on the start of the BWP with respect to the point-A. The reason this is this specific way is that the maximum value of nshift is limited to 268, and a point-A can be 2199 PRBs away from the start of a CC

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

The BWP could change semi-statically or dynamically, so the LMF would not be in position to know which is the active BWP whenever the SRS for positioning needs to be transmited, however we do not consider that there is a need for the LMF to know. To see this, consider the following procedure:

- The LMF requests a serving gNB for available SRS resources.

- The serving gNB can reply with an SRS-Config which contains the frequency domain nshift (taking the values 0…2199) with respect to a point-A (taking the values from ARFCN-ValueNR)

  - This does not mean that from UE-perspective, this SRS is outside BWP. It just means that the serving gNB hides the BWP information and provides an SRS configuration which is defined in absolute terms, to remove any ambiguity of the configuration

- The LMF sends as assistance data the SRS configuration to the UE

- The serving gNB controls the active BWP of the UE, and therefore it may tune the UE to a BWP for which such SRS can be transmitted, or in other words, all the sounded PRBs are within the BWP.

- If, for any reason, any of the configured PRBs for SRS for positioning are not within the active BWP, then the UE would drop the whole SRS transmission.

Proposal 1: The SRS for positioning configuration does not require any association to BWP ID or CC ID. The frequency domain allocation (nshift) can take the values {0,…,2172} with respect to a signalled Point-A taking the values from ARFCN-ValueNR, similar to a band/CC/BWP-agnostic PRS configuration.

- The UE is not expected to transmit the SRS for positioning unless it is fully contained within the active BWP at the time of the SRS transmission.

# Cyclic-shifts

Regarding the cyclic shifts for each comb size, for comb-2 and comb-4 in NR Rel-15 (and LTE), 8 and 12 cyclic shifts are supported for a bandwidth of at least 4 PRBs. This means that up to 2*8=16 ports and up to 4*12=48 ports can be orthogonally multiplexed through FDM and CDM for comb-2 and comb-4 respectively. A similar capacity of up to 48 ports can be supported for comb-8 also, which means 6 cyclic shifts are needed to be specified. This would also allow to have a 4-PRB SRS BW with comb-8 (only 6 resource elements per comb).

Proposal 2: For SRS for positioning, 6 cyclic shifts for comb-8 are supported. No change in the number of cyclic shifts for comb-2 and comb-4 beyond those supported in NR Rel-15.

# Frequency hopping & Repetition Factor for staggered SRS resources

In NR Rel-15, for frequency hopped SRS, the frequency domain starting position of each hop varies over time according to a pre-defined hopping pattern, whereas for non-hopped SRS the frequency domain starting position is fixed over time. The non-hopped SRS is specified as a special case of SRS hopping in which the number of hops in a hopping cycle is only one. For an SRS resource configured with frequency hopping, repetition allows the same set of subcarriers to be sounded in R consecutive OFDM symbols before the next hop occurs. The allowed values of R are 1,2,4 where R≤NsymbSRS. The order of repetition and hopping is always repetition first, then hopping.

Observation 1: In NR Rel-15, for an SRS resource configured with frequency hopping, repetition allows the same set of subcarriers to be sounded in R consecutive OFDM symbols before the next hop occurs. The allowed values of R are 1,2,4 where R≤NsymbSRS. The order of repetition and hopping is always repetition first, then hopping.

For SRS for positioning however, it is important to sound all subcarriers of a hop (fully-staggered patterns), as it is also the agreement, to ensure that no aliasing exists and to have a more robust channel estimation on the UL. Therefore, the principles for hopping for an SRS for positioning should not be to repeat the same subcarriers R times, but rather to repeat a fully-staggered pattern R times before moving to the next hop.

Observation 2: Main two principle of hopping for SRS for positioning:

- Ensure that all subcarriers of one hop have been sounded (i.e. fully-staggered sounding) before the next hop occurs

- Ensure that within one slot, for each of the sounded hop, all subcarriers have been sounded, or in other words, there should not be a hop which was partially sounded within a slot.

For example, for comb-2 and an 8-symbol SRS resource, the following configurations allow for fully-staggered sounding for all sounded hops within a slot.  The reason that we need to ensure that a hop is fully staggered within a slot, is because the UE cannot ensure phase continuity for any SRS symbols in another slot. So, for a receiver to coherently combine

Comb-2 with 8 symbol and different options of repetitions and number of hops

Comb-4 with 8 symbol and different options of repetitions and number of hops

Proposal 3: For a comb type of  KTC={2,4,8} SRS resource spanning NsymbSRS={1,2,4,8,12} OFDM symbols within a slot, configuration of repetition of R={1,2,3,4,6} with R≤NsymbSRS is possible with the following number Nb of frequency hops within the NsymbSRS symbols of the SRS resource:

| Comb/symbols | NsymbSRS=1 | NsymbSRS=2 | NsymbSRS=4 | NsymbSRS=8 | NsymbSRS=12 |
| --- | --- | --- | --- | --- | --- |
| KTC=2 | R=1, Nb=1 | R=1, Nb=1 | R=1, Nb =2 R=2, Nb =1 | R=1, Nb =4 R=2, Nb =2 R=4, Nb =1 | R=1, Nb =6 R=2, Nb =3 R=3, Nb =2 R=6, Nb =1 |
| KTC=4 | R=1, Nb=1 | N/A | R=1, Nb=1 | R=1, Nb =2 R=2, Nb =1 | R=1, Nb =3 R=3, Nb =1 |
| KTC=8 | R=1, Nb=1 | N/A | N/A | R=1, Nb =1 | N/A |

# Sequence hopping

Group and sequence hopping are used for SRS resource sequence randomization which could be useful for interference randomization and avoidance of time-persisstence interference. It makes sense to introduce this feature for SRS for positioning.

Proposal 4: Introduce the group or sequence hopping of NR Rel-15 for SRS for positioning.

# Number of bits for sequenceId

For DL PRS, at least 4096 are used for the scrambling initialization. We think that both DL and UL PRS should have the same number of bits for the scrambing ID to ensure similar randomization across DL and UL.

Proposal 5: The number of bits for sequenceID shall be as many as they are used for the scrambling ID of DL PRS resource.

# Time behavior

The following agreement has been made regarding the time behaviour of an SRS resource:

Agreement: Support the following configurations of SRS for positioning - Semi-persistent configuration - Periodic configuration - Aperiodic configuration

It should be understood that the above agreements not necessarily refer to the DCI-based or MAC-CE based triggered resources that RAN1. On-demand resources can still be configured through high-layer with a start-time, and a duration which can even be potentially one occasion. We propose to leave these details to RAN2 & RAN3 WGs

Proposal 6: Triggering details of how the on-demand SRS are configured, shall be left to RAN2 & RAN3 WGs.

# Power control higher layer parameters

# P0, alpha, pathlossReferenceRS

Even  though it is reasonable to use the P0 values and alpha values that are already supported for NR Rel-15, a special consideration is needed for the case that these parameters are not provided. Specifically, in NR Rel-15,

- when the pathloss reference is not provided, the UE estimates the pathloss using the SSB carrying the MIB, and

- when P0 is not provided, then the UE shall use the P0 of PUSCH.

- When alpha is not provided, the nthe UE shall use alpha = 1

However, for NR Rel-16 and SRS for positioning, when these values are not provided, a different solution would be a better choice. For example, if the pathlossReference is not provided, using the pathlossreference of the SSB would lead the Tx power to be adjusted according to the serving cell, evn though the SRS for positioning is expected to be received by non-neighboring TRPs. Similarly, if P0 is not provided, using the P0 of PUSCH is not a good option for the same reason.

Proposal 7: Support the higher layer parameter of P0, alpha with values reused from NR Rel-15. Discuss further what shall be the default behavior in case for an SRS for positioning:

- P0 is not provided (e.g., use maximum value for P0)

- Alpha is not provided (e.g., use alpha=1 as legacy behavior)

- pathlossReference is not provided (e.g., transmit with full Tx power)

# Number of Ports

No discussion has been made on supporting multiple ports SRS for positioning. It should be understood that the parameter nrofSRS-Ports is not needed.

Proposal 8: Only 1-port SRS resource for positioning is supported.

# RE-level shift

In order to avoid cross-correlation peaks cause by RE holes in frequency domain, the number of SRS symbols shall be equal to the comb-type with the following proposed deterministic sequence of shifts. Such sequence of shift is also helpful for the case of a UE in a high mobility scenario where the receiver may want to perform coherent combining only across a subset of OFDM symbols due to faster decorrelation. Ensuring that consecutive symbols contain subcarriers which are uniformly spaced would facilitate such block-based coherent processing of the SRS of positioning.

Figure 3-1 Comb-4 with suggested frequency offset (vshift) sequence  0→2→1→4

Figure 2 Comb-8 with suggested frequency offset (vshift) sequence  0→4→2→6→1→5→3→7

Proposal 9: For positioning, the frequency domain shifts are chosen according a round robin fashion from the following sequence for all the symbols of the SRS resource inside a slot.

- For comb type 2: 0→1

- For comb type 4: 0→2→1→4

- For comb type 8: 0→4→2→6→1→5→3→7

For example, if the SRS resource spans 12 symbols with comb-4, then the sequence of comb-offsets would be three times the sequence shown above: 0→2→4→1→0→2→4→1→0→2→4→1 .

Proposal 10: With respect to the relation between the comb-type and number of symbols in a PRS resource downselect between the following 2 alternatives:

- A comb-N pattern is only applied for a M-symbol PRS resource with N=M

- A comb-N pattern is only applied for a M-symbol PRS resource with N<=M

# Conclusion

In this contribution, we make the following proposals:

Proposal 1: The SRS for positioning configuration does not require any association to BWP ID or CC ID. The frequency domain allocation (nshift) can take the values {0,…,2172} with respect to a signalled Point-A taking the values from ARFCN-ValueNR, similar to a band/CC/BWP-agnostic PRS configuration.

- The UE is not expected to transmit the SRS for positioning unless it is fully contained within the active BWP at the time of the SRS transmission.

Proposal 2: For SRS for positioning, 6 cyclic shifts for comb-8 are supported. No change in the number of cyclic shifts for comb-2 and comb-4 beyond those supported in NR Rel-15.

Proposal 3: For a comb type of  KTC={2,4,8} SRS resource spanning NsymbSRS={1,2,4,8,12} OFDM symbols within a slot, configuration of repetition of R={1,2,3,4,6} with R≤NsymbSRS is possible with the following number Nb of frequency hops within the NsymbSRS symbols of the SRS resource:

| Comb/symbols | NsymbSRS=1 | NsymbSRS=2 | NsymbSRS=4 | NsymbSRS=8 | NsymbSRS=12 |
| --- | --- | --- | --- | --- | --- |
| KTC=2 | R=1, Nb=1 | R=1, Nb=1 | R=1, Nb =2 R=2, Nb =1 | R=1, Nb =4 R=2, Nb =2 R=4, Nb =1 | R=1, Nb =6 R=2, Nb =3 R=3, Nb =2 R=6, Nb =1 |
| KTC=4 | R=1, Nb=1 | N/A | R=1, Nb=1 | R=1, Nb =2 R=2, Nb =1 | R=1, Nb =3 R=3, Nb =1 |
| KTC=8 | R=1, Nb=1 | N/A | N/A | R=1, Nb =1 | N/A |

Proposal 4: Introduce the group or sequence hopping of NR Rel-15 for SRS for positioning.

Proposal 5: The number of bits for sequenceID shall be as many as they are used for the scrambling ID of DL PRS resource.

Proposal 6: Triggering details of how the on-demand SRS are configured, shall be left to RAN2 & RAN3 WGs.

Proposal 7: Support the higher layer parameter of P0, alpha with values reused from NR Rel-15. Discuss further what shall be the default behavior in case for an SRS for positioning:

- P0 is not provided

- Alpha is not provided

- pathlossReference is not provided

- Proposal 8: Only 1-port SRS resource for positioning is supported.

Proposal 9: For positioning, the frequency domain shifts are chosen according a round robin fashion from the following sequence for all the symbols of the SRS resource inside a slot.

- For comb type 2: 0→1

- For comb type 4: 0→2→1→4

- For comb type 8: 0→4→2→6→1→5→3→7

Proposal 10: With respect to the relation between the comb-type and number of symbols in a PRS resource downselect between the following 2 alternatives:

- A comb-N pattern is only applied for a M-symbol PRS resource with N=M

- A comb-N pattern is only applied for a M-symbol PRS resource with N<=M

# References

[1] R1-1909932, “List of higher layer parameters for NR Positioning”, Intel