3GPP TSG RAN WG1 meeting #98bis	           	  R1-1911245

Chongqing, China, October 14th – 20th, 2019

Agenda item:		7.2.9.1

Source:	Nokia, Nokia Shanghai Bell

Title:		On PDCCH-based power saving techniques

Document for:		Discussion and Decision

# 1	Introduction

UE Power Saving WID was approved in RAN#83 [1]. Further updates were made in RAN#84 [2]. The current objectives are as follows:

The objective is to specify the UE power saving techniques with UE adaption in achieving UE power saving. The power saving technique should address latency and performance in NR as well as network impact. The objective of the UE power saving includes the following, - Specify power saving techniques with PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED mode [RAN1, RAN2, RAN4] - Specify procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence(s) of the drx-onDurationTimer [RAN2, RAN1] NOTE: Any change of PDCCH channel coding and payload interleaver is not in the scope - Specify the procedure of cross-slot scheduling power saving techniques [RAN1, RAN4] NOTE: The procedure is in addition to Rel-15 cross-slot scheduling procedure - Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation - Specify a mechanism for a UE to indicate its preference of transitioning out of RRC_CONNECTED state. [RAN2] - Specify, if agreed, the mechanism to provide UE assistance information [RAN2, RAN1]: - Study and select among the following UE assistance information for RAN2 by RAN#85: power preferred information (baseline LTE PPI in a well-defined manner) and UE's preference on C-DRX, BWP and SCell configuration [RAN2]. - NOTE: additional UE assistance information for RAN1-specific power saving techniques to indicate what setting of parameters related to this Work Item will lead to power savings for the UE can be included if agreed in RAN1. - Specify network-configured mechanism to relax intra and inter-frequency RRM measurement for neighbour cells for RRC_IDLE/INACTIVE with minimal mobility performance impacts [RAN2, RAN4] - Specify RRM measurement relaxation by allowing measurements with longer intervals, and/or by reducing the number of cells/carriers to be measured [RAN4, RAN2] - Define triggering criteria for the UE to move between relaxed and normal RRM measurements, that considers at least if UE is not at cell edge, or if UE is stationary or with low mobility [RAN2, RAN4]. - Evaluate and specify requirements (if needed) depending on RAN2 criteria [ RAN4] NOTE: No new measurement quantities should be introduced

In this contribution, we discuss about the configurations related for the new DCI format.

# 2	Discussion

## 2.1	On terminology

The discussion regarding the design on the power saving channel has been carrying out over several meetings. Based on the made agreements the outside active time indications will be carried by a new RRC configurable DCI format that will be monitored by a specific RNTI, PS-RNTI. No new signal nor channel has been agreed to be introduced for this purpose, i.e. the DCI will be carried by PDCCH. For cross-slot adaptation inside active time it has been agreed to introduce a new 1-bit field to (non-fallback) scheduling DCI’s to carry the indications. Again, this DCI would be carried by PDCCH.

Therefore, in order to avoid confusion and enable smooth progress it is proposed:

Observation 1: No new signal nor channel need to be introduced for triggering adaptation of UE power saving inside or outside active time.

Proposal 1: To have logical naming convention especially in respect to other working groups, the adaptation triggering outside active time would be referred as DCI format monitored using PS-RNTI or PDCCH scrambled by PS-RNTI.

In context of this paper, in merit of space (and time) we will refer to the ‘DCI format monitored using PS-RNTI’ as ‘PDCCH scrambled by PS-RNTI’.

## 2.2	On CORESET for PDCCH scrambled by PS-RNTI

In RAN1#97 following agreement was made allowing the configuration of the PDCCH scrambled scrambled by PS-RNTI. I.e the CORESET can be shared by other PDCCHs so that UE would monitor other DCI formats monitored with other RNTIs e.g. inside the active time on that CORESET.

Agreements: - The CORESET for the PDCCH-based power saving signal/channel outside Active Time can be configured to index to at least one of the CORESET(S) configured for other PDCCH monitoring - FFS whether the indexed CORESET can be exclusively used by the PDCCH-based power saving signal/channel (i.e., not be used for other PDCCH monitoring) - FFS whether or not to increase the number of CORESETs relative to that in Rel-15 - FFS whether or not a BWP is dedicated for PDCCH-based power saving signal/channel

The following working assumptions were made regarding CORESET configuration for PDCCH-based power saving channel outside active time in RAN1#98:

Working assumption: - The CORESET for power saving signal/channel outside Active Time can be associated with (in addition to search space set for power saving signal/channel) other search space set(s). Working assumption: - UE monitors the PDCCH-based power saving signal/channel outside Active Time being configured on the active BWP in an active cell.

Now the first working assumption seems to repeat the agreement that was made in RAN1#97, i.e. UE can be configured to monitor different RNTIs in the same CORESET, e.g. the CORESET can be shared between active time and outside active time. Based on this the working assumption could be considered to be confirmed (or unnecessary).

Observation 2: RAN1#97 already made an agreement that the CORESET can be shared between active time and outside active time.

One aspect that was left FFS in RAN1#97, was whether there can be a CORESET that is solely assigned for PDCCH scrambled by PS-RNTI. The reason for this was to simplify the PDCCH scrambled by PS-RNTI  detection with low power consumption. This could be achieved for example via CORESET configuration with narrower bandwidth. Of course from network point of view and resource usage point of view it would make sense to be able to share the same CORESET configuration among UEs as much as possible within a BWP. Also, like discussed in the next session, it could be also beneficial from UE perspective to share the CORESET with other RNTIs, that are needed to be monitored also outside active time, to simplify the UE operation. Hence, certain UE behavioural aspects would need to be clarified before the merit of having CORESET solely for PDCCH scrambled by PS-RNTI.

Observation 3: Before concluding the feasibility of having independent CORESET for PDCCH scrambled by PS-RNTI, the UE behaviour in relation to other PDCCHs to monitor needs to be clarified.

Other working assumption related to the possibility to have BWP dedicated solely for PDCCH with PS-RNTI was left FFS in RAN1#97. Like discussed in the next session, the UE needs to monitor paging (and SI change) in background also outside active time. Thus, in case of time domain conflict between monitoring for PDCCH scrambled by PS-RNTI on one (dedicated) BWP and monitoring paging/SI on the other (active time BWP), the additional rules would be needed to be determined to handle the conflicts. I.e. UE would be required to perform autonomous BWP switches, which would result interruptions preventing the monitoring. Furthermore, RAN2 has agreed that RRM and RLM measurements are not to be impacted by WUS design (RAN2 naming), i.e. UE should continue to measure the configured reference signals as per RRM requirements regardless of the monitoring of the PDCCH scrambled by PS-RNTI. Possible adaptation between BWPs could hinder these.

Thus, based on above discussion it’s considered feasible to confirm the working assumptions from RAN1#98.

Proposal 2: Based on the agreement preceding the working assumption in RAN1#97 consider the following working assumption to be confirmed:

•	The CORESET for power saving signal/channel outside Active Time can be associated with (in addition to search space set for power saving signal/channel) other search space set(s).

Proposal 3: Confirm working assumption:

•	UE monitors the PDCCH-based power saving signal/channel outside Active Time being configured on the active BWP in an active cell.

## 2.2	DCI format size budget and total number of CORESETs

The following agreement was made in RAN1#98 regarding the number of CORESETs in outside and inside active time as follows:

Agreements: The maximum number of CORESETs for PDCCH-based power saving signal/channel outside Active Time is no larger than the max number that can be configured inside Active Time.

In RAN1#96bis it was agreed that the DCI-format size budget is not increased but left open whether the size budget can be separated inside and outside active time.

Agreements: The assumptions of the DCI design of the PDCCH-based power saving signal/channel include: - No increase of DCI format size budget - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time - Working assumption: no increase of UE BD/non-overlapping CCE limit

When considering the outside active time it’s noted that UE needs to monitor additionally paging (P-RNTI) and system information (if triggered by Short Message or validity expiration) independent of outside and inside active times as C-DRX configuration applies only C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI [3GPP TS 38.321, section 5.7]. Thus, there are CORESETs that are configured inside active time that are required to be monitored also outside active time.

Currently the DCI format size budget per UE is determined in a general manner, i.e. there is no separation for the capability between CONNECTED or IDLE/INACTIVE, nor between outside active time and inside active time. The only separation is done via defining the RNTIs which UE is expected to monitor. Thus, as noted above, there are certain RNTIs (P-RNTI, SI-RNTI, RA-RNTI) that UE may need to monitor outside active time, and the corresponding DCI-formats are accounted in DCI-format size budget. In this light the FFS point from RAN1#96bis is not very evident. The intent of the FSS, in our understanding, was to consider whether the new DCI-format introduced for outside active time use would be counted in the total budget of DCI-format sizes UE has to support. From system flexibility perspective it is of course preferable that the new DCI format that is expected to be solely used outside of active time (probably based on RNTI monitoring rules in MAC spec) would not limit the budget for the DCI-formats that can be used, both inside and outside active time.

Proposal 4: The (new) DCI format scrambled by PS-RNTI is not accounted when determining the DCI format size budget.

Correspondingly also CORESETs can be used both inside and outside active time. Hence it is not evident whether separating the budget for number of CORESET budget is very straight forward. Therefore, for the maximum number of CORESETs outside and inside active time, we consider that the principle where the maximum number of CORESETs is defined per BWP according to UE capability should be followed. From that perspective, the total number of configured CORESETs both outside and inside active time should be less than or equal to UE’s capability.

Proposal 5: The total number of configured CORESETs both outside and inside active time is less than or equal to the number according to UE’s capability per BWP.

## 2.3	Monitoring PDCCH scrambled by PS-RNTI and other procedures outside active time

There are certain procedures that UE may trigger outside active time and the PDCCH monitoring related to those procedures may overlap with monitoring of PDCCH scrambled by PS-RNTI in time. Those are for instance:

- Random access procedure for scheduling request due to lack of SR resources on active BWP

- Random access procedure due to beam failure recovery

So, it is possible that monitoring occasions for PDCCH scrambled by PS-RNTI would collide for example monitoring of PDCCH with RA-RNTI. If the UE is to be able to monitor both, PDCCH scrambled by PS-RNTI and PDCCH with RA-RNTI, it would be simplest from both UE and gNB point of view. This can be achieved at least if the same CORESET configuration would be used for both (from network point of view if CORESET could be shared by multiple UEs). Correspondingly, the PDCCH scrambled by PS-RNTI monitoring could collide with the RAR message reception (when scheduled by PDCCH with RA-RNTI) when UE is still outside active time. Thus, there should be some rules and UE behaviour for the cases e.g. when random access procedure overlaps in time with PDCCH scrambled by PS-RNTI monitoring.

Observation 4: UE behaviour needs to be determined for the cases when UE triggered procedures like RA or BFR overlap in time with PDCCH scrambled by PS-RNTI monitoring.

Proposal 6: Support one of the following options when UE initiates random access procedure outside active time:

- Monitor both RA-RNTI and PS-RNTI

  - Prioritize RA-RNTI monitoring in case QCL assumptions are different for PS-RNTI monitoring

- Monitor only RA-RNTI

In certain options it would be possible that UE would not be able to monitor (and receive) PDCCH scrambled by PS-RNTI. Therefore, it would need to be determined whether UE should in those cases monitor PDCCH with C-RNTI the next onDuration without explicit indication via PDCCH scrambled by PS-RNTI. To avoid latency increase it would be preferable to determine that the UE would, in case of missed monitoring occasion(s) for PDCCH scrambled by PS-RNTI, would monitor PDCCH(s) on next onDuration.

Proposal 7: If the UE misses the monitoring occasion(s) of PDCCH scrambled by PS-RNTI, due to other procedures, UE shall monitor PDCCH(s) on next onDuration.

## 2.4	Monitoring occasions for PDCCH based power saving channel for triggering wake-up

Regarding monitoring occasion for PDCCH based power saving channel for triggering wake-up outside the Active Time RAN1#98 made the following working assumption:

Working assumption: - More than one monitoring occasion can be configured within a slot or multiple slots before the DRX ON

Regarding multiple PDCCH monitoring occasions within a slot it needs to be remembered that only slot-based monitoring, i.e. PDCCH monitoring occasions within the first 1-3 symbols, is mandatory capability for Rel15 UEs. Furthermore, we don’t see big difference in UE power consumption between the following exemplary cases:

- 2 monitoring occasions in one slot (not mandatory feature, i.e. depends on UE capability)

- 1 monitoring occasion per slot and monitoring occasions in two consecutive slots (mandatory feature)

Thus, it’s considered feasible to support only cases where more than one monitoring occasion can be configured within multiple consecutive slots before the DRX ON. Similar to e.g. RMSI scheduling monitoring using default configuration, baseline could be two slots for the monitoring the PDCCH based wake-up.

Proposal 8: More than one monitoring occasion can be configured within multiple consecutive slots before the DRX ON.

Further related to the PDCCH monitoring occasions, the following conclusion was made in RAN1#98:

Conclusion: For next meeting, down select the following two alternatives: The configuration of the offset of the PDCCH-based power saving signal/channel - Alt1: Dedicated configuration with offset relative to the beginning of DRX ON - Alt2: The Offset is based on search space configuration - FFS: whether this applies to long DRX only or long/short DRX.

Based on the agreements the UE would be “indicated” the offset before the DRX onDuration based on which the UE determines the monitoring occasions for the PDCCH based power saving channel outside the Active Time. Different options can be considered how to determine actual monitoring options:

Alt1-1: Offset indicates the starting slot before the onDuration and separate parameter could provide duration in slots as illustrated in Figure 5.

Figure 1 Offset indicates starting positions and separate duration field the number of slots for the monitoring before the onDuration.

Alt1-2: Offset indicates the starting slot before the onDuration from where UE starts monitoring PDCCH for wake-up until the start of the onDuration as illustrated in Figure 6.

Figure 2 Offset indicates starting positions from where UE starts monitoring PDCCH for wake-up until the start of the onDuration.

Alt2: Search space configuration is solely used to determine the monitoring occasions for wake-up outside the active time as illustrated in Figure 7.

Figure 3 Search space configuration determines the monitoring occasions of PDCCH for wake-up outside  active time.

In order to minimize UE’s monitoring time during outside of Active time it could make sense to limit the number of slots where to monitor power saving channel to a few slots (e.g. one or two). Thus, Option Alt1-2 above is less attractive if the X offset is large (would be required e.g. for BWP switching and A-CSI-RS for UEs reported long latencies required for the procedures if they need to be ready before onDuration). On the other hand, downside is the limited scheduling flexibility at the gNB. Similar to e.g. RMSI scheduling monitoring using default configuration, baseline could be two slots for the monitoring the PDCCH based wake-up. Alt-2 would require the search space configuration to be used to set the monitoring occasions prior the onDuration. Currently SS periodicity support period of 2560 slots, while DRX supports cycles up to 10s. Thus, either the wake-up monitoring could not be applied with longer DRX cycles or additional periodicities and offset values would need to be introduced for SS configuration and/or some additional window definition. Consequently, a preferred solution to determine the PDCCH monitoring occasions the following could be used (Alt1-1 above) for outside of Active time (as illustrated in Figure 4):

- X offset: defines the starting slot for PDCCH scrambled by PS-RNTI monitoring before the onDuration

  - applicable only outside of Active time – during Active time UE determines monitoring occasions for the search space set as in Rel15

  - applicable to both short and long DRX cycle

- duration: defines the number of consecutive slots where to monitor PDCCH scrambled by PS-RNTI starting from the slot defined by X offset

Figure 4. X offset and duration to define monitoring occasions outside Active time.

Proposal 9: Define the following parameters within the search space set configuration that are valid only outside of Active time – during Active time UE determines monitoring occasions for the search space set as in Rel15.

- X offset: defines the starting slot for PDCCH scrambled by PS-RNTI monitoring before the onDuration

  - applicable only outside of Active time – during Active time UE determines monitoring occasions for the search space set as in Rel15

  - applicable to both short and long DRX cycle

  - offset needs to take into account reported UE capabilities related to adaptations that can be triggered by the power saving channel

- duration: defines the number of consecutive slots where to monitor PDCCH scrambled by PS-RNTI starting from the slot defined by X offset outside Active time

It was left open whether the PDCCH scrambled by PS-RNTI monitoring is applied for both short and long DRX or only for other. Now the configuration of short DRX in addition to long DRX (in C-DRX configuration) is optional. Therefore, it should not be a pre-requisite that both, short and long DRX cycle, are configured before PDCCH scrambled by PS-RNTI can be configured. When both are configured, it could be that the short-DRX cycle configuration has a very short cycle, i.e. down to 2ms, and therefore it may not be always beneficial nor feasible to have PDCCH scrambled by PS-RNTI monitoring occasion(s) present before on durations when short DRX is applied. Therefore, it should be possible to separately determine whether the PDCCH scrambled by PS-RNTI is monitored before onDuration when short DRX cycle is applied.

Proposal 10: It can be separately configured whether PDCCH scrambled by PS-RNTI is monitored before onDuration when short DRX cycle is applied.

## 2.5	On New DCI format

The following agreements were in RAN1#98:

Agreements: - The new DCI format for power saving signal/channel is configured to be monitored at least in CSS. - FFS for UESS Agreements: - The CRC of new DCI format for power saving signal/channel is scrambled by PS-RNTI outside active time

New DCI format for outside active time is assumed to support triggering adaptation fields for up to a few UEs (e.g. 1-4) in order to keep total payload size in a level that provides enough good detection performance. Thus, it’s expected that there can be multiple different groups of UEs with different PS-RNTIs configured for the new DCI format. In addition, from network and resource consumption point of view it would be beneficial to share a CORESET among UEs monitoring PS-RNTI and active UEs. And further, it makes sense to keep the number of monitoring slots low (e.g. 2) for PS-RNTI monitoring. Consequence from those aspects is that transmitting new DCI format only in CSS which does not utilize hashing operation would be increased blocking probability and thereby requiring larger CORESET size and larger number of monitoring occasions. Thus, it is seen beneficial to allow gNB to configure new DCI format for WUS to be monitored also in UESS.

Proposal 11: Support the new DCI format monitoring configuration also in UESS.

## 2.6	Triggered UE adaptations outside Active Time

RAN1#97 made the following agreements:

Agreements: Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s) - FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update): - Indicating UE to use the aperiodic RS - Aperiodic CSI-RS - Aperiodic SRS - TRS - Triggering aperiodic CSI report - Cross-slot scheduling - Rel-15 DCI-based BWP switching The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

In RAN#85 the possible adaptation schemes were discussed and a way forward was agreed in [3]. Hence RAN1 was tasked to evaluate the possibility to combine SCell dormant behaviour adaptation related indication to a new DCI format carrying triggering to monitor PDCCH in onDuration. This would be treated under MR/DCCA agenda. The aspects related to the Scell dormant behaviour are discussed in our companion paper (in [4]).

Based on the current specifications UE shall wake-up on all active SCells. When the amount of data to be sent to the UE is very low it is considered to be beneficial to be able to limit the active cells for which the UE needs to monitor PDCCH during the onDuration. For that DCI carrying wake-up triggering could have triggering possibility in that domain. However, it does not appear to be needed for very high degree of granularity (i.e. no per CC indication) but rather a binary type triggering command would be feasible: monitor only on PCell, or monitor on PCell and all active SCells. However, benefit of this is heavily dependent on the work carried under DCCA work item to define the possible SCell states and also the delay that can be assumed for SCell activation, should it be needed. E.g. it would need to be possible to “re-enable” scheduling on the SCells quickly enough during the active time.

Observation 5: It could be beneficial to have possibility for frequency domain adaptation in CC domain in the wake-up DCI if the latency for ‘re-enabling’ PDCCH monitoring on SCells is short enough.

Like noted in the agreement in RAN1#97 and based on the way forward, introduction of other additional adaptation schemes would be in strict sense outside the work item scope. Like discussed during the last meeting and also considered in this paper, there could be occasions when UE would not necessarily be able to monitor PDCCH scrambled by PS-RNTI (i.e. misses the monitoring occasion) but would be expected to monitor PDCCH during the next onDuration. Therefore, it may be desirable to introduce such adaptations that would prevent the UE from monitoring the PDCCH without first receiving the PDCCH scrambled by PS-RNTI. For example for this reason in Rel-15 the BWP adaptation is acknowledged (HARQ FB) so that network and UE remain synchronous on the reception condition. Certain BWP related configurations may not be feasibly maintained simultaneously so it won’t be necessarily possible for the network to transmit channels and signals simultaneously based on both BWP configurations.  Therefore, introducing BWP adaptation to the PDCCH scrambled by PS-RNTI might be risky if the BWP adaptation would prevent the PDCCH monitoring on the following onDuration. Naturally. it could be considered to allow only limited adaptation of the BWP configurations, but it is not clear if it would provide the benefit from power saving perspective.

Other considered adaptation has been an A-CSI triggering, to enable early CSI feedback at the start of the onDuration/active time. Consequences of missed monitoring of PDCCH scrambled by PS-RNTI would not be as severe as in case of BWP adaptation. I.e. UE would not send the CSI report, and the allocated PUSCH resources would go unused but UE would be able to receive monitor PDCCH during the onDuration. Hence it would be possible to recover quite simply by just re-sending the request during the onDuration. On the other hand, latency benefit may be rather low when allowing triggering A-CSI in a new DCI format.

For SCell adaptation (to be discussed in DCCA AI), if UE would be prevented to monitor the PDCCH scrambled by PS-RNTI before the onDuration, UE could assume that all Scells are to be monitored and monitor PDCCH accordingly.

As a summary we consider that a new DCI format outside would have the following fields per UE:

- 1 bit indication to trigger PDCCH monitoring inside active time (bit needed per UE because of group common DCI format)

- [1 bit] indication to indicate UE whether to monitor PDCCH in PCell only or both in PCell and all active SCells (subject to progress in DCCA WI)

Proposal 12: New DCI format outside active time has the following indication fields per UE:

- 1 bit indication to trigger PDCCH monitoring inside active time (bit needed per UE because of group common DCI format)

- [1 bit] indication to indicate UE whether to monitor PDCCH in PCell only or both in PCell and all active SCells (subject to progress in DCCA WI)

## 2.7	Failure Detection for PDCCH scrambled by PS-RNTI

One aspect raised in past meetings in context of monitoring of PDCCH scrambled by PS-RNTI outside the Active time, is the handling on beam failure. In a Rel-15 beam failure recovery a beam failure instance is indicated by L1 to L2 when all the BFD-RS in set of q0 are estimated to be in failure condition. Even if one BFD-RS is not in failure condition no instance is indicated to L2. As a result the beam failure detection procedure is not able not track specifically just one or specific set of the PDCCH links the efficient beam failure detection in case of PDCCH scrambled by PS-RNTI may require additional consideration.

In Rel-15, RLM and beam failure recovery, detection and eventually recovery is triggered when all the monitored links are estimated to be in failure condition. UE performs failure detection also in DRX and attempts to recover when all links fail (BFR), it would be able to recover using BFR but e.g. in case  link corresponding to PDCCH scrambled by PS-RNTI is in failure condition the recovery procedure would not be triggered

One of the potential configuration options for PDCCH scrambled by PS-RNTI is that it corresponds to at least one of the configured TCI State for PDCCH reception e.g. based on CORESET. Considering this from the current beam failure detection perspective, it may mean that the beam used for monitoring power saving signal can be the only beam in failure condition (while no other TCI states are) but no beam failure instance is indicated to L2, no beam failure can be detected, and no recovery initiated. Effectively this may mean that UE is not able to decode the transmission of PDCCH scrambled by PS-RNTI but has no means to quickly recover from this situation. Additionally, network is not necessarily aware of UE condition or it may take relatively long time for network to notice it. Even in this case network can only wait for UE to trigger failure recovery. Also, DRX, the beam failure detection is done less frequently (in more relaxed manner) i.e. the indication interval is scaled according to the DRX cycle length. Thus, even if all beams are in failure condition, the declaration of beam failure may take relatively long time.

One potential approach to solve this could be to enhance current beam failure detection procedure to allow UE to e.g. trigger beam failure recovery based only on the quality of the RS corresponding to the PDCCH reception scrambled by PS-RNTI. This would enable the UE to start recovery procedure according to Rel-15 mechanisms.

Furthermore, to proactively prevent such failure conditions, network could also configure UE with periodic beam reporting during the on-duration or every Nth on duration. Based on the UE reporting it would be able perform beam management actions, TCI state reconfiguration etc. However, when PDCCH scrambled by PS-RNTI is configured, network may not able to reconfigure/perform beam management for UE if it has not indicated UE to wake up. After UE has performed beam reporting (e.g. on PUCCH) UE is not monitoring PDCCH. In one approach, when power saving signal/channel is configured and UE is configured to report during the on-duration, it would be required to monitor the PDCCH on duration or at least a part of the on duration after it has sent the beam report.

Proposal 13: Define failure detection and recovery mechanism for PDCCH scrambled by PS-RNTI.

# 3	Conclusions

In this contribution we have discussed about PDCCH-based power saving channel design.

The following observations and proposal were made:

Observation 1: No new signal nor channel need to be introduced for triggering adaptation of UE power saving inside or outside active time.

Proposal 1: To have logical naming convention especially in respect to other working groups, the adaptation triggering outside active time would be referred as DCI format monitored using PS-RNTI or PDCCH scrambled by PS-RNTI.

Observation 2: RAN1#97 already made an agreement that the CORESET can be shared between active time and outside active time.

Observation 3: Before concluding the feasibility of having independent CORESET for PDCCH scrambled by PS-RNTI, the UE behaviour in relation to other PDCCHs to monitor needs to be clarified.

Proposal 2: Based on the agreement preceding the working assumption in RAN1#97 consider the following working assumption to be confirmed:

•	The CORESET for power saving signal/channel outside Active Time can be associated with (in addition to search space set for power saving signal/channel) other search space set(s).

Proposal 3: Confirm working assumption:

•	UE monitors the PDCCH-based power saving signal/channel outside Active Time being configured on the active BWP in an active cell.

Proposal 4: The (new) DCI format scrambled by PS-RNTI is not accounted when determining the DCI format size budget.

Proposal 5: The total number of configured CORESETs both outside and inside active time is less than or equal to the number according to UE’s capability per BWP.

Observation 4: UE behaviour needs to be determined for the cases when UE triggered procedures like RA or BFR overlap in time with PDCCH scrambled by PS-RNTI monitoring.

Proposal 6: Support one of the following options when UE initiates random access procedure outside active time:

- Monitor both RA-RNTI and PS-RNTI

  - Prioritize RA-RNTI monitoring in case QCL assumptions are different for PS-RNTI monitoring

- Monitor only RA-RNTI

Proposal 7: If the UE misses the monitoring occasion(s) of PDCCH scrambled by PS-RNTI, due to other procedures, UE shall monitor PDCCH(s) on next onDuration.

Proposal 8: More than one monitoring occasion can be configured within multiple consecutive slots before the DRX ON.

Proposal 9: Define the following parameters within the search space set configuration that are valid only outside of Active time – during Active time UE determines monitoring occasions for the search space set as in Rel15.

- X offset: defines the starting slot for PDCCH scrambled by PS-RNTI monitoring before the onDuration

  - applicable only outside of Active time – during Active time UE determines monitoring occasions for the search space set as in Rel15

  - applicable to both short and long DRX cycle

  - offset needs to take into account reported UE capabilities related to adaptations that can be triggered by the power saving channel

- duration: defines the number of consecutive slots where to monitor PDCCH scrambled by PS-RNTI starting from the slot defined by X offset outside Active time

Proposal 10: It can be separately configured whether PDCCH scrambled by PS-RNTI is monitored before onDuration when short DRX cycle is applied.

Proposal 11: Support the new DCI format monitoring configuration also in UESS.

Observation 5: It could be beneficial to have possibility for frequency domain adaptation in CC domain in the wake-up DCI if the latency for ‘re-enabling’ PDCCH monitoring on SCells is short enough.

Proposal 12: New DCI format outside active time has the following indication fields per UE:

- 1 bit indication to trigger PDCCH monitoring inside active time (bit needed per UE because of group common DCI format)

- [1 bit] indication to indicate UE whether to monitor PDCCH in PCell only or both in PCell and all active SCells (subject to progress in DCCA WI)

Proposal 13: Define failure detection and recovery mechanism for wake-up link.

# References

- RP-190727, “New WID: UE Power Saving in NR”, CATT, CAICT

- RP-191607, “New WID: UE Power Saving in NR”, CATT, CAICT

- RP-192326, Clarifying RAN1 scope for combining dormancy and power saving signals, Qualcomm, InterDigital

- R1-1910604, Efficient CA design, Nokia, Nokia Shanghai Bell