3GPP TSG RAN WG1 #98bis									    R1-1911279

Chongqing, China, Oct 14th – 20th, 2019

Agenda Item:	7.2.4.4

Source:	InterDigital, Inc.

Title:	In-device Coexistence between LTE and NR V2X Sidelinks

Document for:	Discussion and Decision

# Introduction

In RAN1 #98, the following agreement has been made:

Agreements:

Unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction), then

- It is up to UE implementation to handle LTE Tx/NR Rx overlap.

- It is up to UE implementation to handle NR Tx and LTE Rx overlap.

Agreements:

- RAN1 understand that NR V2X priority field and PPPP are directly comparable i.e. the same numerical value has the same meaning in both the RATs.

  - Ask SA2 to confirm the understanding. If understanding is incorrect, please provide solution.

Draft LS in R1-1909818 (Sudhir, QC), which is approved (with the update “in-device”) with final LS in R1-1909876

In this contribution, we discuss the remaining issues on in-device coexistence between LTE and NR V2X sidelinks.

# Discussion

## Time overlap avoidance

To reduce overlapping between NR sidelink transmission and LTE sidelink transmission/reception, some enhancements on NR sidelink resource allocation can be applied to both mode 1 UE and mode 2 UE.

### Mode 1 UE reporting

When a UE operates in Mode 1 for NR sidelink transmission and in Mode 4 for LTE sidelink transmission, the NR sidelink transmission is scheduled by the network (e.g., gNB). It is beneficial that the network supports a coordinated scheduling scheme such that the simultaneous NR TX and LTE TX is avoided using TDM solutions. To facilitate the network coordination, assistance information can be provided by the UE to the network. For example, a UE may notify the network that it is in dual sidelink mode and the network can use a coordinated scheduling scheme accordingly. The UE may also inform the network about its synchronization source and resource reservations (e.g., periodic scheduling) in LTE sidelink.

A similar scheme can be applied to  avoid overlapping between NR sidelink transmission and LTE sidelink reception. A simultaneous NR TX and LTE RX is not feasible due to the half-duplex limitation when NR and LTE SL operate on adjacent channel. A mode 1 UE receives NR sidelink transmission scheduling from the network (e.g., gNB). To facilitate the scheduling to avoid NR TX at LTE RX instances, the UE can report the LTE SL data reception time and priority information to the network.

Proposal 1: For a NR mode 1 UE, gNB should support a coordinated scheduling scheme such that the simultaneous NR sidelink transmission and LTE sidelink transmission/reception is avoided in TDM solutions.

Proposal 2: To facilitate gNB’s NR sidelink scheduling in coordination of LTE sidelink, a NR mode 1 UE should report to gNB its LTE sidelink resource reservation information, including the reservation timing and the data QoS associated with the reservation.

### Mode 2 UE resource allocation

When a UE operates in Mode 2 for NR sidelink transmission and is in Mode 3 or Mode 4 for LTE sidelink transmission, the UE autonomously reserve the NR sidelink resources for NR sidelink transmission. The UE may reserve LTE SL resources either via network scheduling (for LTE Mode 3) or UE autonomous selection (for LTE Mode 4). The LTE SL resource reservation should be considered in the UE’s NR sidelink resource selection procedure. Specifically, to avoid the simultaneous NR TX and LTE TX in TDM solutions, the time units overlapped by reported reserved resources for LTE sidelink transmission should be excluded in the NR sidelink resource selection procedure.

A similar scheme can be applied to avoid overlapping between NR sidelink transmission and LTE sidelink reception. A simultaneous NR TX and LTE RX may not feasible as discussed above. The UE can obtain the LTE RX resource reservation by sensing transmission of either Mode 3 or Mode 4 UEs. Subsequently, the UE can consider the LTE sidelink reception timing and data priority in its NR sidelink resource selection procedure to avoid simultaneous NR sidelink transmission and LTE sidelink reception.

Proposal 3: For a UE in NR mode 2, the UE’s NR sidelink resource selection procedure should consider the resource reservation in LTE sidelink transmission and reception.

## Determination of SL TX and RX instance and its priority

It is agreed that it is up to UE implementation to handle TX/RX overlap between LTE and NR SL unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction). Thus, it can be inferred that a SL TX or RX instance and its priority can be known prior to the time of collision.

While it is straightforward that a UE can determine a SL TX instance and its priority based on resource selection and the priority of the TB in the buffer, further discussions may be needed regarding how a UE determines the LTE or NR RX instance and its associated priority.

For periodic traffic supported by both LTE and NR V2X, the determination of a future PSSCH RX instance and its priority can be determined based on the semi-persistent resource reservation. Furthermore, the future RX instances can include initial transmission of a TB, blind re-transmission and HARQ re-retransmission, because the resource reservation of the re-transmissions are agreed to be included in the initial transmission of the TB.  The priority of the LTE or NR PSSCH RX at the time of collision can be an estimate based on the priority of past TBs received in the same semi-persistent resource reservation.

For aperiodic traffic supported by NR V2X, a PSSCH RX instance (including blind and/or HARQ re-transmissions) and its priority can be determined based on a resource reservation of an initial transmission and/or retransmissions of aperiodic traffic, e.g. in a stand-alone PSCCH transmission.  For PSFCH transmission, a UE can determine a PSFCH TX/RX instance based on the HARQ timing and its priority can be based on the priority of the associated PSSCH.

Proposal 4: When PSFCH TX/RX overlaps with an LTE SL TX/RX, the priority of PSFCH is derived based on the priority of the associated PSSCH.

# Conclusion

In this contribution, we provided our views on the TDM-based solutions for the coexistence between LTE sidelink and NR sidelink. Our proposals and observation are as follows:

Proposal 1: For a NR mode 1 UE, gNB should support a coordinated scheduling scheme such that the simultaneous NR sidelink transmission and LTE sidelink transmission/reception is avoided in TDM solutions.

Proposal 2: To facilitate gNB’s NR sidelink scheduling in coordination of LTE sidelink, a NR mode 1 UE should report to gNB its LTE sidelink resource reservation information, including the reservation timing and the data QoS associated with the reservation.

Proposal 3: For a UE in NR mode 2, the UE’s NR sidelink resource selection procedure should consider the resource reservation in LTE sidelink transmission and reception.

Proposal 4: When PSFCH TX/RX overlaps with an LTE SL TX/RX, the priority of PSFCH is derived based on the priority of the associated PSSCH.

# References

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #97 Meeting, Reno, USA, May 2019.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #96 Meeting, Athens, Greece, Feb. 2019

- RP-190766, “New WID on 5G V2X with NR sidelink,” Mar. 2019.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #94 Meeting, Gothenburg, Sweden, Aug. 2018.