3GPP TSG RAN WG1 Meeting #99                                                                           R1-1912928

Reno, USA, November 18th – 22th, 2019

Title:	[DRAFT] Reply LS on CSI/SRS reporting

Response to:	R2-1914200

Release:	Release 16

Work Item:	NR_UE_pow_sav

Source:	Qualcomm [RAN1]

To:	RAN2

Contact Person:

Name: 	        Wooseok Nam

E-mail Address:	 wnam@qti.qualcomm.com

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-1914200 on P/SP CSI reporting and SRS transmission when PDCCH-WUS does not wake the UE up.

From the RAN2 agreements in RAN2#107 meeting, RAN1 has the same understanding that the UE would not report P/SP CSI and SRS during the next DRX cycle if PDCCH-WUS does not indicate the UE to wake up. However, in RAN1’s view, enhanced behavior may be needed for the scenarios where P/SP CSI reporting and SRS transmission is necessary for UEs to maintain the radio link quality and serving beam pairs. On the other hand, changing the current understanding based on already made agreements to have P/SP CSI reporting and SRS transmission regardless of WUS would diminish UE power saving and this would not be desirable.

In RAN1#99 meeting, RAN1 will discuss any mechanism enabling CSI reporting and/or SRS transmission when the UE is configured with PDCCH-WUS, without violating RAN2’s agreements. Some of, but not limited to, the following schemes can be considered as the candidate solution:

- Regardless of the traffic status, NW sends PDCCH-WUS to wake the UE up and trigger P/SP CSI reporting and SRS transmission. This solution can already be supported with existing RAN1 and RAN2 agreements.

- When the UE is not indicated to wake-up, the UE is allowed to report CSI in an event-triggered manner (e.g., based on L1-RSRP threshold).

- Regardless of the indication by PDCCH-WUS, the UE is configured to wake-up at anchor DRX cycles (e.g., every Nth DRX cycles) and reports P/SP CSI and SRS.

- At least for some types of CSI (e.g., L1-RSRP reporting), P/SP reporting is not affected by PDCCH-WUS.

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #99	18 – 22 November 2019, Reno, US