3GPP TSG-RAN WG1 Meeting #99	R1-1913175

# Reno, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of two-step RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.2	Transmission timing adjustments

A UE can be provided a value  of a timing advance offset for a serving cell by n-TimingAdvanceOffset for the serving cell. If the UE is not provided n-TimingAdvanceOffset for a serving cell, the UE determines a default value  of the timing advance offset for the serving cell as described in [10, TS 38.133].

If a UE is configured with two UL carriers for a serving cell, a same timing advance offset value  applies to both carriers.

Upon reception of a timing advance command for a TAG, the UE adjusts uplink timing for PUSCH/SRS/PUCCH transmission on all the serving cells in the TAG based on a value  that the UE expects to be same for all the serving cells in the TAG and based on the received timing advance command where the uplink timing for PUSCH/SRS/PUCCH transmissions is the same for all the serving cells in the TAG.

For a band with synchronous contiguous intra-band EN-DC in a band combination with non-applicable maximum transmit timing difference requirements as described in Note 1 of Table 7.5.3-1 of [10, TS 38.133], if the UE indicates ul-TimingAlignmentEUTRA-NR as 'required' and uplink transmission timing based on timing adjustment indication for a TAG from MCG and a TAG from SCG are determined to be different by the UE, the UE adjusts the transmission timing for PUSCH/SRS/PUCCH transmission on all serving cells part of the band with the synchronous contiguous intra-band EN-DC based on timing adjustment indication for a TAG from a serving cell in MCG in the band. The UE is not expected to transmit a PUSCH/SRS/PUCCH in one CG when the PUSCH/SRS/PUCCH is overlapping in time, even partially, with random access preamble transmitted in another CG.

For a SCS of  kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of . The start timing of the random access preamble is described in [4, TS 38.211].

In case of random access response, a timing advance command [11, TS 38.321], , for a TAG indicates  values by index values of  = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of  kHz is .  is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response.

In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of a current  value, , to the new  value, , by index values of  = 0, 1, 2,..., 63, where for a SCS of  kHz, .

If a UE has multiple active UL BWPs, as described in Subclause 12, in a same TAG, including UL BWPs in two UL carriers of a serving cell, the timing advance command value is relative to the largest SCS of the multiple active UL BWPs. The applicable  value for an UL BWP with lower SCS may be rounded to align with the timing advance granularity for the UL BWP with the lower SCS while satisfying the timing advance accuracy requirements in [10, TS38.133].

Adjustment of an  value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing for the TAG by a corresponding amount, respectively.

For a timing advance command received on uplink slot  and for a transmission other than a PUSCH scheduled by a RAR UL grant as described in Subclause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in Subclause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot  where ,  is a time duration in msec of  symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured,  is a time duration in msec of  symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214],  is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits,  is the number of slots per subframe, and  is the subframe duration of 1 msec.  and  are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For , the UE assumes  [6, TS 38.214]. Slot  and  are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG.  is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot  is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].

If a UE changes an active UL BWP between a time of a timing advance command reception and a time of applying a corresponding adjustment for the uplink transmission timing, the UE determines the timing advance command value based on the SCS of the new active UL BWP. If the UE changes an active UL BWP after applying an adjustment for the uplink transmission timing, the UE assumes a same absolute timing advance command value before and after the active UL BWP change.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as described in [10, TS 38.133], the UE changes  accordingly.

If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot.

## *** Unchanged text is omitted ***

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Subclauses 7.1.1, 7.2.1, and 7.3.1.

A PUSCH/PUCCH/SRS/PRACH transmission occasion  is defined by a slot index  within a frame with system frame number , a first symbol  within the slot, and a number of consecutive symbols .

## 7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Subclause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Subclause 7.1.1. For a PUSCH transmission scheduled by a DCI format 0_1 or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook' and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource. The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

-	is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of serving cell  in PUSCH transmission occasion .

-	 is a parameter composed of the sum of a component  and a component  where .

-	If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Subclause 8.3, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or  dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in Subclause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission scheduled by a RAR UL grant as described in Subclause 8.3,

, , and ,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or  dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in Subclause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in Subclause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by preambleReceivedTargetPower and ΔMsgA_PUSCH is provided by msgADeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgADeltaPreamble is not provided, for carrier f of serving cell c

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, ,  is provided by p0-NominalWithoutGrant, or  if p0-NominalWithoutGrant is not provided, and  is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a  value, applicable for all , is provided by p0-NominalWithGrant, or  if p0-NominalWithGrant is not provided, for each carrier  of serving cell  and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the value of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	For

-	For ,

-	if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided,  is the value of msg3-Alpha

-	elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha

-	else,

-	For ,  is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a set of  values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the values of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	 is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  and  is a SCS configuration defined in [4, TS 38.211]

-	is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index  for the active DL BWP, as described in Subclause 12, of carrier  of serving cell

-	If the UE is not provided PUSCH-PathlossReferenceRS or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource from the SS/PBCH block that the UE uses to obtain MIB

-	If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index  in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS

-	If the PUSCH transmission is scheduled by a RAR UL grant as described in Subclause 8.3, or for a PUSCH transmission for Type-2 random access procedure as described in Subclause 8.1A, the UE uses the same RS resource index  as for a corresponding PRACH transmission

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 and a set of PUSCH-PathlossReferenceRS-Id values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the RS resource index  from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP  of each carrier  and serving cell , as described in Subclause 9.2.2, the UE uses the same RS resource index  as for a PUCCH transmission in the PUCCH resource with the lowest index

-	If the PUSCH transmission is scheduled by a DCI format 0_0 and if the UE is not provided a spatial setting for a PUCCH transmission, or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index  with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index  from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission. If the DCI format activating the PUSCH transmission does not include a SRI field, the UE determines a RS resource index  with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

= referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

If the UE is not configured periodic CSI-RS reception, referenceSignalPower is provided by ss-PBCH-BlockPower. If the UE is configured periodic CSI-RS reception, referenceSignalPower is provided either by ss-PBCH-BlockPower or by powerControlOffsetSS providing an offset of the CSI-RS transmission power relative to the SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB.

-	 for  and  for  where  is provided by deltaMCS for each UL BWP  of each carrier  and serving cell . If the PUSCH transmission is over more than one layer [6, TS 38.214], .  and , for active UL BWP  of each carrier  and each serving cell , are computed as below

-	 for PUSCH with UL-SCH data and  for CSI transmission in a PUSCH without UL-SCH data, where

-	 is a number of transmitted code blocks,  is a size for code block , and  is a number of resource elements determined as , where  is a number of symbols for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell ,  is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol , , and ,  are defined in [5, TS 38.212]

-	 when the PUSCH includes UL-SCH data and , as described in Subclause 9.3, when the PUSCH includes CSI and does not include UL-SCH data

-	 is the modulation order and  is the target code rate, as described in [6, TS 38.214], provided by the DCI format scheduling the PUSCH transmission that includes CSI and does not include UL-SCH data

-	For the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  in PUSCH transmission occasion

-	 is a TPC command value included in a DCI format 0_0 or DCI format 0_1 that schedules the PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in Subclause 11.3

-	 if the UE is configured with twoPUSCH-PC-AdjustmentStates and  if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in Subclause 8.3, or if the PUSCH transmission is for a Type-2 random access procedure as described in Subclause 8.1A

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of  is provided to the UE by powerControlLoopToUse

-	If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in DCI format 0_1 and the  value(s) provided by sri-PUSCH-ClosedLoopIndex. If the PUSCH transmission is scheduled by a DCI format 0_1 and if DCI format 0_1 includes a SRI field, the UE determines the  value that is mapped to the SRI field value

-	If the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if a SRI-PUSCH-PowerControl is not provided to the UE,

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is not provided tpc-Accumulation, where

-	The  values are given in Table 7.1.1-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUSCH transmission occasion  and  symbols before PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUSCH power control adjustment state , where  is the smallest integer for which  symbols before PUSCH transmission occasion  is earlier than  symbols before PUSCH transmission occasion

-	If a PUSCH transmission is scheduled by a DCI format 0_0 or DCI format 0_1,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission

-	If a PUSCH transmission is configured by ConfiguredGrantConfig,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	A UE resets accumulation of a PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  to

-	If a configuration for a corresponding  value is provided by higher layers

-	If a configuration for a corresponding  value is provided by higher layers

-	If  and the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, and the UE is provided higher SRI-PUSCH-PowerControl, the UE determines the value of  from the value of  based on an indication by the SRI field for a sri-PUSCH-PowerControlId value associated with the sri-P0-PUSCH-AlphaSetId value corresponding to  and with the sri-PUSCH-ClosedLoopIndex value corresponding to

-	If  and the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field or the UE is not provided SRI-PUSCH-PowerControl,

-	If ,  is provided by the value of powerControlLoopToUse

-	 is the PUSCH power control adjustment state for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	 absolute values are given in Table 7.1.1-1

-	If the UE receives a random access response message in response to a PRACH transmission on active UL BWP  of carrier  of serving cell  as described in Subclause 8

-	, where  and

-	 is a TPC command value indicated in the random access response grant of the random access response message corresponding to the PRACH transmission on active UL BWP  of carrier  in the serving cell , and

-	 and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last random access preamble for carrier  in the serving cell ,  is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP  of carrier  of serving cell , and  is the power adjustment of first PUSCH transmission on active UL BWP  of carrier  of serving cell

-	If the UE transmits the PUSCH on active UL BWP b  of carrier f of serving cell c as described in Subclause 8.1A, fb,f,c(0,l)=ΔPrampup,b,f,c, where

-	l=0, and

-	 and ΔPrampuprequested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers, MRB,b,f,cPUSCH(0) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks, and ΔTF,b,f,c(0) is the power adjustment of the PUSCH transmission

Table 7.1.1-1: Mapping of TPC Command Field in DCI format 0_0, DCI format 0_1, or DCI format 2_2, with CRC scrambled by TPC-PUSCH-RNTI, or DCI format 2_3, to absolute and accumulated  values or  values

| TPC Command Field | Accumulated or [dB] | Absolute or [dB] |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

## 7.2	Physical uplink control channel

If the UE is configured with a SCG, the UE shall apply the procedures described in this subclause for both MCG and SCG.

-	When the procedures are applied for MCG, the term 'serving cell' in this subclause refers to serving cell belonging to the MCG.

-	When the procedures are applied for SCG, the term 'serving cell' in this subclause refers to serving cell belonging to the SCG. The term 'primary cell' in this subclause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this subclause for both primary PUCCH group and secondary PUCCH group.

-	When the procedures are applied for the primary PUCCH group, the term 'serving cell' in this subclause refers to serving cell belonging to the primary PUCCH group.

-	When the procedures are applied for the secondary PUCCH group, the term 'serving cell' in this subclause refers to serving cell belonging to the secondary PUCCH group. The term 'primary cell' in this subclause refers to the PUCCH-SCell of the secondary PUCCH group.

### 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP  of carrier  in the primary cell  using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power  in PUCCH transmission occasion  as

[dBm]

where

-	 is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of serving cell  in PUCCH transmission occasion

-	 is a parameter composed of the sum of a component , provided by p0-nominal, or  dBm if p0-nominal is not provided, for carrier  of primary cell  and, if provided, a component  provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP  of carrier  of primary cell , where .  is a size for a set of  values provided by maxNrofPUCCH-P0-PerSet. The set of  values is provided by p0-Set. If p0-Set is not provided to the UE, ,

-	If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If the UE is not provided PUCCH-SpatialRelationInfo, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set

-	 is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell  and  is a SCS configuration defined in [4, TS 38.211]

-	is a downlink pathloss estimate in dB calculated by the UE using RS resource index  as described in Subclause 7.1.1 for the active DL BWP  of carrier  of the primary cell  as described in Subclause 12

-	If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB

-	If the UE is provided a number of RS resource indexes, the UE calculates  using RS resource with index , where .  is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS

-	If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index  on the active DL BWP of the serving cell

-	If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on a same serving cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	The parameter  is a value of deltaF-PUCCH-f0 for PUCCH format 0, deltaF-PUCCH-f1 for PUCCH format 1, deltaF-PUCCH-f2 for PUCCH format 2, deltaF-PUCCH-f3 for PUCCH format 3, and deltaF-PUCCH-f4 for PUCCH format 4, if provided; otherwise ΔF_PUCCHF=0.

-	 is a PUCCH transmission power adjustment component on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 0 or PUCCH format 1,  where

-	 is a number of PUCCH format 0 symbols or PUCCH format 1 symbols included in a PUCCH resource of a PUCCH resource set indicated by a value of a PUCCH resource indicator field in DCI format 1_0 or DCI format 1_1, or provided by nrofSymbols in PUCCH-format0 or in PUCCH-format1 respectively

-	 for PUCCH format 0

-	 for PUCCH format 1

-	 for PUCCH format 0

-	 for PUCCH format 1, where  is a number of UCI bits in PUCCH transmission occasion

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, , where

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Subclause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Subclause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided with pdsch-HARQ-ACK-Codebook,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Subclause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Subclause 9.2.5.2

-	 is a number of resource elements determined as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Subclause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where

-

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Subclause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Subclause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided pdsch-HARQ-ACK-Codebook,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Subclause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Subclause 9.2.5.2

-	 is a number of CRC bits that the UE determines as described in Subclause 9.2.5

-	 is a number of resource elements that the UE determines as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Subclause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell .

-	For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value and is included in a DCI format 1_0 or DCI format 1_1 for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion  or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Subclause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	If the UE obtains a TPC command value from a DCI format 1_0 or a DCI format 1_1 and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the  value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of  through the link to a corresponding p0-PUCCH-Id index

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the current PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUCCH transmission occasion , where

-	The  values are given in Table 7.1.2-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUCCH transmission occasion  and  symbols before PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUCCH power control adjustment state, where  is the smallest integer for which  symbols before PUCCH transmission occasion  is earlier than  symbols before PUCCH transmission occasion

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

-	Else,

-	, where , and  is

-	the TPC command value indicated in a random access response grant corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant corresponding to PRACH and PUSCH transmissions according to Type-2 random access procedure with RAR message(s) for fallbackRAR, or

-	the TPC command value indicated in a random access response corresponding to PRACH and PUSCH transmissions according to Type-2 random access procedure with RAR message(s) for successRAR, or

-	the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception, and,

if the UE transmits PUCCH on active UL BWP  of carrier  of serving cell ,

;

otherwise,

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of primary cell , and  corresponds to PUCCH format 0 or PUCCH format 1

Table 7.2.1-1: Mapping of TPC Command Field in DCI format 1_0 or DCI format 1_1 or DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI to accumulated  values

| TPC Command Field | Accumulated [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 0 |
| 2 | 1 |
| 3 | 3 |

## *** Unchanged text is omitted ***

# 8	Random access procedure

Prior to initiation of the physical random access procedure, Layer 1 receives from higher layers a set of SS/PBCH block indexes and provides to higher layers a corresponding set of RSRP measurements.

Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a Type-1 random access procedure, as described in Subclauses 8.1 through 8.4, or a Type-2 random access procedure as described in Subclauses 8.1 through 8.2A.

Prior to initiation of the physical random access procedure, Layer 1 receives the following information from the higher layers:

-	Configuration of physical random access channel (PRACH) transmission parameters (PRACH preamble format, time resources, and frequency resources for PRACH transmission).

-	Parameters for determining the root sequences and their cyclic shifts in the PRACH preamble sequence set (index to logical root sequence table, cyclic shift (), and set type (unrestricted, restricted set A, or restricted set B)).

From the physical layer perspective, the Type-1 L1 random access procedure includes the transmission of random access preamble (Msg1) in a PRACH, random access response (RAR) message with a PDCCH/PDSCH (Msg2), and when applicable, the transmission of a PUSCH scheduled by a RAR UL grant, and PDSCH for contention resolution.

From the physical layer perspective, the Type-2 L1 random access procedure includes the transmission of random access preamble in a PRACH and of a PUSCH (MsgA) and the reception of a RAR message with a PDCCH/PDSCH (MsgB).

If a random access procedure is initiated by a PDCCH order to the UE, a PRACH transmission is with a same SCS as a PRACH transmission initiated by higher layers.

If a UE is configured with two UL carriers for a serving cell and the UE detects a PDCCH order, the UE uses the UL/SUL indicator field value from the detected PDCCH order to determine the UL carrier for the corresponding PRACH transmission.

## 8.1	Random access preamble

Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission [4, TS 38.211].

-	A preamble index, a preamble SCS, , a corresponding RA-RNTI, and a PRACH resource.

A PRACH is transmitted using the selected PRACH format with transmission power , as described in Subclause 7.4, on the indicated PRACH resource.

For Type-1 random access procedure, a UE is provided a number  of SS/PBCH blocks associated with one PRACH occasion and a number  of contention based preambles per SS/PBCH block per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. For Type-2 random access procedure with common PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH blocks associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number R of contention based preambles per SS/PBCH block per valid PRACH occasion by msgA-CB-PreamblesPerSSB. For Type-2 random access procedure with separate PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH blocks associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB-msgA when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

If , one SS/PBCH block is mapped to  consecutive valid PRACH occasions and  contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index 0. If ,  contention based preambles with consecutive indexes associated with SS/PBCH block , , per valid PRACH occasion start from preamble index  where  is provided by totalNumberOfRA-Preambles for Type-1 random access procedure, or by msgA-totalNumberOfRA-Preambles for Type-2 random access procedure, and is an integer multiple of .

For link recovery, a UE is provided  SS/PBCH blocks associated with one PRACH occasion by ssb-perRACH-Occasion in BeamFailureRecoveryConfig. For a dedicated RACH configuration provided by RACH-ConfigDedicated, if cfra is provided, a UE is provided  SS/PBCH blocks associated with one PRACH occasion by ssb-perRACH-Occasion in occasions. If , one SS/PBCH block is mapped to  consecutive valid PRACH occasions. If , all consecutive  SS/PBCH blocks are associated with one PRACH occasion.

SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211].

-	First, in increasing order of preamble indexes within a single PRACH occasion

-	Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Fourth, in increasing order of indexes for PRACH slots

An association period, starting from frame 0, for mapping SS/PBCH blocks to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH blocks are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH blocks to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions that are not mapped to  SS/PBCH blocks, no SS/PBCH blocks are mapped to the set of PRACH occasions. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH blocks after an integer number of association periods, if any, are not used for PRACH transmissions.

For a PRACH transmission triggered by a PDCCH order, the PRACH mask index field [5, TS 38.212], if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order.

For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For the indicated preamble index, the ordering of the PRACH occasions is

-	First, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Third, in increasing order of indexes for PRACH slots

For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

For paired spectrum all PRACH occasions are valid. For unpaired spectrum, if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last SS/PBCH block reception symbol, where  is provided in Table 8.1-2.

If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last downlink symbol and at least  symbols after a last SS/PBCH block transmission symbol, where  is provided in Table 8.1-2.

For preamble format B4 [4, TS 38.211], .

Table 8.1-2:  values for different preamble SCS

| Preamble SCS |  |
| --- | --- |
| 1.25 kHz or 5 kHz | 0 |
| 15 kHz or 30 kHz or 60 kHz or 120 kHz | 2 |

If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to  msec, where  is a time duration of  symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214],  if the active UL BWP does not change and  is defined in [10, TS 38.133] otherwise, and  msec for FR1 and  msec for FR2. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines  assuming SCS configuration .

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than  symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where  for  or ,  for  or , and  is the SCS configuration for the active UL BWP.

## 8.1A	PUSCH for Type-2 random access procedure

For a Type-2 random access procedure, a UE transmits a PUSCH after transmitting a PRACH. PUSCH occasions for PUSCH transmissions are defined by a set of frequency resources, time resources, and DMRS configuration.

For the indicated preamble index, the ordering of the PRACH occasions mapped to corresponding PUSCH occasions is

-	first, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

-	second, in increasing order of DMRS indexes within a PUSCH occasion

-	third, in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot

-	fourth, in increasing order of indexes for PUSCH slots

When a UE is provided multiple PUSCH configurations by msgA-PUSCH-configList in an UL BWP, the mapping between PRACH occasions and PUSCH occasions is per PUSCH configuration.

A UE determines time resources and frequency resources for PUSCH occasions in an active UL BWP from msgA-PUSCH-config for the active UL BWP. If the active UL BWP is not the initial UL BWP and msgA-PUSCH-config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-config provided for the initial active UL BWP.

A UE determines a first RB for a first PUSCH occasion in an active UL BWP from frequencyStartMsgAPUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of RBs provided by nrofPRBsperMsgAPO. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgAPUSCH. A number of PUSCH occasions in the frequency domain of an UL BWP is provided by nrMsgAPO-FDM.

A UE determines a first slot for a first PUSCH occasion in an active UL BWP from msgAPUSCH-timeDomainOffset that provides an offset, in number of slots in the active UL BWP, relative to the start of each PRACH slot. The UE expects that a first PUSCH occasion in each slot has a same SLIV [6, TS 38.214] for a PUSCH transmission that is provided by startSymbolAndLengthMsgAPO. If the UE does not have dedicated RRC configuration, the SLIV and PUSCH mapping type are included in a table for time domain resource allocation [4, TS 38.211]. Consecutive PUSCH occasions within each slot are separated by guardPeriodMsgAPUSCH symbols and have a same duration. A number of time domain PUSCH occasions in each slot is provided by nrofMsgAPOperSlot and a number of slots that include PUSCH occasions is provided by nrofSlotsMsgAPUSCH.

A UE is provided a DMRS configuration for a PUSCH transmission in a PUSCH occasion in an active UL BWP by msgA-DMRS-Configuration.

A UE is provided an MCS for data information in a PUSCH transmission for a PUSCH occasion by msgA-MCS.

For a PUSCH transmission with frequency hopping in a slot, when indicated by msgA-intraSlotFrequencyHopping for the active UL BWP, the frequency offset for the second hop [6, TS 38.214] is determined as described in Subclause 8.3, Table 8.3-1 using msgA-HoppingBits instead of .A PUSCH transmission uses a same spatial filter as an associated PRACH transmission.

A UE determines whether or not to apply transform precoding for a PUSCH transmission as described in [6, TS 38.214].

A PUSCH occasion is valid if it does not overlap in time and frequency with any PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2.

## 8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Subclause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in Subclause 10.1. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For , the UE assumes  [6, TS 38.214].

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Subclause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for a secondary cell, the UE may assume the DM-RS antenna port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set for receiving the PDCCH that includes the DCI format 1_0.

A RAR UL grant schedules a PUSCH transmission from the UE. The contents of the RAR UL grant, starting with the MSB and ending with the LSB, are given in Table 8.2-1.

If the value of the frequency hopping flag is 0, the UE transmits the PUSCH without frequency hopping; otherwise, the UE transmits the PUSCH with frequency hopping.

The UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [6, TS 38.214].

The TPC command value  is used for setting the power of the PUSCH transmission, as described in Subclause 7.1.1, and is interpreted according to Table 8.2-2.

The CSI request field is reserved.

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14 |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |

Table 8.2-2: TPC Command  for PUSCH

| TPC Command | Value (in dB) |
| --- | --- |
| 0 | -6 |
| 1 | -4 |
| 2 | -2 |
| 3 | 0 |
| 4 | 2 |
| 5 | 4 |
| 6 | 6 |
| 7 | 8 |

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding RA-RNTI or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

## 8.2A	Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Subclause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in Subclause 10.1. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Subclause 8.2 when the UE detects a RAR UL grant, or

-	transmission of an ACK if the RAR message(s) is for successRAR

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits an ACK if the UE correctly detects the transport block or a NACK if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321]. The UE does not expect to be indicated to transmit a PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding RA-RNTI or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].