3GPP TSG-RAN WG1 Meeting #99	R1-1913682

# Reno, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0081 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of mobility enhancements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2019-12-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of mobility enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for power determination of transmissions and for PDCCH monitoring on the target cell group and on the source cell group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for mobility enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 17	Dual active protocol stack based handover

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and a maximum power PSCG for transmissions on the SCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and with an inter-CG power sharing mode by UplinkPowerSharingDAPS-HO-mode for FR1 and/or by UplinkPowerSharingDAPS-HO-mode for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Subclause 7.6.2 for UplinkPowerSharingDAPS-HO = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Subclause 7.6.2 for UplinkPowerSharingDAPS-HO = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Subclause 7.6.2 for UplinkPowerSharingDAPS-HO = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

If

-	the UE does not provide UplinkPowerSharingDAPS-HO, and

-	UE transmissions on the target cell and the source cell overlap

the UE transmits only on the target cell

UE transmissions on the target cell and the source cell overlap if they are in

-	overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band

-	overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively.

The UE can provide pdcch-BlindDetectionMCG1-UE to indicate a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellstarget downlink cells for the target MCG and pdcch-BlindDetectionMCG2-UE to indicate a capability monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellssource downlink cells for the source MCG. If the UE is provided search space sets on both the target MCG and the source MCG, the UE does not expect to have in any slot any USS set without allocated PDCCH candidates for monitoring on both the target MCG and the source MCG.