3GPP TSG-RAN WG1 NR Ad Hoc #3	R1-1716367

Nagoya, Japan, 18th – 21st, September 2017

Source:	Ericsson

Title:	Analysis of beam indication signalling options

Agenda Item:	6.2.2.6

Document for:	Discussion and Decision

# Introduction

In RAN1#90, the following agreement was made:

Agreement #1:

- For the purposes of beam indication for at least NR unicast PDSCH, support an N-bit indicator field in DCI which provides a reference to a DL RS which is spatially QCL’d with at least one PDSCH DMRS port group

  - An indicator state is associated with at least one index of a DL RS (e.g., CRI, SSB Index) where each index of downlink RS can be associated with a given DL RS type,  e.g., aperiodic CSI-RS, periodic CSI-RS, semi-persistent CSI-RS, or SSB,

    - Note: L1-RSRP reporting on SSB is not yet agreed

    - Note: One possibility to determine DL CSI-RS type is through the resource setting ID, other options are not precluded

  - The value of N is FFS, but is at most [3] bits

  - FFS: The case of more than one DMRS port group

  - FFS: Whether or not to indicate more than one beam indicator, NR strive to minimize the indicator overhead

- FFS: Signalling mechanism for the association of a DL RS index (e.g., CRI, SSB index) to an indicator state, e.g.,

  - The association is explicitly signaled to the UE

  - The association is implicitly determined by the UE

  - Combination of the above is not precluded

- FFS: An indicator state may or may not also include other parameter(s), e.g., for PDSCH to RE mapping purposes analogous to PQI in LTE, other QCL parameters

- FFS: Whether or not an indicator state may be associated with more than one DL RS index

- FFS: PDCCH beam indication may or may not be based on the beam indication states for PDSCH

In this contribution, we will analyse the signalling mechanisms regarding beam indication for PDSCH and PDCCH and describe the error cases. We will also list open issues, and propose solutions.

This is a revision of R1-1714297.

# Discussion

The need for beam indication mainly comes from the use of analog beamforming in the UE receiver. With such a receiver, the UE can only receive data from a certain direction, due to that spatial prefiltering is performed in the analog parts of the receiver. This leads to that signals that impinge at the UE from a direction outside its receive beam are suppressed.

However, the use of analog receivers does not by itself motivate beam indications. In fact, it is possible to operate beam management without beam indication, as demonstrated in [1]. The situations when beam indication is needed are one or more of the following:

- Several BPLs are maintained

- Simultaneous P2/P3 procedure is applied

- Beam grouping is used

In all other cases, beam indication is unnecessary.

Several agreements on beam indication have been made. In agreement #1, it is stated that beam indication should come in the form of a low-overhead QCL indicator. The UE will interpret the beam indicator that the DMRS of either the PDCCH or the PDSCH will be spatially QCL with a previously transmitted reference signal.

In our paper [2], we introduce a Transmission Configuration Indicator (TCI) to represent and convey at least the QCL assumption to the UE so that the UE can store the information. The TCI should be applicable both for the PDCCH and PDSCH beams, although the indication mechanisms could be different.

In [3], we provided a signalling error analysis for PDSCH beam indication over DCI. We will now investigate the various signalling options for PDCCH.

## PDCCH beam configuration/indication

We will now discuss general properties and requirements for MAC CE-based signalling.

### General MAC CE requirements for beam indication

In LTE, MAC CE is used to convey control information to the UE over MAC. Signalling over MAC CE have some properties that differ from the L1 signalling RAN1 usually defines:

- Since the messages are transmitted over MAC, they rely on HARQ. This means that the message is protected by a CRC, and that retransmissions are possible. Furthermore, the NW will know when the UE has successfully received the message.

- Due to the retransmissions, the time it takes to transmit a MAC CE message is unknown. The delay depends on how many retransmissions are required. In general of course, the delay of transmitting the MAC CE message is (at least) one slot. The time it takes until the NW receives the ACK/NAK depends on how the transmissions are allocated in the frame structure, and also on the time alignment between UL and DL. This means that it becomes somewhat tricky to use MAC CE for synchronous configurations.

In some cases, beam indication signalling can be very sensitive. If the UE loses a beam indication message, it could mean that beams become misaligned and that the UE would need to perform beam recovery, since the NW loses its ability to communicate with the UE. Hence, it is very important that beam indications reach the UE, and if the message is lost, it is important that the NW understands that. Here, the fact that the MAC CE is acknowledged helps.

It is also important that the gNB and the UE updates their beams at the same time. Any data that is sent during periods when the beams are misaligned may be lost. E.g., if the gNB updates its TX beam in slot 27, and the UE updates its RX beam in slot 35, everything the NW sends to the UE in slots 28-34 may be lost. Hence, it is important that the NW and the UE have a common understanding on when the beams should change. We note that some MAC CE messages in LTE don’t have this property: e.g., for the secondary cell activation [4], the time when the UE is required to perform the action varies within an interval.

Finally, the beam indication signalling needs to be fast. The RX beam needs to be updated to compensate for UE movement. If the beams are narrow, and the UE is travelling fast, the NW may need to update the beams rather rapidly. We will study this situation in more detail in the following subsection

#### Delay requirements for beam indication

To investigate the delay requirements for PDCCH beam indication, we study the scenario depicted in Figure 1.

Figure 1: Requirement scenario for PDCCH beam switch. A moving UE crosses a beam, and as the gNB updates its Tx beam, the QCL association changes.

In Figure 1, a fast-moving UE passes by a TRP with a Tx beam indicated by the dashed lines. As the UE moves across the beam, it discovers that the new beam becomes better, and transmits a measurement report to the NW. The NW receives the measurement report, and decides to update its Tx beam. The NW sends the beam indication to the UE. At a certain point in time, the TRP and the UE update their beams. The procedure should be completed as soon as possible, for sure it must be completed before the UE moves into the coverage area of another beam.

To make the situation in Figure 1 concrete, assume that a 120-degree sector is covered by 10 horizontal beams, implying that one beam is 12 degrees wide. Further assume that the UE passes 50m from the TRP. The distance over which the beam pointing in the broadside direction of the antenna can be calculate as

d=2×50×tan6*π180 =10 (m)

The beams at the TRP and the UE must be updated well before the UE has travelled 10m. For a UE travelling at 20m/s (72 km/h), this takes 500ms.

The chosen parameters are thought of as typical, rather than extreme values. Clearly, with narrower beams, the distance is shorter. If the UE passes closer to the TRP, the distance is shorter.

The required steps, and the estimated associated delays are summarized in  Table 1.

| Table 1: Estimated delays for beam indication signalling |  |  |
| --- | --- | --- |
| UE performs measurements on SS | 0-20 ms |  |
| The UE reports SSRP periodically | 2 ms | Reasonably aligned with SS transmissions |
| gNB processing + slot alignment: | 5 ms |  |
| (Initial) PDCCH+PDSCH transmission | 1 slot |  |
| PUCCH ACK: 1 slot | 1 slot |  |
| Possible retransmissions | (3 x # of retransmissions) slots |  |
| Activation delay | 5 slots | To discover missing ACK/NAK |
| Sum | 7-27 ms + 7 slots + (reTx) |  |

From Table 1, it is clear that the delay depends on the slot length. The beam indication procedures are mostly relevant for mmW, it is reasonable to assume a short slot length, say 125μs. With that assumption, we can state that the total delay is between 8ms and 29ms.

The delay for beam indication over MAC CE is between 8ms and 29ms.

Clearly, the delay is dominated by measurement delay. The measurement delay may actually be higher, if the UE fails to detect when a new beam becomes better. As with handover procedures, the NW may not update the beams based on an individual measurement.

Note that the beam switch is associated with a relatively large activation delay. This value has been chosen to enable a coordinated switch at the TRP and the UE.

### Signalling error analysis for beam indication signalling over MAC CE

In this section, we provide a detailed analysis of the PDCCH beam indication signalling when MAC CE is used.

#### MAC CE: No signalling error

The case where the MAC CE is correctly decoded by the UE is depicted in the signalling diagram in Figure 1. When the TRP transmits the MAC CE, it starts a timer, and when the UE decodes the MAC CE, it starts a timer. Once the timer expires, the TRP and the UE apply their new beams.

Figure 2: Beam indication to switch beam for PDCCH over MAC CE: no error.

#### MAC CE: PDCCH received, PDSCH decoding failed

The case where the UE receive the PDCCH but fails to decode the PDSCH is depicted in Figure 2. Since the UE decoded the PDCCH, it understands that the NW tried to send data, and sends a NAK over PUCCH to indicate that. The NW realizes that the beam switch did not reach the UE, and retransmits the MAC CE, relying on HARQ. Typically, the NW would have to restart the timer, to make sure that the UE will have time to adjust its beam once it has successfully decoded the beam indication.

Once the TRP receives an ACK of the MAC CE, it may proceed with the beam switch.

Figure 3: Beam indication to switch beam for PDCCH over MAC CE: PDSCH decoding failed.

#### MAC CE: PDSCH decoded, ACK missed

The case where the UE receives the PDSCH, but the NW misses the PUCCH ACK is depicted in Figure 3. The NW is expecting a response to the MAC CE sent over PDSCH, but it is never received. After some time, the NW will retransmit the MAC CE containing the beam indicator, this time not relying on HARQ.  The NW would have to restart the timer, to make sure that the UE will have time to adjust its beam once it has successfully decoded the beam indication.

Once the UE receives the retransmission of the beam switch command, it will stop the pending beam switch, and restart the timer, to make sure that its beam switch time instant coincides with that of the TRP.

Once the TRP receives an ACK of the MAC CE, it may proceed with the beam switch when the timer expires.

Figure 4: Beam indication to switch beam for PDCCH over MAC CE: ACK missed.

#### MAC CE: PDCCH missed

The case where the UE fails to decode the PDCCH is depicted in Figure 4. The NW is expecting a response to the MAC CE sent over PDSCH, but since the UE does not understand that it was scheduled, no response is sent. After some time, the NW will realize that the PDCCH never reached the UE, and retransmit the MAC CE containing the beam indicator.  The NW would have to restart the timer, to make sure that the UE will have time to adjust its beam once it has successfully decoded the beam indication.

Once the UE receives the retransmission of the beam switch command, it will stop the pending beam switch, and restart the timer, to make sure that its beam switch time instant coincides with that of the TRP.

Once the TRP receives an ACK of the MAC CE, it may proceed with the beam switch when the timer expires.

Figure 5: Beam indication to switch beam for PDCCH over MAC CE: PDCCH missed.

#### MAC CE: Unresolved error case: ACK missed, retransmitted PDSCH missed

As have been shown in the above signalling diagrams, the beam indication signalling is able to quite robust to lost messages, as long as only a single message is lost. In this section, we will describe an error case, where beam misalignment may occur. This situation will happen if two consecutive messages are lost.

In Figure 5, the UE decodes the MAC CE, but the ACK is lost, just as described in section 2.2.2.3. Note that UE starts preparing for a beam switch, and starts the timer. In this case, the UE also misses the MAC CE the NW retransmitted since it never received the ACK. Since the UE never received the retransmission, it will proceed with the beam switch as planned. Once the timer expires, the UE will change its Rx beam, but since the TRP never changed its Tx beam, subsequent PDCCH transmissions will be lost.

Figure 6: Beam indication to switch beam for PDCCH over MAC CE: unresolved error case.

We note that the probability of this error event is low: it requires that one PUCCH and one PDCCH are lost in sequence. Also, it may be possible to improve this situation by delaying the beam switch further, to allow the TRP to perform more retransmissions of the MAC CE.

### Robust PDCCH beam indication via DCI

In this section, we will address the FFS on DCI signalling in agreement #1. The reason why DCI was not agreed is a bit unclear, but one explanation could be that there is a concern that since DCI is unacknowledged, there is a higher risk for errors.

However, it is easy to avoid the error cases, by simply transmitting data in the PDSCH associated with the PDCCH where the beam switch command comes. The NW would include data in the PDSCH, and if the NW did not have any data in the buffer, it may simply send dummy data and ask for an ACK of that data. Any response from the UE would mean that the UE could decode the DCI, and hence the beam switch command.

If an ACK or a NAK is received in response to data transmitted over PDSCH is an acknowledgment that the UE was able to decode DCI.

We will use Observation 2 to design a robust beam switching command based on DCI.

#### No error case

As for the other signalling cases, we start with the no-error case, depicted in Figure 6. The TRP transmits a PDCCH containing the beam switch indicator, and at the same time, the TRP schedules data over PDSCH. When the TRP transmits the DCI, it starts a timer, and when the UE decodes the DCI, it starts a timer. When the TRP receives and ACK of the data transmitted over PDSCH, it knows that the PDSCH data was received, and also that the DCI was received. Hence, the TRP knows that the UE received the beam indicator, and may safely proceed with the beam switch, once the timer expires. Note that the timer must be set long enough to make sure that the UE will have time to send the ACK of the data.

Figure 7: Beam indication to switch beam for PDCCH over DCI: no error.

#### PDCCH received, PDSCH decoding failed

We will now investigate the interesting case where the PDCCH was received, but the PDSCH decoding failed. This situation is depicted in Figure 7.

Figure 8: Beam indication to switch beam for PDCCH over DCI. PDCCH reception is successful, but PDSCH decoding fails.

The interesting observation of the procedure in Figure 7 is that the PDSCH NAK the UE sent is still an acknowledgment that the DCI could be decoded. Hence, once the TRP received the NAK, it can proceed with the beam switch as planned! Once the timer expires, the TRP and the UE will switch to the new beam.

#### Other error cases

We realize that the analysis of the signalling cases described in section 2.1.2.3, 2.1.2.4, and 2.1.2.5 are applicable also for the DCI signalling. Basically, all the cases in section 2.1.2.3, 2.1.2.4, and 2.1.2.5 involve PDCCH decoding errors. Hence, the performance of DCI beam indication will never be worse than that of MAC CE.

## Comparison of MAC CE and DCI PDCCH beam indications

Robustness:

We have analysed MAC CE and DCI-based beam indication signalling for PDCCH in section 2.2.2 and 2.2.3, respectively. Based on the comparison we can conclude that

A robust version of DCI-based signalling can provide the same robustness with the same performance as MAC CE signalling.

Possibility to perform synchronized switches:

DCI signalling is inherently synchronous, due to the lack of retransmissions. The NW knows during which slot the DCI is transmitted, and the UE knows in which slot it received it.

Delay:

DCI does not add any signalling delay: the DCI is transmitted during the PDCCH, and the UE decodes it during a fraction of the slot. The uncertainties related to retransmissions are then avoided.

Other aspects:

However, MAC CE has other advantages compared to DCI. MAC CE provides a larger payload making it possible to provide a wider range of QCL parameters.

Note that allowing PDCCH beam indication over DCI is a small addition to what is already agreed. Since the UE would anyway be required to decode the DCI to update the PDSCH beam, that functionality would anyway be implemented in the UE. Since we rely on the PDSCH ACK/NAK to achieve robustness, there is no additional UE implementation required. The functionality that the UE would cancel a pending Rx beam update when it receives a new beam switch command is required for the MAC CE scheme as well:

Support for DCI-based beam switch for PDCCH reuses components already agreed for NR.

Hence, we propose:

NR support beam indication for PDCCH using DCI, in addition to configuration via MAC CE and RRC.

## Single-beam operation to realize DCI PDCCH beam indication

It is important to note that Proposal 1 does not necessarily mean that an additional explicit indication must be introduced. Much of the procedures on beam indication have been designed under the assumption that the PDCCH and PDSCH beams are separately updated. The indication schemes have been designed under the assumption that the UE should adjust its Rx beams differently for PDCCH and PDSCH reception.

However, in many cases, this level of freedom is unnecessary. Both the PDCCH and PDSCH should be transmitted with beams that provide the best SINR at the receiver. In many cases, this is the narrowest beam. Then, the target BLER for the PDCCH and the PDSCH are likely to be different, and that will be achieved by suitable selection of the PDCCH format and the PDSCH scheduling mechanism.

A very common realization is that the PDCCH and PDSCH are transmitted using the same beam.

Furthermore, the implementation of independent PDCCH and PDSCH beams leads to increased signalling. The extra amount depends on how often then the PDCCH and PDSCH beams are updated

In addition, introducing the possibility to switch between the PDCCH and the PDSCH leads to a more complex UE implementation than if the Rx beam switches are constrained to be between slots.

For these reasons, we propose

NR should support a configuration where the beam indications for PDSCH are valid also for PDCCH.

Note that this means that the UE may use the PDSCH Rx beam also when receiving PDCCH. As the beam indication for PDSCH relies on DCI to convey the beam indication, the PDCCH beam indication is also conveyed using DCI.

# Conclusions

In this contribution, we made the following observations:

Observation 1	The delay for beam indication over MAC CE is between 8ms and 29ms.

Observation 2	If an ACK or a NAK is received in response to data transmitted over PDSCH is an acknowledgment that the UE was able to decode DCI.

Observation 3	A robust version of DCI-based signalling can provide the same robustness with the same performance as MAC CE signalling.

Observation 4	Support for DCI-based beam switch for PDCCH reuses components already agreed for NR.

Observation 5	A very common realization is that the PDCCH and PDSCH are transmitted using the same beam.

Based on the discussion in this contribution we propose the following:

Proposal 1	NR support beam indication for PDCCH using DCI, in addition to configuration via MAC CE and RRC.

Proposal 2	NR should support a configuration where the beam indications for PDSCH are valid also for PDCCH.

# References

R1-1716366, “Performance of beam management without beam indication,” Ericsson, RAN1 NR Ad Hoc #3, Nagoya, September 2017

R1-1716350, “On beam indication, measurement, and reporting,” Ericsson, RAN1 NR Ad Hoc #3, Nagoya, September 2017

R1-1711023, “Analysis of beam indication signalling options,” Ericsson, RAN1#89ah, Qingdao, June 2017

3GPP TS 36.213 V14.3.0, “Physical layer procedures”