# 3GPP TSG RAN WG1 Meeting NR#3                                  			   R1-1716892

# Nagoya, Japan, 18th – 21st, September 2017

Source:               ZTE, Sanechips

Title:                    Summary of off-line discussion of AI 6.1.4.1, Remaining details on PRACH formats

Agenda item:      6.1.4.1

Document for:    Discussion and Decision

# Background

This document summarizes the offline discussion for AI 6.1.4.1, PRACH Remaining details on PRACH formats.

The following issues have been discussed:

- Cyclic shift values for preamble format 2 and 3 (long sequence length)

- Sequence length for PRACH preambles based on short sequence length

- Cyclic shift values for preamble format based on short sequence length

- RACH configurations

# Agreements until Tuesday afternoon coffee break

Continue offline discussion – Jan (ZTE)

- Focus on sequence length, configurations, cyclic shifts, etc.

R1-1716762	Summary of off-line discussion of AI 6.1.4.1, Remaining details on PRACH formats	ZTE, Sanechips

Agreements:

- For format 2, same cyclic shift values as for format 0 and 1 are used

Working assumption:

- L = 139 is adopted as the sequence length for the RACH Preamble Formats using the short sequence

Agreements:

- Use one common table for cyclic shift (Ncs) values for short sequence based PRACH formats for all SCS

  - Alt 1: The number of cyclic shift values is up to 16 values represented by 4 bits

  - Alt 2: The number of cyclic shift values is up to 8 values represented by 3bits

  - Down-selection to be done this week. In addition, to come up with the actual set of values

# Cyclic shift values for format 3

Offline working agreement:

For format 3, use table below. With the red marked changes:

- For unrestricted Ncs values, replace Ncs = 28 with Ncs = 13.

- For restricted set type A: Add Ncs value of 36

- For restricted set type B: Add Ncs value 36

| ZeroCorrelationZoneConfig | Sequence length 839, SCS = 5KHz |  |  |
| --- | --- | --- | --- |
|  | Unrestricted | Restricted set type A | Restricted set type B |
| 0 | 0 | 36 | 36 |
| 12 | 26 | 57 | 57 |
| 1 2 | 13 28 | 72 | 60 |
| 3 | 33 | 81 | 63 |
| 4 | 38 | 89 | 65 |
| 5 | 41 | 94 | 68 |
| 6 | 49 | 103 | 71 |
| 7 | 55 | 112 | 77 |
| 8 | 64 | 121 | 81 |
| 9 | 76 | 132 | 85 |
| 10 | 93 | 137 | 97 |
| 11 | 119 | 152 | 109 |
| 12 | 139 | 173 | 122 |
| 13 | 209 | 195 | 137 |
| 14 | 279 | 216 | - |
| 15 | 419 | 239 | - |

# Cyclic shift values for preamble format A/B/C

Offline Agreements:.

- Restricted set is not supported for NR PRACH preamble based on short sequence length

- Use one common table for cyclic shift (Ncs) values for short sequence based PRACH formats for all SCS

  - The number of cyclic shift values is up to 16 values represented by 4 bits, the following table is adopted

| ZeroCorrelationZoneConfig | Ncs values |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 4 |
| 3 | 6 |
| 4 | 8 |
| 5 | 10 |
| 6 | 12 |
| 7 | 13 |
| 8 | 15 |
| 9 | 17 |
| 10 | 19 |
| 11 | 23 |
| 12 | 27 |
| 13 | 34 |
| 14 | 46 |
| 15 | 69 |

# RACH configurations

Offline agreements:

NR defines the pattern of the slots that contain PRACH occasion(s) in to a larger time interval

- FFS: time interval e.g 5/10/20ms

- FFS pattern

- FFS numerology of the slot e.g SS block, UL/DL, Msg1 or PUSCH

FFS: Within each slot

Alt1: RACH resources within a slot are consecutive

Alt2:RACH resources within a slot are not consecutive, e.g to handle the case of CORESET monitoring , in the 2/4/7 symbols

Below part is discussion material:

RACH resource: A time-frequency resource to send RACH preamble (from RAN1#86b)

RACH resource occupies partial or full or multiple RACH slots

- For format 0-3 partial, full and multiple RACH slots are needed

- For format A/B/C partial RACH slots are needed

For preamble formats A/B/C a RACH slot consists of one or more RACH resources

Mapping of RACH preamble formats to RACH resources within a RACH slot

  - Alt1: Only one format (e.g only A)

  - Alt2: More than one format (e.g A and B)

- RACH configuration in RMSI provides time-frequency domain information on RACH resource

- Time domain information of RACH resource

  - RACH slot configuration is provided

    - If a slot is reserved for RACH resource(s) in partial or full, it is called RACH slot

    - RACH slot configuration within one radio frame

  - RACH resource within the RACH slot(s)

    - Starting symbol index within the RACH slot

    - RACH resources are consecutive within the configured RACH slot, as baseline

  - Specify [M] states on the RACH slot configuration (similar to LTE) per PRACH preamble subcarrier spacing

    - For long sequence, RACH slot configuration is provided based on the SCS with either of [30, 15]khz

    - For short sequence, RACH slot configuration is provided based on the SCS of PRACH preamble

    - Note : each state has its own RACH slot intensity and/or periodicity

    - Exact value of M is FFS