3GPP TSG RAN WG1 Meeting AH 1801	                                              	   R1-1800536

Vancouver, Canada, January 22nd – 26th, 2018

Agenda Item:	7.3.2.4

Source:	NEC

Title:	PUCCH resource allocation prior to RRC configuration

Document for:	Discussion and decision

# Introduction

In the last meeting, RAN1 discussed the PUCCH resource allocation before RRC connection set-up/initial access, with the following agreements:

Agreements:(RAN1#91)

- For resource allocation for HARQ-ACK before RRC connection setup:

  - Only PUCCH Format 0 and 1 are supported

  - The resource allocation is derived based on a 4-bit parameter in RMSI

    - FFS other details (no additional RRC impact)

Agreements:

- For PUCCH resource allocation with fallback DCI,

  - The same approach is used as that with normal DCI.

Agreements:

- For resource allocation for HARQ-ACK before RRC connection setup, UE identifies a PUCCH resource from a set of resources derived from RMSI using a similar approach to the case after RRC connection setup.

In this contribution, we discuss how to design and allocate PUCCH resources for Msg4 HARQ-ACK feedback transmission during the initial access/ before RRC connection set-up.

# PUCCH resource allocation prior to RRC configuration

Before RRC configuration, it is necessary that most of the parameters that determine the PUCCH resource allocation to be fixed in the specification except for few parameters that may be signaled either in the DCI format or RMSI. Based on this understanding, we propose to predefine PUCCH formats and their transmission parameters as captured in the following table:

Table 1. PUCCH formats with predefined parameters

| PUCCH format indicator | Number of symbols | Starting symbol | Frequency resource | Hopping | CS index | Time-domain OCC |
| --- | --- | --- | --- | --- | --- | --- |
| Format 0-1symbol | 1 | 13 | PRB is given by an equation | NA | Between 0-11, but to be selected by the proposed equation or signalled by the DCI | NA |
| Format 0-2symbol | 2 | 12 | PRB is given by an equation | Always Hop (Mirroring) | Between 0-11, but to be selected by the proposed equation or signalled by the DCI | NA |
| Format 1 | 10 | 4 | PRB is given by an equation | Always hop (Mirroring) | Between 0-11, but to be selected by the proposed equation or signalled by the DCI | Between 0-1, but to be selected by the proposed table. |
| Format 1 | 14 | 0 | PRB is given by an equation | Always hop (Mirroring) | Between 0-11, but to be selected by the proposed equation or signalled by the DCI | Between 0-1, but to be selected by the proposed table. |

The PUCCH format is one of {Format 0-1symbol, Format 0-2symbol, Format 1-10symbol, Format 1-14symbol} and can be dynamically signalled in the DCI scheduling MSG4 or in the RAR message (MSG2). In this case 2-bits should be enough.

Proposal 1: Predefine four PUCCH formats {Format 0-1symbol, Format 0-2symbol, Format 1-10symbol, Format 1-14symbol} for initial access where one of the formats is signalled dynamically in the DCI scheduling MSG4 or in the RAR message (MSG2).

Proposal 2: Predefine and fix the value of the transmission parameters such as number of symbols, starting symbol, and hopping resource for each PUCCH format.

Proposal 3: Define an equation or method how to obtain the first frequency resource (PRB number), CS index and time-domain OCC for each PUCCH format.

For defining PUCCH resources for the initial access, it is important to include an offset in order to locate the PUCCH resource away from the edges of the bandwidth part (BWP) if needed, for example when the edges of the BWP suffer from high level of interference. From last meeting, it is agreed that 4-bits shall be used for deriving the PUCCH resources during initial access, therefore we propose only 2-bits of 4bits are used for signaling the offset in cell specific manner and the remaining two bits can be either reserved or used for other purpose. In this case, the BWP is divided into 4 virtual sub-bands indexed from 0 to 3 as shown in Figure 1 where each sub-band contains contiguous PRBs to avoid resource fragmentations. This offers flexibility to start the PUCCH resources in a frequency region that suffers from lower interference than other regions of the BWP.

Figure 2 Example of sub-bands within the BWP

Based on the above description, we propose the following equation for PUCCH resource allocation for Msg4 HARQ-ACK feedback transmission:

nPUCCH,RI=iSB_index,BWP. NPRB,SB.K+nCCEindexLmin

located within:

PRBnumber=floornPUCCH,RIK=iSB_index,BWP. NPRB,SB+floornCCEindexLminK

where iSB_index,BWP is the index of SB indicated in RMSI, NPRB,SB is the number of PRBs in each SB, nCCEindex is the starting CCE index of the detected PDCCH/DCI, K is the number of available PUCCH resources (cyclic shifts and/or OCC) within one PRB, and Lmin is the minimum aggregation level supported in the search space.

The main reason of adding the parameter Lmin in to the equation is to increase the density of the usable PUCCH resources within a PRB as the higher aggregation level tends to decrease the density of the usage.

The number of available PUCCH resources within one PRB can be determined by at least two different alternatives as follows:

- Alt-1:  The PUCCH resource within one PRB is identified by an index k given by k=nCCEindexLmin mod K, and the parameter K is given by  K=12Δpucch, where Δpucch = {2,3,4,6} is a cell-specific and signalled using the remaining two bits in the RMSI, and k is mapped to CS index as shown below.

Table 2 for mapping  k  to CS index

| k | Cyclic shift index (CSindex) |
| --- | --- |
| 0 | 0 |
| 1 | Δpucch |
| 2 | 2Δpucch |
| K-1 | (K-1)Δpucch |

The main concern of Alt-1 is that gNB does not have a scheduling flexibility to change the PUCCH resource index because the cyclic shift is implicitly derived from the equation itself.

- Alt-2:  The PUCCH resource within one PRB is identified by an index k which is signalled in the detected PDCCH DCI, and 2 or 3 bits should be sufficient for this, giving K=4 or 8 respectively.

The merit of Alt-2 is that it allows gNB to allocate resource index flexibility, for example in order to avoid collisions for different UEs Ack/Nack feedbacks.

For both Alt-1 and Alt-2, the resource index within one PRB, k can be mapped to a cyclic shift (CS) and Orthogonal Cover Code (OCC) by means of a pre-determined look-up-table.

Since CS separation is more effective in low delay spread environments and OCC is more effective at low UE speed, it may be advantageous to pre-define multiple look-up-tables suitable for different cell types. The look-up-table to be used in each cell could be signalled in the RAR message (MSG2) or in the RMSI using 1-bit of the remaining two bits.

Table 3 for K=4 in low delay spread environment (for all PUCCH formats)

| k | Cyclic shift index (CSindex) | OCC Index |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 3 | 0 |
| 2 | 6 | 0 |
| 3 | 9 | 0 |

Table 4 for K=4 in high delay spread environment (for PUCCH format 1)

| k | Cyclic shift index (CSindex) | OCC Index |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 1 |
| 2 | 6 | 0 |
| 3 | 6 | 1 |

Proposal 4: Define PUCCH resource for Msg4 HARQ-ACK feedback transmission as given by the following equation:

nPUCCH,RI=iSB_index,BWP. NPRB,SB.K+nCCEindexLmin

located within:

PRBnumber=floornPUCCH,RIK=iSB_index,BWP. NPRB,SB+floornCCEindexLminK

where iSB_index,BWP is the index of SB indicated in RMSI, NPRB,SB is the number of PRBs in each SB, nCCEindex is the starting CCE index of the detected DCI, K is the number of available PUCCH resources (cyclic shifts and/or OCC) within one PRB for example K=4 where its index k is signalled by 2-bits in the detected DCI (as captured in Table 3 and 4), and Lmin is the minimum aggregation level supported in the search space.

Proposal 5: Define two different look-up-tables for PUCCH format 1 for the combination of CS and OCC and signal in the RMSI using 1-bit of the remaining two bits.

# Conclusions

In this contribution, we discussed how to design and allocate PUCCH resources for Msg4 HARQ-ACK feedback transmission during the initial access/ before RRC connection set-up, and we have the following proposals.

Proposal 1: Predefine four PUCCH formats {Format 0-1symbol, Format 0-2symbol, Format 1-10symbol, Format 1-14symbol} for initial access where one of the formats is signalled dynamically in the DCI scheduling MSG4 or in the RAR message (MSG2).

Proposal 2: Predefine and fix the value of the transmission parameters such as number of symbols, starting symbol, and hopping resource for each PUCCH format.

Proposal 3: Define an equation or method how to obtain the first frequency resource (PRB number), CS index and time-domain OCC for each PUCCH format.

Proposal 4: Define PUCCH resource for Msg4 HARQ-ACK feedback transmission as given by the following equation:

nPUCCH,RI=iSB_index,BWP. NPRB,SB.K+nCCEindexLmin

located within:

PRBnumber=floornPUCCH,RIK=iSB_index,BWP. NPRB,SB+floornCCEindexLminK

where iSB_index,BWP is the index of SB indicated in RMSI, NPRB,SB is the number of PRBs in each SB, nCCEindex is the starting CCE index of the detected DCI, K is the number of available PUCCH resources (cyclic shifts and/or OCC) within one PRB for example K=4 where its index k is signalled by 2-bits in the detected DCI (as captured in Table 3 and 4), and Lmin is the minimum aggregation level supported in the search space.

Proposal 5: Define two different look-up-tables for PUCCH format 1 for the combination of CS and OCC and signal in the RMSI using 1-bit of the remaining two bits.

# References

[1] 90b-NR-30, Summary of email approval on RRC parameter table for PUCCH resource allocation.

[2] R1-1718702, “PUCCH resource allocation,” Motorola Mobility, Lenovo.

[3] R1-1720380, “Resource allocation for NR PUCCH,” 	NEC