3GPP TSG RAN WG1 NR Ad Hoc Meeting	R1-1801079

Vancouver, Canada, 22-26 January, 2018

Source:	NTT DOCOMO, INC.

Title:	Offline discussion for search space

Agenda Item:	7.3.1.2

Document for: 	Discussion and Decision

- Proposals

Proposal 0:

- CORESET ID of the CORESET configured by PBCH is 0.

- Search space ID of the search space configured by PBCH is 0.

Proposal 1:

- A UE can be configured with a search space configuration by UE-specific RRC signaling which include following:

  - CORESET ID (range: 0 – 11, to indicate which CORESET the search space is mapped to)

    - The search space can be associated with any CORESET configuration

  - Search space ID (range: 0 - 39)

  - Indication to inform whether the search space is CSS or USS

    - If it is a CSS, following are informed as part of search space configuration:

      - Which RNTI(s) is/are used to scramble a DCI to monitor in the search space

        - C-RNTI

        - CS-RNTI (Note: DL SPS and CG Type2 may or may not use the same RNTI)

        - SP-CSI-RNTI

        - RA-RNTI

        - TC-RNTI

        - P-RNTI

        - SI-RNTI

        - INT-RNTI

          - If INT-RNTI is configured, DL-preemption-int (i.e., PI-related parameters) are also configured as part of the search space configuration

            - INT-monitoring-periodicity still necessary?

            - Preemp-DL (the parameter to turn on PI monitoring) still necessary?

        - SFI-RNTI

          - If SFI-RNTI is configured, SFI-PDCCH (i.e., SFI-related parameters) are also configured as part of the search space configuration

            - SFI-monitoring-periodicity is deleted

            - SFI-SS (Defines whether a specific CORESET is used or the CSS) still necessary?

        - TPC-PUSCH-RNTI

        - TPC-PUCCH-RNTI

        - TPC-SRS-RNTI

        - FFS: which RNTI(s) can be configured simultaneously for one search space configuration

    - If it is a USS, following are informed as part of search space configuration:

      - Which RNTI(s) is/are used to scramble a DCI to monitor in the search space

        - C-RNTI

        - CS-RNTI (Note: DL SPS and CG Type2 may or may not use the same RNTI)

        - TC-RNTI (FFS: configuration limitation)

        - SP-CSI-RNTI

    - Monitoring periodicity of slot(s) ()

      - For 15KHz SCS (slots based on 15kHz) without concatenated DL/UL assignments: 1,2,5,10,20

      - For 30KHz SCS (slots based on 30kHz) without concatenated DL/UL assignments: 1, 2, 4, 5, 10, 20

      - For 60KHz SCS(slots based on 60kHz) without concatenated DL/UL assignments: 1, 2, 4, 5, 8, 10, 20

      - For 120KHz SCS(slots based on 120kHz) without concatenated DL/UL assignments: 1, 2, 4, 5, 10, 20

      - FFS: for the case of concatenated DL/UL assignments

    - Monitoring offset of slot(s) ((0, 1, …, N-1) for monitoring periodicity of N)

    - Monitoring symbol within a slot (14-bit bit-map)

    - Number of candidates for AL=1 (0, 1, 2, 3, 4, 5, 6, 8)

    - Number of candidates for AL=2 (0, 1, 2, 3, 4, 5, 6, 8)

    - Number of candidates for AL=4 (0, 1, 2, 3, 4, 5, 6, 8)

    - Number of candidates for AL=8 (0, 1, 2, 3, 4, 5, 6, 8)

    - Number of candidates for AL=16 (0, 1, 2, 3, 4, 5, 6, 8)

Proposal 2:

- A UE can be configured with a search space configuration of CSS by RMSI which may include following:

  - CORESET ID (range: 0 – 1, to indicate which CORESET the search space is mapped to)

    - Default value: 0?

  - Search space ID (fixed to 1?)

  - Monitoring periodicity of slot(s)

    - For 15KHz SCS (slots based on 15kHz) without concatenated DL/UL assignments: 1,2,5,10,20

    - For 30KHz SCS (slots based on 30kHz) without concatenated DL/UL assignments: 1, 2, 4, 5, 10, 20

    - For 60KHz SCS(slots based on 60kHz) without concatenated DL/UL assignments: 1, 2, 4, 5, 8, 10, 20

    - For 120KHz SCS(slots based on 120kHz) without concatenated DL/UL assignments: 1, 2, 4, 5, 10, 20

    - FFS: for the case of concatenated DL/UL assignments

    - Default value:?

  - Monitoring offset of slot(s) ((0, 1, …, N-1) for monitoring periodicity of N)

    - Default value:?

  - Monitoring symbol within a slot (ceil(log2(14))=4-bit)

    - Default value:?

  - Number of candidates for AL=4 (0, 1, 2, 3, 4, 5, 6, 8) (reduce candidates?)

    - Default value:?

  - Number of candidates for AL=8 (0, 1, 2, 3, 4, 5, 6, 8) (reduce candidates?)

    - Default value:?

  - Number of candidates for AL=16 (0, 1, 2, 3, 4, 5, 6, 8) (reduce candidates?)

    - Default value:?

Proposal 3:

- For the CORESET configured by RMSI, following parameters are configurable as part of the CORESET configuration:

  - CORESET ID (or fixed to 1?)

  - CCE-to-REG mapping type (interleaved or non-interleaved)

    - Default: interleaved

  - REG bundle size (2, 3, or 6)

    - Default: 6

  - Interleaver rows

    - Default: 6

  - Time domain duration (1, 2, 3)

    - Default: 2?

  - Frequency domain resource allocation

    - No default?

  - DMRS scrambling sequence initialization value

    - Default: physical cell ID

  - Precoder granularity

    - Default: REG-bundle

Proposal 4:

- DCI format 0_1 and 1_1 are monitored only in USS.

- DCI format 0_0 and 1_0 are monitored in CSS.

  - They have the same DCI payload size between 0_0 and 1_0.

- DCI format 0_0 and 1_0 can be monitored in USS.

  - They have the same DCI payload size.

  - One of the following is configured by RRC signaling for the USS:

    - Monitoring DCI format 0_1 and 1_1 only

    - Monitoring DCI format 0_0 and 1_0 only

- UE is not expected to monitor more than four DCI format sizes for a given BWP of a cell within a slot.

- RAN1 spec is not optimized for the case where monitoring of DCI format 1_1 and 0_1 is not configured.

Proposal 5:

- For a DCI monitored in a CSS which is not configured by UE-specific RRC signaling, the DCI format size is not dependent on UE-specific RRC configuration.

Proposal 6:

- Discuss further about CORESET-BFR.

Proposal 7:

- In order to increase the capacity of the CORESET which is used for RMSI and Paging and random access, following are discussed during the meeting.

  - Opt.1: Define large CORESET configuration

  - Opt.2: Allow additional CORESET configuration for Paging

  - Opt.3: Define very compact DCI

Proposal 8:

- UE is not expected to monitor more than two candidates for AL=8 or AL=16 in the active DL BWP within a slot.