3GPP TSG RAN WG1 Meeting AH 1801	R1-1801187

Vancouver, Canada, 22nd – 26th January, 2018

Source:	Ericsson

Title:	Feature lead summary 3 of beam measurement and reporting

Agenda Item:	7.2.2.3

Document for:	Discussion and Decision

# Introduction

This document contains the feature lead summary of Tdocs submitted under Agenda Item 7.2.2.3 Beam measurement and reporting. Proposals are grouped into those having RRC parameter impact, meaning a new RRC parameter is needed or a value range requires modification, and those not having RRC parameter impact. A list of Tdocs containing text proposals is provided where the TPs are grouped by spec number and section number.

Priority order of open issues to address:

- Proposal 2 (RRC parameter impacting)

- Proposal 1 (RRC parameter impacting)

- Proposal 3 (Review draft LS reply to RAN2 on MAC-CE)

- Proposal 5 (MAC-CE latency)

- Proposal 8 (Default/fallback QCL assumptions for PUSCH, PUCCH)

- Proposal 4 (Beam reporting)

- Proposal 9 (SRS configuration)

- Proposal 7 (UL beam indication)

- Proposal 6 (DL beam indication)

Two WFs are currently discussed on the reflector:

- WF on DL beam management (ZTE)

- WF on SRS for UL beam management (Samsung)

# Summary of open issues with RRC parameter impact

In this section, open issues with RRC parameter impact are summarized. For the purposes of this document, RRC parameter impact is defined as an issue that requires a new RRC parameter or modification of a value range for an existing parameter.

## Beam reporting configuration

The following agreement was made in RAN1#90:

- Support L1-RSRP reporting of measurements on SS block for beam management procedures

- The following configurations for L1-RSRP reporting for beam management are supported

  - SS block only (with mandatory support by UE)

  - CSI-RS only (with mandatory support by UE)

  - SS block + CSI-RS independent L1 RSRP reporting

    - Joint L1-RSRP using QCL-ed SS-block + CSI-RS is optionally supported by UE (with optionally support by UE)

Note: currently supported value range for the RRC parameter ReportQuantity is the following [1]:

- {No Report, CRI/RI/PMI/CQI,  CRI/RI/i1, CRI/RI/i1/CQI, CRI/RI/CQI, CRI/RSRP, SSB index/RSRP, CRI/RI/LI/PMI/CQI}

The following proposals (paraphrased) appear in the Tdocs related to the above agreement, and beam reporting in general

- (Huawei) Support additional value range setting for the RRC parameter ReportQuantity to {SSBID or CSI-RS}/RSRP

- (Nokia) Introduce RRC parameter to inform UE to report on both SSB and CSI-RS

- (Nokia) Introduce RRC parameter to inform UE to identify strongest SSB/CSI-RS resource pairs

- (Nokia) Introduce RRC parameter to inform UE about reported QCL'd SSB/CSI-RS resource pairs

- (Oppo) Add 'CRI/RSRP and/or SSB index/RSRP' as another value of the RRC parameter ReportQuantity

- (vivo) Support joint reporting when the RRC parameter ReportQuantity set to SSBRI-RSRP

- (ZTE) Support additional value range setting for the RRC parameter ReportQuantity as {SSBID-CSI-RS}/RSRP

- (ZTE) The name of higher layer parameter “NZP-CSI-RS-ResourceSetConfigList” is replaced by “ResourceSetConfigList”, which can contain both CSI-RS resources (higher layer parameter NZP-CSI-RS-ResourceConfigList) and SS/PDCH resources used for L1-RSRP computation (higher layer parameterSSBResourceMeasList)

- (MediaTek) Support CSI reporting for P3, where the RRC parameter ReportQuantity is configurable between CRI/RI/PMI/CQI and RI/PMI/CQI

- (Qualcomm) For beam management UE is not expected to report quantities other than CRI/SSBID/RSRP

- (Ericsson) Support CSI reporting for P3. Include value RI/PMI/CQI for the RRC parameter ReportQuantity.

Proposal 1:

- Further offline discussion on Joint L1-RSRP using QCL-ed SS-block + CSI-RS (optionally supported by UE)

- Issues to resolve:

  - Configurability of joint reporting

    - New RRC parameter or addition of value range to existing RRC parameter (e.g., ReportQuantity)

    - From 38.215:

      - For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in 3GPP TS 38.213 [5].

  - How would the NW interpret a measurement if the NW does not know what is measured?

  - How would it limit the CSI-RS transmissions? Note that all CSI-RSs are QCLed with an SSB.

  - How could the NW make sure it gets reports of SSB only or CSI-RS only, which should be supported?

  - What value of ReportQuantity is to be used for joint reporting: CRI/RSRP or SSB index/RSRP?

Offline-conclusion:

Down-selection to one of the following alternatives:

- Alt-1: No higher layer indication of joint reporting configuration

  - Details FFS:

    - Configuration of QCL linkages between resources/resource sets for joint reporting

    - What value(s) of ReportConfig should be allowed

  - Note: 38.215 spec correction needed (remove higher layer indication)

  - Supporting: Huawei/HiSi, CATT, Intel, ITRI, ZTE/Sanechips, Motorola/Lenovo, Samsung, Fujitsu

- Alt-2: Joint L1-RSRP using QCL-ed SS-block + CSI-RS (with optional support by UE) – not supported in Rel-15

  - Note: reversion of agreement

  - Supporting: Nokia/NSB, Qualcomm, ITRI, Ericsson, Huawei/HiSi, CATT, MediaTek

- Note: Neither Alt-1 nor Alt-2 have RRC parameter impact

## TCI state configuration

The following proposals (paraphrased) appear in the Tdocs related to TCI state configuration

- (AT&T) TCI states should be configured per BWP - send LS to RAN2

- (Docomo) Configure only single TCI state for CORESET with lowest ID

- (Ericsson) RAN1 agreement needed on maximum number of TCI states

- (Interdigital) TCI state configuration  includes info about QCL assumptions across carriers

Agreement from RAN1#91:

A candidate set of DL RSs are configured using RRC mechanism

Each state of M TCI states is RRC configured with a downlink RS set used as a QCL reference, and MAC-CE is used to select up to 2^N TCI states out of M for PDSCH QCL indication

The same set of M TCI states are reused for CORESET

K TCI states are configured per CORESET

When K>1, MAC CE can indicate which one TCI state to use for control channel QCL indication

When K=1, no additional MAC CE signaling is necessary

Offline Agreement:

- Maximum number of candidate TCI states configured for a CORESET is K_max

  - K_max = M

    - Note that the value M is for configuration of TCI states only

    - Note: UE is not expected to track the K configured TCI states. The value of K is for configuration of TCI states only.

Include as part of LS to RAN2

## CSI & Beam management framework

- (Huawei) Support triggering multiple CSI-RS resource sets with different triggering offsets X in single triggering state. For beam management, support multiple CSI-RS resource sets with repetition = "ON"

- (ZTE) Support triggering multiple CSI-RS resource sets in single DCI. Support feedback of multiple CRIs.

- (CATT) If multiple resource sets for beam management supported in Rel-15, then support multiple CSI-RS resource sets with repetition = "ON"

- (Ericsson) For beam management, support only single NZP CSI-RS resource set (as specified for Rel-15)

- (Samsung) Two CSI-RS resource sets configuration/triggering is needed for group-based beam reporting.

- (DCM) Support triggering multiple aperiodic CSI-RS resource sets in a single DCI for beam management, multiple equal-size resource sets, all with repetition = “OFF”, and UE reports distinct local CSI-RS resource indicator(s) within one or more resource sets. The UE can assume that the gNB applies the same Tx beams in the same order for each of the sets

## QCL configuration for CSI-RS

The following proposals (paraphrased) appear in the Tdocs related to QCL configuration for CSI-RS:

- (Nokia) (Spatial) QCL reference for target periodic CSI-RS provided by an RRC configured TCI state

- (vivo) QCL reference for target periodic & semi-persistent CSI-RS configured at CSI-RS resource level

- (Nokia) (Spatial) QCL reference for a target ZP/NZP CSI-RS-based IMR provided by an RRC configured TCI state

- (ZTE) UE may assume that the CSI-RS resource(s) in a resource set for channel measurement and the CSI-IM/NZP CSI-RS resource(s) in a resource set for interference measurement are resource-wise spatially quasi co-located in the order of resources listed in the sets.

- (ZTE) QCL reference or TCI state for periodic/semi-persistent CSI-RS is configured at resource level

# Summary of open issues on UE capability

In this section, open issues with respect to UE capability are summarized. The summary below can serve as input to the UE capability discussion planned during RAN1 Ad Hoc – NR#1

## PDSCH scheduling offset threshold

The following proposals (paraphrased) appear in the Tdocs related to the PDSCH scheduling offset K that the UE is expected to support:

- (MediaTek) Support single latency of 26 OFDM symbols at 120 kHz SCS for both PDSCH scheduling offset and aperiodic CSI-RS triggering offset X

- (CMCC) Support PDSCH scheduling offset threshold as UE capability

- (Huawei) Capability includes 14, 28 symbols (assuming 120 kHz SCS)

- (ZTE) Capability includes 1, 2 slots

- (Interdigital) PDSCH scheduling threshold is based on UE capability

- (Ericsson) Common value on capability for PDSCH scheduling offset and aperiodic CSI-RS triggering offset X

- (Samsung) Scheduling offset threshold computed as ceil(tau_0/slot duration) expressed in # of slots where tau_0 is based on UE capability:

  - Note: we have agreed that UE reports the time needed for DCI decoding and switch the Rx beam as part of the UE capability reporting

## Number of UE beam switches

The following proposals (paraphrased) appear in the Tdocs related to the number of UE beam switches per slot that the UE is expected to perform:

- (MediaTek) No need to define number of Tx+Rx beam switches in a slot as a capability

- (Qualcomm) Upper bound on supported number of UE beam switches is a UE capability. Capability should include 7.

## Number of SSB/CSI-RS resources for measurement

The following proposals (paraphrased) appear in the Tdocs related to the supported number of SSB/CSI-RS resources on which the UE is required to measure:

- (MediaTek) For shorter latency, baseline number of resources to consider should be 8 or 16 per slot

- (Qualcomm) Upper bound on number of CSI-RS/SSB resources on which UE needs to compute L1-RSRP is a UE capability. Capability should include 8 and 16.

# Summary of other open issues

In this section, other open issues not captured in the previous two sections are summarized. Note, that while these issues are not categorized as RRC parameter impacting, depending on the outcome of discussions, they may still be RAN2 impacting, i.e., affect ASN.1 in 38.331 or MAC-CE message definition in 38.321.

## Draft LS Reply to RAN2

A draft LS reply to RAN2 on MAC-CE has been prepared by Huawei.

Proposal 3:

- Offline discussion to review draft.

Offline conclusion:

- Huawei will update the draft based on offline discussion and circulate on the reflector

## Beam reporting

### Differential RSRP reporting

- (CATT) CRI, RSRP ordering needs to be clarified

- (Huawei) Specify mapping of RSRP range [-140, -44] dBm with 1dB step size to 7-bit quantization levels. Introduce table into spec.

- (Huawei) 4-bit differential RSRP should include a state to indicate out-of-range

- (NEC) 4-bit differential RSRP should include a state to indicate out-of-range

- (Nokia) For joint SSB/CSI-RS RSRP reporting, both differential and non-differential L1-RSRP reporting for SSB + CSI-RS supported

- (Nokia) For joint SSB/CSI-RS RSRP reporting, report contains 2 elements: 1st element for differential RSRPs, 2nd element for SSBID/CRIs

- (Samsung) Reported CRIs ordered in decending order of coresponding RSRPs; bits are arranged as: CRI1., largest RSRP of CRI1; CRI2, differential RSRP  of CRI2; CRI3, differential RSRP of CRI3, ...

- (ZTE) Support differential L1-RSRP reporting for group-based beam reporting

Also, in R1-180005, RAN4 provide the following input to RAN2 and RAN1:

RAN4 discussed the reporting range for NR RSRP and signal quality measurements. RAN4 conclusion is

- Reporting range of SS-RSRP and CSI RSRP can be represented with up to 128 reported values (7 bits) and the granularity shall be 1dB.

- Reporting range of SS-RSRQ, CSI-RSRQ, SS-SINR and CSI-SINR can be represented with up to 128 reported values (7 bits) and the granularity shall be 0.5dB

- The range in the signaling may be larger than the guaranteed accuracy range.

- RAN4 will specify the mapping between reported value and measured quantity value in the NR performance work item.

The assumption is that the mapping table will go into 38.133.

Proposal 4:

- Reuse the RSRP mapping derived by RAN4 for the maximum RSRP

- Sort the RSRP elements in the report in the order of decreasing RSRP

- Offline discussion on the following aspects

  - 4-bit quantization of differential RSRP including possible out-of-range state

  - Support of differential RSRP reporting for joint CSI-RS/SSB RSRP reporting

  - Support of differential RSRP reporting for group-based RSRP reporting

WF for group-based reporting (ZTE):

Proposal:

- The reporting format of differential L1-RSRP for non-group based beam reporting is reused for group based beam reporting.

  - Support UE to report its capability of maximum number of reported beams for group based beam reporting, and its candidate value is {1, 2}.

### Priority rules for reporting

- (Huawei) Support the following priority ordering:

  - AP beam reporting > P CSI reporting

  - AP CSI reporting > P beam reporting

  - AP beam reporting > P beam reporting

  - Beam reporting part 1 > CSI reporting> beam reporting part 2

- (Intel) Priority rules between PUCCH and SRS when PUCCH carries beam reporting should be the same as between PUCCH and SRS when PUCCH carries CSI report

- (NEC) SP-CSI > P-CSI; AP-BM > SP-BM > P-BM

- (NEC) Support flexible priority order between beam reporting/CSI reporting

- (Samsung) Support the following priorities

  - AP Beam report > AP CSI report. P/SP: Beam report < CSI report.

  - CRI/RSRP > SSBRI/RSRP

  - normal beam report > Group based beam report

  - Collision of two beam reports: (a) Send higher priority report only if merged size > threshold; or (b) Merge reports if merged size < threshold "

- (vivo) AP beam reporting > P beam reporting; P beam reporting > P CSI reporting

### Reporting format

- (Samsung) For periodic beam reporting on short PUCCH: (1) 2 beams ==> 1 PUCCH instances, (2) 4 beams ==> 2 PUCCH instances

- (Samsung) Aperiodic beam report on PUSCH comprised of one part only

### Group-based beam reporting

- (Samsung) Two RSRPs/CRIs reported. UE configured with two sets; 1 CSI-RS from each set reported, where CRIs correspond to the two different sets

- (ZTE) Support 4 beams per group

## DL beam indication

Several companies have pointed out issues with 38.213 Sections 10.1, 9.2.1 and 38.214 Section 5.1.5. For example, the following issues (non-exhaustive) have been identified:

- Alignment of text in 38.213 with agreement on MAC-CE activation of TCI states for PDCCH and MAC-CE activation of spatial relations for PUCCH

  - DCI format 1_0 does not contain a TCI field

- Alignment of text in 38.214 with agreement on MAC-CE activation of TCI states for PDSCH

- Clarification of description in 38.214 regarding UE behaviour with respect to TCI-PresentInDCI Enabled/Disabled and PDSCH scheduling offset less/greater than threshold

- …

Two companies identified that timing of the MAC-CE messages for beam indication requires specification. The following proposals appear in Tdocs

- (Docomo) Activation delay of TCI states/spatial relation for PDSCH/PDCCH/PUCCH = 2*k, where k = PDSCH HARQ reception latency in slots

- (Qualcomm) Activation delay = 20 slots

In addition to the above there various proposals from companies for modifications/extensions of the agreed TCI framework such as

- Default QCL assumption for aperiodic CSI-RS resource set if scheduling offset is less than a threshold (like for PDSCH)

- Clarification of timing between DCI reception and PDSCH transmission (scheduling offset), including when PDSCH is FDM with QCL’d SSB

- MAC-CE activation of TCI states even if TCI field is not present in DCI or if the PDSCH scheduling offset is less than a threshold

- (Intel) the CORESET used to determine default PDSCH TCI when scheduling offset is below a threshold should exclude the CORESET for beam failure recovery

- …

Proposal 5:

- Offline discussions to consider the details for determining the activation/deactivation latencies based on MAC CE

  - Include as part of LS to RAN2

Offline conclusion:

Response to RAN2: “These are candidate values that RAN1 is considering. It will be narrowed down to one.”

Downlink-related:

  - Time between the ACK transmission for the PDSCH carrying the MAC-CE activation message and the time that the UE is able to apply the QCL assumption carried by the MAC-CE message. Assumption is that 120 kHz SCS is used as a reference point.

  - For PDSCH

    - Candidate values (slots): 8, 10, 20, 40, 80 (considering the minimum 10 ms periodicity for TRS)

  - For PDCCH:

    - Candidate values (slots): 8, 10, 20, 40, 80

  - For SP-CSI-RS:

    - Candidate values (slots): 8, 10, 20, 40, 80

Uplink-related:

  - FFS

Proposal 6:

- Offline discussion to identify potential areas where the agreed TCI framework may or may not require modification/extension

  - e.g., Clarify the CORESET used to determine the default PDSCH TCI, e.g. whether or not CORESET for beam failure recovery can be considered

## DL beam management

- (Huawei) UE provides requested # of CSI-RS resources for UE Rx beam sweeping

- (MediaTek) Specify P1, P2, P3 behavior through configuration of repetition ON vs. OFF and whether or not spatial QCL information is provided

- (MediaTek) UE not expected to receive CSI-RS for beam management and PDSCH in the same OFDM symbol

## UL beam indication

In RAN1#91, UL beam indication was agreed for PUCCH, whereby a list of spatial relations is configured and MAC-CE activates one of them. The elements of the list contain the ID of an SSB, CSI-RS, or SRS resource. In this way, dynamic beam indication is achieved. Interestingly, no such agreement has been made for PUSCH. Several proposals appear in Tdocs in this area, illustrating 3 different approaches. One approach is based on using SRI for PUSCH beam indication. A second approach is based on a rule whereby PUSCH uses a spatial relation to the DL RS contained in the TCI state used for PDCCH QCL indication. A third approach reuses the TCI framework from the DL by including a TCI field in UL DCI, and potentially including SRS in the TCI state configuration.

- (CATT) UL grant contains SRI pointing to a previously transmitted SRS, CSI-RS, or SSB

- (CATT) For multi-panel, UL grant contains SRI where the SRI points to one or multiple previously transmitted SRS

- (Ericsson) For PUSCH beam indication, if SRS not configured, UE assumes spatial relation to the DL RS provided by the TCI state used for PDCCH QCL indication (i.e., the PDCCH carrying the DCI that schedules PUSCH)

- (Interdigital) Reuse TCI framework for UL. Add SRS to TCI state definition.(LGE) We do not see any need to define a separate beam indication for PUSCH. For non-codebook based transmission, SRI(s) are indicated in the UL related DCI so that UE can apply the indicated SRS beam for the PUSCH transmission. For codebook based transmission, an SRI is indicated together with TPMI/TRI so that UE can apply the indicated SRS beam for the PUSCH transmission. Besides, it may be clarified (if necessary), before SRS is configured for codebook-based or non-codebook-based UL, UE may transmit PUSCH in the same direction as Msg 3 PUSCH.

- (HW) We share similar view with LGE and Samsung where current SRI-based framework is sufficient for PUSCH. If needed, further specification clarification can be discussed to link spatial relationship between SRS ports and PUSCH ports.

- (Samsung): The function of ‘beam indication’ for PUSCH has been agreed and specified in CB-based and non-CB-based PUSCH session. For CB-based PUSCH: one SRI in DCI. For non-CB-based: one or multiple SRI in DCI. We do not discuss this here

One company identified that no default spatial relation has yet been defined prior to MAC-CE activation of the spatial relation of PUCCH. The proposal in the Tdoc is as follows

- (Fujitsu) After RRC configuration of PUCCH-SpatialRelationInfo and before MAC-CE activation of spatial relation, UE transmits PUCCH in same direction as Msg 3 PUSCH

- (LGE) After RRC configuration of PUCCH-SpatialRelationInfo and before MAC-CE activation of spatial relation, UE transmits PUCCH using the first entry of PUCCH-SpatialRelationInfo.

Proposal 7:

- Offline discussion targeting a WF on PUSCH beam indication

Proposal 8:

- Default/Fallback QCL assumptions for PUSCH, PUCCH

  - If no SRS configured, UE may assume that the spatial relation for PUSCH is given by the TCI state used for PDCCH spatial QCL indication, i.e., the PDCCH carrying the DCI that schedules PUSCH

  - After RRC configuration of PUCCH-SpatialRelationInfo and before MAC-CE activation of spatial relation, UE assumes may assume default spatial relation is the SSB identified during the RACH procedure.

## UL beam management

Several proposals appear in Tdocs on the configuration of SRS for the purposes of UL beam sweeping as follows:

- (MediaTek) When SRS-SpatialRelationInfo not configured for SRS resources within a set, UE is not expected to transmit SRS with a fixed beam

- (Samsung) To support U3, SRS-SetUse = "BeamManagement": 1 or subset of SRS resources in set configured with SRS-SpatialRelationInfo. Alternatively, for SRS-SetUse = "U3-BeamManagement": SRS-SpatialRelationInfo configured for whole SRS set

- (vivo) Support indication between U2/U3 for SRS beam sweeping

- (Samsung) For resource set configure for “Beam management”, same bandwidth is configured for all SRS resources in the resource set

- (Interdigital) UE provides beam correspondence status, beam sweeping type (local sweep/full sweep), requested # of SRS resources

- (ZTE) To introduce the following value range for higher layer parameter SRS-SpatialRelationInfo

    - {SSB Index, NZP-CSI-RS-ResourceConfigId, or SRS-ResourceConfigId}

- (LGE) Add a clarifying sentence in S6.2.1 that “When the higher layer parameter SRS-SetUse is set to ‘BeamManagement’ for a configured SRS resource set and when any of the SRS resources within the SRS resource set are not configured with higher layer parameter SRS-SpatialRelationInfo, UE should apply independent spatial domain transmission filter for each configured SRS resource.”

- (LGE) Add a clarifying sentence in S6.2.1 that “UE should apply the same spatial domain transmission filter on different OFDM symbols for a SRS resource.”

Proposal 9:

- Offline discussion targeting a WF on configuration of an SRS resource sets for the purposes of U2/U3 UL beam management procedures

WF on SRS for UL beam management (Samsung)

# References

R1-1721581, “RRC Parameters,” Ericsson, RAN1#91, December 2017.

R1-180005, “LS on RSRP and signal quality measurement report mapping,” RAN4, January 2018

# Appendix A: List of Text Proposals

In this section, a list of Tdocs containing text proposals is provided, where the TPs are grouped by spec number and section number. This can serve as input on harmonizing text proposals as well as useful input to the spec editors of 38.213 and 38.214.

| Tdoc # | Company | High-level categorization | Issue | Spec Number | Spec Section Number(s) |
| --- | --- | --- | --- | --- | --- |
| R1-1800660 | Docomo | MAC-CE timing | Activation/deactivation timing for PUCCH undefined | 38.213 | 9.4 |
| R1-1800100 | Huwaei | DL beam indication | Agreement #2 not captured | 38.213 | 10.1 |
| R1-1800401 | Lenovo | DL beam indication | No TCI in fallback DCI (Format 1_0) | 38.213 | 10.1 |
| R1-1800362 | LG | DL beam indication | Agreement #2 not captured | 38.213 | 10.1 |
| R1-1800185 | vivo | DL beam indication | Agreement #7 not captured | 38.213 | 10.1 |
| R1-1800660 | Docomo | MAC-CE timing | Activation/deactivation timing for PDCCH undefined | 38.213 | 10.2 |
| R1-1800126 | Fujitsu | UL beam indication | Default behavior before MAC-CE activation of spatial relation for PUCCH | 38.213 | 9.2.1 |
| R1-1800110 | ZTE | UL beam indication | Agrrement #6 not captured | 38.213 | 9.2.1 |
| R1-1800751 | Nokia | QCL assumption for ZP/NZP CSI-RS-based IMR | Linkage to CSI-RS for CM undefined | 38.214 | 5.1.4.2, 5.2.2.4 |
| R1-1800126 | Fujitsu | DL beam indication | Clarfication of scheduling offset needed | 38.214 | 5.1.5 |
| R1-1800100 | Huwaei | DL beam indication | Agreement #3 not correctly captured | 38.214 | 5.1.5 |
| R1-1800100 | Huwaei | DL beam indication | Possible QCL reference ambiguity | 38.214 | 5.1.5 |
| R1-1800100 | Huwaei | DL beam indication | QCL type ambiguity | 38.214 | 5.1.5 |
| R1-1800311 | Intel | DL beam indication | Default TCI state for PDSCH | 38.214 | 5.1.5 |
| R1-1800498 | OPPO | DL beam indication | Agreement #8 not captured completely | 38.214 | 5.1.5 |
| R1-1800498 | OPPO | DL beam indication | QCL assumptions for PDSCH | 38.214 | 5.1.5 |
| R1-1800432 | Samsung | DL beam indication | Scheduling offset threshold | 38.214 | 5.1.5 |
| R1-1800185 | vivo | DL beam indication | QCL assumptions for PDSCH and PDCCH | 38.214 | 5.1.5 |
| R1-1800185 | vivo | DL beam indication | QCL assumptions for PDSCH | 38.214 | 5.1.5 |
| R1-1800110 | ZTE | DL beam indication | Agreement #2 not correctly captured | 38.214 | 5.1.5 |
| R1-1800110 | ZTE | UE capability | Scheduling offset threshold | 38.214 | 5.1.5 |
| R1-1800159 | MediaTek | Reception of CSI-RS | Reception of CSI-RS and PDSCH | 38.214 | 5.1.6.1.2 |
| R1-1800498 | OPPO | Reception of CSI-RS | Reception of CSI-RS for CSI-RS-ResourceRep="OFF" | 38.214 | 5.1.6.2 |
| R1-1800660 | Docomo | MAC-CE timing | Activation/deactivation timing for PDSCH undefined | 38.214 | 5.1.7 |
| R1-1800498 | OPPO | CSI & BM framework | Joint SSB, CSI-RS beam reporting | 38.214 | 5.2.1, 5.2.1.1, 5.2.1.4 |
| R1-1800100 | Huwaei | CSI & BM framework | Agreement #5 not correctly captured | 38.214 | 5.2.1.2 |
| R1-1800110 | ZTE | CSI & BM framework | Editorial TP | 38.214 | 5.2.1.2 |
| R1-1800100 | Huwaei | Beam reporting | Agreement #4 not captured | 38.214 | 5.2.1.4 |
| R1-1800159 | MediaTek | Beam reporting | Clarification on allowed reporting quantities | 38.214 | 5.2.1.4 |
| R1-1800751 | Nokia | Beam reporting | Details of joint SSB, CSI-RS beam reporting undefined | 38.214 | 5.2.1.4 |
| R1-1800751 | Nokia | Beam reporting | Details of joint SSB, CSI-RS beam reporting undefined | 38.214 | 5.2.1.4 |
| R1-1800751 | Nokia | Beam reporting | Details of joint SSB, CSI-RS beam reporting undefined | 38.214 | 5.2.1.4 |
| R1-1800751 | Nokia | Beam reporting | Details of joint SSB, CSI-RS beam reporting undefined | 38.214 | 5.2.1.4 |
| R1-1800751 | Nokia | Beam reporting | Details of joint SSB, CSI-RS beam reporting undefined | 38.214 | 5.2.1.4 |
| R1-1800859 | Qualcomm | Beam reporting | Clarficiation on allowed reporting quantities | 38.214 | 5.2.1.4 |
| R1-1800432 | Samsung | Beam reporting | Reporting order | 38.214 | 5.2.1.4 |
| R1-1800185 | vivo | Beam reporting | Joint SSB, CSI-RS beam reporting | 38.214 | 5.2.1.4 |
| R1-1800110 | ZTE | Beam reporting | Local vs. global CRI | 38.214 | 5.2.1.4 |
| R1-1800110 | ZTE | Beam reporting | Joint SSB, CSI-RS beam reporting | 38.214 | 5.2.1.4 |
| R1-1800110 | ZTE | CSI & BM framework | Number of resources in a resource setting | 38.214 | 5.2.1.4 |
| R1-1800159 | MediaTek | DL beam management | CSI-RS resource set configuration | 38.214 | 5.2.1.4 |
| R1-1800159 | MediaTek | DL beam management | CSI-RS resource set configuration | 38.214 | 5.2.1.4 |
| R1-1800159 | MediaTek | DL beam management | CSI-RS resource set configuration | 38.214 | 5.2.1.4 |
| R1-1800751 | Nokia | Group-based beam reporting | Differential L1-RSRP reporting not defined | 38.214 | 5.2.1.4 |
| R1-1800859 | Qualcomm | Group-based beam reporting | Typo in allowed reporting quanitities | 38.214 | 5.2.1.4 |
| R1-1800432 | Samsung | Group-based beam reporting | Reporting order | 38.214 | 5.2.1.4 |
| R1-1800110 | ZTE | Group-based beam reporting | Differential L1-RSRP reporting not defined, number of beams per group limited to 2 | 38.214 | 5.2.1.4 |
| R1-1800660 | Docomo | Non-group based beam reporting | Agreement #1 not correctly captured | 38.214 | 5.2.1.4 |
| R1-1800859 | Qualcomm | UE capability | Number of RSRP measurements | 38.214 | 5.2.1.4 |
| R1-1800660 | Docomo |  | Editorial TP | 38.214 | 5.2.1.4 |
| R1-1800110 | ZTE | CSI & BM framework | Editorial TP | 38.214 | 5.2.1.5.1 |
| R1-1800751 | Nokia | QCL assumption CSI-RS | QCL reference for target p-CSI-RS undefined | 38.214 | 5.2.2.3.1 |
| R1-1800432 | Samsung | Beam reporting | Reporting format | 38.214 | 5.2.4 |
| R1-1800432 | Samsung | Beam reporting | Priority rules for reporting | 38.214 | 5.2.5 |
| R1-1800433 | Samsung | Beam reporting | Aperiodic reporting on PUSCH | 38.214 | 5.2.5 |
| R1-1800159 | MediaTek | SRS configuration | Configuration details for aperiodic SRS triggering require specification | 38.214 | 6.1.1.2, 6.2.1 |
| R1-1800159 | MediaTek | SRS configuration | Configuration details for non-codebook based UL precoding require specification | 38.214 | 6.2.1 |
| R1-1800159 | MediaTek | UL beam indication | UE behavior when SRS-SpatialRelationInfo not configured is undefined | 38.214 | 6.2.1 |
| R1-1800159 | MediaTek | UL beam indication | Conflict between different layers indicating spatial relation for SRS | 38.214 | 6.2.1 |
| R1-1800363 | LG | UL beam management | Agreement #8 not captured completely | 38.214 | 6.2.1 |
| R1-1800432 | Samsung | UL beam management | Configuration of UL beam sweep | 38.214 | 6.2.1 |
| R1-1800432 | Samsung | UL beam management | Configuration of UL beam sweep | 38.214 | 6.2.1 |

# Appendix B: Prior agreements

In this section, a list of prior RAN1 agreements is provided which are referenced in the above text.

Agreement #1

- For non-grouping based beam reporting, support the following reports parameters:

  - Maximal number of configured Tx beams for beam measurement: K equals 64

  - Maximal number of configured Tx beams to be reported in one instance: N_max = 2, 4  where a subset of N (N<=N_max where N = 1, 2, 4) beams can be selected by the gNB and indicated to the UE (FFS signaling mechanism)

  - Reporting differential L1-RSRP when multiple beams are reported in one reporting instance. Reference is the largest L1-RSRP in that reporting instance. FFS other reference for differential reporting.

    - FFS applicable reporting channels and number of beams, and associated reporting contents

    - FFS: the UE adjusts the L1-RSRP of multiple RS resources according to the power offset between them

  - Bit-width: 7bit for L1-RSRP ranging from -140dBm to -44dBm with 1dB stepping size (analogous with LTE) and 4bit for differential L1-RSRP

    - FFS stepping size of differential quantization

Agreement #2

- A candidate set of DL RSs are configured using RRC mechanism

  - Each state of M TCI states is RRC configured with a downlink RS set used as a QCL reference, and MAC-CE is used to select up to 2^N TCI states out of M for PDSCH QCL indication

    - The same set of M TCI states are reused for CORESET

    - K TCI states are configured per CORESET

    - When K>1, MAC CE can indicate which one TCI state to use for control channel QCL indication

    - When K=1, no additional MAC CE signaling is necessary

Agreement #3

- When the scheduling offset is <=k, and the PDSCH uses QCL assumption that is based on a default TCI state

  - The default TCI state corresponds to the TCI state used for control channel QCL indication for the lowest CORESET ID in that slot

Agreement #4

- For L1-RSRP and/or beam resource indicators (e.g. CRI or SSB index) reporting for beam management, support the following UL channels:

  - Short/long PUCCH

  - PUSCH

- Support the following reporting types for beam mgmt. on the above channels

  - For Periodic, support long PUCCH and short PUCCH

  - Semi-persistent – support all channels

  - Aperiodic – support PUSCH and short PUCCH

Agreement #5

- Support Alt#1 for SSB: configuration of SSB resources within a resource setting for beam management. L1-RSRP measurement on those configured resources is reported.

Agreement #6

- Modify the RRC parameter PUCCH-Spatial-relation-info as list.

  - Each entry can be SSB ID or, a CRI, or a SRI

  - One or multiple SpatialRelationInfo IE(s) is included in the list.

- Introduce MAC-CE signalling to provide spatial relation information for a PUCCH resource to one of the entries in PUCCH-Spatial-relation-info

- If PUCCH-Spatial-relation-info includes one SpatialRelationInfo IE, UE applies the configured SpatialRelationInfo and no MAC-CE is used.

- MAC-CE Impact:

| TS38.214 |  | Indication of spatial relation for PUCCH | Provides the spatial relation for a PUCCH resource | PUCCH resource ID ¦ Bitmap of size [8] (Bitmap activates one of the [8] entries within the RRC parameter PUCCH-Spatial-relation-info) |
| --- | --- | --- | --- | --- |

- RRC modification:

| PUCCH-SpatialRelationInfo | New | PUCCH- SpatialRelationInfo | List of configurations of the spatial relation between reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. SSB Index, NZP-CSI-RS-ResourceConfigId, or SRS-ResourceConfigId |  | UE-Specific |  | 38.331 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement #7

- Between initial RRC configuration and MAC CE activation of TCI states, the UE may assume that both PDCCH and PDSCH DMRS are spatially QCL-ed with the SSB determined during initial access

Agreement #8

- For UL beam management,

  - NR supports gNB configuration of transmitting SRS with same Tx beam across multiple symbols via either of followings

    - configuring one SRS resource spanning multiple symbols

    - configuring UE to apply the same Tx beam across the SRS resources in a SRS resource set.

  - UE can apply different Tx beams to different SRS resources if it is not configured to apply the same Tx beam across SRS resources in a SRS resource set, where the beams can be determined either (1) via a gNB-transparent way, or (2) via gNB indication.

Agreement #9

- TCI-State structure for one RS set is either:

  - DL Reference RS1| QCL_type1, DL Reference RS2| QCL_type2

    - Note that there is no overlap of QCL_type1 and QCL_type2

  - DL Reference RS1| QCL_type1

  - Note that QCL_type1 and QCL_type2 are selected from {QCL types A, QCL types B, QCL types C, QCL types D}

  - Note that if DL Reference RS1 and DL Reference RS2 are the same, both QCL types apply

Agreement #10

- Working assumption from RAN1#90 is confirmed:

  - For beam management CSI-RS, NR supports higher layer configuration of a set of single-symbol CSI-RS resources where

    - The set configuration contains an information element (IE) indicating whether repetition is “on/off”

  - Note: In this context, repetition “on/off” means:

    - “On”: The UE may assume that the gNB maintains a fixed Tx beam

    - “Off”: The UE can not assume that the gNB maintains a fixed Tx beam

  - Note: This does NOT necessarily mean that the CSI-RS resources in a set occupy adjacent symbols

- Furthermore, the following details are agreed

  - CSI-RS resources in the resource set are TDMed if repetition is ON

    - If repetition is ON, The UE does not expect different values for the following parameters across different CSI-RS resources within a resource set

      - Transmission periodicity

      - Number of antenna port subject to RAN4 decision

      - FFS for other parameters