3GPP TSG RAN WG1 Ad-Hoc Meeting 1901					R1-1900123

Taipei, 21st – 25th January, 2019

Source:	vivo

Title:	On coexistence of NR and LTE sidelink for V2X

Agenda Item:	7.2.4.5

Document for:	Discussion and Decision

- Introduction

The feasibility and mechanism of coexistence of LTE and NR sidelink technologies have been discussed in the RAN1 #95 meeting, and the following agreements were achieved [1]:

Agreements: - Consider solutions for sidelink coexistence for the following: - Potential LTE V2X Tx and NR V2X Tx - Potential LTE V2X Tx and NR V2X Rx - Potential LTE V2X Rx and NR V2X Tx - FFS the case of potential LTE V2X Rx and NR V2X Rx, e.g., whether or not it can be handled implementation Agreements: RAN1 will identify both TDM and FDM solutions for coexistence. The specific support for each solution is FFS. For FDM solutions: - For both dynamic and semi-static power allocation solutions, RAN1 assumes synchronization between NR and LTE V2X sidelinks, for a NR V2X UE when NR and LTE V2X sidelinks are intra-band - The case of inter-band is FFS Note: If the identified solutions can be applied to systems that are not synchronized, then RAN1 may revisit this assumption.

In this contribution, we provide our view on the coexistence issues of LTE and NR sidelink technologies.

- Synchronization

It has been agreed that synchronization is assumed not only for TDM, but also for FDM, as least for intra-band coexistence of NR sidelink and LTE sidelink in V2X. Thus, the synchronization accuracy should be specified to support both TDM solutions and FDM solutions for in-device coexistence.

For TDM solutions, as well as some FDM solutions such as dynamic power sharing, it seems at least subframe or slot level synchronization is needed. For some of the long term TDM scheme, DFN level synchronization is needed because the resource pool is defined based on the DFN.

Proposal 1: Both DFN level and slot level synchronization should be assumed for coexistence of NR sidelink and LTE sidelink.

For the case of inter-band coexistence of NR sidelink and LTE sidelink in V2X, with separate PA implementations and sufficient frequency separation, at least some of the solutions, e.g. semi-static power sharing, can be adopted without synchronization between NR sidelink and LTE sidelink. Such kink of coexistence scheme enables separate deployment of NR and LTE sidelink modules in the vehicle, which is beneficial for the vehicle vendor.

Proposal 2: At least for some FDM solutions, synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink.

- TDM solution

TDM solution has beneficial on no adjacent channel leakage nor power limitation between LTE and NR sidelink transmissions. However, TDM solution would inevitably degrade the latency and reliability to some extent. The details of coordination and time scale should be studied.

  - Long term time-scale coordination

If the resource pools provided to a UE for LTE and NR sidelink transmissions are overlapping in time domain, the simplest TDM solution is to exclude the overlapping resources in one side. For example, the overlapping resources are always excluded from the NR resource pool, in order to avoid any specification impact and performance degradation to LTE. This solution does not require dynamic coordination nor resource reservation results between LTE and NR sidelink modules. Resource exclusion is done only when reconfiguration of resource pool occurs.

Observation 1: The simplest TDM solution is to exclude the overlapping resources from the resource pool, which does not require dynamic coordination nor resource reservation results.

Coordinated sensing resource reservation

If LTE sidelink and NR sidelink modules may have coordinated procedures, some potential solutions based on the information sharing between two modules can be considered for TDM. For example, if both LTE sidelink and NR sidelink are operating in autonomous resource selection mode, the sensing and measurement results, as well as the selected resources, can be shared between them, so that the sidelink transmissions for LTE and NR can be separated in time. Thanks to the semi-static resource reservation mechanism adopted in LTE, this solution is feasible at least by simply excluding the slots of NR sidelink overlapping with the reserved sources of LTE sidelink.

However, the impacts to the LTE sensing procedure should be investigated. For example, due to the transmission in NR sidelink, sensing is not possible in the LTE subframe at the same time. It may significantly reduce the candidate subframes in the selection window. Another issue is that during the transmission time of NR sidelink, receiving broadcast (e.g., basic safety message) in LTE sidelink is not possible due to the half-duplex constraint. The performance of packet reception of LTE sidelink would be significantly degraded.

Proposal 3: Semi-static coordination on sensing and resource reservation between LTE and NR sidelink modules can be used to prevent overlapping of NR and LTE V2X sidelink transmissions.

  - Short term time-scale coordination

Joint resource selection

If short term inter-module coordination can be employed between LTE sidelink and NR sidelink modules, a joint resource selection mechanism for LTE and NR sidelink can also be considered. Given that the physical resource configurations may be distinct significantly between LTE sidelink and NR sidelink, the sensing procedure and monitoring may be sidelink-specific. However, the resource selection may be performed over the superset of candidate resources from both sidelink, so that the collision between LTE and NR sidelink transmission can be avoided.

Dropping based on QoS

By this way, independent resource allocation between LTE sidelink and NR sidelink can be applied for better spectrum utilization. In any case, if a collision between LTE and NR sidelink transmission occurs, one of the transmission occasions should be dropped based on the QoS requirement.

Proposal 4: Dynamic resource allocation (such as joint resource selection, dropping based on QoS, etc.) can be considered if short term coordination is possible between LTE and NR sidelink modules.

- FDM solution

FDM solution has beneficial on supporting independent resource allocation between LTE and NR sidelink modules. The following issues are identified for FDM solution:

Power limited issue

In the case of both LTE and NR sidelink transmissions occur at the same time, the LTE sidelink transmission may limit the power available for NR sidelink transmission and vice versa. The issue of how to share the power between LTE and NR sidelink in order to meet the QoS requirements should also be addressed.

Self-interference issue

When a UE is configured with both LTE and NR sidelink, there could be some self-interference issues due to coexistence, namely inter-modulation distortion and harmonic, depending on the exact LTE and NR band combinations.

Half duplex issue

If the LTE and NR sidelink are deployed in adjacent channels, it may be challenging for simultaneous transmission and/or reception on both modes due to leakage from TX to RX. This will lead to a half-duplex issue between LTE and NR sidelink operations, i.e., a UE transmits on one side may not be able to monitor and perform proper measurement on the other side. This issue is more severe than that of LTE CA. Unlike the LTE CA case, the LTE and NR sidelink technologies may be implemented in separate modules. Real-time coordination for simultaneous transmission and/or reception between them may be infeasible.

Proposal 5: The issues of power limited, self-interference and half-duplex in FDM solution need to be resolved for LTE and NR sidelink coexistence.

Dynamic power sharing between LTE sidelink and NR sidelink can be used to mitigate the power-limit issue. Similar to the TDM solution, dropping is also feasible for any issues identified above. Both of them require prioritizing the transmission between different RATs. The prioritization of the transmissions can either base on the QoS requirement, or base on the RAT type (such as to avoid impact to LTE). Nevertheless, both of them rely on short-term/real-time coordination between LTE and NR modules.

Proposal 6: Dynamic power sharing or packet dropping can be considered for FDM solution if dynamic inter-module coordination is achieved.

On the other hand, semi-static power sharing without short-term/real-time coordination between LTE and NR modules can be considered. The minimum guaranteed power can be configured to each module. In case of power limitation, each RAT is given at least its minimum guaranteed power. The remaining power can be simply given to the transmission from one RAT (e.g. LTE), or to the earlier transmission between RATs.

Proposal 7: Semi-static power sharing without dynamic coordination can be considered for FDM solution without stringent synchronization between LTE and NR sidelink transmissions.

- Coexistence in mode-1 operation

If either TDM or FDM solution is applied to a UE, where scheduled resource allocation mode is applied to one sidelink, it would be beneficial to inform the network that coexistence issue may happen and to provide some assistance information to the network for proper scheduling. Moreover, the in-device conflict may also happen between Uu and sidelink, e.g. simultaneous sidelink transmission and uplink transmission. Assistance information is also needed in this case.

The assistance info, for example, may include the affected frequency band or carrier, or time domain resources, etc. If both sidelink are controlled by the network, some kinds of coordination on the network side can be assumed.

Proposal 8: A UE can inform the network that the coexistence issue would happen (either between different sidelink modules or between Uu and sidelink), and provides assistance information to the network to resolve the coexistence issues.

On the other hand, if the autonomous resource selection mode is enabled in one sidelink, the scheduling node of one RAT may not be aware of the autonomous resource selection result of the other RAT and vice versa, thus not possible to avoid collision between them. Therefore, it is helpful to take into account the resources reserved by the network in the other sidelink, during the resource sensing and reservation procedure. As a result, the problematic resource candidates can be excluded, and the potential in-device coexistence issues can be mitigated.

Proposal 9: During the sensing and reservation procedure in one sidelink, a UE should consider the resources reserved by the network in the other sidelink to mitigate the potential coexistence issue.

- Limited Tx/Rx capability

The limited Tx capability issue has been discussed for LTE CA in eV2X. A similar issue also occurs in the case of LTE and NR sidelink coexistence. The solution in LTE CA is done in the MAC, by either excluding the subframe exceeds its TX capability limitation, or triggering resource reselection until the resultant transmission resources can be supported. It may not be simply reused for the coexistence case, because a separate MAC entity is employed for each RAT and real-time coordination between may be challenging.

On the other hand, a UE with limited Rx capability may not be able to monitor all possible channels at the same time. Even if it is possible, it is suboptimal in terms of power consumption and UE complexity, and not always necessary. For instance, the UE may need to monitor the basic safety message in LTE sidelink all the time. However, for advanced V2X service, the UE may not always need to monitor the same channel. Therefore, some enhancements should be introduced to mitigate the limited Tx/Rx capability issue.

Proposal 10: The issues of limited Tx/Rx capability need to be resolved for LTE and NR sidelink coexistence.

If the SA and the data can be transmitted in different carriers or different sidelink, it is possible that the UE performs sensing on only one carrier can still be aware of the resource reservation on another carrier or another sidelink. As a result, only a limited set of carriers are required for sensing, which can mitigate the issue of limited Tx/Rx capability. Moreover, it is useful if a UE is required to keep monitoring on one carrier or sidelink for some specific services (e.g., basic safety), while still be able to sense and transmit on the other carrier or sidelink for other services.

Proposal 11: Cross-carrier/-sidelink sensing and reservation can be considered for mitigating the limited Tx/Rx capability issues.

- Conclusion

In the contribution, we provide our view on the coexistence issues of LTE and NR sidelink technologies with the following observations and proposals:

Observation 1: The simplest TDM solution is to exclude the overlapping resources from the resource pool, which does not require dynamic coordination nor resource reservation results.

Proposal 1: Both DFN level and slot level synchronization should be assumed for coexistence of NR sidelink and LTE sidelink.

Proposal 2: At least for some FDM solutions, synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink.

Proposal 3: Semi-static coordination on sensing and resource reservation between LTE and NR sidelink modules can be used to prevent overlapping of NR and LTE V2X sidelink transmissions.

Proposal 4: Dynamic resource allocation (such as joint resource selection, dropping based on QoS, etc.) can be considered if short term coordination is possible between LTE and NR sidelink modules.

Proposal 5: The issues of power limited, self-interference and half-duplex in FDM solution need to be resolved for LTE and NR sidelink coexistence.

Proposal 6: Dynamic power sharing or packet dropping can be considered for FDM solution if dynamic inter-module coordination is achieved.

Proposal 7: Semi-static power sharing without dynamic coordination can be considered for FDM solution without stringent synchronization between LTE and NR sidelink transmissions.

Proposal 8: A UE can inform the network that the coexistence issue would happen (either between different sidelink modules or between Uu and sidelink), and provides assistance information to the network to resolve the coexistence issues.

Proposal 9: During the sensing and reservation procedure in one sidelink, a UE should consider the resources reserved by the network in the other sidelink to mitigate the potential coexistence issue.

Proposal 10: The issues of limited Tx/Rx capability need to be resolved for LTE and NR sidelink coexistence.

Proposal 11: Cross-carrier/-sidelink sensing and reservation can be considered for mitigating the limited Tx/Rx capability issues.

- Reference

- Chairman’s Notes, 3GPP TSG RAN WG1 meeting#95