3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	         R1-1900496

Taipei, Taiwan, 21st – 25th January, 2019

Source:	Intel Corporation

Title:	On enhancements to Scheduling/HARQ/CSI processing

Agenda item:	7.2.6.1.4

Document for:	Discussion and Decision

# Introduction

In this contribution, we present our views on potential enhancements related to DL/UL data scheduling and HARQ operations towards achieving the objectives for Rel-16 studies on URLLC, including potential enhancements to minimum UE processing times for DL HARQ and UL scheduling, and enhancements to CSI processing times.

# Achievability of latency budget with NR Rel-15 processing times

In this section, we present feasibility analysis of achieving different required latency budgets. For UL, transmission using Configured Grant (CG) as well as grant-based PUSCH, and for DL, dynamic scheduling are assumed.

Detailed assumptions on the transmission settings and constraints can be found in Annex-A.

Latency evaluations are performed corresponding to single-shot transmission, as well as 1 to 3 re-transmissions. Figures 1-2, demonstrate examples of timelines to compute the latency for DL and grant-based (GB) UL transmissions, respectively, along with the detailed components contributing to the overall latency as explained in Annex.

Further, we note that all possible contributors to alignment delays (including PDCCH/PUCCH/SR occasions as well as PDSCH/PUSCH scheduling constraints) need to be considered for a particular configuration, when accounting for latency components, with the data arrival at Layer 3/2 interface at different locations within a slot to determine the “worst alignment delay”.

Particularly, by sweeping the start of timeline over all possible symbols (and slots e.g., in case of TDD), the worst case latency can be obtained for each set of configuration assumptions. The major contributors to the worst case latencies are in fact the alignment delay(s) before transmission of different control/data channels due to either the constraint from slot-boundary or from availability of corresponding occasions of the control channel.

Given a certain set of transmission durations and starting occasions based on Rel-15 NR or any potential updated values, and other configuration assumptions, e.g., as listed in the Annex-A and Annex-B, we present the total number of cases satisfying the latency target(s), under each SCS, PDCCH periodicity, and set of processing times as in the summary tables below.

Figure 1. DL transmission timeline and contributing components to the overall latency (N1 includes the overlap margin due to simultaneous PDCCH and PDSCH TX) – the numbered itemized contributing factors are also listed in the Annex

Figure 2. Grant-based UL transmission timeline and contributing components to the overall latency - the numbered itemized contributing factors are also listed in the Annex

Table 1 – Summary of latency evaluations for baseline Capability 2 in Rel-15, FDD (based on Tables A1-A3 in Annex-C)

| NR FDD |  |  | UE capability 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
|  | UL CG, total 16 evaluated cases | # cases met 0.5ms target | 3 | 3 | 3 | 1 | 1 | 1 | 0 | 0 | 0 |
|  |  | # cases met 1ms target | 7 | 6 | 6 | 4 | 4 | 4 | 1 | 1 | 1 |
|  |  | # cases met 2ms target | 14 | 13 | 13 | 10 | 9 | 8 | 4 | 4 | 4 |
|  |  | # cases met 3ms target | 16 | 16 | 16 | 14 | 14 | 13 | 8 | 8 | 8 |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | # cases met 1ms target | 3 | 3 | 3 | 2 | 2 | 1 | 0 | 0 | 0 |
|  |  | # cases met 2ms target | 12 | 12 | 11 | 8 | 8 | 8 | 3 | 2 | 2 |
|  |  | # cases met 3ms target | 15 | 15 | 15 | 13 | 13 | 11 | 6 | 5 | 5 |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target | 3 | 3 | 3 | 2 | 2 | 1 | 0 | 0 | 0 |
|  |  | # cases met 1ms target | 7 | 7 | 7 | 4 | 4 | 4 | 2 | 2 | 1 |
|  |  | # cases met 2ms target | 14 | 14 | 14 | 10 | 9 | 9 | 5 | 5 | 4 |
|  |  | # cases met 3ms target | 16 | 16 | 16 | 14 | 14 | 13 | 10 | 10 | 9 |

Table 2 – Summary of latency evaluations for baseline Capability 2 in Rel-15, TDD 1 (based on Tables A4-A6 in Annex-C)

| NR TDD, where slots are of the form …/UL/DL/UL/DL/… (in this tdoc we call it TDD mode 1) |  |  | UE capability 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, TDD 1 |  |  | SCS 30kHz, TDD 1 |  |  | SCS 15kHz, TDD 1 |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
|  | UL CG, total 12 evaluated cases | # cases met 0.5ms target | 1 | 1 | 1 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 1ms target | 4 | 4 | 4 | 1 | 1 | 1 |  |  |  |
|  |  | # cases met 2ms target | 10 | 10 | 9 | 4 | 4 | 4 |  |  |  |
|  |  | # cases met 3ms target | 12 | 12 | 12 | 7 | 7 | 7 |  |  |  |
|  | UL GB, total 12 evaluated cases | # cases met 0.5ms target | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 1ms target | 1 | 1 | 1 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 2ms target | 7 | 7 | 7 | 3 | 3 | 3 |  |  |  |
|  |  | # cases met 3ms target | 12 | 12 | 12 | 6 | 6 | 6 |  |  |  |
|  | DL, total 12 evaluated cases | # cases met 0.5ms target | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 1ms target | 3 | 3 | 3 | 2 | 2 | 1 |  |  |  |
|  |  | # cases met 2ms target | 9 | 9 | 9 | 5 | 5 | 4 |  |  |  |
|  |  | # cases met 3ms target | 12 | 12 | 12 | 8 | 8 | 7 |  |  |  |

Table 3 – Summary of latency evaluations for baseline Capability 2 in Rel-15, TDD 2 (based on Tables A7-A9 in Annex-C)

| NR TDD, where each slot has 7 DL symbols first and then 7 UL symbols (in this tdoc we call it TDD mode 2) |  |  | UE capability 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, TDD 2 |  |  | SCS 30kHz, TDD 2 |  |  | SCS 15kHz, TDD 2 |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
|  | UL CG total 12 evaluated cases | # cases met 0.5ms target | 2 | 2 | 2 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 1ms target | 5 | 5 | 4 | 3 | 3 | 3 |  |  |  |
|  |  | # cases met 2ms target | 10 | 10 | 9 | 7 | 7 | 6 |  |  |  |
|  |  | # cases met 3ms target | 12 | 12 | 12 | 10 | 10 | 10 |  |  |  |
|  | UL GB, total 12 evaluated cases | # cases met 0.5ms target | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 1ms target | 2 | 2 | 2 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 2ms target | 8 | 8 | 7 | 5 | 5 | 4 |  |  |  |
|  |  | # cases met 3ms target | 11 | 11 | 10 | 9 | 9 | 8 |  |  |  |
|  | DL, total 12 evaluated cases | # cases met 0.5ms target | 2 | 2 | 1 | 0 | 0 | 0 |  |  |  |
|  |  | # cases met 1ms target | 4 | 4 | 4 | 3 | 3 | 3 |  |  |  |
|  |  | # cases met 2ms target | 10 | 10 | 8 | 7 | 7 | 6 |  |  |  |
|  |  | # cases met 3ms target | 12 | 12 | 12 | 11 | 11 | 11 |  |  |  |

As it can be seen from the detailed latency evaluation tables in Annex-C, as well as the summary tables above, depending on the choices of subcarrier spacing, transmission duration and PDCCH periodicity, certain scenarios satisfy the 0.5ms (e.g., for TSN)/1ms/2ms/3ms latency budgets for UL and DL cases (the green/yellow/blue/pink highlights in the latency tables presented in Annex-C, respectively). While it is challenging to fit more than 1 retransmission at maximum within the 0.5ms and 1ms latency budgets for most of the cases, more number of retransmissions can fulfill the targets of 2ms and 3ms.

# Improved UE minimum processing time in Rel-16

While the minimum processing times specified in NR are already significantly improved compared to their counterparts in LTE, further reduction in the minimum processing times, if feasible, may offer additional benefits in terms of improved HARQ-based operation (by realization of shorter TTIs), improved scheduling and multiplexing of different channels and signals (no need to schedule too far into the future), more up-to-date CSI feedback, etc.

At least for PDSCH processing (N1) and PUSCH preparation (N2) times, the currently specified Capability #2 numbers may be considered as the starting point. While there may not be a significant room to reduce the minimum processing times across the board, RAN1 could further study specific cases with room for further improvement. In this regard, it should be considered that most of the latency-critical use cases and analyses inherently consider relatively short data channel durations.

For PUSCH preparation times (N2), there is no dependency on the duration of the scheduled PUSCH. However, for PDSCH with short durations, additional margins are provisioned in consideration of the very limited time the UE may have in “catching-up” from the initial delay incurred due to the time needed for decoding of the scheduling DCI and channel estimation and demodulation efforts prior to decoding. Such margins are quite necessary in facilitating practical UE implementation and have been carefully factored in during Rel-15 specification development. However, for some cases, the margins can be seen to leave room for further improvement – a primary example is the case of PDSCH mapping type A with short durations.

For PDSCH mapping type A with durations 3 to 6 symbols, additional symbols are added to the nominal N1 value such that the PDSCH end is aligned with symbol #6 of a slot. For short PDSCH allocations (3~4 symbols) with mapping type A starting from symbols #0 or #1, this implies a significant additional delay incurred. From a UE processing time perspective, as long as there is sufficient consideration on the DMRS position, additional margin may not be essential.

Another case that leaves room for improvement and is quite relevant to URLLC operation is that of 60 kHz SCS, wherein the values for Capability #2 leave some room for improvement. Such enhancements could apply to both the N1 and N2 values.

For PUSCH preparation times (N2), the current Capability #2 numbers also leave some room for possible improvement for lower SCS values. A reduction by approximately a symbol duration can benefit in a non-negligible manner the worst-case latency performance as well as in cases with small TA usage.

We note that potential scheduling restrictions may be considered to enable very fast turn-around times, including limited number of CCs, number of layers, and number of scheduled PRBs.

## Achievability of latency budget with improved processing time in Rel-16

In order to study the impact of UE minimum processing times N1/N2 on the overall latency, we validate the gains with reductions to the N1/N2 values beyond NR Capability #2. Towards this, we evaluate the latency corresponding to the following potential N1/N2 choices (marked in red). This provides an estimation on what level of reduction of the minimum UE processing times may be pursued for each of SCS choices beyond NR Capability #2, as detailed in the table below.

Table 4 – Improved processing times beyond Capability 2 in Rel-15

| DMRS for CE | Processing time | 15 KHz |  |  | 30 KHz |  |  |  | 60 KHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | R15 | R16_a | R16_b [2] | R15 | R16_a | R16_b | R16_c [2] | R15 |  | R16_a |  | R16_b | R16_c [2] |
| Front-loaded DMRS only | N1 | 3 | 2.5 | 2.5 | 4.5 | 4 | 3 | 2.5 | 9 |  | 7 |  | 6 | 5 |
| Frequency First | N2 | 5 | 3.5 | 2.5 | 5.5 | 4 | 3 | 2.5 | 11 | 7 |  | 6 |  | 5 |

Given the coupling between N1 and N2 values in both DL and UL evaluations, at least due to the consideration of symmetric turn-around times at the BS side, we may need to consider other combinations, as well (also, this is the reason we have N1 = 2.5 in both columns for 15 kHz SCS).

Table 5 – Summary of latency evaluations for processing times beyond Capability 2 in Rel-15, FDD (based on Tables A10-A18 in Annex)

| NR FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SCS 60kHz, FDD |  |  |  | SCS 30kHz, FDD |  |  |  |  |  |  | SCS 15kHz, FDD |  |  |  |
|  |  |  |  | PDCCH every symbol |  | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol |  | PDCCH every 2-symbol |  | PDCCH every 4-symbol |  |  | PDCCH every symbol | PDCCH every 2-symbol |  | PDCCH every 4-symbol |
| Rel 16a | UL CG, total 16 evaluated cases | # cases met 0.5ms target |  | 3 |  | 3 | 3 | 1 |  | 1 |  | 1 |  |  | 1 | 1 |  | 1 |
|  |  | # cases met 1ms target |  | 8 |  | 8 | 8 | 4 |  | 4 |  | 4 |  |  | 2 | 2 |  | 2 |
|  |  | # cases met 2ms target |  | 14 |  | 14 | 14 | 10 |  | 10 |  | 10 |  |  | 6 | 5 |  | 5 |
|  |  | # cases met 3ms target |  | 16 |  | 16 | 16 | 14 |  | 14 |  | 14 |  |  | 10 | 9 |  | 9 |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target |  | 1 |  | 1 | 0 | 0 |  | 0 |  | 0 |  |  | 0 | 0 |  | 0 |
|  |  | # cases met 1ms target |  | 6 |  | 6 | 6 | 3 |  | 3 |  | 2 |  |  | 0 | 0 |  | 0 |
|  |  | # cases met 2ms target |  | 13 |  | 13 | 13 | 10 |  | 10 |  | 9 |  |  | 4 | 3 |  | 2 |
|  |  | # cases met 3ms target |  | 15 |  | 15 | 15 | 14 |  | 14 |  | 13 |  |  | 6 | 6 |  | 5 |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target |  | 3 |  | 3 | 3 | 2 |  | 2 |  | 1 |  |  | 0 | 0 |  | 0 |
|  |  | # cases met 1ms target |  | 8 |  | 8 | 8 | 5 |  | 5 |  | 4 |  |  | 2 | 2 |  | 2 |
|  |  | # cases met 2ms target |  | 14 |  | 14 | 14 | 12 |  | 12 |  | 10 |  |  | 5 | 5 |  | 5 |
|  |  | # cases met 3ms target |  | 16 |  | 16 | 16 | 14 |  | 14 |  | 14 |  |  | 10 | 9 |  | 9 |
| Rel 16b | UL CG, total 16 evaluated cases | # cases met 0.5ms target |  | 4 |  | 4 | 4 | 2 |  | 2 |  | 2 |  |  | 1 | 1 |  | 1 |
|  |  | # cases met 1ms target |  | 9 |  | 9 | 8 | 6 |  | 6 |  | 5 |  |  | 2 | 2 |  | 2 |
|  |  | # cases met 2ms target |  | 15 |  | 15 | 14 | 12 |  | 12 |  | 11 |  |  | 6 | 6 |  | 5 |
|  |  | # cases met 3ms target |  | 16 |  | 16 | 16 | 14 |  | 14 |  | 14 |  |  | 10 | 10 |  | 10 |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target |  | 1 |  | 1 | 1 | 0 |  | 0 |  | 0 |  |  | 0 | 0 |  | 0 |
|  |  | # cases met 1ms target |  | 8 |  | 7 | 7 | 4 |  | 4 |  | 3 |  |  | 0 | 0 |  | 0 |
|  |  | # cases met 2ms target |  | 15 |  | 15 | 14 | 11 |  | 11 |  | 10 |  |  | 4 | 4 |  | 3 |
|  |  | # cases met 3ms target |  | 16 |  | 16 | 15 | 14 |  | 14 |  | 14 |  |  | 8 | 8 |  | 7 |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target |  | 4 |  | 4 | 3 | 2 |  | 2 |  | 2 |  |  | 1 | 1 |  | 0 |
|  |  | # cases met 1ms target |  | 9 |  | 9 | 9 | 6 |  | 6 |  | 5 |  |  | 2 | 2 |  | 2 |
|  |  | # cases met 2ms target |  | 15 |  | 15 | 15 | 12 |  | 12 |  | 12 |  |  | 6 | 6 |  | 5 |
|  |  | # cases met 3ms target |  | 16 |  | 16 | 16 | 14 |  | 14 |  | 14 |  |  | 10 | 10 |  | 10 |
| Rel 16c | UL CG, total 12 evaluated cases |  | # cases met 0.5ms target | 4 | 4 |  | 4 |  | 2 |  | 2 |  | 2 |  |  |  |  |  |
|  |  |  | # cases met 1ms target | 10 | 10 |  | 9 |  | 6 |  | 6 |  | 5 |  |  |  |  |  |
|  |  |  | # cases met 2ms target | 15 | 15 |  | 15 |  | 12 |  | 12 |  | 11 |  |  |  |  |  |
|  |  |  | # cases met 3ms target | 16 | 16 |  | 16 |  | 14 |  | 14 |  | 14 |  |  |  |  |  |
|  | UL GB, total 12 evaluated cases |  | # cases met 0.5ms target | 2 | 2 |  | 2 |  | 0 |  | 0 |  | 0 |  |  |  |  |  |
|  |  |  | # cases met 1ms target | 9 | 8 |  | 8 |  | 4 |  | 4 |  | 3 |  |  |  |  |  |
|  |  |  | # cases met 2ms target | 15 | 15 |  | 15 |  | 11 |  | 11 |  | 10 |  |  |  |  |  |
|  |  |  | # cases met 3ms target | 16 | 16 |  | 16 |  | 14 |  | 14 |  | 14 |  |  |  |  |  |
|  | DL, total 12 evaluated cases |  | # cases met 0.5ms target | 4 | 4 |  | 4 |  | 2 |  | 2 |  | 2 |  |  |  |  |  |
|  |  |  | # cases met 1ms target | 10 | 10 |  | 10 |  | 6 |  | 6 |  | 5 |  |  |  |  |  |
|  |  |  | # cases met 2ms target | 15 | 15 |  | 15 |  | 12 |  | 12 |  | 12 |  |  |  |  |  |
|  |  |  | # cases met 3ms target | 16 | 16 |  | 16 |  | 14 |  | 14 |  | 14 |  |  |  |  |  |

Table 6 – Summary of latency evaluations for processing times beyond Capability 2 in Rel-15, TDD 1 (based on Tables A19-A27 in Annex)

| NR TDD, where slots are of the form …/UL/DL/UL/DL/… (in this tdoc we call it TDD mode 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SCS 60kHz, TDD 1 |  |  |  | SCS 30kHz, TDD 1 |  |  |  |  |  |  |
|  |  |  |  | PDCCH every symbol |  | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol |  | PDCCH every 2-symbol |  | PDCCH every 4-symbol |  |  |
| Rel 16a | UL CG, total 16 evaluated cases | # cases met 0.5ms target |  | 1 |  | 1 | 1 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 4 |  | 4 | 4 | 1 |  | 1 |  | 1 |  |  |
|  |  | # cases met 2ms target |  | 10 |  | 10 | 10 | 4 |  | 4 |  | 4 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 7 |  | 7 |  | 7 |  |  |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target |  | 0 |  | 0 | 0 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 2 |  | 2 | 2 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 2ms target |  | 8 |  | 8 | 8 | 3 |  | 3 |  | 3 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 6 |  | 6 |  | 6 |  |  |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target |  | 1 |  | 1 | 1 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 4 |  | 4 | 4 | 2 |  | 2 |  | 1 |  |  |
|  |  | # cases met 2ms target |  | 10 |  | 10 | 10 | 5 |  | 5 |  | 4 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 8 |  | 8 |  | 7 |  |  |
| Rel 16b | UL CG, total 16 evaluated cases | # cases met 0.5ms target |  | 1 |  | 1 | 1 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 4 |  | 4 | 4 | 2 |  | 2 |  | 2 |  |  |
|  |  | # cases met 2ms target |  | 10 |  | 10 | 10 | 5 |  | 5 |  | 5 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 8 |  | 8 |  | 8 |  |  |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target |  | 0 |  | 0 | 0 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 3 |  | 3 | 3 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 2ms target |  | 9 |  | 9 | 9 | 3 |  | 3 |  | 3 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 6 |  | 6 |  | 6 |  |  |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target |  | 1 |  | 1 | 1 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 4 |  | 4 | 4 | 2 |  | 2 |  | 2 |  |  |
|  |  | # cases met 2ms target |  | 10 |  | 10 | 10 | 5 |  | 5 |  | 5 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 8 |  | 8 |  | 8 |  |  |
| Rel 16c | UL CG, total 12 evaluated cases |  | # cases met 0.5ms target | 1 | 1 |  | 1 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 1ms target | 4 | 4 |  | 4 |  | 2 |  | 2 |  | 2 |
|  |  |  | # cases met 2ms target | 10 | 10 |  | 10 |  | 5 |  | 5 |  | 5 |
|  |  |  | # cases met 3ms target | 12 | 12 |  | 12 |  | 8 |  | 8 |  | 8 |
|  | UL GB, total 12 evaluated cases |  | # cases met 0.5ms target | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 1ms target | 3 | 3 |  | 3 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 2ms target | 9 | 9 |  | 9 |  | 3 |  | 3 |  | 3 |
|  |  |  | # cases met 3ms target | 12 | 12 |  | 12 |  | 6 |  | 6 |  | 6 |
|  | DL, total 12 evaluated cases |  | # cases met 0.5ms target | 2 | 2 |  | 1 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 1ms target | 5 | 5 |  | 4 |  | 2 |  | 2 |  | 2 |
|  |  |  | # cases met 2ms target | 11 | 11 |  | 10 |  | 5 |  | 5 |  | 5 |
|  |  |  | # cases met 3ms target | 12 | 12 |  | 12 |  | 8 |  | 8 |  | 8 |

Table 7 – Summary of latency evaluations for processing times beyond Capability 2 in Rel-15, TDD 2 (based on Tables A28-A36 in Annex)

| NR TDD, where each slot has 7 DL symbols first and then 7 UL symbols (in this tdoc we call it TDD mode 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SCS 60kHz, TDD 2 |  |  |  | SCS 30kHz, TDD 2 |  |  |  |  |  |  |
|  |  |  |  | PDCCH every symbol |  | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol |  | PDCCH every 2-symbol |  | PDCCH every 4-symbol |  |  |
| Rel 16a | UL CG, total 16 evaluated cases | # cases met 0.5ms target |  | 3 |  | 3 | 3 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 6 |  | 5 | 5 | 3 |  | 3 |  | 3 |  |  |
|  |  | # cases met 2ms target |  | 12 |  | 11 | 11 | 8 |  | 8 |  | 8 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 11 |  | 11 |  | 11 |  |  |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target |  | 0 |  | 0 | 0 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 4 |  | 3 | 3 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 2ms target |  | 10 |  | 9 | 9 | 5 |  | 5 |  | 5 |  |  |
|  |  | # cases met 3ms target |  | 11 |  | 11 | 11 | 10 |  | 10 |  | 10 |  |  |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target |  | 3 |  | 3 | 3 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 5 |  | 5 | 5 | 3 |  | 3 |  | 3 |  |  |
|  |  | # cases met 2ms target |  | 11 |  | 11 | 11 | 8 |  | 8 |  | 8 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 12 |  | 12 |  | 11 |  |  |
| Rel 16b | UL CG, total 16 evaluated cases | # cases met 0.5ms target |  | 3 |  | 3 | 3 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 7 |  | 7 | 6 | 3 |  | 3 |  | 3 |  |  |
|  |  | # cases met 2ms target |  | 12 |  | 12 | 12 | 9 |  | 9 |  | 9 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 12 |  | 11 |  | 11 |  |  |
|  | UL GB, total 16 evaluated cases | # cases met 0.5ms target |  | 0 |  | 0 | 0 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 5 |  | 4 | 4 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 2ms target |  | 11 |  | 10 | 10 | 6 |  | 6 |  | 5 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 12 |  | 10 |  | 10 |  |  |
|  | DL, total 16 evaluated cases | # cases met 0.5ms target |  | 3 |  | 3 | 3 | 0 |  | 0 |  | 0 |  |  |
|  |  | # cases met 1ms target |  | 7 |  | 7 | 6 | 3 |  | 3 |  | 3 |  |  |
|  |  | # cases met 2ms target |  | 12 |  | 12 | 12 | 9 |  | 9 |  | 9 |  |  |
|  |  | # cases met 3ms target |  | 12 |  | 12 | 12 | 12 |  | 12 |  | 12 |  |  |
| Rel 16c | UL CG, total 12 evaluated cases |  | # cases met 0.5ms target | 3 | 3 |  | 3 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 1ms target | 7 | 7 |  | 7 |  | 3 |  | 3 |  | 3 |
|  |  |  | # cases met 2ms target | 12 | 12 |  | 12 |  | 9 |  | 8 |  | 8 |
|  |  |  | # cases met 3ms target | 12 | 12 |  | 12 |  | 12 |  | 11 |  | 11 |
|  | UL GB, total 12 evaluated cases |  | # cases met 0.5ms target | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 1ms target | 5 | 5 |  | 5 |  | 0 |  | 0 |  | 0 |
|  |  |  | # cases met 2ms target | 11 | 11 |  | 11 |  | 6 |  | 5 |  | 5 |
|  |  |  | # cases met 3ms target | 12 | 12 |  | 12 |  | 12 |  | 10 |  | 10 |
|  | DL, total 12 evaluated cases |  | # cases met 0.5ms target | 3 | 3 |  | 3 |  | 1 |  | 0 |  | 0 |
|  |  |  | # cases met 1ms target | 8 | 8 |  | 7 |  | 4 |  | 3 |  | 3 |
|  |  |  | # cases met 2ms target | 12 | 12 |  | 12 |  | 10 |  | 9 |  | 9 |
|  |  |  | # cases met 3ms target | 12 | 12 |  | 12 |  | 12 |  | 12 |  | 12 |

Based on the presented tables, we have the following observations:

Observation 1:

- Potential gains from reducing the UE minimum processing times, are more pronounced for SCS of 60 kHz compared to 15 and 30 kHz.

- Gain margins are larger for GB PUSCH compared to CG PUSCH or DL scheduling cases due to multiple contributions from UE and gNB processing times.

- Potential gains in terms of satisfying the tight URLLC requirements (0.5ms or 1ms latency budgets) are limited, especially for lower SCS values.

- Use cases with latency target of 3ms and above can be satisfied efficiently using Rel-15 design.

Consequently, we have the following proposal:

Proposal 1:

- Focus on larger SCS cases wherein there is larger potential room for improved spectral efficiency from reduced minimum processing times (from a system spectral efficiency perspective).

- In terms of further prioritization, it would be instructive to focus on DL scheduling and CG PUSCH cases rather than the case of GB PUSCH when considering very strict latency budgets like 0.5ms or 1ms.

# Enhancements to processing times for A-CSI feedback

Regarding CSI computation times, beyond the possible path of reducing currently specified processing times, new characterization of CSI computation times may be needed in light of potential enhancements to CSI feedback triggering and reporting mechanisms. However, while the benefits from a very fast CSI feedback for fast adaptation for retransmission scheduling may be limited in practice, such enhancements can severely increase the UE complexity.

Supporting very fast CSI feedback, e.g., triggered by a NACK to a PDSCH reception and with similar processing times as HARQ-ACK feedback without consideration of CSI reporting, can incur significant burden on UE complexity. At the same time, such a feature is unlikely to bring material benefits to URLLC operation considering the fact that, even when assuming a HARQ-retransmission-based operation, the initial BLER should still be quite low to meaningfully impact overall latency/reliability performance and resource utilization. The real benefits on top of outer-loop link adaptation and from “fine-tuned retransmission scheduling” are questionable at best. In addition, in order to make such tight CSI feedback feasible it has been suggested to consider PDSCH/PDCCH DMRS-based CSI feedback. This essentially offers a “fast but coarse” CSI feedback that is unlikely to be very useful towards realizing the objective of optimized scheduling of a retransmission.

For URLLC operations, robust scheduling is of utmost importance and it is unclear as to how much real benefits can be realized from rare (e.g., triggered by a NACK) instances of a coarse CSI feedback.

At the RAN1 #95 meeting, it was suggested that since the mandatory UE CSI processing capability requires a UE to support calculation of 5 simultaneous CSI reports, the CSI requirement 2 is about 5x longer than what it would be if the requirement were that only a single CSI report would need to be computed. However, this observation may hold only for the case of serial processing of CSI reports. The multiple CSI reports may be prepared using serial or parallel processing approaches. For the latter case, reducing the number of reports from five does not necessarily reduce the minimum turn-around time. In our view, for similar setups (i.e., a single report computation), Rel-15 already supports fast CSI feedback based on CSI computation delay requirement 1.

Observation 2:

- Benefits of supporting a fast CSI feedback, triggered by failed reception of scheduled PDSCH are unclear and may not be warranted considering the impact to UE complexity and specification efforts.

- It is not necessarily feasible to linearly reduce the CSI processing times for CSI computation delay requirement 2 by reducing the number of CSI reports.

# Conclusion

In this contribution, we presented our views on potential enhancements related to DL/UL data scheduling and HARQ operations towards achieving the objectives for Rel-16 studies on URLLC.

Our analysis, showed that depending on the choice of subcarrier spacing, transmission duration, and the PUSCH/PDSCH mapping type, Rel-15 NR can satisfy the 0.5ms/1ms latency budget for both UL and DL in certain cases. Still, some improvements may be considered regarding the reduction of UE minimum processing times, etc. As such, we have the following observations and proposals.

Observation 1:

- Potential gains from reducing the UE minimum processing times, are more pronounced for SCS of 60 kHz compared to 15 and 30 kHz.

- Gain margins are larger for GB PUSCH compared to CG PUSCH or DL scheduling cases due to multiple contributions from UE and gNB processing times.

- Potential gains in terms of satisfying the tight URLLC requirements (0.5ms or 1ms latency budgets) are limited, especially for lower SCS values.

- Use cases with latency target of 3ms and above can be satisfied efficiently using Rel-15 design.

Proposal 1:

- Focus on larger SCS cases wherein there is larger potential room for improved spectral efficiency from reduced minimum processing times (from a system spectral efficiency perspective).

- In terms of further prioritization, it would be instructive to focus on DL scheduling and CG PUSCH cases rather than the case of GB PUSCH when considering very strict latency budgets like 0.5ms or 1ms.

Observation 2:

- Benefits of supporting a fast CSI feedback, triggered by failed reception of scheduled PDSCH are unclear and may not be warranted considering the impact to UE complexity and specification efforts.

- It is not necessarily feasible to linearly reduce the CSI processing times for CSI computation delay requirement 2 by reducing the number of CSI reports.

# References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, Washington, USA, Nov. 12th – 16th, 2018

- R1-1812156, “Scheduling/HARQ/CSI Processing Timeline Enhancements for NR URLLC,” Ericsson, 3GPP TSG RAN WG1 Meeting #95, Spokane, Washington, USA, Nov. 12th – 16th, 2018

# Annex-A

## Evaluation assumptions

## Assumptions regarding the data channel transmission

GF UL transmission uses the following start symbols (to most extent aligned with IMT-2020 submission assumptions for UP latency evaluations as endorsed by RAN1 in R1-1809947):

  - For 2-symbol PUSCH, the start symbol can be symbols {0,2,4,6,8,10,12} for PUSCH resource mapping type B

  - For 4-symbol PUSCH, the start symbol can be:

    - For PUSCH resource mapping type B: symbols {0,3,7}

    - For PUSCH resource mapping type A: symbol 0;

  - For 7-symbol PUSCH, the start symbol can be:

    - For PUSCH resource mapping type B: symbols {0, 7}

    - For PUSCH resource mapping type A: symbol 0;

  - For 14-symbol PUSCH, the start symbol can be at symbol #0 for PUSCH resource mapping type A and B.

Regarding the DL case, we note that for PDSCH mapping type A,

- Starting symbol can be symbol index #0, 1, 2, 3 in a slot

- Length of the PDSCH is at least 3 symbols, up to 14 symbols  within a slot, such that slot boundary is not crossed

For PDSCH mapping type B,

- Starting symbol can be any position within a slot, such that slot boundary is not crossed

- Length of the PDSCH can be 2, 4, or 7 symbols

Grant-based UL transmission duration are also very similar to those of DL. Hence, for DL and GB UL,

- For 2-symbol PDSCH/PUSCH, the start symbol can be symbols [0:12] for resource mapping type B

- For 4-symbol, the start symbol can be:

    - For PDSCH/PUSCH resource mapping type B: symbols [0:10]

    - For PDSCH resource mapping type A: symbols [0:3]

- For 7-symbol, the start symbol can be:

    - For PDSCH/PUSCH resource mapping type B: symbols [0:7]

    - For PDSCH resource mapping type A: symbol [0:3]

- For 14-symbol PDSCH/PUSCH, the start symbol can be at symbol #0 for resource mapping type A and B.

In the evaluations of this contribution, we focus on latency evaluations for mapping type B.

## Assumptions regarding transmission of channels other than the data channels:

- We assume that PUCCH are available every OFDM symbol [0:13], i.e., it is assumed that the first available PUCCH is located at the first symbol of slot.

- As of PDCCH, as can be seen in the tables, periodicities of every symbol, every 2-symbol, and every 4-symbol ([0:13], [0:2:13], [0:4:13], respectively) have been considered], i.e., it is assumed that the first available PDCCH is located at the first symbol of slot.

- For DL, we assume that PDCCH and PDSCH start at the same symbol. Hence, if PDCCH is not available every symbol, the union of constraints from the PDSCH starting occasions and from PDCCH availability is considered.

- Also, for PDSCH mapping type B with durations of 2 and 4 symbols (in all cases), in the processing time values, we have considered the N1 value with additional margin (as a function of the time-domain overlap between the PDCCH and PDSCH, i.e., 1OS for PDCCH length of 1OS, etc.).

  - Further, though no mapping type A has been considered in this tdoc, for PDSCH with mapping type A, for the 4-symbol case, an additional margin of 3 symbols to the Capability 2 N1 value, should be considered.

- To obtain the latency values for UL GB case, we note that alignment delay for SR transmission, SR transmission duration, gNB SR receiving processing time, alignment delay for PDCCH carrying transmission grant, PDCCH transmission duration, as well UE PDCCH receive processing times should be included in the timeline.

- We assume SR TX positions are at symbols [1:2:13] of slot (for [0:13] symbols in the slot).

## Assumptions on gNB processing times

We follow the IMT-2020 assumptions of considering the reciprocal relationship to the UE processing times. For instance, in UL, the time required for PUSCH reception and processing, and potential re-scheduling of reTX, is considered as N1. Similarly, for DL, the reciprocal of N2 is considered, as the time that gNB takes to process ACK/NACK feedback from the UE and then prepare for a reTX of the packet and the scheduling, etc.

# Annex-B

## Detailed contributing factors to the overall latency:

Here, we also follow the IMT-2020 approach that considers from the Layer 3 to Layer 2 ingress at the Tx (implying need to consider initial processing/scheduling time for DL scheduling example) up until Layer 2 to 3 egress at the Rx (implying we do not need to consider the time to prepare the HARQ-ACK feedback for the very last reception instance), with approximating the decoding and preparation times as half of the corresponding N1/N2 values.

## DL scenario:

- Initial TX:

- gNB processing time for preparing N2/2 (e.g., assuming gNB processing times are equal to those of UEs)

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PDCCH+PDSCH duration

- UE  processing time for decoding, i.e., (N1 + d11)/2

- 1st reTX:

- gNB processing time for preparing, e.g., N2/2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PDCCH+PDSCH duration

- UE  processing time, i.e., N1 + d11

- Alignment delay until the start of next symbol

- PUCCH length

- gNB processing time, e.g., N2

- Alignment delay until the start of next symbol

- PDCCH+PDSCH duration

- UE  processing time for decoding, i.e., (N1 + d11)/2

## UL GB scenario:

- Initial TX:

- UE processing time for preparing N2/2

- Alignment delay for SR TX opportunity

- SR TX duration

- SR reception to initial PUSCH grant processing time at the gNB, e.g., N1

- Alignment delay for PDCCH TX to transmit the UL grant

- PDCCH  duration

- UE processing time N2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PUSCH duration

- gNB processing time for decoding, e.g., (N1)/2

- 1st reTX:

- UE processing time for preparing N2/2

- Alignment delay for SR TX opportunity

- SR TX duration

- gNB processing time, e.g., N1

- Alignment delay for PDCCH TX to transmit the UL grant

- PDCCH  duration

- UE processing time N2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PUSCH duration

- gNB processing time, e.g., N1

- Alignment delay for PDCCH TX

- PDCCH length

- UE processing time N2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PUSCH duration

- gNB processing time for decoding, e.g., (N1)/2

## UL CG scenario:

- Initial TX:

- UE processing time for preparing N2/2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PUSCH duration

- gNB processing time for decoding, e.g., (N1)/2

- 1st reTX:

- UE processing time for preparing N2/2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PUSCH duration

- gNB processing time, e.g., N1

- Alignment delay for PDCCH TX

- PDCCH length

- UE processing time N2

- Alignment delay until the next transmission occasion  that allows transmission not crossing slot border

- PUSCH duration

- gNB processing time for decoding, e.g., (N1)/2

# Annex-C

## Evaluation results

## Baseline – FDD Configuration

Table A1– UL user plane worst-case latency (CG PUSCH) for NR FDD with grant free transmission (msec)

| UL user plane latency (Grant free) – NR FDD |  |  | UE capability 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.2500 | 0.2500 | 0.2500 | 0.3214 | 0.3214 | 0.3214 | 0.5714 | 0.5714 | 0.5714 |
|  |  | 1 reTX | 0.6786 | 0.6786 | 0.7143 | 0.8214 | 0.8929 | 0.9643 | 1.4286 | 1.4286 | 1.5714 |
|  |  | 2reTX | 1.1071 | 1.1071 | 1.1429 | 1.3214 | 1.4643 | 1.6071 | 2.2857 | 2.2857 | 2.5714 |
|  |  | 3reTX | 1.5357 | 1.5357 | 1.6071 | 1.8214 | 2.0357 | 2.2500 | 3.1429 | 3.1429 | 3.5714 |
|  | M=4 (4OS) | Initial TX | 0.3750 | 0.3750 | 0.3750 | 0.5714 | 0.5714 | 0.5714 | 1.0714 | 1.0714 | 1.0714 |
|  |  | 1 reTX | 0.8750 | 0.8750 | 0.8750 | 1.2143 | 1.2143 | 1.3214 | 2.0714 | 2.0714 | 2.0714 |
|  |  | 2reTX | 1.3750 | 1.3750 | 1.3750 | 1.8214 | 1.8214 | 2.0714 | 3.0714 | 3.0714 | 3.0714 |
|  |  | 3reTX | 1.8750 | 1.8750 | 1.8750 | 2.5714 | 2.5714 | 2.8214 | 4.0714 | 4.0714 | 4.0714 |
|  | M=7 (7OS) | Initial TX | 0.4286 | 0.4286 | 0.4286 | 0.6786 | 0.6786 | 0.6786 | 1.2857 | 1.2857 | 1.2857 |
|  |  | 1 reTX | 0.9286 | 1.0536 | 1.0536 | 1.4286 | 1.4286 | 1.6786 | 2.7857 | 2.7857 | 2.7857 |
|  |  | 2reTX | 1.4286 | 1.5536 | 1.6786 | 2.1786 | 2.1786 | 2.6786 | 4.2857 | 4.2857 | 4.2857 |
|  |  | 3reTX | 1.9286 | 2.0536 | 2.3036 | 2.9286 | 2.9286 | 3.6786 | 5.7857 | 5.7857 | 5.7857 |
|  | M=14 (14OS) | Initial TX | 0.6786 | 0.6786 | 0.6786 | 1.1786 | 1.1786 | 1.1786 | 2.2857 | 2.2857 | 2.2857 |
|  |  | 1 reTX | 1.4286 | 1.4286 | 1.4286 | 2.1786 | 2.1786 | 2.6786 | 4.2857 | 4.2857 | 4.2857 |
|  |  | 2reTX | 2.1786 | 2.1786 | 2.1786 | 3.1786 | 3.1786 | 4.1786 | 6.2857 | 6.2857 | 6.2857 |
|  |  | 3reTX | 2.9286 | 2.9286 | 2.9286 | 4.1786 | 4.1786 | 5.6786 | 8.2857 | 8.2857 | 8.2857 |
| # 0.5 |  |  | 3 | 3 | 3 | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 7 | 6 | 6 | 4 | 4 | 4 | 1 | 1 | 1 |
| # 2 |  |  | 14 | 13 | 13 | 10 | 9 | 8 | 4 | 4 | 4 |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 13 | 8 | 8 | 8 |

Table A2 – UL user plane latency (GB PUSCH) for NR FDD with grant-based transmission (msec)

| UL user plane latency (Grant based) – NR FDD |  |  | UE capability 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2- symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.6607 | 0.6607 | 0.6964 | 0.7857 | 0.8571 | 0.8929 | 1.3571 | 1.3571 | 1.5000 |
|  |  | 1 reTX | 1.0714 | 1.0893 | 1.1250 | 1.2857 | 1.3929 | 1.3929 | 2.1429 | 2.2143 | 2.5000 |
|  |  | 2reTX | 1.4821 | 1.5179 | 1.5893 | 1.7857 | 1.8929 | 1.8929 | 2.9286 | 3.0714 | 3.5000 |
|  |  | 3reTX | 1.9107 | 1.9464 | 2.0179 | 2.2857 | 2.3929 | 2.3929 | 3.7143 | 3.9286 | 4.5000 |
|  | M=4 (4OS) | Initial TX | 0.7321 | 0.7321 | 0.7321 | 0.9286 | 1.0000 | 1.0714 | 1.6429 | 1.6429 | 1.6429 |
|  |  | 1 reTX | 1.2321 | 1.2321 | 1.2321 | 1.5000 | 1.6071 | 1.7500 | 2.6429 | 2.6429 | 2.6429 |
|  |  | 2reTX | 1.7321 | 1.7321 | 1.7321 | 2.0714 | 2.1786 | 2.3214 | 3.6429 | 3.6429 | 3.6429 |
|  |  | 3reTX | 2.2321 | 2.2321 | 2.2321 | 2.6429 | 2.7500 | 3.0714 | 4.6429 | 4.6429 | 4.6429 |
|  | M=7 (7OS) | Initial TX | 0.8214 | 0.8571 | 0.8571 | 1.1786 | 1.1786 | 1.1786 | 2.0000 | 2.1429 | 2.1429 |
|  |  | 1 reTX | 1.3214 | 1.3571 | 1.3929 | 1.8571 | 1.8929 | 1.8929 | 3.1429 | 3.2857 | 3.4286 |
|  |  | 2reTX | 1.8214 | 1.8571 | 1.8929 | 2.6786 | 2.6786 | 2.6786 | 4.2857 | 4.4286 | 4.5714 |
|  |  | 3reTX | 2.3214 | 2.3571 | 2.3929 | 3.3571 | 3.3929 | 3.3929 | 5.4286 | 5.5714 | 6.1429 |
|  | M=14 (14OS) | Initial TX | 1.0893 | 1.0893 | 1.1250 | 1.6429 | 1.7143 | 1.7857 | 3.0714 | 3.0714 | 3.0714 |
|  |  | 1 reTX | 1.8393 | 1.8393 | 1.8750 | 2.6429 | 2.7143 | 3.2857 | 5.0714 | 5.0714 | 5.0714 |
|  |  | 2reTX | 2.5893 | 2.5893 | 2.6250 | 3.6429 | 3.7143 | 4.7857 | 7.0714 | 7.0714 | 7.0714 |
|  |  | 3reTX | 3.3393 | 3.3393 | 3.3750 | 4.6429 | 4.7143 | 6.2857 | 9.0714 | 9.0714 | 9.0714 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 3 | 3 | 3 | 2 | 2 | 1 | 0 | 0 | 0 |
| # 2 |  |  | 12 | 12 | 11 | 8 | 8 | 8 | 3 | 2 | 2 |
| # 3 |  |  | 15 | 15 | 15 | 13 | 13 | 11 | 6 | 5 | 5 |

Table A3 - DL user plane latency for NR FDD (msec)

| DL user plane latency – NR FDD |  |  | UE capability 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.2589 | 0.2589 | 0.2946 | 0.3393 | 0.3393 | 0.4107 | 0.6071 | 0.6071 | 0.7500 |
|  |  | 1 reTX | 0.6875 | 0.6875 | 0.7232 | 0.8750 | 0.9107 | 1.0536 | 1.4643 | 1.4643 | 1.7500 |
|  |  | 2reTX | 1.1161 | 1.1161 | 1.1875 | 1.4107 | 1.4821 | 1.6964 | 2.3214 | 2.3214 | 2.7500 |
|  |  | 3reTX | 1.5446 | 1.5446 | 1.6161 | 1.9464 | 2.0536 | 2.2679 | 3.1786 | 3.1786 | 3.7500 |
|  | M=4 (4OS) | Initial TX | 0.3304 | 0.3304 | 0.3661 | 0.4821 | 0.4821 | 0.5536 | 0.8929 | 0.8929 | 1.0357 |
|  |  | 1 reTX | 0.8304 | 0.8304 | 0.8661 | 1.0893 | 1.1250 | 1.1964 | 1.8929 | 1.8929 | 2.0357 |
|  |  | 2reTX | 1.3304 | 1.3304 | 1.3661 | 1.6964 | 1.7679 | 1.8393 | 2.8929 | 2.8929 | 3.0357 |
|  |  | 3reTX | 1.8304 | 1.8304 | 1.8661 | 2.3036 | 2.4821 | 2.5536 | 3.8929 | 3.8929 | 4.0357 |
|  | M=7 (7OS) | Initial TX | 0.4286 | 0.4464 | 0.4821 | 0.6786 | 0.7143 | 0.7857 | 1.2857 | 1.3571 | 1.5000 |
|  |  | 1 reTX | 0.9286 | 0.9464 | 0.9821 | 1.3571 | 1.4286 | 1.7857 | 2.4286 | 2.5000 | 2.7857 |
|  |  | 2reTX | 1.4286 | 1.4464 | 1.4821 | 2.1786 | 2.2143 | 2.7857 | 3.5714 | 3.6429 | 4.5000 |
|  |  | 3reTX | 1.9286 | 1.9464 | 1.9821 | 2.8571 | 2.9286 | 3.7857 | 4.7143 | 4.7857 | 5.7857 |
|  | M=14 (14OS) | Initial TX | 0.6786 | 0.6786 | 0.6786 | 1.1786 | 1.1786 | 1.1786 | 2.2857 | 2.2857 | 2.2857 |
|  |  | 1 reTX | 1.4286 | 1.4286 | 1.4286 | 2.1786 | 2.1786 | 2.1786 | 4.2857 | 4.2857 | 4.2857 |
|  |  | 2reTX | 2.1786 | 2.1786 | 2.1786 | 3.1786 | 3.1786 | 3.1786 | 6.2857 | 6.2857 | 6.2857 |
|  |  | 3reTX | 2.9286 | 2.9286 | 2.9286 | 4.1786 | 4.1786 | 4.1786 | 8.2857 | 8.2857 | 8.2857 |
| # 0.5 |  |  | 3 | 3 | 3 | 2 | 2 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 7 | 7 | 7 | 4 | 4 | 4 | 2 | 2 | 1 |
| # 2 |  |  | 14 | 14 | 14 | 10 | 9 | 9 | 5 | 5 | 4 |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 13 | 10 | 10 | 9 |

We note that there exist some (few) cases where the timeline starting position(s) which leads to the maximum overall latency, is not necessarily same or may not have any common value, for all or some of initial TX, 1st reTX, 2nd reTX, and/or 3rd reTX. In such cases, the worst-case latency for each of these transmissions is computed based on its own corresponding starting position.

- For example, for GF PUSCH TX of length 4, with PUSCH transmission positions of symbols [0,3,7] in the slot and with PDCCH available every symbol or every other symbol, the worst case timeline starting position for the initial TX and 3rs reTX is the 8th symbol of the slot, while for the 1st reTX is the 4th symbol, and for the 2nd reTX is any of [1,4,8]-th symbols of the slot. However, in this case, when PDCCH is available every four symbol, the worst case timeline starting position for all transmissions is the 8TH symbol of the slot.

## Baseline - TDD Configuration

Table A4– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant free transmission (msec)

| UL user plane latency (Grant free) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.5000 | 0.5000 | 0.5000 | 0.8214 | 0.8214 | 0.8214 |
|  |  | 1 reTX | 1.0000 | 1.0000 | 1.0000 | 1.8214 | 1.8214 | 1.8214 |
|  |  | 2reTX | 1.5000 | 1.5000 | 1.5000 | 2.8214 | 2.8214 | 2.8214 |
|  |  | 3reTX | 2.0000 | 2.0000 | 2.0000 | 3.8214 | 3.8214 | 3.8214 |
|  | M=4 (4OS) | Initial TX | 0.6250 | 0.6250 | 0.6250 | 1.0714 | 1.0714 | 1.0714 |
|  |  | 1 reTX | 1.1250 | 1.1250 | 1.1250 | 2.0714 | 2.0714 | 2.0714 |
|  |  | 2reTX | 1.6250 | 1.6250 | 1.6250 | 3.0714 | 3.0714 | 3.0714 |
|  |  | 3reTX | 2.1250 | 2.1250 | 2.1250 | 4.0714 | 4.0714 | 4.0714 |
|  | M=7 (7OS) | Initial TX | 0.6786 | 0.6786 | 0.6786 | 1.1786 | 1.1786 | 1.1786 |
|  |  | 1 reTX | 1.1786 | 1.3036 | 1.3036 | 2.1786 | 2.1786 | 2.1786 |
|  |  | 2reTX | 1.6786 | 1.8036 | 2.1786 | 3.1786 | 3.1786 | 3.1786 |
|  |  | 3reTX | 2.1786 | 2.3036 | 2.8036 | 4.1786 | 4.1786 | 4.1786 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 1 | 1 | 1 |
| # 2 |  |  | 10 | 10 | 9 | 4 | 4 | 4 |
| # 3 |  |  | 12 | 12 | 12 | 7 | 7 | 7 |

Table A5– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant-based transmission (msec)

| UL user plane latency (Grant-based) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.9821 | 0.9821 | 0.9821 | 1.7857 | 1.7857 | 1.7857 |
|  |  | 1 reTX | 1.4821 | 1.4821 | 1.4821 | 2.7857 | 2.7857 | 2.7857 |
|  |  | 2reTX | 1.9821 | 1.9821 | 1.9821 | 3.7857 | 3.7857 | 3.7857 |
|  |  | 3reTX | 2.4821 | 2.4821 | 2.4821 | 4.7857 | 4.7857 | 4.7857 |
|  | M=4 (4OS) | Initial TX | 1.0179 | 1.0179 | 1.0179 | 1.8571 | 1.8571 | 1.8571 |
|  |  | 1 reTX | 1.5179 | 1.5179 | 1.5179 | 2.8571 | 2.8571 | 2.8571 |
|  |  | 2reTX | 2.0179 | 2.0179 | 2.0179 | 3.8571 | 3.8571 | 3.8571 |
|  |  | 3reTX | 2.5179 | 2.5179 | 2.5179 | 4.8571 | 4.8571 | 4.8571 |
|  | M=7 (7OS) | Initial TX | 1.0714 | 1.1071 | 1.1071 | 1.9643 | 1.9643 | 1.9643 |
|  |  | 1 reTX | 1.5714 | 1.6071 | 1.6429 | 2.9643 | 2.9643 | 2.9643 |
|  |  | 2reTX | 2.0714 | 2.1071 | 2.1429 | 3.9643 | 3.9643 | 3.9643 |
|  |  | 3reTX | 2.5714 | 2.6071 | 2.6429 | 4.9643 | 4.9643 | 4.9643 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 2 |  |  | 7 | 7 | 7 | 3 | 3 | 3 |
| # 3 |  |  | 12 | 12 | 12 | 6 | 6 | 6 |

Table A6– DL user plane worst-case latency for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) (msec)

| DL user plane latency – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.5089 | 0.5089 | 0.5089 | 0.8393 | 0.8393 | 0.8393 |
|  |  | 1 reTX | 1.0089 | 1.0089 | 1.0089 | 1.8393 | 1.8393 | 1.8393 |
|  |  | 2reTX | 1.5089 | 1.5089 | 1.5089 | 2.8393 | 2.8393 | 2.8393 |
|  |  | 3reTX | 2.0089 | 2.0089 | 2.0089 | 3.8393 | 3.8393 | 3.8393 |
|  | M=4 (4OS) | Initial TX | 0.5804 | 0.5804 | 0.6161 | 0.9821 | 0.9821 | 1.0536 |
|  |  | 1 reTX | 1.0804 | 1.0804 | 1.1161 | 1.9821 | 1.9821 | 2.0536 |
|  |  | 2reTX | 1.5804 | 1.5804 | 1.6161 | 2.9821 | 2.9821 | 3.0536 |
|  |  | 3reTX | 2.0804 | 2.0804 | 2.1161 | 3.9821 | 3.9821 | 4.0536 |
|  | M=7 (7OS) | Initial TX | 0.6786 | 0.6964 | 0.7321 | 1.1786 | 1.2143 | 1.2857 |
|  |  | 1 reTX | 1.1786 | 1.1964 | 1.2321 | 2.1786 | 2.2143 | 2.2857 |
|  |  | 2reTX | 1.6786 | 1.6964 | 1.7321 | 3.1786 | 3.2143 | 3.2857 |
|  |  | 3reTX | 2.1786 | 2.1964 | 2.2321 | 4.1786 | 4.2143 | 4.2857 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 3 | 3 | 3 | 2 | 2 | 1 |
| # 2 |  |  | 9 | 9 | 9 | 5 | 5 | 4 |
| # 3 |  |  | 12 | 12 | 12 | 8 | 8 | 7 |

Table A7– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant free transmission (msec)

| UL user plane latency (Grant free) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3929 | 0.3929 | 0.3929 | 0.6071 | 0.6071 | 0.6071 |
|  |  | 1 reTX | 0.8929 | 0.8929 | 0.9643 | 1.1071 | 1.1786 | 1.2500 |
|  |  | 2reTX | 1.3929 | 1.3929 | 1.4643 | 1.6071 | 1.7500 | 2.1071 |
|  |  | 3reTX | 1.8929 | 1.8929 | 1.9643 | 2.1071 | 2.6071 | 2.7500 |
|  | M=4 (4OS) | Initial TX | 0.5000 | 0.5000 | 0.5000 | 0.8214 | 0.8214 | 0.8214 |
|  |  | 1 reTX | 1.0000 | 1.0000 | 1.2500 | 1.8214 | 1.8214 | 1.8214 |
|  |  | 2reTX | 1.5000 | 1.5000 | 2.0000 | 2.8214 | 2.8214 | 2.8214 |
|  |  | 3reTX | 2.0000 | 2.0000 | 2.7500 | 3.8214 | 3.8214 | 3.8214 |
|  | M=7 (7OS) | Initial TX | 0.5536 | 0.5536 | 0.5536 | 0.9286 | 0.9286 | 0.9286 |
|  |  | 1 reTX | 1.3036 | 1.3036 | 1.3036 | 1.9286 | 1.9286 | 1.9286 |
|  |  | 2reTX | 2.0536 | 2.0536 | 2.0536 | 2.9286 | 2.9286 | 2.9286 |
|  |  | 3reTX | 2.8036 | 2.8036 | 2.8036 | 3.9286 | 3.9286 | 3.9286 |
| # 0.5 |  |  | 2 | 2 | 2 | 0 | 0 | 0 |
| # 1 |  |  | 5 | 5 | 4 | 3 | 3 | 3 |
| # 2 |  |  | 10 | 10 | 9 | 7 | 7 | 6 |
| # 3 |  |  | 12 | 12 | 12 | 10 | 10 | 10 |

Table A8– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant-based transmission (msec)

| UL user plane latency (Grant-based) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.8929 | 0.8929 | 0.8929 | 1.1071 | 1.1071 | 1.1071 |
|  |  | 1 reTX | 1.3929 | 1.3929 | 1.3929 | 1.6071 | 1.6071 | 1.6071 |
|  |  | 2reTX | 1.8929 | 1.8929 | 1.8929 | 2.1071 | 2.1071 | 2.1071 |
|  |  | 3reTX | 2.3929 | 2.3929 | 2.3929 | 2.6071 | 2.6071 | 2.6071 |
|  | M=4 (4OS) | Initial TX | 0.9643 | 0.9643 | 1.0000 | 1.1786 | 1.2500 | 1.3214 |
|  |  | 1 reTX | 1.4643 | 1.4643 | 1.7500 | 1.7500 | 1.8214 | 2.3214 |
|  |  | 2reTX | 1.9643 | 1.9643 | 2.5000 | 2.6786 | 2.7500 | 3.3214 |
|  |  | 3reTX | 2.4643 | 2.4643 | 3.2500 | 3.2500 | 3.3214 | 4.3214 |
|  | M=7 (7OS) | Initial TX | 1.0179 | 1.0179 | 1.0536 | 1.4286 | 1.4286 | 1.4286 |
|  |  | 1 reTX | 1.7679 | 1.7679 | 1.8036 | 2.4286 | 2.4286 | 2.4286 |
|  |  | 2reTX | 2.5179 | 2.5179 | 2.5536 | 3.4286 | 3.4286 | 3.4286 |
|  |  | 3reTX | 3.2679 | 3.2679 | 3.3036 | 4.4286 | 4.4286 | 4.4286 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 2 | 2 | 2 | 0 | 0 | 0 |
| # 2 |  |  | 8 | 8 | 7 | 5 | 5 | 4 |
| # 3 |  |  | 11 | 11 | 10 | 9 | 9 | 8 |

Table A9– DL user plane worst-case latency for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) (msec)

| DL user plane latency – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3839 | 0.4018 | 0.4018 | 0.5893 | 0.6250 | 0.6250 |
|  |  | 1 reTX | 0.8839 | 0.9018 | 0.9732 | 1.1250 | 1.1964 | 1.2679 |
|  |  | 2reTX | 1.3839 | 1.4018 | 1.4732 | 1.6607 | 1.7679 | 2.1250 |
|  |  | 3reTX | 1.8839 | 1.9018 | 1.9732 | 2.1964 | 2.3393 | 2.7679 |
|  | M=4 (4OS) | Initial TX | 0.4554 | 0.4732 | 0.5089 | 0.7321 | 0.7679 | 0.8393 |
|  |  | 1 reTX | 1.0446 | 1.0804 | 1.2589 | 1.3393 | 1.4107 | 1.4821 |
|  |  | 2reTX | 1.5446 | 1.5804 | 2.0089 | 2.2321 | 2.2679 | 2.3393 |
|  |  | 3reTX | 2.0446 | 2.0804 | 2.7589 | 2.8393 | 2.9107 | 2.9821 |
|  | M=7 (7OS) | Initial TX | 0.5536 | 0.5536 | 0.5536 | 0.9286 | 0.9286 | 0.9286 |
|  |  | 1 reTX | 1.1429 | 1.1607 | 1.3036 | 1.6071 | 1.6429 | 1.9286 |
|  |  | 2reTX | 1.6429 | 1.6607 | 2.0536 | 2.4286 | 2.4286 | 2.9286 |
|  |  | 3reTX | 2.1429 | 2.1607 | 2.8036 | 3.1071 | 3.1429 | 3.9286 |
| # 0.5 |  |  | 2 | 2 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 3 | 3 | 3 |
| # 2 |  |  | 10 | 10 | 8 | 7 | 7 | 6 |
| # 3 |  |  | 12 | 12 | 12 | 11 | 11 | 11 |

## Achievability of latency budget with improved processing time in Rel-16 – FDD configuration

Table A10– DL user plane latency for NR FDD (msec), for Rel-16 beyond Capability 2 processing times – R16a

| DL user plane latency– NR FDD |  |  | R16a |  |  | R16a |  |  | R16a |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.2054 | 0.2054 | 0.2411 | 0.3036 | 0.3036 | 0.3750 | 0.5357 | 0.5357 | 0.6786 |
|  |  | 1 reTX | 0.5268 | 0.5268 | 0.5982 | 0.7321 | 0.7321 | 0.8750 | 1.3214 | 1.3929 | 1.6786 |
|  |  | 2reTX | 0.8482 | 0.8482 | 0.9196 | 1.1607 | 1.1607 | 1.3750 | 2.1071 | 2.2500 | 2.6786 |
|  |  | 3reTX | 1.1696 | 1.1696 | 1.2411 | 1.5893 | 1.5893 | 1.8750 | 2.8929 | 3.1071 | 3.6786 |
|  | M=4 (4OS) | Initial TX | 0.2768 | 0.2768 | 0.3125 | 0.4464 | 0.4464 | 0.5179 | 0.8214 | 0.8214 | 0.9643 |
|  |  | 1 reTX | 0.6339 | 0.6339 | 0.7054 | 0.9464 | 0.9464 | 1.0179 | 1.8214 | 1.8214 | 1.9643 |
|  |  | 2reTX | 1.0268 | 1.0268 | 1.0982 | 1.4464 | 1.4464 | 1.5179 | 2.8214 | 2.8214 | 2.9643 |
|  |  | 3reTX | 1.3839 | 1.3839 | 1.4554 | 1.9464 | 1.9464 | 2.0179 | 3.8214 | 3.8214 | 3.9643 |
|  | M=7 (7OS) | Initial TX | 0.3750 | 0.3929 | 0.4286 | 0.6429 | 0.6786 | 0.7500 | 1.2143 | 1.2857 | 1.4286 |
|  |  | 1 reTX | 0.8750 | 0.8929 | 0.9286 | 1.2143 | 1.2500 | 1.3929 | 2.2857 | 2.4286 | 2.7143 |
|  |  | 2reTX | 1.3750 | 1.3929 | 1.4286 | 1.7857 | 1.8214 | 2.2500 | 3.3571 | 3.5714 | 4.4286 |
|  |  | 3reTX | 1.8750 | 1.8929 | 1.9286 | 2.3571 | 2.3929 | 2.8929 | 4.4286 | 4.7143 | 5.7143 |
|  | M=14 (14OS) | Initial TX | 0.6250 | 0.6250 | 0.6250 | 1.1429 | 1.1429 | 1.1429 | 2.2143 | 2.2143 | 2.2143 |
|  |  | 1 reTX | 1.3750 | 1.3750 | 1.3750 | 2.1429 | 2.1429 | 2.1429 | 4.2143 | 4.2143 | 4.2143 |
|  |  | 2reTX | 2.1250 | 2.1250 | 2.1250 | 3.1429 | 3.1429 | 3.1429 | 6.2143 | 6.2143 | 6.2143 |
|  |  | 3reTX | 2.8750 | 2.8750 | 2.8750 | 4.1429 | 4.1429 | 4.1429 | 8.2143 | 8.2143 | 8.2143 |
| # 0.5 |  |  | 3 | 3 | 3 | 2 | 2 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 8 | 8 | 8 | 5 | 5 | 4 | 2 | 2 | 2 |
| # 2 |  |  | 14 | 14 | 14 | 12 | 12 | 10 | 5 | 5 | 5 |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 | 10 | 9 | 9 |

Table A11– DL user plane latency for NR FDD (msec), for Rel-16 beyond Capability 2 processing times – R16b

| DL user plane latency– NR FDD |  |  | R16b |  |  | R16b |  |  | R16b |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.1875 | 0.1875 | 0.2232 | 0.2679 | 0.2679 | 0.3393 | 0.5000 | 0.5000 | 0.6429 |
|  |  | 1 reTX | 0.4732 | 0.4732 | 0.5446 | 0.6250 | 0.6250 | 0.6964 | 1.2143 | 1.2143 | 1.3571 |
|  |  | 2reTX | 0.7589 | 0.7589 | 0.8661 | 0.9821 | 0.9821 | 1.1250 | 1.9286 | 1.9286 | 2.2143 |
|  |  | 3reTX | 1.0446 | 1.0446 | 1.1518 | 1.3393 | 1.3393 | 1.4821 | 2.6429 | 2.6429 | 2.9286 |
|  | M=4 (4OS) | Initial TX | 0.2589 | 0.2589 | 0.2946 | 0.4107 | 0.4107 | 0.4821 | 0.7857 | 0.7857 | 0.9286 |
|  |  | 1 reTX | 0.5804 | 0.5804 | 0.6161 | 0.9107 | 0.9107 | 0.9821 | 1.7857 | 1.7857 | 1.9286 |
|  |  | 2reTX | 0.9018 | 0.9018 | 0.9375 | 1.4107 | 1.4107 | 1.4821 | 2.7857 | 2.7857 | 2.9286 |
|  |  | 3reTX | 1.2589 | 1.2589 | 1.2946 | 1.9107 | 1.9107 | 1.9821 | 3.7857 | 3.7857 | 3.9286 |
|  | M=7 (7OS) | Initial TX | 0.3571 | 0.3750 | 0.4107 | 0.6071 | 0.6429 | 0.7143 | 1.1786 | 1.2500 | 1.3929 |
|  |  | 1 reTX | 0.7143 | 0.7321 | 0.9107 | 1.1071 | 1.1429 | 1.2143 | 2.1786 | 2.2500 | 2.3929 |
|  |  | 2reTX | 1.1071 | 1.1250 | 1.4107 | 1.6071 | 1.6429 | 1.7143 | 3.1786 | 3.2500 | 3.3929 |
|  |  | 3reTX | 1.4643 | 1.4821 | 1.9107 | 2.1071 | 2.1429 | 2.2143 | 4.1786 | 4.2500 | 4.3929 |
|  | M=14 (14OS) | Initial TX | 0.6071 | 0.6071 | 0.6071 | 1.1071 | 1.1071 | 1.1071 | 2.1786 | 2.1786 | 2.1786 |
|  |  | 1 reTX | 1.1071 | 1.1071 | 1.1071 | 2.1071 | 2.1071 | 2.1071 | 4.1786 | 4.1786 | 4.1786 |
|  |  | 2reTX | 1.6071 | 1.6071 | 1.6071 | 3.1071 | 3.1071 | 3.1071 | 6.1786 | 6.1786 | 6.1786 |
|  |  | 3reTX | 2.1071 | 2.1071 | 2.1071 | 4.1071 | 4.1071 | 4.1071 | 8.1786 | 8.1786 | 8.1786 |
| # 0.5 |  |  | 4 | 4 | 3 | 2 | 2 | 2 | 1 | 1 | 0 |
| # 1 |  |  | 9 | 9 | 9 | 6 | 6 | 5 | 2 | 2 | 2 |
| # 2 |  |  | 15 | 15 | 15 | 12 | 12 | 12 | 6 | 6 | 5 |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 | 10 | 10 | 10 |

Table A12– DL user plane latency for NR FDD (msec), for Rel-16 beyond Capability 2 processing times – R16c

| DL user plane latency– NR FDD |  |  | R16c |  |  | R16c |  |  | R16c |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.1696 | 0.1696 | 0.2054 | 0.2500 | 0.2500 | 0.3214 |  |  |  |
|  |  | 1 reTX | 0.4196 | 0.4196 | 0.4554 | 0.6071 | 0.6071 | 0.6786 |  |  |  |
|  |  | 2reTX | 0.6696 | 0.6696 | 0.7054 | 0.9643 | 0.9643 | 1.1071 |  |  |  |
|  |  | 3reTX | 0.9196 | 0.9196 | 0.9554 | 1.3214 | 1.3214 | 1.4643 |  |  |  |
|  | M=4 (4OS) | Initial TX | 0.2411 | 0.2411 | 0.2768 | 0.3929 | 0.3929 | 0.4643 |  |  |  |
|  |  | 1 reTX | 0.5268 | 0.5268 | 0.5982 | 0.8929 | 0.8929 | 0.9643 |  |  |  |
|  |  | 2reTX | 0.8125 | 0.8125 | 0.9196 | 1.3929 | 1.3929 | 1.4643 |  |  |  |
|  |  | 3reTX | 1.0982 | 1.0982 | 1.2768 | 1.8929 | 1.8929 | 1.9643 |  |  |  |
|  | M=7 (7OS) | Initial TX | 0.3393 | 0.3571 | 0.3929 | 0.5893 | 0.6250 | 0.6964 |  |  |  |
|  |  | 1 reTX | 0.6607 | 0.6786 | 0.7143 | 1.0893 | 1.1250 | 1.1964 |  |  |  |
|  |  | 2reTX | 1.0893 | 1.1071 | 1.1429 | 1.5893 | 1.6250 | 1.6964 |  |  |  |
|  |  | 3reTX | 1.4107 | 1.4286 | 1.4643 | 2.0893 | 2.1250 | 2.1964 |  |  |  |
|  | M=14 (14OS) | Initial TX | 0.5893 | 0.5893 | 0.5893 | 1.0893 | 1.0893 | 1.0893 |  |  |  |
|  |  | 1 reTX | 1.0893 | 1.0893 | 1.0893 | 2.0893 | 2.0893 | 2.0893 |  |  |  |
|  |  | 2reTX | 1.5893 | 1.5893 | 1.5893 | 3.0893 | 3.0893 | 3.0893 |  |  |  |
|  |  | 3reTX | 2.0893 | 2.0893 | 2.0893 | 4.0893 | 4.0893 | 4.0893 |  |  |  |
| # 0.5 |  |  | 4 | 4 | 4 | 2 | 2 | 2 |  |  |  |
| # 1 |  |  | 10 | 10 | 10 | 6 | 6 | 5 |  |  |  |
| # 2 |  |  | 15 | 15 | 15 | 12 | 12 | 12 |  |  |  |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 |  |  |  |

Table A13 – UL user plane latency (CG PUSCH) for NR FDD with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16a

| UL user plane latency (Grant free) – NR FDD |  |  | R16a |  |  | R16a |  |  | R16a |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.1964 | 0.1964 | 0.1964 | 0.2857 | 0.2857 | 0.2857 | 0.5000 | 0.5000 | 0.5000 |
|  |  | 1 reTX | 0.5179 | 0.5179 | 0.5536 | 0.7143 | 0.7143 | 0.7857 | 1.2143 | 1.3571 | 1.5000 |
|  |  | 2reTX | 0.8393 | 0.8393 | 0.9107 | 1.1429 | 1.1429 | 1.2857 | 1.9286 | 2.2143 | 2.5000 |
|  |  | 3reTX | 1.1607 | 1.1607 | 1.2321 | 1.5714 | 1.5714 | 1.7857 | 2.6429 | 3.0714 | 3.5000 |
|  | M=4 (4OS) | Initial TX | 0.3214 | 0.3214 | 0.3214 | 0.5357 | 0.5357 | 0.5357 | 1 | 1 | 1 |
|  |  | 1 reTX | 0.6964 | 0.6964 | 0.6964 | 1.0357 | 1.0357 | 1.1786 | 2 | 2 | 2 |
|  |  | 2reTX | 1.0714 | 1.0714 | 1.0714 | 1.5357 | 1.5357 | 1.6786 | 3 | 3 | 3 |
|  |  | 3reTX | 1.4464 | 1.4464 | 1.4464 | 2.0357 | 2.0357 | 2.1786 | 4 | 4 | 4 |
|  | M=7 (7OS) | Initial TX | 0.3750 | 0.3750 | 0.3750 | 0.6429 | 0.6429 | 0.6429 | 1.2143 | 1.2143 | 1.2143 |
|  |  | 1 reTX | 0.8750 | 0.8750 | 0.8750 | 1.3929 | 1.3929 | 1.3929 | 2.7143 | 2.7143 | 2.7143 |
|  |  | 2reTX | 1.3750 | 1.3750 | 1.3750 | 2.1429 | 2.1429 | 2.1429 | 4.2143 | 4.2143 | 4.2143 |
|  |  | 3reTX | 1.8750 | 1.8750 | 1.8750 | 2.8929 | 2.8929 | 2.8929 | 5.7143 | 5.7143 | 5.7143 |
|  | M=14 (14OS) | Initial TX | 0.6250 | 0.6250 | 0.6250 | 1.1429 | 1.1429 | 1.1429 | 2.2143 | 2.2143 | 2.2143 |
|  |  | 1 reTX | 1.3750 | 1.3750 | 1.3750 | 2.1429 | 2.1429 | 2.1429 | 4.2143 | 4.2143 | 4.2143 |
|  |  | 2reTX | 2.1250 | 2.1250 | 2.1250 | 3.1429 | 3.1429 | 3.1429 | 6.2143 | 6.2143 | 6.2143 |
|  |  | 3reTX | 2.8750 | 2.8750 | 2.8750 | 4.1429 | 4.1429 | 4.1429 | 8.2143 | 8.2143 | 8.2143 |
| # 0.5 |  |  | 3 | 3 | 3 | 1 | 1 | 1 | 1 | 1 | 1 |
| # 1 |  |  | 8 | 8 | 8 | 4 | 4 | 4 | 2 | 2 | 2 |
| # 2 |  |  | 14 | 14 | 14 | 10 | 10 | 10 | 6 | 5 | 5 |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 | 10 | 9 | 9 |

Table A14 – UL user plane latency (CG PUSCH) for NR FDD with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16b

| UL user plane latency (Grant free) – NR FDD |  |  | R16b |  |  | R16b |  |  | R16b |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.1786 | 0.1786 | 0.1786 | 0.2500 | 0.2500 | 0.2500 | 0.4643 | 0.4643 | 0.4643 |
|  |  | 1 reTX | 0.4643 | 0.4643 | 0.5000 | 0.6071 | 0.6071 | 0.6786 | 1.1786 | 1.1786 | 1.3214 |
|  |  | 2reTX | 0.7500 | 0.7500 | 0.8214 | 0.9643 | 0.9643 | 1.0357 | 1.8929 | 1.8929 | 2.0357 |
|  |  | 3reTX | 1.0357 | 1.0357 | 1.1429 | 1.3214 | 1.3214 | 1.4643 | 2.6071 | 2.6071 | 2.8929 |
|  | M=4 (4OS) | Initial TX | 0.3036 | 0.3036 | 0.3036 | 0.5000 | 0.5000 | 0.5000 | 0.9643 | 0.9643 | 0.9643 |
|  |  | 1 reTX | 0.6250 | 0.6250 | 0.6786 | 1.0000 | 1.0000 | 1.0000 | 1.9643 | 1.9643 | 1.9643 |
|  |  | 2reTX | 0.9286 | 0.9286 | 1.0536 | 1.5000 | 1.5000 | 1.5000 | 2.9643 | 2.9643 | 2.9643 |
|  |  | 3reTX | 1.3036 | 1.3036 | 1.4286 | 2.0000 | 2.0000 | 2.0000 | 3.9643 | 3.9643 | 3.9643 |
|  | M=7 (7OS) | Initial TX | 0.3571 | 0.3571 | 0.3571 | 0.6071 | 0.6071 | 0.6071 | 1.1786 | 1.1786 | 1.1786 |
|  |  | 1 reTX | 0.7321 | 0.7321 | 0.8571 | 1.1071 | 1.3571 | 1.3571 | 2.1786 | 2.6786 | 2.6786 |
|  |  | 2reTX | 1.1071 | 1.1071 | 1.3571 | 1.6071 | 1.8571 | 2.1071 | 3.1786 | 3.6786 | 4.1786 |
|  |  | 3reTX | 1.4821 | 1.4821 | 1.8571 | 2.1071 | 2.3571 | 2.8571 | 4.1786 | 4.6786 | 5.6786 |
|  | M=14 (14OS) | Initial TX | 0.6071 | 0.6071 | 0.6071 | 1.1071 | 1.1071 | 1.1071 | 2.1786 | 2.1786 | 2.1786 |
|  |  | 1 reTX | 1.1071 | 1.1071 | 1.3571 | 2.1071 | 2.1071 | 2.1071 | 4.1786 | 4.1786 | 4.1786 |
|  |  | 2reTX | 1.6071 | 1.6071 | 2.1071 | 3.1071 | 3.1071 | 3.1071 | 6.1786 | 6.1786 | 6.1786 |
|  |  | 3reTX | 2.1071 | 2.1071 | 2.8571 | 4.1071 | 4.1071 | 4.1071 | 8.1786 | 8.1786 | 8.1786 |
| # 0.5 |  |  | 4 | 4 | 4 | 2 | 2 | 2 | 1 | 1 | 1 |
| # 1 |  |  | 9 | 9 | 8 | 6 | 6 | 5 | 2 | 2 | 2 |
| # 2 |  |  | 15 | 15 | 14 | 12 | 12 | 11 | 6 | 6 | 5 |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 | 10 | 10 | 10 |

Table A15 – UL user plane latency (CG PUSCH) for NR FDD with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16c

| UL user plane latency (Grant free) – NR FDD |  |  | R16c |  |  | R16c |  |  | R16c |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.1607 | 0.1607 | 0.1607 | 0.2321 | 0.2321 | 0.2321 |  |  |  |
|  |  | 1 reTX | 0.4107 | 0.4107 | 0.4464 | 0.5893 | 0.5893 | 0.6607 |  |  |  |
|  |  | 2reTX | 0.6607 | 0.6607 | 0.6964 | 0.9464 | 0.9464 | 1.0179 |  |  |  |
|  |  | 3reTX | 0.9107 | 0.9107 | 0.9464 | 1.3036 | 1.3036 | 1.4464 |  |  |  |
|  | M=4 (4OS) | Initial TX | 0.2857 | 0.2857 | 0.2857 | 0.4821 | 0.4821 | 0.4821 |  |  |  |
|  |  | 1 reTX | 0.6071 | 0.6071 | 0.6607 | 0.9821 | 0.9821 | 0.9821 |  |  |  |
|  |  | 2reTX | 0.9107 | 0.9107 | 1.0357 | 1.4821 | 1.4821 | 1.4821 |  |  |  |
|  |  | 3reTX | 1.2857 | 1.2857 | 1.4107 | 1.9821 | 1.9821 | 1.9821 |  |  |  |
|  | M=7 (7OS) | Initial TX | 0.3393 | 0.3393 | 0.3393 | 0.5893 | 0.5893 | 0.5893 |  |  |  |
|  |  | 1 reTX | 0.7143 | 0.7143 | 0.7143 | 1.0893 | 1.3393 | 1.3393 |  |  |  |
|  |  | 2reTX | 1.0893 | 1.0893 | 1.0893 | 1.5893 | 1.8393 | 2.0893 |  |  |  |
|  |  | 3reTX | 1.4643 | 1.4643 | 1.4643 | 2.0893 | 2.3393 | 2.8393 |  |  |  |
|  | M=14 (14OS) | Initial TX | 0.5893 | 0.5893 | 0.5893 | 1.0893 | 1.0893 | 1.0893 |  |  |  |
|  |  | 1 reTX | 1.0893 | 1.0893 | 1.0893 | 2.0893 | 2.0893 | 2.0893 |  |  |  |
|  |  | 2reTX | 1.5893 | 1.5893 | 1.5893 | 3.0893 | 3.0893 | 3.0893 |  |  |  |
|  |  | 3reTX | 2.0893 | 2.0893 | 2.0893 | 4.0893 | 4.0893 | 4.0893 |  |  |  |
| # 0.5 |  |  | 4 | 4 | 4 | 2 | 2 | 2 |  |  |  |
| # 1 |  |  | 10 | 10 | 9 | 6 | 6 | 5 |  |  |  |
| # 2 |  |  | 15 | 15 | 15 | 12 | 12 | 11 |  |  |  |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 |  |  |  |

Table A16 – UL user plane latency (GB PUSCH) for NR FDD with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16a

| UL user plane latency (Grant based) – NR FDD |  |  | R16a |  |  | R16a |  |  | R16a |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.5000 | 0.5000 | 0.5357 | 0.6786 | 0.6786 | 0.7500 | 1.1429 | 1.2857 | 1.4286 |
|  |  | 1 reTX | 0.8036 | 0.8214 | 0.8929 | 1.0714 | 1.1071 | 1.2500 | 1.8571 | 2.0714 | 2.4286 |
|  |  | 2reTX | 1.1071 | 1.1429 | 1.2143 | 1.4643 | 1.5357 | 1.7500 | 2.5714 | 2.7857 | 3.4286 |
|  |  | 3reTX | 1.4107 | 1.4643 | 1.5357 | 1.8571 | 1.9643 | 2.2500 | 3.2857 | 3.5000 | 4.4286 |
|  | M=4 (4OS) | Initial TX | 0.5714 | 0.5714 | 0.6071 | 0.8214 | 0.8214 | 0.8214 | 1.4286 | 1.5714 | 1.5714 |
|  |  | 1 reTX | 0.9107 | 0.9286 | 0.9643 | 1.3214 | 1.3214 | 1.3214 | 2.4286 | 2.5714 | 2.5714 |
|  |  | 2reTX | 1.2500 | 1.3214 | 1.3571 | 1.8214 | 1.8214 | 1.8214 | 3.4286 | 3.5714 | 3.5714 |
|  |  | 3reTX | 1.5893 | 1.6786 | 1.7143 | 2.3214 | 2.3214 | 2.3214 | 4.4286 | 4.5714 | 4.5714 |
|  | M=7 (7OS) | Initial TX | 0.6607 | 0.6964 | 0.6964 | 1.0000 | 1.0000 | 1.0714 | 1.9286 | 1.9286 | 2.0714 |
|  |  | 1 reTX | 1.1607 | 1.1964 | 1.1964 | 1.5714 | 1.6071 | 1.6786 | 3.0000 | 3.0714 | 3.2857 |
|  |  | 2reTX | 1.6607 | 1.6964 | 1.6964 | 2.1429 | 2.1786 | 2.2857 | 4.0714 | 4.2143 | 4.5000 |
|  |  | 3reTX | 2.1607 | 2.1964 | 2.1964 | 2.7143 | 2.7500 | 3.0714 | 5.1429 | 5.3571 | 6.0714 |
|  | M=14 (14OS) | Initial TX | 0.9286 | 0.9286 | 0.9643 | 1.5357 | 1.5357 | 1.5357 | 2.8571 | 3 | 3 |
|  |  | 1 reTX | 1.6786 | 1.6786 | 1.7143 | 2.5357 | 2.5357 | 2.5357 | 4.8571 | 5 | 5 |
|  |  | 2reTX | 2.4286 | 2.4286 | 2.4643 | 3.5357 | 3.5357 | 3.5357 | 6.8571 | 7 | 7 |
|  |  | 3reTX | 3.1786 | 3.1786 | 3.2143 | 4.5357 | 4.5357 | 4.5357 | 8.8571 | 9 | 9 |
| # 0.5 |  |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 6 | 6 | 6 | 3 | 3 | 2 | 0 | 0 | 0 |
| # 2 |  |  | 13 | 13 | 13 | 10 | 10 | 9 | 4 | 3 | 2 |
| # 3 |  |  | 15 | 15 | 15 | 14 | 14 | 13 | 6 | 6 | 5 |

Table A17 – UL user plane latency (GB PUSCH) for NR FDD with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16b

| UL user plane latency (Grant based) – NR FDD |  |  | R16b |  |  | R16b |  |  | R16b |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4464 | 0.4464 | 0.4643 | 0.5714 | 0.5714 | 0.6429 | 1.1071 | 1.1071 | 1.2500 |
|  |  | 1 reTX | 0.7143 | 0.7321 | 0.7857 | 0.8929 | 0.9286 | 1.0000 | 1.7500 | 1.8214 | 1.9643 |
|  |  | 2reTX | 0.9821 | 1.0179 | 1.1071 | 1.2143 | 1.2857 | 1.4286 | 2.3929 | 2.5357 | 2.8214 |
|  |  | 3reTX | 1.2500 | 1.3036 | 1.3929 | 1.5714 | 1.6429 | 1.7857 | 3.1071 | 3.2500 | 3.5357 |
|  | M=4 (4OS) | Initial TX | 0.5179 | 0.5179 | 0.5536 | 0.7143 | 0.7143 | 0.7857 | 1.3929 | 1.3929 | 1.5357 |
|  |  | 1 reTX | 0.8214 | 0.8393 | 0.9107 | 1.2143 | 1.2143 | 1.2857 | 2.3929 | 2.3929 | 2.5357 |
|  |  | 2reTX | 1.1250 | 1.1607 | 1.3036 | 1.7143 | 1.7143 | 1.7857 | 3.3929 | 3.3929 | 3.5357 |
|  |  | 3reTX | 1.4286 | 1.5179 | 1.6607 | 2.2143 | 2.2143 | 2.2857 | 4.3929 | 4.3929 | 4.5357 |
|  | M=7 (7OS) | Initial TX | 0.6071 | 0.6071 | 0.6071 | 0.8929 | 0.9643 | 1.0357 | 1.7500 | 1.8929 | 2.0357 |
|  |  | 1 reTX | 0.9643 | 0.9821 | 0.9821 | 1.3929 | 1.4643 | 1.6071 | 2.7500 | 2.8929 | 3.1786 |
|  |  | 2reTX | 1.3571 | 1.3571 | 1.3750 | 1.8929 | 1.9643 | 2.1071 | 3.7500 | 3.8929 | 4.1786 |
|  |  | 3reTX | 1.7143 | 1.7321 | 1.7321 | 2.3929 | 2.4643 | 2.6071 | 4.7500 | 4.8929 | 5.1786 |
|  | M=14 (14OS) | Initial TX | 0.8750 | 0.8750 | 0.9107 | 1.4286 | 1.4286 | 1.5000 | 2.8214 | 2.8214 | 2.9643 |
|  |  | 1 reTX | 1.3750 | 1.3750 | 1.6607 | 2.4286 | 2.4286 | 2.5000 | 4.8214 | 4.8214 | 4.9643 |
|  |  | 2reTX | 1.8750 | 1.8750 | 2.4107 | 3.4286 | 3.4286 | 3.5000 | 6.8214 | 6.8214 | 6.9643 |
|  |  | 3reTX | 2.3750 | 2.3750 | 3.1607 | 4.4286 | 4.4286 | 4.5000 | 8.8214 | 8.8214 | 8.9643 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 8 | 7 | 7 | 4 | 4 | 3 | 0 | 0 | 0 |
| # 2 |  |  | 15 | 15 | 14 | 11 | 11 | 10 | 4 | 4 | 3 |
| # 3 |  |  | 16 | 16 | 15 | 14 | 14 | 14 | 8 | 8 | 7 |

Table A18 – UL user plane latency (GB PUSCH) for NR FDD with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16c

| UL user plane latency (Grant based) – NR FDD |  |  | R16c |  |  | R16c |  |  | R16c |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  | SCS 15kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3929 | 0.3929 | 0.4286 | 0.5536 | 0.5536 | 0.6250 |  |  |  |
|  |  | 1 reTX | 0.6429 | 0.6429 | 0.6786 | 0.8750 | 0.9107 | 0.9821 |  |  |  |
|  |  | 2reTX | 0.8929 | 0.8929 | 0.9286 | 1.1964 | 1.2679 | 1.4107 |  |  |  |
|  |  | 3reTX | 1.1429 | 1.1429 | 1.1786 | 1.5536 | 1.6250 | 1.7679 |  |  |  |
|  | M=4 (4OS) | Initial TX | 0.4643 | 0.4643 | 0.4643 | 0.6964 | 0.6964 | 0.7679 |  |  |  |
|  |  | 1 reTX | 0.7321 | 0.7500 | 0.7857 | 1.1964 | 1.1964 | 1.2679 |  |  |  |
|  |  | 2reTX | 1.0000 | 1.0357 | 1.1071 | 1.6964 | 1.6964 | 1.7679 |  |  |  |
|  |  | 3reTX | 1.2679 | 1.3214 | 1.3929 | 2.1964 | 2.1964 | 2.2679 |  |  |  |
|  | M=7 (7OS) | Initial TX | 0.5536 | 0.5893 | 0.5893 | 0.8750 | 0.9464 | 1.0179 |  |  |  |
|  |  | 1 reTX | 0.8750 | 0.9107 | 0.9464 | 1.3750 | 1.4464 | 1.5893 |  |  |  |
|  |  | 2reTX | 1.3036 | 1.3393 | 1.3393 | 1.8750 | 1.9464 | 2.0893 |  |  |  |
|  |  | 3reTX | 1.6250 | 1.6607 | 1.6964 | 2.3750 | 2.4464 | 2.5893 |  |  |  |
|  | M=14 (14OS) | Initial TX | 0.8214 | 0.8214 | 0.8214 | 1.4107 | 1.4107 | 1.4821 |  |  |  |
|  |  | 1 reTX | 1.3214 | 1.3214 | 1.3214 | 2.4107 | 2.4107 | 2.4821 |  |  |  |
|  |  | 2reTX | 1.8214 | 1.8214 | 1.8214 | 3.4107 | 3.4107 | 3.4821 |  |  |  |
|  |  | 3reTX | 2.3214 | 2.3214 | 2.3214 | 4.4107 | 4.4107 | 4.4821 |  |  |  |
| # 0.5 |  |  | 2 | 2 | 2 | 0 | 0 | 0 |  |  |  |
| # 1 |  |  | 9 | 8 | 8 | 4 | 4 | 3 |  |  |  |
| # 2 |  |  | 15 | 15 | 15 | 11 | 11 | 10 |  |  |  |
| # 3 |  |  | 16 | 16 | 16 | 14 | 14 | 14 |  |  |  |

## Achievability of latency budget with improved processing time in Rel-16 - TDD configuration

Table A19– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16a

| UL user plane latency (Grant free) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4464 | 0.4464 | 0.4464 | 0.7857 | 0.7857 | 0.7857 |
|  |  | 1 reTX | 0.9464 | 0.9464 | 0.9464 | 1.7857 | 1.7857 | 1.7857 |
|  |  | 2reTX | 1.4464 | 1.4464 | 1.4464 | 2.7857 | 2.7857 | 2.7857 |
|  |  | 3reTX | 1.9464 | 1.9464 | 1.9464 | 3.7857 | 3.7857 | 3.7857 |
|  | M=4 (4OS) | Initial TX | 0.5714 | 0.5714 | 0.5714 | 1.0357 | 1.0357 | 1.0357 |
|  |  | 1 reTX | 1.0714 | 1.0714 | 1.0714 | 2.0357 | 2.0357 | 2.0357 |
|  |  | 2reTX | 1.5714 | 1.5714 | 1.5714 | 3.0357 | 3.0357 | 3.0357 |
|  |  | 3reTX | 2.0714 | 2.0714 | 2.0714 | 4.0357 | 4.0357 | 4.0357 |
|  | M=7 (7OS) | Initial TX | 0.6250 | 0.6250 | 0.6250 | 1.1429 | 1.1429 | 1.1429 |
|  |  | 1 reTX | 1.1250 | 1.1250 | 1.1250 | 2.1429 | 2.1429 | 2.1429 |
|  |  | 2reTX | 1.6250 | 1.6250 | 1.6250 | 3.1429 | 3.1429 | 3.1429 |
|  |  | 3reTX | 2.1250 | 2.1250 | 2.1250 | 4.1429 | 4.1429 | 4.1429 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 1 | 1 | 1 |
| # 2 |  |  | 10 | 10 | 10 | 4 | 4 | 4 |
| # 3 |  |  | 12 | 12 | 12 | 7 | 7 | 7 |

Table A20– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16b

| UL user plane latency (Grant free) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4286 | 0.4286 | 0.4286 | 0.7500 | 0.7500 | 0.7500 |
|  |  | 1 reTX | 0.9286 | 0.9286 | 0.9286 | 1.7500 | 1.7500 | 1.7500 |
|  |  | 2reTX | 1.4286 | 1.4286 | 1.4286 | 2.7500 | 2.7500 | 2.7500 |
|  |  | 3reTX | 1.9286 | 1.9286 | 1.9286 | 3.7500 | 3.7500 | 3.7500 |
|  | M=4 (4OS) | Initial TX | 0.5536 | 0.5536 | 0.5536 | 1 | 1 | 1 |
|  |  | 1 reTX | 1.0536 | 1.0536 | 1.0536 | 2 | 2 | 2 |
|  |  | 2reTX | 1.5536 | 1.5536 | 1.5536 | 3 | 3 | 3 |
|  |  | 3reTX | 2.0536 | 2.0536 | 2.0536 | 4 | 4 | 4 |
|  | M=7 (7OS) | Initial TX | 0.6071 | 0.6071 | 0.6071 | 1.1071 | 1.1071 | 1.1071 |
|  |  | 1 reTX | 1.1071 | 1.1071 | 1.1071 | 2.1071 | 2.1071 | 2.1071 |
|  |  | 2reTX | 1.6071 | 1.6071 | 1.6071 | 3.1071 | 3.1071 | 3.1071 |
|  |  | 3reTX | 2.1071 | 2.1071 | 2.1071 | 4.1071 | 4.1071 | 4.1071 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 2 | 2 | 2 |
| # 2 |  |  | 10 | 10 | 10 | 5 | 5 | 5 |
| # 3 |  |  | 12 | 12 | 12 | 8 | 8 | 8 |

Table A21– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16c

| UL user plane latency (Grant free) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4107 | 0.4107 | 0.4107 | 0.7321 | 0.7321 | 0.7321 |
|  |  | 1 reTX | 0.9107 | 0.9107 | 0.9107 | 1.7321 | 1.7321 | 1.7321 |
|  |  | 2reTX | 1.4107 | 1.4107 | 1.4107 | 2.7321 | 2.7321 | 2.7321 |
|  |  | 3reTX | 1.9107 | 1.9107 | 1.9107 | 3.7321 | 3.7321 | 3.7321 |
|  | M=4 (4OS) | Initial TX | 0.5357 | 0.5357 | 0.5357 | 0.9821 | 0.9821 | 0.9821 |
|  |  | 1 reTX | 1.0357 | 1.0357 | 1.0357 | 1.9821 | 1.9821 | 1.9821 |
|  |  | 2reTX | 1.5357 | 1.5357 | 1.5357 | 2.9821 | 2.9821 | 2.9821 |
|  |  | 3reTX | 2.0357 | 2.0357 | 2.0357 | 3.9821 | 3.9821 | 3.9821 |
|  | M=7 (7OS) | Initial TX | 0.5893 | 0.5893 | 0.5893 | 1.0893 | 1.0893 | 1.0893 |
|  |  | 1 reTX | 1.0893 | 1.0893 | 1.0893 | 2.0893 | 2.0893 | 2.0893 |
|  |  | 2reTX | 1.5893 | 1.5893 | 1.5893 | 3.0893 | 3.0893 | 3.0893 |
|  |  | 3reTX | 2.0893 | 2.0893 | 2.0893 | 4.0893 | 4.0893 | 4.0893 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 2 | 2 | 2 |
| # 2 |  |  | 10 | 10 | 10 | 5 | 5 | 5 |
| # 3 |  |  | 12 | 12 | 12 | 8 | 8 | 8 |

Table A22– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16a

| UL user plane latency (Grant-based) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.9286 | 0.9286 | 0.9286 | 1.7500 | 1.7500 | 1.7500 |
|  |  | 1 reTX | 1.4286 | 1.4286 | 1.4286 | 2.7500 | 2.7500 | 2.7500 |
|  |  | 2reTX | 1.9286 | 1.9286 | 1.9286 | 3.7500 | 3.7500 | 3.7500 |
|  |  | 3reTX | 2.4286 | 2.4286 | 2.4286 | 4.7500 | 4.7500 | 4.7500 |
|  | M=4 (4OS) | Initial TX | 0.9643 | 0.9643 | 0.9643 | 1.8214 | 1.8214 | 1.8214 |
|  |  | 1 reTX | 1.4643 | 1.4643 | 1.4643 | 2.8214 | 2.8214 | 2.8214 |
|  |  | 2reTX | 1.9643 | 1.9643 | 1.9643 | 3.8214 | 3.8214 | 3.8214 |
|  |  | 3reTX | 2.4643 | 2.4643 | 2.4643 | 4.8214 | 4.8214 | 4.8214 |
|  | M=7 (7OS) | Initial TX | 1.0179 | 1.0179 | 1.0179 | 1.9286 | 1.9286 | 1.9286 |
|  |  | 1 reTX | 1.5179 | 1.5179 | 1.5179 | 2.9286 | 2.9286 | 2.9286 |
|  |  | 2reTX | 2.0179 | 2.0179 | 2.0179 | 3.9286 | 3.9286 | 3.9286 |
|  |  | 3reTX | 2.5179 | 2.5179 | 2.5179 | 4.9286 | 4.9286 | 4.9286 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 2 | 2 | 2 | 0 | 0 | 0 |
| # 2 |  |  | 8 | 8 | 8 | 3 | 3 | 3 |
| # 3 |  |  | 12 | 12 | 12 | 6 | 6 | 6 |

Table A23– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16b

| UL user plane latency (Grant-based) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.9107 | 0.9107 | 0.9107 | 1.7143 | 1.7143 | 1.7143 |
|  |  | 1 reTX | 1.4107 | 1.4107 | 1.4107 | 2.7143 | 2.7143 | 2.7143 |
|  |  | 2reTX | 1.9107 | 1.9107 | 1.9107 | 3.7143 | 3.7143 | 3.7143 |
|  |  | 3reTX | 2.4107 | 2.4107 | 2.4107 | 4.7143 | 4.7143 | 4.7143 |
|  | M=4 (4OS) | Initial TX | 0.9464 | 0.9464 | 0.9464 | 1.7857 | 1.7857 | 1.7857 |
|  |  | 1 reTX | 1.4464 | 1.4464 | 1.4464 | 2.7857 | 2.7857 | 2.7857 |
|  |  | 2reTX | 1.9464 | 1.9464 | 1.9464 | 3.7857 | 3.7857 | 3.7857 |
|  |  | 3reTX | 2.4464 | 2.4464 | 2.4464 | 4.7857 | 4.7857 | 4.7857 |
|  | M=7 (7OS) | Initial TX | 1.0000 | 1.0000 | 1.0000 | 1.8929 | 1.8929 | 1.8929 |
|  |  | 1 reTX | 1.5000 | 1.5000 | 1.5000 | 2.8929 | 2.8929 | 2.8929 |
|  |  | 2reTX | 2.0000 | 2.0000 | 2.0000 | 3.8929 | 3.8929 | 3.8929 |
|  |  | 3reTX | 2.5000 | 2.5000 | 2.5000 | 4.8929 | 4.8929 | 4.8929 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 2 |  |  | 9 | 9 | 9 | 3 | 3 | 3 |
| # 3 |  |  | 12 | 12 | 12 | 6 | 6 | 6 |

Table A24– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16c

| UL user plane latency (Grant-based) – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.8929 | 0.8929 | 0.8929 | 1.6964 | 1.6964 | 1.6964 |
|  |  | 1 reTX | 1.3929 | 1.3929 | 1.3929 | 2.6964 | 2.6964 | 2.6964 |
|  |  | 2reTX | 1.8929 | 1.8929 | 1.8929 | 3.6964 | 3.6964 | 3.6964 |
|  |  | 3reTX | 2.3929 | 2.3929 | 2.3929 | 4.6964 | 4.6964 | 4.6964 |
|  | M=4 (4OS) | Initial TX | 0.9286 | 0.9286 | 0.9286 | 1.7679 | 1.7679 | 1.7679 |
|  |  | 1 reTX | 1.4286 | 1.4286 | 1.4286 | 2.7679 | 2.7679 | 2.7679 |
|  |  | 2reTX | 1.9286 | 1.9286 | 1.9286 | 3.7679 | 3.7679 | 3.7679 |
|  |  | 3reTX | 2.4286 | 2.4286 | 2.4286 | 4.7679 | 4.7679 | 4.7679 |
|  | M=7 (7OS) | Initial TX | 0.9821 | 0.9821 | 0.9821 | 1.8750 | 1.8750 | 1.8750 |
|  |  | 1 reTX | 1.4821 | 1.4821 | 1.4821 | 2.8750 | 2.8750 | 2.8750 |
|  |  | 2reTX | 1.9821 | 1.9821 | 1.9821 | 3.8750 | 3.8750 | 3.8750 |
|  |  | 3reTX | 2.4821 | 2.4821 | 2.4821 | 4.8750 | 4.8750 | 4.8750 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 2 |  |  | 9 | 9 | 9 | 3 | 3 | 3 |
| # 3 |  |  | 12 | 12 | 12 | 6 | 6 | 6 |

Table A25– DL user plane worst-case latency for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) (msec), for Rel-16 beyond Capability 2 processing times – R16a

| DL user plane latency – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4554 | 0.4554 | 0.4554 | 0.8036 | 0.8036 | 0.8036 |
|  |  | 1 reTX | 0.9554 | 0.9554 | 0.9554 | 1.8036 | 1.8036 | 1.8036 |
|  |  | 2reTX | 1.4554 | 1.4554 | 1.4554 | 2.8036 | 2.8036 | 2.8036 |
|  |  | 3reTX | 1.9554 | 1.9554 | 1.9554 | 3.8036 | 3.8036 | 3.8036 |
|  | M=4 (4OS) | Initial TX | 0.5268 | 0.5268 | 0.5625 | 0.9464 | 0.9464 | 1.0179 |
|  |  | 1 reTX | 1.0268 | 1.0268 | 1.0625 | 1.9464 | 1.9464 | 2.0179 |
|  |  | 2reTX | 1.5268 | 1.5268 | 1.5625 | 2.9464 | 2.9464 | 3.0179 |
|  |  | 3reTX | 2.0268 | 2.0268 | 2.0625 | 3.9464 | 3.9464 | 4.0179 |
|  | M=7 (7OS) | Initial TX | 0.6250 | 0.6429 | 0.6786 | 1.1429 | 1.1786 | 1.2500 |
|  |  | 1 reTX | 1.1250 | 1.1429 | 1.1786 | 2.1429 | 2.1786 | 2.2500 |
|  |  | 2reTX | 1.6250 | 1.6429 | 1.6786 | 3.1429 | 3.1786 | 3.2500 |
|  |  | 3reTX | 2.1250 | 2.1429 | 2.1786 | 4.1429 | 4.1786 | 4.2500 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 2 | 2 | 1 |
| # 2 |  |  | 10 | 10 | 10 | 5 | 5 | 4 |
| # 3 |  |  | 12 | 12 | 12 | 8 | 8 | 7 |

Table A26– DL user plane worst-case latency for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) (msec), for Rel-16 beyond Capability 2 processing times – R16b

| DL user plane latency – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4375 | 0.4375 | 0.4375 | 0.7679 | 0.7679 | 0.7679 |
|  |  | 1 reTX | 0.9375 | 0.9375 | 0.9375 | 1.7679 | 1.7679 | 1.7679 |
|  |  | 2reTX | 1.4375 | 1.4375 | 1.4375 | 2.7679 | 2.7679 | 2.7679 |
|  |  | 3reTX | 1.9375 | 1.9375 | 1.9375 | 3.7679 | 3.7679 | 3.7679 |
|  | M=4 (4OS) | Initial TX | 0.5089 | 0.5089 | 0.5446 | 0.9107 | 0.9107 | 0.9821 |
|  |  | 1 reTX | 1.0089 | 1.0089 | 1.0446 | 1.9107 | 1.9107 | 1.9821 |
|  |  | 2reTX | 1.5089 | 1.5089 | 1.5446 | 2.9107 | 2.9107 | 2.9821 |
|  |  | 3reTX | 2.0089 | 2.0089 | 2.0446 | 3.9107 | 3.9107 | 3.9821 |
|  | M=7 (7OS) | Initial TX | 0.6071 | 0.6250 | 0.6607 | 1.1071 | 1.1429 | 1.2143 |
|  |  | 1 reTX | 1.1071 | 1.1250 | 1.1607 | 2.1071 | 2.1429 | 2.2143 |
|  |  | 2reTX | 1.6071 | 1.6250 | 1.6607 | 3.1071 | 3.1429 | 3.2143 |
|  |  | 3reTX | 2.1071 | 2.1250 | 2.1607 | 4.1071 | 4.1429 | 4.2143 |
| # 0.5 |  |  | 1 | 1 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 4 | 4 | 2 | 2 | 2 |
| # 2 |  |  | 10 | 10 | 10 | 5 | 5 | 5 |
| # 3 |  |  | 12 | 12 | 12 | 8 | 8 | 8 |

Table A27– DL user plane worst-case latency for NR TDD (mode 1 - slots: …/UL/DL/UL/DL/…) (msec), for Rel-16 beyond Capability 2 processing times – R16c

| DL user plane latency – NR TDD mode 1 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.4196 | 0.4196 | 0.4196 | 0.7500 | 0.7500 | 0.7500 |
|  |  | 1 reTX | 0.9196 | 0.9196 | 0.9196 | 1.7500 | 1.7500 | 1.7500 |
|  |  | 2reTX | 1.4196 | 1.4196 | 1.4196 | 2.7500 | 2.7500 | 2.7500 |
|  |  | 3reTX | 1.9196 | 1.9196 | 1.9196 | 3.7500 | 3.7500 | 3.7500 |
|  | M=4 (4OS) | Initial TX | 0.4911 | 0.4911 | 0.5268 | 0.8929 | 0.8929 | 0.9643 |
|  |  | 1 reTX | 0.9911 | 0.9911 | 1.0268 | 1.8929 | 1.8929 | 1.9643 |
|  |  | 2reTX | 1.4911 | 1.4911 | 1.5268 | 2.8929 | 2.8929 | 2.9643 |
|  |  | 3reTX | 1.9911 | 1.9911 | 2.0268 | 3.8929 | 3.8929 | 3.9643 |
|  | M=7 (7OS) | Initial TX | 0.5893 | 0.6071 | 0.6429 | 1.0893 | 1.1250 | 1.1964 |
|  |  | 1 reTX | 1.0893 | 1.1071 | 1.1429 | 2.0893 | 2.1250 | 2.1964 |
|  |  | 2reTX | 1.5893 | 1.6071 | 1.6429 | 3.0893 | 3.1250 | 3.1964 |
|  |  | 3reTX | 2.0893 | 2.1071 | 2.1429 | 4.0893 | 4.1250 | 4.1964 |
| # 0.5 |  |  | 2 | 2 | 1 | 0 | 0 | 0 |
| # 1 |  |  | 5 | 5 | 4 | 2 | 2 | 2 |
| # 2 |  |  | 11 | 11 | 10 | 5 | 5 | 5 |
| # 3 |  |  | 12 | 12 | 12 | 8 | 8 | 8 |

Table A28– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16a

| UL user plane latency (Grant free) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3393 | 0.3393 | 0.3393 | 0.5714 | 0.5714 | 0.5714 |
|  |  | 1 reTX | 0.6607 | 0.6607 | 0.6607 | 1.0714 | 1.0714 | 1.0714 |
|  |  | 2reTX | 1.0893 | 1.0893 | 1.0893 | 1.5714 | 1.5714 | 1.5714 |
|  |  | 3reTX | 1.4107 | 1.4107 | 1.4107 | 2.0714 | 2.0714 | 2.0714 |
|  | M=4 (4OS) | Initial TX | 0.4464 | 0.4464 | 0.4464 | 0.7857 | 0.7857 | 0.7857 |
|  |  | 1 reTX | 0.9464 | 0.9464 | 0.9464 | 1.2857 | 1.2857 | 1.7857 |
|  |  | 2reTX | 1.4464 | 1.4464 | 1.4464 | 1.7857 | 1.7857 | 2.7857 |
|  |  | 3reTX | 1.9464 | 1.9464 | 1.9464 | 2.2857 | 2.2857 | 3.7857 |
|  | M=7 (7OS) | Initial TX | 0.5000 | 0.5000 | 0.5000 | 0.8929 | 0.8929 | 0.8929 |
|  |  | 1 reTX | 1.0000 | 1.2500 | 1.2500 | 1.8929 | 1.8929 | 1.8929 |
|  |  | 2reTX | 1.5000 | 2.0000 | 2.0000 | 2.8929 | 2.8929 | 2.8929 |
|  |  | 3reTX | 2.0000 | 2.7500 | 2.7500 | 3.8929 | 3.8929 | 3.8929 |
| # 0.5 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 1 |  |  | 6 | 5 | 5 | 3 | 3 | 3 |
| # 2 |  |  | 12 | 11 | 11 | 8 | 8 | 8 |
| # 3 |  |  | 12 | 12 | 12 | 11 | 11 | 11 |

Table A29– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16b

| UL user plane latency (Grant free) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B0 | M=2 (2OS) | Initial TX | 0.3214 | 0.3214 | 0.3214 | 0.5357 | 0.5357 | 0.5357 |
|  |  | 1 reTX | 0.6071 | 0.6071 | 0.6429 | 1.0357 | 1.0357 | 1.0357 |
|  |  | 2reTX | 0.8929 | 0.8929 | 1.0714 | 1.5357 | 1.5357 | 1.5357 |
|  |  | 3reTX | 1.3214 | 1.3214 | 1.3929 | 2.0357 | 2.0357 | 2.0357 |
|  | M=4 (4OS) | Initial TX | 0.4286 | 0.4286 | 0.4286 | 0.7500 | 0.7500 | 0.7500 |
|  |  | 1 reTX | 0.9286 | 0.9286 | 0.9286 | 1.2500 | 1.2500 | 1.2500 |
|  |  | 2reTX | 1.4286 | 1.4286 | 1.4286 | 1.7500 | 1.7500 | 1.7500 |
|  |  | 3reTX | 1.9286 | 1.9286 | 1.9286 | 2.2500 | 2.2500 | 2.2500 |
|  | M=7 (7OS) | Initial TX | 0.4821 | 0.4821 | 0.4821 | 0.8571 | 0.8571 | 0.8571 |
|  |  | 1 reTX | 0.9821 | 0.9821 | 0.9821 | 1.3571 | 1.8571 | 1.8571 |
|  |  | 2reTX | 1.4821 | 1.4821 | 1.4821 | 1.8571 | 2.8571 | 2.8571 |
|  |  | 3reTX | 1.9821 | 1.9821 | 1.9821 | 2.3571 | 3.8571 | 3.8571 |
| # 0.5 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 1 |  |  | 7 | 7 | 6 | 3 | 3 | 3 |
| # 2 |  |  | 12 | 12 | 12 | 9 | 9 | 9 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 11 | 11 |

Table A30– UL user plane worst-case latency (CG PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant free transmission (msec), for Rel-16 beyond Capability 2 processing times – R16c

| UL user plane latency (Grant free) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3036 | 0.3036 | 0.3036 | 0.5179 | 0.5179 | 0.5179 |
|  |  | 1 reTX | 0.5536 | 0.5536 | 0.5893 | 1.0179 | 1.0179 | 1.0179 |
|  |  | 2reTX | 0.8036 | 0.8036 | 0.8750 | 1.5179 | 1.5179 | 1.5179 |
|  |  | 3reTX | 1.0536 | 1.0536 | 1.1250 | 2.0179 | 2.0179 | 2.0179 |
|  | M=4 (4OS) | Initial TX | 0.4107 | 0.4107 | 0.4107 | 0.7321 | 0.7321 | 0.7321 |
|  |  | 1 reTX | 0.9107 | 0.9107 | 0.9107 | 1.2321 | 1.2321 | 1.2321 |
|  |  | 2reTX | 1.4107 | 1.4107 | 1.4107 | 1.7321 | 1.7321 | 1.7321 |
|  |  | 3reTX | 1.9107 | 1.9107 | 1.9107 | 2.2321 | 2.2321 | 2.2321 |
|  | M=7 (7OS) | Initial TX | 0.4643 | 0.4643 | 0.4643 | 0.8393 | 0.8393 | 0.8393 |
|  |  | 1 reTX | 0.9643 | 0.9643 | 0.9643 | 1.3393 | 1.8393 | 1.8393 |
|  |  | 2reTX | 1.4643 | 1.4643 | 1.4643 | 1.8393 | 2.8393 | 2.8393 |
|  |  | 3reTX | 1.9643 | 1.9643 | 1.9643 | 2.3393 | 3.8393 | 3.8393 |
| # 0.5 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 1 |  |  | 7 | 7 | 7 | 3 | 3 | 3 |
| # 2 |  |  | 12 | 12 | 12 | 9 | 8 | 8 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 11 | 11 |

Table A31– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16a

| UL user plane latency (Grant-based) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.6250 | 0.6429 | 0.6786 | 1.0714 | 1.0714 | 1.0714 |
|  |  | 1 reTX | 0.9286 | 1.0893 | 1.1071 | 1.5714 | 1.5714 | 1.5714 |
|  |  | 2reTX | 1.3750 | 1.3929 | 1.4286 | 2.0714 | 2.0714 | 2.0714 |
|  |  | 3reTX | 1.6786 | 1.8393 | 1.8571 | 2.5714 | 2.5714 | 2.5714 |
|  | M=4 (4OS) | Initial TX | 0.6964 | 0.6964 | 0.8750 | 1.1429 | 1.1429 | 1.1429 |
|  |  | 1 reTX | 1.1964 | 1.1964 | 1.3750 | 1.6429 | 1.6429 | 1.7143 |
|  |  | 2reTX | 1.6964 | 1.6964 | 1.8750 | 2.1429 | 2.1429 | 2.2143 |
|  |  | 3reTX | 2.1964 | 2.1964 | 2.3750 | 2.6429 | 2.6429 | 2.7143 |
|  | M=7 (7OS) | Initial TX | 0.9286 | 0.9286 | 0.9643 | 1.2500 | 1.2500 | 1.3214 |
|  |  | 1 reTX | 1.4286 | 1.6786 | 1.7143 | 2.2500 | 2.2500 | 2.3214 |
|  |  | 2reTX | 1.9286 | 2.4286 | 2.4643 | 3.2500 | 3.2500 | 3.3214 |
|  |  | 3reTX | 2.4286 | 3.1786 | 3.2143 | 4.2500 | 4.2500 | 4.3214 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 4 | 3 | 3 | 0 | 0 | 0 |
| # 2 |  |  | 10 | 9 | 9 | 5 | 5 | 5 |
| # 3 |  |  | 11 | 11 | 11 | 10 | 10 | 10 |

Table A32– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16b

| UL user plane latency (Grant-based) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.5714 | 0.5714 | 0.5714 | 1.0357 | 1.0357 | 1.0357 |
|  |  | 1 reTX | 0.8393 | 0.8571 | 0.8929 | 1.5357 | 1.5357 | 1.5357 |
|  |  | 2reTX | 1.1071 | 1.1429 | 1.3214 | 2.0357 | 2.0357 | 2.0357 |
|  |  | 3reTX | 1.3750 | 1.5714 | 1.6429 | 2.5357 | 2.5357 | 2.5357 |
|  | M=4 (4OS) | Initial TX | 0.6429 | 0.6429 | 0.6786 | 1.1071 | 1.1071 | 1.1071 |
|  |  | 1 reTX | 0.9464 | 1.1429 | 1.1786 | 1.6071 | 1.6071 | 1.6071 |
|  |  | 2reTX | 1.3929 | 1.6429 | 1.6786 | 2.1071 | 2.1071 | 2.1071 |
|  |  | 3reTX | 1.6964 | 2.1429 | 2.1786 | 2.6071 | 2.6071 | 2.6071 |
|  | M=7 (7OS) | Initial TX | 0.7321 | 0.7321 | 0.7321 | 1.2143 | 1.2143 | 1.2857 |
|  |  | 1 reTX | 1.2321 | 1.2321 | 1.2321 | 1.7143 | 2.2143 | 2.2857 |
|  |  | 2reTX | 1.7321 | 1.7321 | 1.7321 | 2.2143 | 3.2143 | 3.2857 |
|  |  | 3reTX | 2.2321 | 2.2321 | 2.2321 | 2.7143 | 4.2143 | 4.2857 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 5 | 4 | 4 | 0 | 0 | 0 |
| # 2 |  |  | 11 | 10 | 10 | 6 | 6 | 5 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 10 | 10 |

Table A33– UL user plane worst-case latency (GB PUSCH) for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) with grant-based transmission (msec), for Rel-16 beyond Capability 2 processing times – R16c

| UL user plane latency (Grant-based) – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.5536 | 0.5536 | 0.5536 | 1.0179 | 1.0179 | 1.0179 |
|  |  | 1 reTX | 0.8036 | 0.8036 | 0.8036 | 1.5179 | 1.5179 | 1.5179 |
|  |  | 2reTX | 1.0536 | 1.0536 | 1.0536 | 2.0179 | 2.0179 | 2.0179 |
|  |  | 3reTX | 1.3036 | 1.3036 | 1.3036 | 2.5179 | 2.5179 | 2.5179 |
|  | M=4 (4OS) | Initial TX | 0.5893 | 0.5893 | 0.5893 | 1.0893 | 1.0893 | 1.0893 |
|  |  | 1 reTX | 0.8571 | 0.8750 | 0.9107 | 1.5893 | 1.5893 | 1.5893 |
|  |  | 2reTX | 1.1250 | 1.1607 | 1.3393 | 2.0893 | 2.0893 | 2.0893 |
|  |  | 3reTX | 1.3929 | 1.5893 | 1.6607 | 2.5893 | 2.5893 | 2.5893 |
|  | M=7 (7OS) | Initial TX | 0.6786 | 0.7143 | 0.7143 | 1.1964 | 1.1964 | 1.2679 |
|  |  | 1 reTX | 1.1786 | 1.2143 | 1.2143 | 1.6964 | 2.1964 | 2.2679 |
|  |  | 2reTX | 1.6786 | 1.7143 | 1.7143 | 2.1964 | 3.1964 | 3.2679 |
|  |  | 3reTX | 2.1786 | 2.2143 | 2.2143 | 2.6964 | 4.1964 | 4.2679 |
| # 0.5 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| # 1 |  |  | 5 | 5 | 5 | 0 | 0 | 0 |
| # 2 |  |  | 11 | 11 | 11 | 6 | 5 | 5 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 10 | 10 |

Table A34– DL user plane worst-case latency for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) (msec), for Rel-16 beyond Capability 2 processing times – R16a

| DL user plane latency – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3304 | 0.3482 | 0.3482 | 0.5536 | 0.5893 | 0.5893 |
|  |  | 1 reTX | 0.6518 | 0.6696 | 0.7411 | 1.0536 | 1.0893 | 1.0893 |
|  |  | 2reTX | 1.0982 | 1.1339 | 1.2411 | 1.5536 | 1.5893 | 1.5893 |
|  |  | 3reTX | 1.4196 | 1.4554 | 1.7411 | 2.0536 | 2.0893 | 2.0893 |
|  | M=4 (4OS) | Initial TX | 0.4018 | 0.4196 | 0.4554 | 0.6964 | 0.7321 | 0.8036 |
|  |  | 1 reTX | 0.7589 | 0.7768 | 0.9554 | 1.1964 | 1.2321 | 1.3036 |
|  |  | 2reTX | 1.2232 | 1.2411 | 1.4554 | 1.6964 | 1.7321 | 1.8036 |
|  |  | 3reTX | 1.7232 | 1.7411 | 1.9554 | 2.1964 | 2.2321 | 2.3036 |
|  | M=7 (7OS) | Initial TX | 0.5000 | 0.5000 | 0.5000 | 0.8929 | 0.8929 | 0.8929 |
|  |  | 1 reTX | 1.0179 | 1.0357 | 1.0714 | 1.4643 | 1.4643 | 1.5357 |
|  |  | 2reTX | 1.5179 | 1.5357 | 1.5714 | 2.3929 | 2.3929 | 2.3929 |
|  |  | 3reTX | 2.0179 | 2.0357 | 2.0714 | 2.9643 | 2.9643 | 3.0357 |
| # 0.5 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 1 |  |  | 5 | 5 | 5 | 3 | 3 | 3 |
| # 2 |  |  | 11 | 11 | 11 | 8 | 8 | 8 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 12 | 11 |

Table A35– DL user plane worst-case latency for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) (msec), for Rel-16 beyond Capability 2 processing times – R16b

| DL user plane latency – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.3125 | 0.3304 | 0.3304 | 0.5179 | 0.5536 | 0.5536 |
|  |  | 1 reTX | 0.5982 | 0.6161 | 0.6518 | 1.0179 | 1.0536 | 1.0536 |
|  |  | 2reTX | 0.8839 | 0.9018 | 1.0804 | 1.5179 | 1.5536 | 1.5536 |
|  |  | 3reTX | 1.3125 | 1.3304 | 1.4018 | 2.0179 | 2.0536 | 2.0536 |
|  | M=4 (4OS) | Initial TX | 0.3839 | 0.4018 | 0.4375 | 0.6607 | 0.6964 | 0.7679 |
|  |  | 1 reTX | 0.7054 | 0.7232 | 0.7589 | 1.1607 | 1.1964 | 1.2679 |
|  |  | 2reTX | 1.1339 | 1.1518 | 1.1875 | 1.6607 | 1.6964 | 1.7679 |
|  |  | 3reTX | 1.4554 | 1.4732 | 1.5089 | 2.1607 | 2.1964 | 2.2679 |
|  | M=7 (7OS) | Initial TX | 0.4821 | 0.4821 | 0.4821 | 0.8571 | 0.8571 | 0.8571 |
|  |  | 1 reTX | 0.9821 | 0.9821 | 0.9821 | 1.3571 | 1.3571 | 1.3571 |
|  |  | 2reTX | 1.4821 | 1.4821 | 1.4821 | 1.8571 | 1.8571 | 1.8571 |
|  |  | 3reTX | 1.9821 | 1.9821 | 1.9821 | 2.3571 | 2.3571 | 2.3571 |
| # 0.5 |  |  | 3 | 3 | 3 | 0 | 0 | 0 |
| # 1 |  |  | 7 | 7 | 6 | 3 | 3 | 3 |
| # 2 |  |  | 12 | 12 | 12 | 9 | 9 | 9 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 12 | 12 |

Table A36– DL user plane worst-case latency for NR TDD (mode 2 – each slot has 7 DL symbols, and 7 symbols for UL) (msec), for Rel-16 beyond Capability 2 processing times – R16c

| DL user plane latency – NR TDD mode 2 |  |  | UE capability 2 |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS 60kHz, FDD |  |  | SCS 30kHz, FDD |  |  |
|  |  |  | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol | PDCCH every symbol | PDCCH every 2-symbol | PDCCH every 4-symbol |
| Resource mapping Type B | M=2 (2OS) | Initial TX | 0.2946 | 0.3125 | 0.3125 | 0.5000 | 0.5357 | 0.5357 |
|  |  | 1 reTX | 0.5625 | 0.5625 | 0.5625 | 1.0000 | 1.0357 | 1.0357 |
|  |  | 2reTX | 0.8125 | 0.8125 | 0.8125 | 1.5000 | 1.5357 | 1.5357 |
|  |  | 3reTX | 1.0625 | 1.0625 | 1.0625 | 2.0000 | 2.0357 | 2.0357 |
|  | M=4 (4OS) | Initial TX | 0.3661 | 0.3839 | 0.4196 | 0.6429 | 0.6786 | 0.7500 |
|  |  | 1 reTX | 0.6518 | 0.6696 | 0.7411 | 1.1429 | 1.1786 | 1.2500 |
|  |  | 2reTX | 0.9375 | 0.9554 | 1.1696 | 1.6429 | 1.6786 | 1.7500 |
|  |  | 3reTX | 1.3661 | 1.3839 | 1.4911 | 2.1429 | 2.1786 | 2.2500 |
|  | M=7 (7OS) | Initial TX | 0.4643 | 0.4643 | 0.4643 | 0.8393 | 0.8393 | 0.8393 |
|  |  | 1 reTX | 0.7857 | 0.7857 | 0.7857 | 1.3393 | 1.3393 | 1.3393 |
|  |  | 2reTX | 1.2143 | 1.2143 | 1.2143 | 1.8393 | 1.8393 | 1.8393 |
|  |  | 3reTX | 1.5357 | 1.5357 | 1.5357 | 2.3393 | 2.3393 | 2.3393 |
| # 0.5 |  |  | 3 | 3 | 3 | 1 | 0 | 0 |
| # 1 |  |  | 8 | 8 | 7 | 4 | 3 | 3 |
| # 2 |  |  | 12 | 12 | 12 | 10 | 9 | 9 |
| # 3 |  |  | 12 | 12 | 12 | 12 | 12 | 12 |