3GPP TSG RAN WG1 Ad-Hoc Meeting #AH1901	R1-1900690

Taipei, Taiwan, January 21st – 25th, 2019

Agenda item:       7.2.8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	CSI Enhancements for MU-MIMO

Document for:     Discussion and Decision

# 1	Introduction

Enhancements on MU-MIMO support were approved to be studied and specified as part of the MIMO Enhancements WID in RAN#80 [1] and revised slightly in RAN#81 [2]. The objectives for enhancing MU-MIMO support are as follows [1] [2]:

The work item aims to specify the enhancements identified for NR MIMO. The detailed objectives are as follows. - Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2

In RAN1#95, the following items were agreed (Agreements from [3], with other items as cited):

Agreement For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002 - Note: The same DFT-based compression scheme is extended for Type II port selection codebook - Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme - FFS: detailed signaling mechanism - Note: Additional compression scheme(s) are not precluded Alt1 is [4]: - Wf is determined from a set of predefined DFT vectors (or possibly DCT vectors). - PMI feedback: W1 (wideband), W2 (subband), possibly a small number of parameters for Wf (wideband) Alt1.1 is [4]: - Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH - P=2N1N2= #SD dimensions - N3= #FD dimensions - FFS value and unit of N3 - Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank) - Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=Wf0,…,Wf2L-1 where Wf(i)=fki,0 fki,1…fki,Mi-1, where fki,mm=0Mi-1 are Mi size-N3×1 orthogonal DFT vectors for SD-component i=0,…,2L-1 - Number of FD-components {Mi} or i=02L-1Mi is configurable, FFS value range - FFS: choose one of the following alternatives - Alt1. common basis vectors: Wf=fk0 fk1…fkM-1, i.e. Mi=M ∀i and ki,mm=0Mi-1 are identical (i.e., ki,m=km, i=0,…,2L-1) - Alt2. independent basis vectors: Wf=Wf0,…,Wf2L-1, where Wf(i)=fki,0 fki,1…fki,Mi-1, i.e. Mi frequency-domain components (per SD-component) are selected - Note: kmm=0M-1 or ki,mm=0Mi-1,i=0,…,2L-1 are all selected from the index set {0,1,…,N3-1} from the same orthogonal basis group - FFS: If oversampled DFT basis or DCT basis is used instead of orthogonal DFT basis - FFS: Same or different FD-basis selection across layers - Linear combination coefficients (for a layer) - FFS if W2 is composed of K=2LM or K=i=02L-1Mi linear combination coefficients - FFS if only a subset K0<K of coefficients are reported (coefficients not reported are zero). - FFS if layer compression is applied so that i=02L-l-1Mi transformed coefficients are used to construct W2 for layer l (where the transformed coefficients are the reported quantity) - FFS quantization/encoding/reporting structure Note: The terminology “SD-compression” and “FD-compression” are for discussion purposes only and are not intended to be captured in the specification Agreement In RAN1 NR-AH 1901, decide (agree on) at least the following aspects of DFT-based compression: - Frequency-domain compression unit: same subband size as CQI vs. RB (or multiple of RBs) different from CQI - Basis subset selection for the 2L beams: common (including the possibility of reporting a subset of 2LM W2 coefficients) vs. independent Agreement For RAN1 NR-AH 1901: - Identify the remaining details required to finalize Type II rank 1-2 compression, e.g. range of values and configuration for each DFT-based compression parameter, CBSR utilization, detailed UCI design (such as reporting of coefficients associated with strongest beam/polarization) - Companies are encouraged to evaluate the options A, B, C, D, and E (“other schemes”) summarized in Table 3 of R1-1813002 for potential support for Type II rank 1-2 overhead reduction In particular, option C is “Enhanced compression for multi-layer PMI” with the description “compression across layers by parameterizing W2 through Givens rotations” [4]. Agreement: In RAN1 NR-AH 1901, select one of the following alternatives for precoder/PMI FD compression unit, taking into account UPT vs. overhead and complexity - Alt1. Subband (SB), wherein the SB size for precoder/PMI compression is the same as the CQI subband size - Alt2. X resource blocks (RBs), different from CQI subband size. Three sub-alternatives - Alt2.1 X = 1 - Alt2.2 X = CQI SB size / R where R>1 is a predetermined integer - Only one R value is supported. FFS: the value of R - Alt2.3 X = {2, 4} where X is higher-layer configured Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes. Agreement The first offline agreement in section 2.2 of R1-1814201 on ‘Basis subset or linear combination (LC) coefficient selection for the 2L beams’ is agreed. The offline agreement from section 2.2 of R1-1814201 [5] is: Offline agreement: In RAN1 NR-AH 1901, select one of the following alternatives for basis subset selection scheme for each layer - Alt1A. Common selection for all the 2L beams, wherein M coefficients are reported for each beam - Wf=fk0 fk1…fkM-1 - W2 is composed of K=2LM linear combination coefficients - The value of M (applied to all 2L beams) is higher-layer configured and the M basis vectors are dynamically selected (hence reported with CSI) - Alt1B. Common selection for all the 2L beams, but only a size- K0<2LM subset of coefficients are reported (not reported coefficients are treated as zero) - Wf=fk0 fk1…fkM-1 - W2 is composed of K=2LM linear combination (LC) coefficients, but (K-K0) of its coefficients are zero - The value of M (applied to all 2L beams) is higher-layer configured and the M basis vectors are dynamically selected (hence reported with CSI) - For evaluation, companies should state their assumption on the selection of K0 LC coefficients (applied to all 2L beams), e.g. - The value of K0 is fixed or higher-layer configured, and the K0 LC coefficients are dynamically selected by the UE (hence reported with CSI), or - The K0 LC coefficients and its size are dynamically selected by the UE (hence reported with CSI) - Alt2. Independent selection for all the 2L beams, wherein Mi coefficients are reported for the i-th beam (i=0, 1, …, 2L-1) - Wf=Wf0,…,Wf2L-1, where Wf(i)=fki,0 fki,1…fki,Mi-1, i.e. Mi frequency-domain components (per beam) are selected - W2 is composed of K=i=02L-1Mi linear combination coefficients - The value of K (applied to all 2L beams) is higher-layer configured - For evaluation, companies should state their assumption on size-Mi basis subset selection (applied to the i-th beam), e.g. for i=0, 1, …, 2L-1 - The size-Mi subset and the value of Mi are dynamically selected by the UE (hence reported with CSI) - The size-Mi subset is dynamically selected by the UE (hence reported with CSI), but the value of Mi is determined by a predefined rule in specification - The size-Mi subset is dynamically selected by the UE (hence reported with CSI), but the value of Mi is higher-layer configured - The size-Mi subset can be chosen either from the fixed basis set or from a beam-common UE-selected intermediate subset of the fixed basis set Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes. Agreement: In RAN1 NR-AH 1901, select one of the following alternatives for DFT basis oversampling factor(s) O3: - Alt1. O3 = 4 - Alt2. O3 = 1 (critically sampled) - Alt3. O3 is fixed for and depends on a given length of the DFT vector (N3) and/or bandwidth part, exact dependence is FFS Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes. Agreement: In RAN1 NR-AH 1901, companies are encouraged to evaluate the following alternatives for compression basis (Wf) subset selection scheme across different layers when RI=2. Select one of the following alternatives in RAN1#96: - Alt1. Basis subset selection (Wf) for the 1st is the same as that for the 2nd layer - Alt2. Basis subset selection (Wf) for the 1st can be different from 2nd layer Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2 quantization for evaluation purposes. For next meeting In RAN1 NR-AH 1901, companies are encouraged to study the following issues for finalizing the remaining details on DFT-based compression in RAN1#96: - Supported values for the number of FD compression units before compression, or the DFT vector length (N3), by considering, e.g. - Whether one compression is performed across the entire CSI reporting band or a segment of the CSI reporting band - Supported values for the number of FD components after compression (M for common selection or {Mi} for independent selection) Agreement: For each layer, the following alternatives for quantizing each of the coefficients in W2 are to be studied for down selection in RAN1#96: - Alt1A. Rel.15 3-bit amplitude; Rel.15 QPSK and 8PSK co-phasing - Alt1B. Rel.15 3-bit amplitude; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing - Alt2A. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK and 8PSK co-phasing - Alt2B. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing - Alt2C. Rel.15 3-bit wideband amplitude + Rel.15 QPSK and 8PSK wideband co-phasing for each beam, 2/3-bit differential amplitude and co-phasing for FD coefficients; - Alt3. A-bit amplitude for each of 2L beams, B-bit amplitude for each of M FD components, 1-bit differential amplitude and 8PSK co-phasing for each of the 2LM FD coefficients - Alt4. For each beam, - B0-bit amplitude and C0-bit phase for coefficients for the P0 strongest coefficients, - B1-bit amplitude and C1-bit phase for coefficients for the P1 2nd strongest coefficients, … - … - BQ-1-bit amplitude and CQ-1-bit phase for coefficients for the PQ-1 Qth strongest coefficients - Alternatively, amplitude/phase can be replaced with real/imaginary - Alt5. Special case of Alt4: Q=2, B0=C0=3; B1=C1=2 on amplitude/phase

In this contribution, we will build upon such agreements and focus on several open aspects of the DFT-based compression of the matrix W2, and analyze the performance of a proposed DFT-based scheme designed according to such aspects.

# 2	DFT-based FD compression

The agreements reached during RAN1 #95 identified a direction to perform a frequency compression of the matrix of linear combination coefficients W2, to reduce the overhead of the PMI payload. A DFT-based scheme has been agreed upon, designed according to a parametric approach. Several aspects of this scheme have been left open for discussion, to guarantee that adequate investigations could be carried out before taking final decisions on the structure of the solution for Type II overhead reduction in Rel-16. Accordingly, the following aspects are discussed in this section:

- Frequency domain compression unit

- Basis subset selection for the DFT-based compression

- Oversampling factor for the DFT codebook

- Supported values for M and K0

- Quantization of the DFT-compressed matrix W2.

This section will then be followed by some considerations on the so-called layer compression, currently considered as a possible complementary item to the DFT-based solution.

## 2.1 Frequency domain compression unit

In the Rel-15 specification, CQI and PMI granularity coincide. However, preserving this condition in the context of Rel-16 DFT-based compression may not be the best choice when larger BWPs are adopted, e.g., larger than 10 MHz. In fact, the accuracy of the PMI when the subband size is larger than 4 PRBs may not be enough to offer good performance after the DFT-compression. Accordingly, a finer PMI granularity, i.e., decoupled from the CQI granularity, may be a viable way to guarantee consistency of the performance across different values of the BWPs. However, increasing the PMI granularity should not be considered a cost-free operation, given that

- It may result in complexity increase at the devices due to the larger number of multiply-and-add operations needed to perform the DFT-based compression, e.g., basis selection and actual compression;

- Increasing the number of frequency units over which W2 is defined, i.e., its number of columns, may also entail an increase of the size of the basis used for the DFT-based compression, to be able to capture all the relevant compressed information. In other words, this may increase the number of columns of a suitable Wf.

Therefore, if on the one hand special care should then be taken to make sure that the performance for larger BWPs would not be penalized by the DFT-based compression, on the other hand we should ensure that the number of frequency units does not grow excessively.

In this context, the agreement reached during RAN1 #95 on this aspect identified four options. If we let X be the number of RBs in a frequency unit for PMI calculation, the possible alternatives are:

- Alt1. CQI and PMI granularity coincide, as in Rel-15.

- Alt2.1 X = 1

- Alt2.2 X = CQI SB size / R where only one predetermined integer value of R > 1 is supported.

- Alt2.3 X = {2, 4} where X is higher-layer configured

These alternatives are characterized by a rather heterogeneous behavior in terms of the corresponding number of frequency units over which the PMI is calculated. To better assess this aspect, we calculate explicitly such number for each of these options and present the result in Table 1 (where we limit the value of R for Alt 2.2 to 2, in order to capture a compromise between Alt 1 and Alt 2.3).

| BWP (#PRBs) |  | Alt 1 | Alt 2.1 | Alt 2.2 [R=2] | Alt 2.3 |
| --- | --- | --- | --- | --- | --- |
| < 24 |  | #PRBs | #PRBs | #PRBs | #PRBs |
| 24 – 72 |  | 3, 4, 5, 6, 7, 8, 9, 10, 13, 17 | #PRBs | 6, 7, 8, 10, 12, 13, 16, 17, 19, 26, 33 | 6, 7, 8, 10, 12, 13, 16, 17, 19, 26, 33 |
| 73 – 144 |  | 5, 6, 7, 8, 9, 10, 12, 14, 16, 17 | #PRBs | 10, 12, 14, 16, 17, 20, 24, 27, 31, 33, 34 | 20, 24, 27, 31, 33, 34, 39, 40, 47, 53, 54, 61, 66, 67, 68 |
| 145 – 275 |  | 5, 6, 7, 8, 9, 10, 11, 12, 14, 16, 17, 18 | #PRBs | 10, 11, 12, 14, 16, 17, 18, 20, 21, 24, 27, 28, 31, 34, 35 | 40, 41, 48, 54, 55, 62, 66, 68, 69, 80, 81, 95, 108, 109, 123, 132, 135, 137 |

Table 1 . Size X of PMI subband in number of PRBs for different BWPs, according to the alternative definitions of X.

Now, from a quantitative perspective, it is rather evident that both Alt. 2.1 and Alt 2.3 would need to consider a very large number of frequency units, e.g., up to an order of magnitude more than Rel-15 PMI granularity, in turn significantly increasing the size of the codebook and the computational complexity of the subsequent DFT-based compression. Additionally, as previously mentioned, this may negatively impact the size of the basis used for the DFT-based compression, to be able to capture all the relevant compressed information. Conversely, Alt 2.2, with R=2, offers an interesting compromise since it does not change the order of magnitude of the frequency units over which the PMI is calculated, as compared to Alt. 1, and yet offers a finer granularity. This is particularly interesting in case of larger BWPs, where the higher number of frequency units may improve the accuracy of the DFT-based compression as compared to what would be observed if Alt. 1 were to be adopted.

Observation 1: Alt.2.2, with R=2, offers the best compromise in terms of PMI granularity for Rel-16 DFT-based compression. Note that R=1 is also supported as default option.

Proposal 1: Support Alt. 2.2, with R=2.

## 2.2 Basis subset selection

After introducing frequency domain (FD) compression, a Type II precoder can be expressed by the formula:

| W=W1W2=W1W2WfH | (1) |
| --- | --- |

Basis selection is the operation by which we select an orthogonal basis from an oversampled DFT codebook of size N3×O3N3. We refer to the N3 orthogonal vectors of this basis as FD candidate components. The FD compression matrix Wf of size N3×M consists of a size-M subset selected from the basis candidate components, which represent the FD components used for compression.

Once the M FD components have been identified, the PMI report needs to communicate a restricted subset of the linear combination coefficients generated by the projection operation

| W2=W2Wf | (2) |
| --- | --- |

where W2 is the matrix of linear combination coefficients generated by the spatial domain (SD) compression. This subset of nonzero coefficients has a maximum size defined by the parameter K0 and they are referred to as FD coefficients. The report of up to K0 FD coefficients should include information about their location in the size-(2L×M) matrix W2 as well as their quantized value. All the remaining coefficients in W2 that are not reported are assumed to be zero by the gNB. Note that because of quantization, the number of nonzero coefficients in W2 may be smaller or larger than K0. In the first case, K1<K0 coefficients may be reported, whilst in the second case only the strongest K0 coefficients are reported. In summary, there are three logical units of information that needs to be reported when FD compression is applied:

1)  index corresponding to the basis subset selection of M FD components

2) 	location of the K1≤K0 nonzero FD coefficients

3) 	quantized values of the K1 FD coefficients

Let us consider the basis subset selection problem and the alternative methods identified in [5]. In our view, Alt.1 (common basis subset selection) and Alt.2 (independent basis subset selection) offer the same flexibility in selecting up to K0 nonzero FD coefficients. However, common basis subset selection has a more favourable overhead when the sets of {Mi} for spatial beams i=0,1,…,2L-1 are not disjoint. This is the case for FD compression because we observe that some of the “strongest” FD components tend to be the same for multiple spatial beams.

Moreover, by comparing the two sub-options Alt.1A and Alt.1B, we observe that Alt.1A is too restrictive because it forces K0 to be fixed to 2LM, which implies an excessive maximum overhead for the reporting of the quantized values of the FD coefficients. Therefore, our conclusion is that Alt.1B offers the best compromise between flexibility and overhead

Proposal 2: Common basis subset selection based on Alt. 1B.

Let us consider the reporting of the (up to) K0 nonzero FD coefficients. The most straightforward option, which we refer to as free selection, for reporting the location of the nonzero coefficients is to do so by means of either a bitmap of size 2L×M or by combinatorial indexing, i.e., by a table for any pair of values (2LM,K0), which associates an index to every possible combination of K0 out of 2LM locations. Let us consider the example depicted in Figure 1, with L=4, M, K0=(5,13) and a typical value for N3=13.

By using a bitmap, the overhead due to 1) and 2) for free selection is calculated as:

| log2N3M+2LM=51 bits | (3) |
| --- | --- |

whereas combinatorial indexing yields:

| log2N3M+log22LMK0=45 bits. | (4) |
| --- | --- |

Although combinatorial indexing is naturally the most efficient signalling method in terms of overhead, it has the significant practical drawback of the table sizes and the need to define a very large number of indexed tables.

Figure 1. Illustration of the nonzero FD coefficients (dark squares) in the 2L×M matrix W2

As an alternative to free selection, one may consider reporting the locations separately for each spatial beam after indicating the {Mi} selection per beam. One big disadvantage of this solution is the sheer number of possible combinations. For example, for K0=11 or K0=37 there exit 29184 combinations of {Mi}! Hence, we need to select a smaller subset of typical combinations and indicate which one provides the best approximation to the actual distributions of the nonzero coefficients. We refer to this method as per-beam selection. If we restrict the number of possible combinations of {Mi} to 16 (4 bits), and use combinatorial indexing, we obtain the following overhead for the example of Figure 1:

| log2N3M+4+i=02L-1log2MMi=38 bits | (5) |
| --- | --- |

This technique yields a very low overhead; however it introduces a rather crude approximation by restricting the possible distributions of nonzero coefficients. Our conclusion is that free selection of the nonzero coefficients is a simple and efficient way of reporting the locations of these coefficients without introducing further approximations that impact on performance.

Proposal 3: Free selection of K1≤K0 nonzero FD coefficients by means of a bitmap.

Further attempts can be made at reducing the overhead 2) and 3) by reducing the size of the bitmap used for signaling. In Figure 2 we plot the average number of nonzero FD coefficients per spatial beam, with the spatial beams on the horizontal axis arranged in decreasing order of magnitude of their FD component ‘0’. We observe that:

1) FD component ‘0’ provides a good approximation of a beam’s wideband (WB) amplitude. Therefore, FD component ‘0’ can serve as the WB component for all spatial beams.

2) The strongest beam has fewer nonzero FD components (strong LOS component), hence it can be approximated by a frequency flat beam as in Rel-15 Type II

3) The weakest beams have fewer significant FD components, hence they can also be approximated with a frequency flat beam by reporting only FD component ‘0’

Figure 2. Average number of nonzero FD coefficients per spatial beam.

Correspondingly, we can introduce three possible reductions to the 2L×M bitmap illustrated, as an example, in Figure 3:

1) FD component ‘0’ is always included in the common basis set (WB component) and its 2L-1 coefficients are always reported (except for the strongest spatial beam). The other M-1  FD components are dynamically reported with log2N3-1M-1 bits

2) The strongest spatial beam is indicated with log22L bits and not reported (its power is used for amplitude normalization)

3) Only the L0≤2L-1 spatial beams with the largest WB components are part of the bitmap. Therefore, K0-2L+1 coefficients are freely selected inside a reduced size table of size L0×(M-1). L0 can be fixed or defined by a rule or dynamically configured. This allows to control the bitmap density. For ex. for L0=⌈2(K0-2L+1)/(M-1)⌉ the bitmap density is ½.

We refer to this variant of free selection with optimizations as optimized free selection.

Figure 3. Illustration of three possible optimizations to reduce the size of the bitmap

In Figure 4 and Figure 5 we compare the overhead 1) and 2) of the schemes presented above in the case L=4 and M=6, as a function of K0

Figure 4. Overhead comparison for different K0-subset selection schemes with a bitmap.

Figure 5. Overhead comparison for different K0-subset selection schemes with combinatorial indexing.

Observation 2: Several optimizations can be introduced to further reduce the size of the FD component bitmap with significant overhead saving.

### 2.2.1 Range of values for M and K0

Regarding the choice of values for the parameters of FD compression, we observe that, for a given value of N3, by fixing M and varying K0 we have enough flexibility to control the overhead/performance trade-off. Moreover, in case of larger BWP when a finer PMI granularity is configured (R=2), it is desirable to keep the same overhead as for the case R=1. We observe that a ratio MN3/R≃12 is a good compromise.

Proposal 4: Fix M for each value of N3, for example such that M=N3/(2R).

As for values of K0, we observe that, if we define the bitmap density as the ratio: K02LM, the higher the density the smaller the overhead but the more limited the choice of K0 elements. Hence, a convenient way of directly controlling this trade-off is by setting this ratio to a couple of RRC configurable values. Suitable simulations have been carried out to evaluate the normalized UE throughput achieved by means of the two approaches discussed in this section, i.e., free selection and optimized free selection, when different density values are adopted (i.e., d={14,13}) and simulations are carried out according to EVM configuration in the Appendix. In particular, a basis subset selection as per Alt 1B is performed for both considered approaches, with K0 fixed or higher-layer configured, and dynamic reporting of K1≤K0 FD coefficients. We will refer to this variant as Alt 1B-1 and depict the normalized UE throughput in Figure 6, where

- The performance of Rel-15 Type II codebook (with L=2,3,4, SB amplitude ON and phase quantized using 8-PSK) is used as a benchmark;

- All the results are normalized w.r.t. the performance achieved by the considered Rel-15 Type II codebook for L=2;

- The normalized performance of Rel.15 Type II, with L=2, SB amplitude OFF and phase quantized using QPSK) is plotted as a reference (e.g., a crude lower bound for the overhead);

- An oversampled DFT codebook has been used, with oversampling factor 4 (more details about this aspect will be given in the following section).

Figure 6. Normalized UE throughput for different values of d

The optimized free selection outperforms the free selection and offers better a throughput/overhead tradeoff regardless of the considered density. Concerning this last parameter, and observing the overhead values of the two approaches as compared to Rel-15 Type II, we can note that supported density values could exceed 13. In this regard, and considering the possible advantage brought by a larger range of supported values, a good compromise can be given by densities d=14 and 12.

Proposal 5: The value of K0 is RRC configured, such that K0=2LMd, with, for example, d=14,12.

## 2.3 Oversampling the DFT codebook

A DFT-based codebook for frequency compression has several advantages in terms of complexity and specification; however it also has an important drawback. When W2 is compressed across frequency units using such a codebook, we can observe the occurrence of spurious components in W2 (i.e., the compressed version of W2). This may alter the amount of useful information, e.g., energy, carried by the M columns of W2, thus the overall accuracy of the compression. A suitable method to combat this issue is to resort to an oversampled version of the DFT codebook F. We first start by recalling the definition of the non-oversampled DFT codebook, given by:

Fnm=1N3e-j2πnmN3

where n,m∈[0,1,…, N3-1]. Alternatively, the oversampled DFT codebook can be defined as a function of an oversampling factor O3≥1 as

Fnm=1N3e-j2πnmN3O3

where n∈[0,1,…, N3-1] and m∈[0,1,…, O3N3-1]. In this regard, we note that when O3=1, a regular non-oversampled DFT codebook is obtained. An important difference between the two codebooks is as follows:

- When O3=1, the M columns of matrix Wf would be chosen from a set of N3M subsets of orthogonal vectors;

- When O3>1, the M columns of matrix Wf can be chosen from a set of O3N3M subsets of orthogonal vectors.

We now calculate the normalized UE throughput achieved by means of the DFT-based compression when these two codebooks are adopted, and depict it in Figure 7, where benchmarks, lower bound and normalizations are set/performed as for Figure 6.

Figure 7. Normalized UE throughput in case of oversampled DFT codebook

As can be seen from the figure, the normalized throughput is larger when the DFT codebook is oversampled.

Observation 3: The normalized throughput improves when the DFT-based compression is performed by means of an oversampled codebook.

Proposal 6: Support oversampling of the codebook used to perform the DFT-based compression, with O3=4.

## 2.4 Quantization of the DFT-compressed W2 matrix

For the design of the quantization scheme we can start by looking at the statistics of amplitude and phase of the FD coefficients. In Figure 8 we plot the average normalized amplitude of the elements of W2 for all N3 possible candidate FD components. We observe a strong correlation between the coefficient amplitude and FD component index. FD component ‘0’ is the strongest and the second strongest is half the amplitude of the strongest one on average. This is not surprising considering the properties of DFT-based signal compression.

Figure 8. Average normalized amplitude of FD coefficients of W2 for all N3 candidate FD components. The average is also across spatial beams.

Note that this correlation is only observed when applying FD compression. This is expected as Fourier-based compression of a correlated signal tend to focus the signal energy in the “low-pass” components. Conversely, if we look at the statistics of amplitude of the coefficients of W2 before FD compression, we observe that they all have have the same average amplitude, as shown in Figure 9.

Figure 9. Average normalized amplitude of coefficients in W2 for all N3 subbands, before applying FD compression. The average is also across spatial beams.

Considering the amplitude statistics of W2, it makes sense to adjust the quantization granularity to the dynamic range of the FD components by introducing different quantization groups of FD components. In the simplest case, we can assign one extra bit for amplitude quantization and one extra for phase to FD component ‘0’

We now look at the distribution of both amplitude and phase of the W2 coefficients for each spatial beam. These are plotted for L=4 in Figure 10 for all 8 spatial beams. For each beam the distribution of all the N3 candidate FD components are shown, with the thick black curve marking the strongest beam used for phase and amplitude normalization. Note that the strongest beam tends to be the first beam of either of the two polarizations. For comparison, we also show the same distributions of the coefficients of W2, before FD compression, in Figure 11.

Figure 10. Distributions of amplitude and phase of W2 coefficients for all spatial beams.

Figure 11. Distributions of amplitude and phase of W2 coefficients for all spatial beams.

We observe that the amplitude distributions of W2 are far from uniform and skewed, whereas the phase distributions are generally uniform, except for beams 0 and 4, which corresponds to the strongest beams, used for normalization in the proposed approach, i.e., optimized free selection. Similar distributions are observed in the coefficients of W2 before FD compression; however, we note that FD compression reduces the variance of the coefficient amplitudes, thus making it even less uniform. Optimizing the amplitude quantizer for such nonuniform distributions is possible, for example by using Lloyd’s design for the quantization codebook. Aside from this last consideration, and to conclude this section, we perform a last set of simulations to assess the merit of the proposed quantization scheme as compared to the quantization strategy labeled as Alt 1A in RAN1 #95 agreements. To this end, we will evaluate the normalized UE throughput achieved when both these quantization strategies are adopted in conjunction with the two considered approaches for basis and coefficients selection, i.e., free selection and optimized free selection. We illustrate the result of this comparison in Figure 12, where benchmarks, lower bound and normalizations are set/performed as for Figure 6 and 7. We note that we will refer to the proposed quantization strategy in this section as Alt 4, given its compliance with the quantization approach labeled as Alt 4 in RAN1 #95 agreements.

Figure 12. Normalized UE throughput when quantization strategies Alt 1A and Alt 4 are used in conjunction with free selection and optimized free selection

As can be seen from the figure, the proposed quantization strategy consistently outperforms Alt 1A both in case of free selection and optimized free selection. Furthermore, we observe that the combination of optimized free selection and proposed quantization strategy is the best performer out of the considered approaches. In particular it allows to achieve better performance/overhead tradeoff than Rel. 15 Type II, while outperforming the latter in terms of normalized UE throughput.

Proposal 7: For each spatial beam, use 4 bits for amplitude and 4 bits for phase quantization of the first FD component; 3 bits amplitude and 3 bits phase for the remaining M-1 FD components.

# 3 	Layers compression

During RAN1 #94b and #95 [5], a further optimization meant to reduce the number of coefficients of W2 prior to FD compression was discussed. This approach, labelled as layers compression aims at removing the redundancy in the layers representation before applying compression in frequency. More precisely, the number of linear combination coefficients resulting from the spatial compression can be reduced by exploiting the unit-norm and orthogonality properties of the layers. As a result, 2 real coefficients can be saved in the representation of layer 1, 4 real coefficients in the representation of layer 2, etc. Note that the saving achieved for layer 1 is equivalent to the amplitude and phase normalization performed by Rel-15 Type II. During RAN1 #95, layers compression was included in the list of «Other schemes» which could complement the core scheme which will be agreed upon in RAN1 AH19_01 [5]. Indeed, the flexibility of this approach allows it to be combined with DFT-based compression rather straightforwardly, regardless of how the latter is configured. For this reason, no further element about it has been included in this contribution.

Observation 4: Prior to frequency compression, the number of linear combination coefficients for each frequency resource can be reduced by exploiting the unit-norm and orthogonality of layers. These coefficients are then rearranged by layers ready for applying frequency compression. This approach can complement a DFT-based compression regardless of how the latter is designed and configured.

# 4	Conclusions

In this contribution, we have presented our proposal for overhead reduction of the NR Type II codebook.  Our proposals are:

- Frequency domain compression unit

Proposal 1: Support Alt. 2.2, with R=2.

- Basis subset selection

Proposal 2: Common basis subset selection based on Alt.1B.

Proposal 3: Free selection of K1≤K0 nonzero FD coefficients by means of a bitmap.

- Supported values for M and K0

Proposal 4: Fix M for each value of N3, for example such that M=N3/(2R).

Proposal 5: The value of K0 is RRC configured, such that K0=2LMd, with, for example, d=14,12.

- Oversampling factor for the DFT codebook

Proposal 6: Support oversampling of the codebook used to perform the DFT-based compression, with O3=4.

- Quantization of the DFT-compressed matrix W2.

Proposal 7: For each spatial beam, use 4 bits for amplitude and 4 bits for phase quantization of the first FD component; 3 bits amplitude and 3 bits phase for the remaining M-1 FD components.

# References

| [1] | Samsung, RP-181453, “New Draft WI proposal on NR MIMO enhancements”, 2018. |
| --- | --- |
| [2] | Samsung, RP-182067, “Revised WID: Enhancements on MIMO for NR”, 2018. |
| [3] | MCC Support, "Draft Report of 3GPP TSG RAN WG1 #95 v0.2.0," Spokane, WA USA, 12-16 November 2018. |
| [4] | Samsung, R1-1813002, "Summary of CSI enhancement for MU-MIMO support," RAN1#95, 12-16 November 2018. |
| [5] | Samsung, R1-1814201, "Outcome of offline session for CSI enhancement for MU-MIMO support," Spokane, WA, USA, 12-15 November 2018. |
| [6] | 3GPP, Chairman's notes RAN1 94b final, 2018. |
| [7] | Nokia/NSB, R1-1811406, "CSI Enhancements for MU-MIMO Support", 2018. |

# Appendix

Table 2.  System Simulation Parameters

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban |
| Carrier frequency |  | 4GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | UMa, according to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | As in TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 |
| CSI Feedback bit allocation | LD | Nϕ=3 bits and Nψ=2 bits |
|  | FD-DFT | Namp=3 bits and Nphase=3 bits |
| Simulation bandwidth |  | 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | Maximum MU layers = 12 |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Overhead |  | 2 symbols/slot |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% target utilization. Arrival rates are: - 4 users/sec for 16 antenna ports results in 62% RU - 5 users/sec for 32 antenna ports results in 60% RU |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Non-ideal |