3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1901077

Taipei, Taiwan, 21st – 25th January, 2019

Agenda item:	7.2.8.3

Source: 	Samsung

Title: 	Enhancements on multi-beam operations

Document for:	Discussion and Decision

# Introduction

The following agreement/conclusion was reached in RAN1#95 [1]:

Agreement (I)

RAN1 has identified the following scenarios to be important for SCell BFR

- Scenario 1: SCell with both uplink and downlink

- Scenario 2: SCell with downlink only

- PCell can be in FR1 or FR2 for scenarios above

Agreement (II)

- Support L1-SINR measured from

  - For signal part, SSB and/or NZP CSI-RS

  - FFS: For interference part

- Companies are encouraged to provide simulation results on how to measure/define L1-SINR, e.g. whether interference is measured from dedicated IMR

  - For example, take Rel-15 L1-RSRP and/or SINR specified in 38.215 as a comparative reference for evaluation purposes

Agreement (III)

For interference part, down-select at least one from the following alternative:

- Alt 1: Dedicated resource(s) for interference measurement

  - FFS: UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214

  - FFS: whether resource(s) for interference measurement can be NZP based or ZP based or both

  - FFS: whether/how to reuse NZP CSI-RS resource(s) configured for channel measurement as resource(s) for interference measurement

- Alt 2: The same reference signal as signal part as specified in 38.215

- Alt 3: Alt1 when SSB is used for signal part, Alt2 when CSI-RS is used for signal part

- Companies are encouraged to provide simulation results for down-selection

Agreement (IV)

In Rel-16, an identifier (ID) that can be used at least for indicating panel-specific UL transmission is supported, where detailed usages for the panel-specific UL transmission are FFS

- The ID should be defined considering the possibility to reuse/modification of Rel-15 specification support or introducing new ID

- Note: RAN1 to avoid unnecessary specification support requiring UE to explicitly disclose its UL antenna panel implementation

- FFS: Whether UE capability signalling is introduced for panel-specific UL transmission

Agreement (V)

Decide (agree on) either one of the followings in RAN1 NR-AH 1901:

- Alt.1: Support sub-time unit for beam management RS shorter than 1 OFDM symbol in a reference numerology.

  - No new RS for beam management is introduced in Rel-16.

  - FFS: details including IFDMA-based, DFT-based, larger subcarrier spacing based, etc, or limited to only for P-3.

- Alt.2: No support of sub-time unit for beam management RS shorter than 1 OFDM symbol.

Agreement I led to a revised WID with the following amended wording [2]:

- Specify beam failure recovery for SCell with DL/UL as well as DL-only, where PCell can be operating in FR1 as well as FR2

With the above guideline starting point, Samsung’s views on enhancements to multi-beam operation and evaluation methodology are presented in this contribution.

2) L1-SINR:

(1) 38.215 SINR shows loss - should not be supported

(2) Study new SINR definition and interference measurement

3) Latency reduction:

(1) SRS based DL beam indication

4) UL beam indication

(1) Support of STxMP

(2) TCI based UL beam indication

SCell BFR

## Design issues for SCell BFR

In Rel.15, PCell BFR includes the following functions: beam failure detection, new beam identification (NBI), transmitting beam failure recovery request, and the gNB response. Beam failure detection is needed for SCell BFR. Here, the Rel.15 beam failure detection procedure can be reused. The gNB can configure the set q0 of beam failure detection RSs for an SCell to the UE. The UE can declare the beam failure of the SCell when the number of consecutive beam failure instances reaches the threshold.

Regarding new beam identification (NBI), in Rel.15 the UE is configured with an RS set q1. When beam failure occurs, the UE chooses one RS from the set q1 and recommends it to the serving gNB. Then the gNB uses that RS as the Tx beam to transmit the gNB response. For SCell BFR, however, NBI is not needed. NBI is supported in Rel.15 because no DL beam connection is reliable when the beam failure event occurs. In contrast, that is not the case for SCell. When beam failure occurs in one SCell (but not in PCell), the beam connection in PCell is still good and can be used for transmitting gNB response and follow-up “beam switch” signaling on the SCell. Likewise, in the deployment scenarios where PCell is in FR1 and SCell is in FR2, the connection in PCell is still good when beam failure occurs in one SCell and the PCell can be used to switch/update the beams for that SCell. Therefore, the UE does not need to identify a new beam for that SCell. The same applies in deployment scenarios where both PCell and SCell are in FR2.

Proposal: For SCell BFR, Rel.15 beam failure detection procedure is reused for SCell while new beam identification (NBI) is not needed.

Regarding the mechanism for transmitting SCell BFR request, the following alternatives have been proposed:

- Alt-1: The UE uses contention-free RACH configured on the UL of that SCell to transmit the BFR request and then monitors the gNB response on CORESET-BFR configured in that SCell.

- Alt-2: The UE uses contention-free RACH configured on the UL of that SCell to transmit the BFR request and then monitors the gNB response on CORESET-BFR of PCell.

- Alt-3: The UE uses contention-free RACH on the UL of PCell to transmit the BFR request for the SCell and monitors the gNB response on PCell.

- Alt-4: The UE uses MAC-CE message to report SCell beam failure.

- Alt-5: The UE uses PUCCH to report SCell beam failure.

We can evaluate and compare the alternatives in various deployment scenarios.

|  | DL-only SCell | SCell with both DL/UL | PCell in FR1 and SCell in FR2 | PCell in FR2 and SCell in FR2 |
| --- | --- | --- | --- | --- |
| Alt.1 | No support (no SCell UL) | Support | Support for SCell with both DL/UL | Support for SCell with both DL/UL |
| Alt.2 | No support (no SCell UL) | Support | Support for SCell with both DL/UL | Support for SCell with both DL/UL |
| Alt.3 | Support | Support | Support but with excessive RACH resource usage. To support BFR of SCell, we would need configure Nnew × NScell RACH resources in PCell for each UE to the association between RACH resources and Tx beams (the number of Tx beams could be large). Typically, the association between RACH resources and large number of Tx beam is not needed since PCell is in FR1. Nnew is the number of RSs in q1 and NScell is the number of SCells | Support |
| Alt.4 | Support | Support | Support | Support |
| Alt.5 | Support | Support | Support | Support |

As evident, only Alt.4 and Alt.5 work well for all the deployment scenarios. While it is plausible to support different schemes for different scenarios, it adds to specification and/or deployment complexity. Therefore, it is preferred to support a single scheme that works for all scenarios.

Observation: Either Alt.4 (MAC-CE) or Alt.5 (PUCCH) facilitates supporting a single scheme that works for all deployment scenarios.

Between Alt.4 and Alt.5, the tradeoff is clear: MAC-CE based solution imposes higher latency than PUCCH-based, but offers much better reliability. The higher latency comes from using L2 control mechanism. Yet it has been pointed out that the overall latency for SCell BFR is not dominated by BF reporting. At the same time, the control information is protected by HARQ (which allows error detection in addition to error correction). When PUCCH is used, the latency is expected to be the same as, e.g. HARQ-ACK or CSI feedback. However, PUCCH lacks error detection capability which makes it more prone to error.

Assuming that SCell BFR is essential for CA deployment in FR2, high reliability takes precedence. Here, MAC-CE based solution is a better alternative considering its superior reliability without imposing excessively additional latency.

Proposal: Support Alt.4 (MAC-CE based solution) for SCell BFR considering its superior trade-off between reliability and latency.

# Support for L1-SINR

In Rel.15, DL TX beam quality (where a TX beam corresponds to one NZP CSI-RS resource with at most 2 ports) is represented with L1-RSRP. While CSI could have been reused for this purpose, it could be argued that the unit for CQI (spectral efficiency) is less suitable for representing energy (dBm). In addition, the use of CSI (which is typically associated with tracking short-term channel characteristics and/or possibly large number of antenna ports) is not aligned with the intention of DL TX beam selection. Therefore, one could argue, for instance, that even if CSI could be used, it would be “too complex” for multi-beam operation. This lends some room for defining a new “low-complexity” CSI parameter that is essentially the SNR of a DL RS. No interference measurement is needed.

Such arguments had been understandable before the CSI acquisition framework were solidified. However, it should now be apparent that such arguments lack substance at least for the following reasons. First, with sufficiently fine granularity, CQI can be easily correlated with SNR. For instance, it is possible to define another 7-bit CQI table (along with its 4-bit differential CQI) that functions the same as L1-RSRP. Second, with properly defined Reporting Setting (e.g. time/frequency-domain behavior of CSI reporting) and Resource Setting (e.g. # ports for CSI-RS resource = 1 or 2, QCL-Type-D-ed with the resource(s) used for CSI acquisition), a CQI report that exhibits the same behavior as L1-RSRP can be made. Hence, strictly speaking, L1-RSRP is a redundant metric.

Observation: Given the flexibility of CSI, it can be argued that even Rel.15 L1-RSRP is redundant.

During the discussion of Rel.16 WI scope, several companies argued that L1-RSRP lacks “interference awareness.” This is of course the case since L1-RSRP, as mentioned above, is defined as an SNR metric which does not account for interference. To account for interference (inter-beam-intra-cell + inter-cell), it seems that not only an IMR is needed, but also a new “interference-aware” beam metric. Therefore, introducing L1-SINR, perhaps along with some specification support for interference measurement, is justified.

This rationale, however, overlooks the following aspects. First, to start with, L1-RSRP was chosen to offer the “simplest” possible DL TX beam quality metric (with very little regard of the configuration used for CSI acquisition, including “interference awareness”). Second, there has been no evaluation-based evidence that the lack of “interference awareness” results in catastrophic failure for multi-beam operation and, moreover, that introducing L1-SINR would remedy the issue. Third, even if the case of “interference-aware” beam metric (possibly along with interference measurement mechanism) could be well established, the case for introducing a new beam metric would still be unclear. The NW may be able to configure CSI that achieves such function in a similar (or even superior) manner. As mentioned, with properly configured Reporting Setting and Resource Setting, spatial/time-domain characteristics and “interference awareness” (with properly configured IMR) can be attained.

Observation: If “interference awareness” is indeed beneficial, the intended function of L1-SINR can be achieved by properly configured CSI. Therefore, the need for supporting L1-SINR is unclear.

Even if L1-SINR is to be specified in Rel.16, its definition is still to be discussed. One alternative is to reuse the CSI/SS-SINR given in TS 38.215 as follows (for CSI-SINR) [2]:

| Definition | CSI signal-to-noise and interference ratio (CSI-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying CSI reference signals divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying CSI reference signals reference signals within the same frequency bandwidth. For CSI-SINR determination CSI reference signals transmitted on antenna port 3000 according to 3GPP TS 38.211 [4] shall be used. For intra-frequency CSI-SINR measurements, if the measurement gap is not configured, UE is not expected to measure the CSI-RS resource(s) outside of the active downlink bandwidth part. For frequency range 1, the reference point for the CSI-SINR shall be the antenna connector of the UE. For frequency range 2, CSI-SINR shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported CSI-SINR value shall not be lower than the corresponding CSI-SINR of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

In TS 38.215 CSI/SS-SINR is defined as the ratio between average signal power and average noise-plus-interference power measured based on the same CSI-RS or SSB. At least a few drawbacks can be identified. First, a metric defined as a ratio between average signal power (across channel ensembles) and average noise-plus-interference power (across interference channel ensembles) is not reflective of the real metric of interest, i.e. the average/mean of the ratio between signal power and noise-plus-interference power, taken across channel and interference ensembles. Second, the manner in which receive diversity (the use of multiple receive chains) is incorporated is ad-hoc. While it accounts for selection receive diversity, it is unclear how maximum ratio combining can be accounted satisfactorily. Third, although signal and noise-plus-interference power can be measured from the same signal resource (either the same NZP CSI-RS or SSB), its accuracy degrades in the presence of strong and dominant interference.

While there could be more complex beam metrics, the following modified L1-SINR metric is proposed: Let t=0, 1, …,NT-1 and r=0, 1, …,NR-1 denote the indices of transmit (TX) and receive (RX) beams. The weighted SINR metric for a given TX-RX beam pair t,r is then given by

Mt,r=γt,r×SINRt,r,

where

  - γt,r=Pt,rr=0NR-1t=0NT-1Pt,r is a weighting factor assigned to TX-RX beam pair (t,r)

  - Pt,r= RSRP for TX-RX beam pair t,r

  - SINRt,r= 38.215 SINR TX-RX beam pair t,r.

Termed the RSRP-weighted SINR, this metric combines the advantageous features from both L1-RSRP (stronger TX beam power) and L1-SINR (weaker interference). In some scenarios, it is expected to offer some gain over L1-RSRP since interference is taken into account. The benefit over L1-SINR is anticipated in scenarios where a TX beam with the highest L1-SINR but lower received power is chosen (which is not beneficial for TX beam selection).

In a companion contribution [3], a system-level study on the CSI-SINR in TS 38.215 is performed which results in the following observation (reproduced here).

Observation:

- There is no observable difference in distributions of gNB and UE beam pair selection according to Rel.15 L1-RSRP and TS 38.215 CSI-SINR metrics.

- The TS 38.215 CSI-SINR metric results in slightly lower average and 50% UPT performance than Rel.15 L1-RSRP metric. Therefore, there is no perceivable benefit from 38.215 SINR in UPT.

- The proposed weighted SINR metric shows large performance gain over both Rel. 15 L1-RSRP and TS 38.215 CSI-SINR metrics.

This leads to the following proposal.

Proposal: If “interference aware” SINR-based beam reporting metric is to be supported in Rel.16, SINR definitions other than that from TS 38.215, such as the RSRP-weighted L1-SINR, should be studied and evaluated

# DL/UL beam indication with reduced latency and overhead

One of the main issues on Rel.15 multi-beam design is the excessive RRC configuration. This was intended to accommodate generous flexibility without costing too much burden on L1 DL control signaling. Since a number of the controlling RRC parameters may require frequent reconfiguration, multi-beam operation may require excessive use of RRC (re)configuration (L1  L3  L1) which imposes high latency. As this involves L3, frequent RRC reconfiguration essentially negates the purpose of “beam management” which is to circumvent higher-layer procedure(s) akin to L3 mobility. In addition, RRC reconfiguration consumes PDSCH resources.

Therefore, one study that can result in significant reduction in latency and overhead is to assess features where the need for RRC (re) configuration for DL/UL beam indication can be alleviated. More specifically, this implies more reliance on L1 control signaling (DCI) or, if needed, L2 control signaling (MAC CE). The increase in PDCCH overhead can be minimized by revisiting the flexibility offered in Rel.15 – whether simplification can be done to reduce the number of options (in beam reporting, indication, and resource configuration used for beam measurement) in Rel.16.

Proposal: Revisit Rel.15 features and assess possible reduction in reconfiguration signaling requirement for DL/UL beam indication (for data and control channels)

- Streamline options/states (e.g. the number of CSI-RS resources, QCL associations) defined in RRC and/or use L1 control signaling instead

Another area where latency and resource usage can be reduced lies not so much in the flexibility of each feature, but rather the use cases. For instance, Rel.15 beam measurement and reporting tend to be (over-)designed to accommodate worst-case scenarios. In terms of UE capability, they were designed to enable UEs without beam correspondence. While the discussion whether beam (BPL) correspondence should be mandatory or not is still unresolved, it suffices to claim that for FR2 (where DL/UL reciprocity is quite common due to TDD or TDD-like operation), beam correspondence allows not only more flexibility, but also opportunities to reduce latency and overhead.

For DL TX beam indication, Figure 1 depicts a timing diagram on the so-called “beam switching” which involves a sequence of UE procedures: receiving beam reporting trigger, (after some timing offset) receiving CSI-RS, measuring CSI-RS and calculating beam reporting, reporting beam metric, receiving DL beam indication (DCI reception).

Figure 1 DL TX beam switching based on Rel.15 design (CSI-RS based measurement)

If beam correspondence holds, the “beam switching” process can be simplified by utilizing SRS instead of CSI-RS as depicted in Figure 2. Compared to Figure 1, it is apparent that using SRS as the reference for DL beam indication at least avoids the latency caused by the time offset between the UL-related DCI for CSI request and AP-CSI-RS transmission. This can be done by simply introducing SRS resource ID (in addition to CSI-RS and SSB IDs) in the TCI state definition for DL beam indication.

Proposal: Introduce the use of SRS for aiding DL beam indication by including SRS resource ID in TCI state definition

Figure 2 DL TX beam switching with SRS-based measurement

Introducing sub-time unit for CSI-RS configured for multi-beam operation (e.g. L1-RSRP reporting with 1 or 2 ports and repetition) reduces the time for the UE to perform RX beam selection for a given DL TX beam. In particular, with NRXB RX beam candidates, the required time is reduced by up to NRXB times since the UE can perform RX beam sweeping within one slot as opposed to NRXB slots. This in turn reduces the required time needed for calculating beam report (CRI(s) and the associated beam metric(s)) by up to a factor of NRXB. As a result, DL beam selection latency is significantly reduced. Therefore, it is beneficial to introduce sub-time unit for CSI-RS configured for multi-beam operation.

Observation: Sub-time unit for CSI-RS configured for multi-beam operation reduces the time required for beam reporting and hence facilitates latency reduction for DL beam selection.

Proposal: Introduce sub-time unit for CSI-RS configured for multi-beam operation.

In Rel.15, DCI format 0_0 cannot be used with an FR2 SCell if there is no PUCCH resource configured for that SCell. This limitation should be removed at least for the following reasons:

- The payload of DCI format 0_0 is almost half that of DCI format 0_1 (32 vs. 57 information bits) and thus permits better coverage.

- Coverage limitation from not being able to use DCI format 0_0 is especially relevant for CA scenarios. For instance, consider one carrier in FR1 as the primary cell and a few SCells in FR2. In this case, even if PUCCH resource is configured for the PCell, DCI format 0_0 cannot be used even for cross carrier scheduling of any of the SCells.

Observation: In Rel.15, not allowing DCI format 0_0 in SCell with no PUCCH resource configuration penalizes CA deployment, especially in terms of DL control coverage.

Proposal: Support DCI format 0_0 for any SCell in FR2 even when PUCCH resource configuration is absent.

# UL beam indication for multi-panel UE

Rel.15 only supports signaling one UL TX beam to the UE since only a single SRI field is included in DCI 0_1. Likewise, SRS-Resource (with only one target SRS resource ID) is linked to a reference RS via SpatialRelationInfo (which includes only one reference RS ID). However, for UEs with N panels, N different UL TX beams can be “simultaneously” transmitted.

At least for UL codebook-based transmission, the use of target SRS (linked to a reference RS) for UL beam indication does not seem necessary. While target SRS can correspond to the (SRS) antenna ports associated with PDSCH transmission, the necessity of multiple target SRS resources (hence the linkage between multiple target SRS resources and reference RS resources in SpatialRelationInfo) is unclear. Even if a UE can be configured with up to 2 SRS resources (for the purpose of UL CSI acquisition at the gNB), only one SRS resource is needed for UL data transmission.

Therefore, for beam training and indication purposes, it suffices for the UE to be configured with multiple reference RS resources. Therefore, for UL beam indication, the same TCI-based approach can be used where the linkage between target SRS resources and reference RS resources is unnecessary. Instead, the UL TCI state definition includes a list of reference RS resources (SRS, CSI-RS, and/or SSB). The current SRI field can be reused to select a TCI state from the configured set, or a new DCI field (for illustrative purposes, termed the UL-TCI field) in DCI 0_1 can be defined for this purpose.

With this in mind, a mechanism to support signaling multiple UL TX beams to the UE can be introduced for the purpose of simultaneous transmission across multiple panels (STxMP). By facilitating such functionality, the UE can “simultaneously” transmit via multiple panels for increasing diversity and/or multiplexing, including UL transmission with panel selection. To enable such functionality, as a starting point for further enhancements, multiple SRI fields (or at least some extension of the SRI field) or multiple UL-TC fields in DCI 0_1 can be supported.

Proposal: For Rel.16 UL beam indication, introduce the following features:

- At least for UL codebook-based transmission, circumvent the unnecessary use of target SRS by introducing UL TCI states (analogous to DL TCI) which are associated with reference RS resource IDs

  - The DCI field used for UL beam indication selects the UL TCI state, either a new DCI field or reusing the existing SRI field

- Mechanism to support multiple UL TX beam indication (STxMP) such as multiple SRI fields (or at least some extension of the existing SRI field) or multiple UL-TCI fields

# Conclusions

In this contribution, Samsung’s views on enhancements to multi-beam operation and evaluation methodology are presented. The following observations and proposals are made:

Observation:

- SCell BFR:

  - Either Alt.4 (MAC-CE) or Alt.5 (PUCCH) facilitates supporting a single scheme that works for all deployment scenarios.

- L1-RSRQ/SINR:

  - Given the flexibility of CSI, it can be argued that even Rel.15 L1-RSRP is redundant.

  - If “interference awareness” is indeed beneficial, the intended function of L1-SINR can be achieved by properly configured CSI. Therefore, the need for supporting L1-SINR is unclear.

  - There is no observable difference in distributions of gNB and UE beam pair selection according to Rel.15 L1-RSRP and TS 38.215 CSI-SINR metrics.

  - The TS 38.215 CSI-SINR metric results in slightly lower average and 50% UPT performance than Rel.15 L1-RSRP metric. Therefore, there is no perceivable benefit from 38.215 SINR in UPT.

  - The proposed RSRP-weighted SINR metric shows large performance gain over both Rel. 15 L1-RSRP and TS 38.215 CSI-SINR metrics.

- DL/UL beam indication with reduced latency/overhead:

  - Sub-time unit for CSI-RS configured for multi-beam operation reduces the time required for beam reporting and hence facilitates latency reduction for DL beam selection

  - In Rel.15, not allowing DCI format 0_0 in SCell with no PUCCH resource configuration penalizes CA deployment, especially in terms of DL control coverage.

Proposal:

- SCell BFR:

  - Rel.15 beam failure detection procedure is reused for SCell while new beam identification (NBI) is not needed

  - Support Alt.4 (MAC-CE based solution) for SCell BFR considering its superior trade-off between reliability and latency.

- L1-RSRQ/SINR:

  - If “interference aware” SINR-based beam reporting metric is to be supported in Rel.16, SINR definitions other than that from TS 38.215, such as the RSRP-weighted L1-SINR, should be studied and evaluated

- DL/UL beam indication with reduced latency/overhead:

  - Revisit Rel.15 features and assess possible reduction in reconfiguration signaling requirement for DL/UL beam indication (for data and control channels)

    - Streamline options/states (e.g. the number of CSI-RS resources, QCL associations) defined in RRC and/or use L1 control signaling instead

  - Introduce the use of SRS for aiding DL beam indication by including SRS resource ID in TCI state definition

  - Introduce sub-time unit for CSI-RS configured for multi-beam operation

  - Support DCI format 0_0 for any SCell in FR2 even when PUCCH resource configuration is absent

- UL beam indication for multi-panel UE: Introduce the following features:

  - At least for UL codebook-based transmission, circumvent the unnecessary use of target SRS by introducing UL TCI states (analogous to DL TCI) which are associated with reference RS resource IDs

    - The DCI field used for UL beam indication selects the UL TCI state, either a new DCI field or reusing the existing SRI field

  - Mechanism to support multiple UL TX beam indication (STxMP) such as multiple SRI fields (or at least some extension of the existing SRI field) or multiple UL-TCI fields

# References

- 3GPP RAN1, RAN1#95 Chairman’s Notes

- 3GPP RAN, RP-182863, Samsung, Revised WID: Enhancements on MIMO for NR

- 3GPP, TS 38.215, NR, Physical Layer Measurements

- 3GPP RAN1, R1-1901084, Samsung, Evaluation on SINR metrics for beam selection