3GPP TSG RAN WG1 AH-1901	 R1-1901083

Taipei, Taiwan, 21st – 25th January 2019

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	Preliminary SLS evaluation on Type II rank 3-4 extension

Document for:	Discussion and Decision

# Introduction

The following agreement was made in RAN1#94bis [1].

Agreement The study and, if needed, work on Type II higher rank extension is performed as follows: - Only for rank 3 and 4 by taking into account the outcome of Type II overhead reduction for rank 1-2 - Simple extension of Rel.15 Type II without any additional optimization (which results in ~3-4x overhead over rank-1) is ruled out

This contribution provides simulation results for the rank 3-4 Type II CSI codebook proposed in the companion contribution [2]. This contribution is a revision of R1-1810890.

# Simulation results for rank 3-4 Type II CSI

For performance evaluation, the non-full-buffer system-level evaluation is carried out for UMi channel model in low (20% target RU) traffic loading scenario, and SU-MIMO is considered in the simulation. The results are provided for 16 antenna ports with (N1, N2) = (4, 2), where we assume that the first dimension is horizontal and the second dimension is vertical. The relevant simulation parameters are enlisted in Table 2 in Appendix.

The simulation results for different rank 3-4 Type II CSI codebook alternatives/schemes discussed in [2] are provided. In particular, the following alternatives are compared.

- Alt 0 (no R34): Type II CSI reporting is restricted up to rank 2. This is the current (Rel. 15) solution.

- Alt 1 (Type I R34): Similar to Rel. 14 advanced CSI codebook, Type I rank > 2 codebooks are used to report rank > 2 Type II CSI.

- Alt 2 (Type II extn.): The Rel. 15 rank 2 Type II CSI codebook design is extended to rank > 2.

- Alt 3 (proposed in [2]): The rank 3-4 CSI payload is comparable to rank 2 CSI payload. The following schemes are compared. The details of the scheme can be found in [2].

  - Scheme 0: Unequal #beams across layers

  - Scheme 1: Unequal #coefficients across layers.

The results are provided in Figure 1 for L = 2, 3, and 4, where Alt 0 (no R34) with L = 2 is considered as reference. The per SB payload and rank distribution of these alternatives/schemes are shown in Table 1 and Figure 2, respectively. We can observe the following.

Observation 1:

- The significantly large percentage of UEs report rank 3-4 CSI. In particular, the order of Alt 0-3 based on the increasing percentage of rank 3-4 CSI is as follows: Alt 0 < Alt 1 < Alt 3, Scheme 0 < Alt 3, Scheme 1 < Alt 2.

- For L = 2, Alt 2 and Alt 3 are identical, hence show identical performance, and performance gap between Alt 1 and Alt 2-3 is small (~3%).

- The performance gap between Alt 1 (Type I R34) and Alt 2 (Type II extn) is large: ~ 20% for L = 4. The SB overhead of Alt 2 (Type II extn), however, is large: 84 bits/SB for rank 4, which is 2 times of rank 2.

- The proposed scheme (Alt 3, Scheme 0 and 1) achieves good performance vs. overhead trade-off.

  - The payload is comparable (≤) to rank 2.

  - The performance is close to Alt 2 (Type II extn.).

L = 4

L = 3

L = 2

Figure 1: Average UPT performance for different candidate rank 3-4 codebooks

Table 1: Per SB payload (phase only) in bits

| Rank | Scheme | SB payload (phase) |
| --- | --- | --- |
| 1 |  | 21 |
| 2 |  | 42 |
| 3 | Alt 0: no R34 | 0 |
|  | Alt 1: Type I R34 | 1 |
|  | Alt 2: Type II extn | 63 |
|  | Alt 3: Scheme 0 (uneq #beams) | 39 |
|  | Alt 3: Scheme 1 (uneq #coefficients) | 42 |
| 4 | Alt 0: no R34 | 0 |
|  | Alt 1: Type I R34 | 1 |
|  | Alt 2: Type II extn | 84 |
|  | Alt 3: Scheme 0 (uneq #beams) | 36 |
|  | Alt 3: Scheme 1 (uneq #coefficients) | 42 |

Figure 2: Rank distribution

# Conclusions

In this contribution, simulation results are provided for the rank 3-4 Type II CSI codebook design proposed in [2]. The observations made are summarized as follows.

Observation 1:

- The significantly large percentage of UEs report rank 3-4 CSI. In particular, the order of Alt 0-3 based on the increasing percentage of rank 3-4 CSI is as follows: Alt 0 < Alt 1 < Alt 3, Scheme 0 < Alt 3, Scheme 1 < Alt 2.

- For L = 2, Alt 2 and Alt 3 are identical, hence show identical performance, and performance gap between Alt 1 and Alt 2-3 is small (~3%).

- The performance gap between Alt 1 (Type I R34) and Alt 2 (Type II extn) is large: ~ 20% for L = 4. The SB overhead of Alt 2 (Type II extn), however, is large: 84 bits/SB for rank 4, which is 2 times of rank 2.

- The proposed scheme (Alt 3, Scheme 0 and 1) achieves good performance vs. overhead trade-off.

  - The payload is comparable (≤) to rank 2.

  - The performance is close to Alt 2 (Type II extn.).

# References

- Chairman’s notes, RAN1#94bis

- R1-1900566, “CSI enhancement for MU-MIMO,” Samsung

# Appendix

Table 2: Simulation assumptions

| Parameters | Value |
| --- | --- |
| Simulation Type | FTP1, 20% RU, Packet size = 500 kB |
| Channel model | UMi-2GHz |
| Number of gNB (H,V) antenna elements, (M,N,P) | (8,4,2), subarray partition, 100° downtilt |
| (N1,N2), 1st dim = horizontal | 16 ports: (4,2) |
| gNB (H,V) antenna spacing | (0.5, 0.8)λ |
| gNB/UE antenna polarizations | gNB: (+45°,-45°); UE: (0°, 90°) |
| Number of UE antennas | 4, dual-pol, (N1,N2) = (2,1), 0.5λ spacing |
| SU/MU pre-coding | CB based |
| Scheduling | SU based on PF metric |
| Channel estimation | Non-ideal |
| Transmission rank (per UE) | 1,2,3,4 |
| Receiver | MMSE-IRC |
| CSI schemes | Ref: Rank 1-2 only Candidate schemes: for rank > 2 Type II CSI |
| Overhead | DMRS, CSI-RS, PDCCH |