3GPP TSG-RAN WG1 Ad-Hoc Meeting 1901 	R1-1901344

Taipei, Taiwan, 21st – 25th January, 2019

Agenda item:	7.2.9.1

Source: 	Qualcomm Incorporated

Title: 	Summary for UE Power Saving Evaluation

Document for:	Discussion/Decision

# Introduction

For this meeting, contributions in the below reference section contain discussion on evaluation results and some remaining issues in methodology for the UE power saving study item. In this summary, we capture the discussions on the remaining issues in evaluation methodology, and provide some comments and recommendation for actions to follow during the meeting. Evaluation results are captured along with power saving scheme discussion in the summaries for the other agenda items (7.2.9.2 and 7.2.9.3).

# Power Model

## Confirmation of FR2 power numbers

In [1], the following is proposed:

- Remove the square brackets in Table 3 of TR 38.840.

- Remove the square brackets for the PDSCH-only slot for FR2 in Table 4 of TR 38.840.

| Power State | Characteristics | Relative Power |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 (for reference only) |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | [175] |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell.. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. (Note 2 SSBs in a slot for the ref. config.) | 100 | [175] |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 | [350] |
| UL | Long PUCCH or PUSCH. | 250 ( 0 dBm) 700 (23 dBm) | [350] (FFS Tx power level) |

Table 3: UE power consumption model for FR2

| PDSCH-only slot | 280 for FR1 [325] for FR2 | This assumes the same number of PDSCH symbols as in the PDCCH+PDSCH case. |
| --- | --- | --- |

Table 4: UE power consumption scaling for adaptation

Along the same line, it can be discussed whether to also confirm the FR2 power numbers for RRM power modelling. Possible proposal:

- Remove the square brackets in Table 5, 7, 8 of TR 38.840

| N: Number of cells for intra-frequency measurement | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 150 | [225] | 170 | [285] |
| N=4 | 120 | [195] | 140 | [255] |

Table 5: UE power consumption for the RRM measurements

| N: Number of cells for intra-frequency measurement & search | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 200 | [320] | 220 | [380] |
| N=4 | 170 | [290] | 190 | [350] |

Table 7: UE power consumption of the combined neighbor cell measurements and cell search

| FR1 | FR2 |
| --- | --- |
| 150 | [270] |

Table 8: UE power consumption of neighboring cell search

Recommendation: If there is no concern, confirm the numbers and remove square brackets.

## CSI-RS processing

Existing agreement for CSI processing power is as follows:

| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells . TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| --- | --- | --- |

In [16], the following scaling for processing other configurations of CSI-RS is proposed:

The CSI-RS power consumption model for 2 symbol of CSI-RS configuration is 100, which is derived from 2 symbols of CSI-RS processing and 2 symbols of transition time not in the micro sleep and (240*(2+2) +10*45)/14. The generalized model for the CSI-RS configuration of 1, 2, 4 OFDM symbols would also assume that 2 additional OFDM symbols for transition and not going to micro sleep. The generic power consumption model for N-symbol CSI-RS configuration is (240*(N+2)+(12-N)*45)/14.

CSI-RS power consumption model

| Case type | 1OFDM symbol | 2OFDM symbols | 4OFDM symbols |
| --- | --- | --- | --- |
| Power consumption per slot | 87 | 100, | 129 |

In [2], the following scaling for CSI-RS processing is proposed:

Pm, n=α∙β∙Pt

Where, α=0.75, if m<M1, if m≥M, β=0.75, if n<N1, if n≥N,  Pt=100, M=4, N=3

Pt is the power units for CSI-RS with >=M symbols and >=N CSI-RS ports. Pm, n is the power units for CSI-RS with m symbols and n CSI-RS ports.

Recommendation: Discuss further. Identify power saving scheme which requires this modelling support.

## Ramp-up/down for sleep transition

In [14], the need to further define the ramp-up / ramp-down details for sleep transition is discussed, and the following is proposed:

Transition energy and time for state transition between active state and sleep state.

| Sleep type | Transition energy |  | Transition time |  |
| --- | --- | --- | --- | --- |
|  | Ramp up | Ramp down | Ramp up | Ramp down |
| Deep sleep | 380 | 70 | 16ms | 4ms |
| Light sleep | 85 | 15 | 5ms | 1ms |

Recommendation: Discuss further

## BWP and CA transition

In [12], it is proposed that microsleep power (i.e. 45 power units) is used also to model BWP transition power, because during BWP transition, RF is not transmitting nor receiving, which is similar to microsleep.

In [16], it is proposed BWP transition power level should be modeled as 50 power units.

In [1], it is proposed that the transition energy for BWP switching and CA activation/deactivation is modelled as

Et=(Pbefore+Pafter)*Tt2.

Possible proposal: BWP transition power level is 50 power units

## Power scaling

### PDCCH candidate reduction for cross-slot scheduling

In RAN1 #95, the following agreement was made:

- For power scaling for PDCCH candidate reduction (for same slot scheduling only):

P(α) = α ∙ Pt + (1 – α) ∙ 0.7Pt

where α is the ratio of PDCCH candidates to the max number of PDCCH candidates in the reference configuration (α>0). Pt is the PDCCH-only power for same-slot scheduling.

In [16], it is proposed that power scaling for PDCCH candidate reduction for cross-slot scheduling be defined as:

P(α) = α ∙ Pcross-slot + (1 – α) ∙ 50

Where Pcross-slot = 70.

Recommendation: The following agreement is already made:

- Scaling for the power reduction due to PDCCH candidates processing (e.g. AL/CCE/BD) reduction is modelled solely based on its effect on micro sleep portion of the PDCCH-only slot

For cross-slot scheduling, microsleep starts right after the end of the last PDCCH symbol, so PDCCH candidate reduction generally does not impact microsleep, and the same power number as cross-slot scheduled PDCCH-only can be used.

### Scaling for sleep state transition

In [14], it is proposed that the same power scaling agreed in last meeting can be used to scale energy consumption for state transition to-and-from sleep state.

Recommendation: Discuss further. Lower priority.

### Power scaling for FR1+FR2 CA

Scaling for CA is agreed for up to 4 carriers for FR1 or FR2.

In [13], power scaling for FR1+FR2 CA is discussed and the following scaling is proposed.

Power scaling for CA (FR1+FR2)

| CA (FR1+FR2) | 2CC (FR1+FR2) is 0.85 * (1CC for FR1 + 1CC for FR2) 4CC (N1 FR1 + N2 FR2) is 0.85 * (N1 * 1CC for FR1 + N2 *1CC for FR2) where N1 and N2 is the number of cells in FR1 and FR2, respectively. |
| --- | --- |

Recommendation: Discuss further.

### Power saving signal

In [16], the following is proposed:

For the evaluation purpose, the power consumption of power saving signal could be modeled by reusing the deep sleep power consumption model 1 unit/slot, or 0.1unit/slot, even less than 0.1unit/slot.

In [14], the following power assumption for PDCCH-based power saving signal is proposed:

Power saving signal power consumption assumption

| Receiver design | Power saving signal | Detection approach | BW | Power consumption |
| --- | --- | --- | --- | --- |
| Independent wake-up radio | PDCCH-based | Time-domain correlation | -- | 0.1 unit |
|  |  | Frequency domain decoding | 1MHz | 3 unit |
|  |  |  | 10 MHz | 9 unit |
|  |  |  | 20 MHz | 15 unit |

In [13], the following power model for power saving signal is proposed:

Power model for PoSS

| PoSS structure | Relative power | Note |
| --- | --- | --- |
| Sequence-based | 55 – 70 | The minimum power 55 is obtained by adding 10 more power to the power for micro sleep (45). Depending on the configurations of the sequences, it can be scaled up to 70. |
| PDCCH-based | 70 – 100 | Same as PDCCH-only state. Depending on the number of BDs, the power level can be varied from 70 to 100. |
| PDSCH-based | 300 | Same as PDCCH+PDSCH state |

In [12], it is proposed that PDCCH-WUS power consumption can be modelled as roughly 1/3 of PDCCH-only power for WUS and 1/3 of deep sleep transition overhead. More specifically:

- For PDCCH-WUS, the slot-averaged power can be assumed to be at most 30 units

- Deep sleep transition overhead for waking up for PDCCH-WUS, detecting nothing, and going back to deep sleep can be modeled as 1/3 of the baseline, i.e. 150 power units * msec. For the case UE detects PDCCH-WUS, the full baseline overhead should be assumed; BWP transition can be assumed to model the transition from the end of PDCCH-WUS to UE being ready to receive PDSCH.

CSI-RS-based WUS should follow similar power model assumption as PDCCH-WUS.

In [18], the following power model for WUS is proposed:

The power for detecting wake-up signal is assumed as 20 and a 3 slots gap between wake-up signal and the beginning of DRX On-duration was assumed.

Recommendation: Discuss further

## Mini-slot power modeling

During RAN1#95 there was extensive offline discussion on UE power modelling for same-slot scheduling of mini-slot transmission of PDSCH. One of the most promising modelling schemes discussed is the following scheme based on interpolation of two already defined power states.

The following scaling model is proposed for modelling the slot-averaged power as a function of the number of PDSCH symbols for two cases: (i) with PDCCH, (ii) without PDCCH, for FR1 only.

Case (i): with PDCCH:

Linear interpolation between above two endpoints:

P(x) = x∙300 + (1-x)∙Y

Where

x = min((N+m),12)/12.

N is the number of PDSCH symbols.

m is the overhead (in number of symbols) for transition into/out-of microsleep. m>=[2].

[70] < Y <= [100].

FFS: minimum value of N

Assume TDRA is configured such that UE can potentially go into lower power state during symbols not schedulable for PDSCH.

No consensus was reached last meeting. Several companies have concern about the overhead for microsleep for small gap, and the overall feasibility for employing microsleep to achieve power saving for such scenarios.

For the contributions submitted to AH1901, above scheme is discussed in [11] and the following parameter values are proposed: Y=90 and m=[1~3]

An alternate scheme is proposed in [6]: (2*240 +β*280 + γ*45)/14, with the following assumptions:

- The number of symbols for PDCCH decoding is assumed to be 2.  .

- β is the number of symbol for  PDSCH reception.

- γ is symbol the number of OFDM symbol for UE micro sleep and γ >7 (the gap is large enough to allow UE micro sleep.

- (2+β+ γ)=14

Recommendation: Continue discussion

Case (ii): without PDCCH

FFS

Recommendation: Define Case (i) first.

## RRM power modelling

In [12], the following clarification is proposed:

- The scaling factors for BWP adaptation and number of antenna reduction are intended to be applicable to active power states except for the states corresponding to RRM.

In [5], the following power model for partial cell search is proposed:

the following power consumption model for partial cell search is proposed:

P_FR1 = 90+60*N/1008

P_FR2 = 210+60*N/1008

Where N is number of cells needs to be detected.

# Traffic Modelling

## Model Proposals

### Web-browsing / interactive content-pull model

In [12], the following is proposed:

For the recommended web-browsing traffic model based on R1-070674, the reading time parameter should be adjusted.

It is further discussed that the reading time of 4 sec is used in the evaluation.

# Evaluation

## Periodic activities

In [14], it is proposed that assumption on the sync condition in deep sleep state should be clarified.

In [3], the following proposal is made:

- Support of periodic activity such as periodic beam acquisition/reporting is necessary in order not to degrade system reliability

The periodic activites are further elaborated as:

These activities include CSI-RS acquisition, beam management reporting using PUCCH, CSI reporting using PUCCH, and Periodic SRS transmission. These periodic background activities are required for several functionalities that guarantee reliable connection between the gNodeB and the UE, such as control link maintenance by the gNodeB, CSI acquisition by the gNodeB, beam management, and power control loop.

In [17], the following rule is proposed:

Periodic CSI-RS acquisition and PUCCH reporting are still maintained for beam management with

Period = max(DRX, 160ms)

This is not cancelled even if power saving signal cancels PDCCH monitoring for an DRX ON duration.

Recommendation: Discuss

## DoU power metric

In [12], the following is proposed:

- Time weights can be assigned to the applications based on real-life typical usage. The weighted-average of DoU power contribution across the applications is the overall DoU power for the UE.

- Overall DoU power has a direct relationship to UE battery life and can be considered as a comprehensive evaluation metric for power saving gain.

## RRM

In [10], the following proposals are discussed:

- It is necessary to investigate the negative impact of RRM measurement relaxation (e.g. reduction of measured cell and/or SSB slots) together with UE power consumption reduction.

- For evaluation UE power consumption reduction in RRM measurement, UEs in motion are modelled. UE dropping and mobility modelling in TR37.885 is considered as a baseline.

## DRX assumptions

In [15], analysis based on the calibration assumptions with some modifications shows that short DRX results in lower UE energy consumption and similar latency when combined with longer DRX cycles (>100 ms) as compared to using only long DRX cycles. Hence, the following is proposed:

RAN1 should include short DRX in benchmark evaluations. Short DRX cycles of 5, 10 and 20ms could be considered.

In Nokia’s contribution for RAN1#95, it was proposed to define the group paging rate (for PO) as 20%. If desired some other value could be evaluated in addition.

In [8], DRX configurations for new traffic models such as Gaming, Web browsing and Video streaming, are proposed:

DRX configurations for Gaming, Web browsing and Video streaming

| Traffic model | DRX cycle, ms | On duration, ms | Inactivity timer, ms |
| --- | --- | --- | --- |
| Gaming | 40 | 4 | 10 |
| Web browsing | 320 | 10 | 80 |
| Video streaming | 40 | 4 | 10 |

Recommendation: Discuss

## UE Power Metric

In [17], the following UE energy efficiency metric based on the agreed UE power consumption model is proposed:

Define UE energy efficiency by weighted average over the component metric:

The traffic per second received by a UE (in bits/s)The power consumption estimated by RAN1 NR UE power model for the traffic (in Joule/s)

Note that optimization of UE energy efficiency is to improve user experience with NR UEs.

# Simulation Assumptions

[12] has the following proposal:

Baseline assumption is to run system-level simulation without DRX. In the case DRX needs to be modelled, simplifications in the DRX configuration such as aligning DRX cycles across users can be considered. The DRX cycle’s offset to other periodic signals (e.g. SSB) can be randomized over multiple runs to avoid biasing effects.

# Evaluation Results

Evaluation results submitted to 7.2.9.1 are captured in respective summaries in 7.2.9.2 and 7.2.9.3 along with discussion of the power saving schemes.

# References

- R1-1900039, Remaining issues for evaluation methodology for UE power saving, Huawei, HiSilicon

- R1-1900144, Evaluation Results and remaining issues on evaluation methodology for UE power saving, vivo

- R1-1900191, Evaluation of NR power saving designs, MediaTek Inc.

- R1-1900225, Evaluation results of UE power saving, ZTE Corporation

- R1-1900304, Evaluation methodology for UE power saving, OPPO

- R1-1900343, Evaluation Results of UE Power Saving Schemes, CATT

- R1-1900379, Evaluation results for power saving study, sony

- R1-1900507, Evaluation results of UE power saving signals, Intel Corporation

- R1-1900578, Evaluation results for UE power savings, Samsung

- R1-1900598, Numerical results for UE power saving schemes, LG Electronics

- R1-1900752, Evaluation of UE power saving techniques, Apple Inc.

- R1-1900910, Evaluation Methodology and Results for UE Power Saving, Qualcomm Incorporated

- R1-1901086, Evaluation results for UE power savings, Samsung

- R1-1901165, Evaluations and modeling of UE power consumption, Ericsson

- R1-1901187, Initial evaluation results and consideration on DRX settings, Nokia, Nokia Shanghai Bell

- R1-1901273, Evaluation results of UE Power Saving Schemes (revision of R1-1900343), CATT

- R1-1900192, Adaptation design for NR UE power saving, Mediatek

- R1-1900305, UE Adaptation to the Traffic and UE Power Consumption, OPPO