3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	Tdoc R1-1901352

Taipei, Taiwan, 21st – 25th January 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Urban Macro Scenario at 700 MHz

Document for:	Discussion, Decision

# Introduction

Power distribution and transport industry are two main use cases for macro scenario considered for Rel. 16 eURLLC SI [1]. The initial performance evaluation for power distribution and transport industry was done for the mid band carrier. In RAN1#95 [2], there is an agreement to consider also performance evaluation of URLLC macro scenario at the low band, i.e., 700 MHz carrier.

In this contribution we evaluate reliability and latency performance of URLLC urban macro scenario at 700 MHz following the assumptions agreed in [2] and the ITU evaluation methodology [3].

The results show certain reliability and latency performance that can be achieved for URLLC urban macro scenario, i.e., power distribution use case.

This contribution does not include similar evaluation of transport industry at 700 MHz. For transport industry, with large packet size of up to 5220 bytes, limited number of RBs, and small number of transmit/receive antennas, it is challenging to achieve reliability and latency requirements (e.g., 99.999%, 3ms) at reasonable SNR.

# Discussion

For the evaluation, we consider requirement in Table 1 below for power distribution use case.

Table 1: Assumption of reliability requirement for power distribution use case

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model |
| --- | --- | --- | --- |
| Power distribution | 99.9999 | 5(end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms |
|  | 99.999 | 15(end to end latency) Note: 6-7 ms air interface latency | DL & UL: 250 bytes Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs |

More specifically, we focus on the strict requirement, i.e., 99.9999% reliability within 3ms latency for the packet size of 100 bytes. We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate performance of URLLC for power distribution using the ITU methodology. That is, we first obtain SINR distribution from a system level simulation. Then reliability is computed from link level simulation results at SNR corresponding to 5%-tile value which typically represents the cell-edge user in the macro scenario.

## System Level Simulation

Based on system level simulation assumption in Table A-1 in the appendix, we obtain DL and UL SINR geometry with full buffer assumption as shown in Fig. 1 and 2 for different BS antenna configurations (4Tx/Rx (8,2,2,1,1,1;1,2) and 2Tx/Rx (8,1,2,1,1,1;1,1)) and ISD values. We note that the full buffer assumption here can be seen as a worst-case traffic assumption in terms of interference.

Figure 1: DL and UL SINR geometry for different BS antenna configurations with inter-site distance of 500 m for full-buffer traffic model.

Figure 2: DL and UL SINR geometry for different BS antenna configurations with inter-site distance of 150 m for full-buffer traffic model.

Based on the results in Fig. 1 with ISD = 500m, for 2Tx/Rx (8,1,2,1,1,1; 1,1) the 5%-tile SINR are -1.96 dB (DL) and -1.51 dB (UL), while for 4Tx/Rx (8,2,2,1,1,1; 1,2) the 5%-tile SINR are 0 dB (DL) and 0.2 dB (UL).

Based on the results in Fig. 2 with ISD = 150m, for 2Tx/Rx (8,1,2,1,1,1; 1,1) the 5%-tile SINR are -2.92 dB (DL) and -2.63 dB (UL), while for 4Tx/Rx (8,2,2,1,1,1; 1,2) the 5%-tile SINR are -0.8 dB (DL) and -0.92 dB (UL).

With the system level simulation assumption in Table A-1 for urban macro scenario with full-buffer assumption, for 2Tx/Rx (8,1,2,1,1,1; 1,1) BS antenna configuration,  the 5%-tile SINR are -1.96 dB (DL) and -1.51 dB (UL) for ISD = 500m, and -2.92 dB (DL) and -2.63 dB (UL) for ISD =150m, respectively.

With the system level simulation assumption in Table A-1 for urban macro scenario with full-buffer assumption, for 4Tx/Rx (8,2,2,1,1,1; 1,2) BS antenna configuration,  the 5%-tile SINR are 0 dB (DL) and 0.2 dB (UL) for ISD = 500m, and -0.8 dB (DL) and -0.92 dB (UL) for ISD =150m, respectively.

Based on system level simulation assumptions in Table A-2 in the appendix we obtain for FTP model 3 traffic model the percentage of UEs meeting URLLC requirements according to Table 2. For both DL and UL, grant-based and slot-based single-transmission is used. Capability 2 processing times is used for UE. For gNB, the SR->grant time is 1 slot (1 PDCCH monitoring occasion per slot) and the PUSCH processing time is assumed to be 7 symbols.

- Table 1: Percentage of UEs fulfilling URLLC requirements (3 ms, 99.9999%)

| Downlink | Uplink |
| --- | --- |
| 99% | 95% |

With the system level simulation assumption in Table A-2 for urban macro scenario with FTP model 3, for 4Tx/Rx (8,2,2,1,1,1; 1,2) BS antenna configuration, 95% of UEs can meet 3 ms latency and 99.9999% reliability for both DL and UL.

## Link Level Simulation

Based on link level simulation assumption in Table A-3 in the appendix, we present BLER performances of PDCCH and PDSCH/PUSCH.

### PDCCH

For PDCCH, we consider DCI size =40 bits excluding CRC, AL4,8 and 1os CORESET. We note that with limited bandwidth of 20 MHz and SCS of 30 kHz, AL8 is the highest AL for PDCCH with 1os duration.  PDCCH BLER for AL4 and AL8 are given in Fig. 3.

Figure 3: PDCCH BLER with DCI size 40 for different AL

### PUCCH

For PUCCH, we consider PUCCH format 0 with 2os and frequency hopping, carrying 1-bit ACK/NACK. Different error rates are given in Fig. 4.

Figure 4: PUCCH error rates with Pr(DTXtoACK) ≤10-2

### PDSCH/PUSCH

For data channels, we consider packet size of 100 bytes, transmission duration of 7 OFDM symbols with 1 DMRS symbol overhead. We note that with TBS of 100 bytes, limited bandwidth of 20 MHz, and SCS of 30 kHz, the lowest supported MCS from Table 5.1.3.1-3 [4] is MCS 6 in this case. BLER for different MCSs (MCS6 and 7) with single transmission and one retransmission with soft combining are given in Fig. 5 and 6.

Figure 5: BLER of DL data (2Tx/4Rx) for single transmission and one retransmission

Figure 6: BLER of UL data (2Tx/2Rx) for single transmission and one retransmission

## Reliability evaluation

According to the definition in [3], reliability is defined as a success probability of a packet transmission within certain latency target. In this contribution, we consider the reliability requirement of 99.9999% within one-way latency of 3 ms according to the requirement of power distribution use case.

Based on the latency analysis in [5], we see that it is possible to have two DL and two DL CG-UL with 7os duration transmissions (including retransmission) within 3 ms latency.

In the following we provide an expression for the success probability of DL and UL data transmission based on probability terms in Table 3.

Table 3: Success probabilities for calculating total reliability

| Probability | Success probability of |
| --- | --- |
| p1 | PDCCH |
| p2 | PDSCH |
| p3 | PUCCH NACK reception |
| p4 | PUCCH DTX detection |

DL HARQ (re)transmission

In the DL we can describe the total reliability after N transmission as:

pt=n=1Ni=1nn-1n-i(1-p1)p4n-ip1p3i-1∙j=1i-1(1-p2,j)∙p1p2,i

where for any positive integer k,  p2,k is the successful probability of a data being correctly received after k attempts of PDSCH transmissions are soft combined conditioned that previous soft-combined k-1 attempts of PDSCH transmissions fail. In this expression the DL control transmissions are assumed to be uncorrelated with each other and with data. This approximation can be motivated by e.g. interleaved PDCCH mapping where different candidates may be used between attempts. The data transmission attempts are assumed to be correlated with each other where the soft combining effect is captured.

Configured grant UL transmission

With configured grant UL, the first UL transmission is done without scheduling DCI. The total reliability can be described similarly as:

pt=p2,1+1-p2,1n=1N-1i=1nn-1n-i1-p1n-ip1i-1∙j=1i-1(1-p2,j+1)∙p1p2,i+1

Here the PDCCH reliability only has an effect starting from the first retransmission.

Based on results in Figures 1-6 (assuming PDCCH AL8 and PDSCH/PUSCH MCS index 6), we have the link level results at the 5%-tile DL and UL SINR summarized in Table 4. The total reliability after one retransmission is also given.

Table 4: Success probabilities

| Probability | BS antenna 2Tx/Rx (8,1,2,1,1,1; 1,1), ISD = 500m | BS antenna 2Tx/Rx (8,1,2,1,1,1; 1,1), ISD = 150m | BS antenna 4Tx/Rx (8,2,2,1,1,1; 1,2), ISD = 500m | BS antenna 4Tx/Rx (8,2,2,1,1,1; 1,2), ISD = 150m |
| --- | --- | --- | --- | --- |
| p1 | ≈1 | ≈1 | ≈1 | ≈1 |
| p2,1 for DL (MCS6) | 1-8*10-7 | 1-8*10-6 | ≈1 | ≈1 |
| p2,2 for DL (MCS6) | ≈1 | ≈1 | ≈1 | ≈1 |
| p2,1 for UL (MCS6) | 1-1.5*10-3 | 1-7*10-3 | 1-8*10-5 | 1-5*10-4 |
| p2,2 for UL (MCS6) | 1-8*10-5 | 1-5*10-4 | 1-4*10-6 | 1-3*10-5 |
| p3 | 1-3*10-2 | 1-5.5*10-2 | 1-1.2*10-2 | 1-2.1*10-2 |
| p4 | 1-2*10-2 | 1-2*10-2 | 1-2*10-2 | 1-2*10-2 |
| DL total reliability after one retransmission | 99.9999976% | 99.999956% | >99.9999% | >99.9999% |
| CG UL total reliability after one retransmission | 99.999988% | 99.99965% | >99.9999% | 99.9999985% |

That is, we show that for certain BS antenna configurations and ISD assumptions reliability of 99.9999% within 3ms one-way latency for DL and CG-UL transmissions can be achieved at the 5%-tile SINR for power distribution scenario at 700 MHz.

For the BS antenna configuration 2Tx/Rx (8,1,2,1,1,1; 1,1) with ISD = 150m, reliability of 99.99965% can be achieved.

It is possible to have up to 2 DL and CG-UL transmissions with 7os duration in a FDD configuration with 30 kHz SCS within 3 ms one-way latency.

Reliability requirement of 99.9999% within 3 ms one-way latency for power distribution use case can be achieved for certain BS antenna configurations.

# Conclusion

In section 2 we made the following observations:

Observation 1	With the system level simulation assumption in Table A-1 for urban macro scenario with full-buffer assumption, for 2Tx/Rx (8,1,2,1,1,1; 1,1) BS antenna configuration,  the 5%-tile SINR are -1.96 dB (DL) and -1.51 dB (UL) for ISD = 500m, and -2.92 dB (DL) and -2.63 dB (UL) for ISD =150m, respectively.

Observation 2	With the system level simulation assumption in Table A-1 for urban macro scenario with full-buffer assumption, for 4Tx/Rx (8,2,2,1,1,1; 1,2) BS antenna configuration,  the 5%-tile SINR are 0 dB (DL) and 0.2 dB (UL) for ISD = 500m, and -0.8 dB (DL) and -0.92 dB (UL) for ISD =150m, respectively.

Observation 3	With the system level simulation assumption in Table A-2 for urban macro scenario with FTP model 3, for 4Tx/Rx (8,2,2,1,1,1; 1,2) BS antenna configuration, 95% of UEs can meet 3 ms latency and 99.9999% reliability for both DL and UL.

Observation 4	It is possible to have up to 2 DL and CG-UL transmissions with 7os duration in a FDD configuration with 30 kHz SCS within 3 ms one-way latency.

Observation 5	Reliability requirement of 99.9999% within 3 ms one-way latency for power distribution use case can be achieved for certain BS antenna configurations.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- 3GPP RAN1 #95 Chairman’s note

- ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

- 3GPP TS 38.214 V15.3.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019

# Appendix

Table A-1: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m, 150m |
| Carrier frequency | 700 MHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 2Tx/2Rx ports (M, N, P, Mg, Ng; Mp, Np) = (8, 1, 2, 1, 1; 1, 1), dH = 0.5λ, dV = 0.8λ; 9 degrees electrical antenna tilt 4Tx/4Rx ports (M, N, P, Mg, Ng; Mp, Np) = (8, 2, 2, 1, 1; 1, 2), dH = 0.5λ, dV = 0.8λ; 9 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 20 MHz DL and 20 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors |
| Traffic | Full buffer |

Table A-2: System level simulation assumption (power distribution use case)

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 4 Tx/4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 2, 2, 1, 1; 1, 2) dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports, Isotropic |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | URLLC: 10 (on average) |
| Simulation bandwidth | 20 MHz DL and 20 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |
| UE power control | P0=-70 dBm, α=0.8 |
| HARQ/repetition | Single transmission |
| Channel estimation | No channel estimation error |
| Traffic | FTP model 3, arrival interval 100 ms, TBS=100 |
| URLLC requirements | Latency: 3 ms, Reliability 99.9999% |

Table A-3: Link level simulation assumption (Urban macro scenario)

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 700MHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for modeling fading channel |
| BS antenna configuration | 2 Tx/2 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| System bandwidth | 20 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |