3GPP TSG RAN WG1 Ad Hoc Meeting #1901					 R1-1901423

Taipei, January 21-25, 2019				(Revision of R1-1901389)

Source:	Intel Corporation

Title:	Summary of Contributions and Initial Outcome of Offline Discussion for NR-Positioning AI - 7.2.10.1.2 UL only based positioning

Agenda item:	7.2.10.1.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide summary of the main technical aspects being discussed in contributions submitted to the NR Positioning AI – “7.2.10.1.2 UL only positioning techniques” and provide tentative proposals for further discussion in RAN1 WG aiming to facilitate discussion during the meeting week.

Based on review of the submitted contributions [1]-[10], the following list of open aspects was identified:

NR UL positioning techniques

NR UL reference signals for positioning, including reference physical structure

gNB measurements and reporting for NR UL positioning

The contribution is a revision of the R1-1901389 that captures additional aspect on the use of pi/2 BPSK proposed in [2]. In the next sections we provide additional details and initial proposals to trigger further discussion and decision in RAN1 WG. The original proposals suggested for RAN1 discussions are marked in magenta and the outcome of the offline discussion is colored by cyan.

- Overview of Contributions and Potential Proposals

NR UL positioning techniques

- The majority of submitted contributions discuss UL timing (UL-TDOA) and angle based positioning (UL-AoA) techniques. One contributions additionally proposes to support UL carrier phase measurements [3]. The explicit support for UL-TDOA is mentioned in [1], [2], [3], while in other contributions it is rather suggested to prioritize [4], [10] or study of UL-TDOA and UL-AoA [5], [8]. One contribution says that UL-TDOA can be considered as a special case of multi-cell RTT [6]. The need for additional studies on UL-AoA techniques in NLOS conditions is highlighted in [7].

- Proposal for discussion

NR supports UL measurements to enable timing (UL-TDOA) and angle (UL-AoA) based positioning technique in FR1 and FR2

  - FFS if UL-TDOA and UL-AoA are separately defined or specified under the same UL positioning framework

  - FFS measurements for carrier phase techniques

- The proposal above was discussed during online session on NR Positioning. Related agreements were made by RAN1 WG.

NR Reference Signals for UL Positioning

As for NR UL positioning reference signals (NR UL PRS), majority of contributions mention SRS as a primary candidate for UL positioning reference signals and considered SRS as a starting point for UL PRS design discussion, emphasizing that enhancements may be needed to improve coverage, support gNB RX beam training, improve accuracy of estimation of signal location parameters. In addition to NR UL SRS, the support of NR PRACH was discussed and proposed in [4], [8]. One contribution [3] proposed to introduce C-PRS for carrier phase based measurements and another contribution also proposed to additionally consider DMRS [8].

- Proposal for discussion

RAN1 to continue study and select between the following two alternatives for NR UL positioning reference signal (NR UL PRS):

  - Alt1. Both NR UL SRS and NR PRACH are considered as a starting point for NR UL PRS design

    - FFS if new UL PRS is introduced or UL SRS/PRACH is enhanced

  - Alt2. NR UL SRS is used as a starting point for NR UL PRS design

    - FFS if new UL PRS is introduced or UL SRS is enhanced

- The following offline consensus was reached based on discussion of the above proposal.

- Offline consensus

NR UL SRS is used as a starting point for design and analysis of UL PRS

  - Further study if and which enhancements are needed

  - FFS NR UL PRS relationship with UL BWP and component carrier

On properties of NR reference signals for UL positioning

- Multiple contributions provide description of the NR UL positioning reference signal design aspects [2], [3], [4], [5], [6], [9]. Based on review, we have following proposal for discussion/decision.

- Proposal for discussion

NR UL PRS design for FR1 and FR2 supports

    - Configurable NR UL PRS signal bandwidth (except based on NR PRACH signal)

  - Configurable NR UL PRS signal numerology (SCS, CP)

  - Configurable NR UL PRS frequency and time allocation

  - Possibility of UL TX beam sweeping

  - Possibility of UL RX beam sweeping

  - NR UL PRS transmissions with periodic, semi-persistent and aperiodic resource allocation

  - Allocation of dedicated NR UL PRS resources

    - i.e. not multiplexed with other cellular transmissions from gNB and system perspective

- The following offline consensus was reached based on discussion of the above proposal.

- Offline consensus

- Analyze further aspects of NR UL PRS design with respect to the following options

  - Use of UL beam sweeping at FR2

    - Beam sweeping includes possibility of quasi-omni transmission

  - Use of UL beam alignment at FR2 through DL reception and beam correspondence

  - UL Power control aspects

  - UL timing advance aspects

On NR UL PRS physical structure

- According to [4], [5], [6], [7], [9] main design aspects for NR UL PRS physical structure are summarized in the following proposal for discussion/decision.

Proposal for discussion

- RAN1 to continue discussion and evaluation of the NR UL PRS physical structure for NR UL positioning

  - FFS NR UL PRS pattern, density of REs, etc.

  - FFS number of antenna ports

  - FFS NR UL PRS signal and sequence and number of IDs for generation of NR UL PRS sequences

  - FFS NR UL PRS transmission schedule (i.e. transmission schedule from UEs)

  - FFS relationship with UL BWPs

- The proposal above was not discussed.

On NR UL PRS beam sweeping

Given that UL is typically more coverage limited than DL, the importance of UL RX beamforming at gNB was mentioned in [4]. The aspects of UL TX and UL RX beam correspondence were also discussed in [6], [7], [8], [9]. In [3], it was proposed to include the beam information of the DL/UL reference signals in the messages exchanged between UE, LMU, and LMF for supporting UL positioning.

Proposal for discussion

RAN1 to continue discussion on whether and how to support TX and RX beam-sweeping / beam management procedures for NR UL positioning

- The proposal above was not discussed, however the related agreements were made in section on NR UL PRS Design properties.

On NR UL PRS measurements

- Multiple companies expressed their view on measurements that can be used for NR UL positioning [3], [4], [5], [6], [7], [8], [10]. Following proposal is suggested for discussion/decision.

Proposal for discussion

- The following measurements for NR UL PRS at serving and neighbor gNBs are supported:

  - UL RSTD measurements

    - FFS details (e.g. RSTD b/w different configured UL PRS resources, beams, etc.)

  - UL Angle Of Arrival (AOA) measurements (including Azimuth and Zenith Angles)

  - UL RSRP (reference signal received power) measurements

    - FFS if UL RSRP measurement for the first arrival path is defined

    - FFS if UL RSRQ/SINR measurements are introduced for NR UL PRS

- The following offline consensus was reached based on discussion of the above proposal.

- Offline consensus

The following measurements for NR UL PRS at serving and neighbor gNBs are supported:

  - UL RTOA measurements

  - UL Angle Of Arrival (AOA) measurements (including Azimuth and Zenith Angles)

  - UL RSRP (reference signal received power) measurements

  - gNB RX-TX time difference measurements

- In [2], it was proposed to use pi/2-BPSK signal for UL PRS transmission at least for DFT-s-OFDM waveform. The proposal was raised during online discussion however no decision was made.

On gNB reporting for NR UL positioning

- As it was clearly seen in [3], [4], [5], [6], [7], [8], [10], positioning technique correlates with set of positioning measurements and reporting, thus, following proposal, which describes the report structure for NR UL positioning, is suggested for discussion/decision.

- Proposal for discussion

NR UL positioning in FR1 and FR2 supports

  - UL RSTD measurement for detected first arrival path and measurement quality

  - UL Angle Of Arrival measurement (Azimuth and Zenith)

  - UL RSRP measurements for NR UL PRS

    - FFS whether reports for all UL RX beams or subset of UL TX beams, etc.

The proposal above was not discussed.

On LMU support for NR UL positioning

In [1], the support of three LMU classes for NR UL positioning is proposed. Given that assumption on LMU deployment may have impact on physical layer aspects the following proposal is made:

- Proposal for discussion

RAN1 to discuss support of NR LMU for UL-based positioning with the following three classes:

  - Integrated with gNB

  - Co-sited with gNB and share antennas with gNB

  - Standalone with its own antennas

The proposal above was discussed. Based on discussion no impact on RAN1 was identified at this stage.

- Summary

In this contribution, we have provided overview of contributions on NR UL positioning aiming to select main aspects for discussion. Based on analysis of submitted contributions we have made potential proposals to be used as a starting point to facilitate and trigger further RAN1 discussion.

- References

R1-1900037, “System level evaluations for NR positioning”, Huawei, HiSilicon, 3GPP RAN1 Ad-Hoc 1901.

R1-1900150, “Views on NR uplink positioning techniques”, vivo, 3GPP RAN1 Ad-Hoc 1901.

R1-1900311, “NR RAT-dependent UL Positioning”, CATT, 3GPP RAN1 Ad-Hoc 1901.

R1-1900513, “Analysis of Techniques for NR UL Positioning”, Intel Corporation, 3GPP RAN1 Ad-Hoc 1901.

R1-1900630, “Discussion on UL only based Positioning”, LG Electronics, 3GPP RAN1 Ad-Hoc 1901.

R1-1900915, “RAT-dependent UL-only NR positioning techniques”, Qualcomm Incorporated, 3GPP RAN1 Ad-Hoc 1901.

R1-1901023, “Potential Positioning Techniques - UL based solutions”, Nokia, Nokia Shanghai Bell, 3GPP RAN1 Ad-Hoc 1901.

R1-1901091, “UL only based Positioning”, Samsung, 3GPP RAN1 Ad-Hoc 1901.

R1-1901183, “Aspects of UL-based NR positioning techniques”, Fraunhofer IIS, Fraunhofer HHI, 3GPP RAN1 Ad-Hoc 1901.

R1-1901196, “Uplink Positioning Solutions: design and evaluations”, Ericsson, 3GPP RAN1 Ad-Hoc 1901.