3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                                        					R1-190xxxx

Taipei, Taiwan, 21st- 25th Janunary 2019

Title:	Summary of UE Power Saving Schemes

Agenda item:	7.2.9.2.1

Source:	CATT

Document for:	Discussion

- 7.2.9.2.1 UE adaptation to the traffic and UE power consumption characteristics

- For power saving scheme with UE adaptation in time domain for further study

Offline Proposals (Captured in TR38.840)

The UE power saving schemes with UE adaptation to the traffic arrival in the time domain includes cross slot scheduling, single slot scheduling with a gap between PDCCH and PDSCH reception, and multi-slot scheduling.  For both cross-slot scheduling and single slot scheduling, the UE may achieve power consumption by switching to micro sleep after PDCCH reception.  For multi-slot scheduling, the UE may achieve power consumption by skipping the PDCCH monitoring at subsequent slots of PDSCH/PUSCH transmission.

Cross-slot scheduling

- Minimum K0 > 0 and aperiodic CSI-RS triggering offset is not within the duration - UE could switch to micro sleep right away after PDCCH reception – no addition PDSCH and CSI-RS signals reception within the given duration (e.g. the same slot)

  - It is known to the UE at PDCCH decoding

  - Extended micro sleep time and reduce the PDCCH processing in reducing UE power consumption

  - minimum K2 > 0 is essential to avoid the requirements of fast PDCCH processing

Offline Proposals

The general procedure for the study of the power saving scheme when cross-slot scheduling is used

- gNB semi-statically configures TDRA to the UE.

- All schedulable TDRA values have K0 > = x and K2 >= x where x > 0 ( taking account potential effects of BWP switching)

- All aperiodic CSI-RS triggering offsets are not smaller than the value  x

- UE decodes PDCCH and  retrieves the index of schedulable TDRA values

- UE could go to micro sleep after reception of last PDCCH symbol

- UE processes PDSCH at the indicated starting time from TDRA values

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HW[1] | Cross-slot scheduling with DRX, inactivity timer configuration, and On Duration. | [17.3%-26.6%] |  |  |  | (DRX cycle, inactivityTimer, onDurationTimer) Case 1(160,100,8), Case 2(160,40,8), Case 3(40,25,4), Case 4 (40,10,4) | Higher power saving gain observed from longer DRX cycle |
| ZTE [4][26] | Cross-slot scheduling with K0 > 0 indicated to the UE in advance | [24%-25%] |  |  |  | DRX configuration ( 160-100-8) |  |
| CATT[6] | Cross-slot scheduling with K0 > 0 indicated to the UE in advance | [0.39%-16.63%] |  |  |  | DRX cycle (160, 320) ms, FTP3 traffic model with traffic load (0.1, 3.2, 5, 10) | Higher power saving gain observed from higher traffic load |
| Apple [12][32] | Cross slot scheduling K0>0 The control of TDRA | [0%-30%] |  |  |  | New power model for same slot scheduling |  |
| QC[15][33] | Configuration of minimum DL scheduling offset that applies to K0 and A-CSI triggering offset | [6%-26%(FR1) 9%-27%(FR2)] |  |  |  | Method: SLS No C-DRX Traffic model: FTP3 |  |
| Samsung [19] | Cross-slot scheduling | [30%] |  |  |  | No C-DRX Traffic model: FTP3 x = 1 |  |
| Nokia, NSB [36][22] | Cross-slot scheduling K >0 | [25%] |  |  |  | DRX configuration (160, 40, 8) Trafic model FTP3 |  |

Update the text in 38.840 as follows:

Same slot scheduling

- Adaptation of TDRA configurations to achieve UE power saving – ensure the gap between PDCCH reception and PDSCH transmission known to the UE

- Adjustment of TDRA configuration

- Selection of TDRA entry in the TDRA table (e.g. K0 > 0) [38.214]

Agreements:

The general procedure for the study of the power saving scheme when same slot scheduling is used

- gNB semi-statically configures TDRA to the UE, subject to UE capability (if any).

- All schedulable TDRA values with K0=0 include at least Y symbols between the last PDCCH symbol and the first PDSCH symbol

  - Y is FFS

    - Note: the value of Y is necessary for power modelling in evaluation, although the specification (if specified) of Y value may also be done in the WI phase

  - Note: in case there is a TDRA entry with K0>0, it is assumed that the gap between the last PDCCH symbol in a first slot and the first PDSCH symbol in a second slot is no less than Y

- All aperiodic CSI-RS triggering offsets are greater than Y symbols

- UE decodes PDCCH and retrieves the index of schedulable TDRA values

- UE could go to micro sleep after reception of last PDCCH symbol

- UE processes PDSCH at the indicated starting time from TDRA values

- The following table is subject to further update, particularly regarding evaluation results/assumptions

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apple [12][32] | Same slot scheduling The control of TDRA | [15% ~ 30%] |  |  |  | New power model for same slot scheduling |  |

Offline Proposals (Captured in TR38.840)

Multi-slot scheduling – PDCCH decoding in one slot (e.g., one DCI, multiple DCI) supports scheduled PDSCH/PUSCH transmission over multiple slots.

- Achieving UE power consumption reduction by potentially skipping PDCCH monitoring at subsequent slots of PDSCH/PUSCH transmission.

Proposal:

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ZTE [4][26] | Multi-slot scheduling | [0.65%-1.9%] |  |  |  | DRX configuration ( 160-100-8) |  |
| Samsung[19] | Multi-slot scheduling:1 DCI format schedule N TBs across N consecutive slots. | [0%-45%] | N = 1-8 |  | 0-1.8ms | Baseline: N=1, PDCCH monitoring periodcity 1 slot, Trafic moddel FTP3, No C-DRX |  |

- For power saving scheme with UE adaptation to the DRX operation

The power saving scheme of power saving signal triggering UE adaptation to DRX operation is to configure the power saving signal before or at the beginning of the DRX ON duration to trigger UE waking up only when there is DL data arrival.   UE is not required to wake up at the DRX ON at least for PDCCH monitoring.  if the power saving signal is not detected.   The go-to-sleep signaling is used as the indication allowing UE going back to sleep state after completion of PDSCH reception during the DRX ON period to further reduce the UE power consumption.

Agreement

The general procedure for the study of UE adaptation to the DRX operation  is as follows,

  - UE adaptation of its behavior to the DRX operation for UE power consumption reduction

    - When is configured with power saving signal/channel, power saving signal/channel asthe indication whether to wake up or not before or at the beginning of DRX ON duration

      - At least for the indication of PDCCH monitoring

    - Preparation period is used for ,( e.g., to perform channel tracking, CSI measurements, beam tracking),

      - Preparation period can be used in preparation for the PDCCH/PDSCH decoding

      - Preparation period could be before or during the DRX ON duration

      - Network can indicate UE to report CSI before or after the power saving signal/channel (if configured) during the preparation period

      - Network can indicate additional RS transmission (e.g., CSI-RS, TRS, SSB and power saving signal) at the preparation period

    - Go-to-sleep signaling as the indication allowing UE going  to sleep state, e.g.,

      - MAC-CE

      - DCI

      - Power saving signal/channel

    - Constraints on scheduling DCI during DRX_ON

- The following table is subject to further update, particularly regarding evaluation results/assumptions

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZTE [4][26] | One stage and two stage power saving signal triggering UE wake up Power saving signal triggering UE go to sleep | [6%-27%] [28%-50%] |  |  |  | DRX configuration (160-10-8,160-40-8,160-100-8) DRX configuration (40-10-4, 40-25-4, 160-40-8, 160-100-8, 320-80-10, 320-200-10) |  |  |
| Oppo [5][27] | WUS triggering DRX | [4%-71%] |  |  |  | Traffic model: FTP3, IM |  |  |
| CATT[6] | Power saving signal triggering UE wakeup before DRX ON with on-demand PS-RS at pre-processing window | [36%-79%] |  |  |  | DRX cycle (40,160, 320) ms, FTP3 traffic model with traffic load (0.1, 0.8,3.2) |  |  |
| Intel [30] | Power saving signal triggering UE wakeup and go-to-sleep signal triggering UE back to sleep | [9%-73% for WUS 52%-82% for GTS ] |  |  |  | DRX configuration (40,10,4),(160, 100, 8), (320,80, 10) Traffic model: FTP3, IM, VoIP, web browsing, video streaming |  |  |
| LG[31] | WUS signal triggered DRX with inactivity timer running | [1%-75%] |  |  |  | Traffic model: FTP3 with file size 0.5 Mbyte |  | UPT loss |
| QC[15][33] | PDCCH-based WUS signal triggering UE wakeup | [2%-70%] |  |  |  | Method: Numerical simulation DRX configuration (40,10) (40,25) (160,100) Traffic model: FTP3, IM, Web browsing |  | CSI measurements in pre-processing before the 1st PDSCH reception |
| Samsung [19] | 1-bit Power Saving signal triggering UE wakeup at DRX ON | [0%-90%] |  |  |  | DRX configurations DRX cycle:80/160/320ms Inactivity Timer: 10-100ms OnDuration:2-8ms) Baseline : DRX cycle:160ms, Inactivity timer:100ms, on Duration:8ms |  |  |
| Huawei [37,38,39] | Power saving signal triggering UE wake-up or power saving signal triggering go-to-sleep signal | [WUS: up to 40% GTS: (40% - 85% for packet size 0.1Mbytes, 35% - 75% for 0.5Mbytes)] |  |  |  | DRX cycle (320/160/40), Inactivity timer(200/100/80/40/25/10) On duration timer (10/8/4) Packet sizes 0.1/0.5Mbytes |  |  |
| MediaTek [25] | Power saving signal triggers DRX OnDuration, where power saving signal is allocated in the preparation windown allowing periodic BM/CSI report and/or SRS transmission around a SS burst | [9%-30%] |  |  |  | Traffic model: FTP3, IM, VoIP |  |  |
| Apple[40] | PDCCH based Go-to-sleep signal | [GTS 37%~71%] |  |  |  | FTP3 traffic (inter-arrival time = 2.5ms, 100ms) |  | With no DRX |
| Sony [41] | WUS triggering wakeup for DRX | [7-11% for PDCCH-based WUS 11-20% for low power WUS] |  |  |  | FTP3 traffic. DRX = {160,100,8} and {160,40,8} |  |  |

- Dynamic DRX configuration –

  - UE is configured with multiple DRX configuration

  - Dynamic selection of DRX configuration by gNB from multiple DRX configurations (e.g., traffic, mobility)

    - UE assistance information may be considered

  - Adaptive parameters setting of one DRX configuration

    - UE assistance information may be considered

    - DRX parameters are indicated by gNB

    - Adaptive UE behavior in the DRX operation (e.g, restart the inactivity timer)

- The following table is subject to further update, particularly regarding evaluation results/assumptions

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HW[1] | DCI indicated Inactivity timer | [7.6%-42.6%] |  |  |  | DCI to trigger the restart of the Inactivity timer |  |
| MediaTek [3] [25] | Adaptative DRX configuration based on traffic type | [42%-70%] |  |  |  | DRX configuration based on the traffic type |  |
| Oppo [5][27] | Adaptation of Inactivity timer | [38%-46%] |  |  |  | Traffic model: FTP3, IM |  |
| Sony[7][41] | - DRX with dynamic inactivity timer - Dynamic DRX indication | [20%-55% 7%-11%] |  |  |  | (1) DRX configuration (320,200, 10) traffic model: FTP3, IM (2): DRX configuration (160,100,10) and (160,40,10). Traffic model: FTP3 |  |
| Nokia, NSB [36][22] | Cross-slot scheduling K >0 | [25%] |  |  |  | DRX configuration (160, 40, 8) Trafic model FTP3 |  |
| Nokia, NSB [36][22] | Static DRX configuration (short DRX embedded in long DRX as per 38.321 Section 5.7) | [10%-65%] |  |  |  | DRX configuration LongCycle: 320, 160, 20 drx-InactivityTimer 200, 25, 20, 10, 4 drx-onDurationTimer: 10, 8, 4, 2 drx-ShortCycle: 40, 25, 4 drx-shortCycleTimer: 16, 8, 4 Trafic model FTP3 | The case for the evaluations would change one parameters at a time |

- For power saving scheme with UE adaptation in frequency domain for further study

Offline Proposal

The UE BWP adaptation  is to dynamically switch the BWP by gNB  based on the traffic to support efficient operation of BWP switching in reducing the UE power consumption.

The UE adaptation to the CA/DC is to fast activation/deactivation of the SCell by gNB based on the traffic to support efficient operation for fast SCell activation/deactivation in achieving UE power saving.

The UE power saving schemes for the UE adaptation in frequency domain for further study are as follows,

  - BWP -  UE adaptation to different BWP

    - RS to assist UE channel tracking and measurements to assist BWP switching

      - Including CSI measurements (UE processes one BWP at a time)

    - Enhancement of L1 signaling, e.g., power saving signal or DCI for power saving, in triggering the BWP switching

    - Association of BWP and DRX configuration

    - UE assistance information could be considered

  - CA/DC –

    - Quick activation/de-activation (e.g.,L1 signaling, MAC CE enhancement)

    - Adaptation of PDCCH monitoring/search space on activated SCell

      - Including cross carrier scheduling

    - Power adaptation based on the operation in a group of cell in power efficient way

    - CSI/RRM measurements and beam management at non-active SCell

    - UE assistance information could be considered

- The following table is subject to further update, particularly regarding evaluation results/assumptions

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek [3][25] | BWP adaptation in combination of SCell activation | [30%-65%] |  |  |  | Traffic model: FTP/Video, IM , VoIP |  |
| CATT[6] | Power saving signal triggering BWP adaptation with on-demand power saving RS (PS-RS) | [5%-44%] |  |  |  | DRX cycle (40,160, 320) ms, FTP3 traffic model with traffic load (0.1, 0.8,3.2) |  |
| Apple [12][32] | UE feedbacks the preferred BWP UE-initiated Fast SCell activation | [Without GTS: 33~65% With GTS: 45%~80% 3%-43%] |  |  |  | FTP3 with size (0.1, 0.5, 2 and 4 Mbyte) | Power saving gain number is against fixed bandwidth part operation. |
| Samsung [19] | Cross-carrier scheduling among carriers in FR1 and FR2 | [40%-57%] |  |  |  |  |  |
| Qualcomm [41] | SCell dormancy. Applicable to the following schemes: - Dormant BWP - SP search space config | [33%] |  |  |  | Method: SLS Traffic model: FTP3. 4CC | Traffic scales with number of carriers |

- Power saving scheme with UE adaptation in antenna domain for further study

The UE power saving schemes with UE adaptation to the traffic is to reduce the number of antenna/panels or MIMO layers indicated by the network semi-statically or dynamically to achieve the UE power saving.

Proposal:

- The following table is subject to further update, particularly regarding evaluation results/assumptions

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HW[1] | 2Rx for PDCCH-only slot. Between the two slots with PDSCH, within the timer, 4Rx is used for the PDCCH-only slot; For the PDSCH + PDCCH slot and PDSCH-only slot, 4Rx is used | [12%-25%] |  |  |  | FTP 3 with 0.1Mbytes and 0.5Mbytes C-DRX configurations are set to Case1(160, 10, 8), Case2(160, 40, 8) and Case 3(160, 100, 8) | No system performance impact |
| ZTE[4] | Adaptive configuration with 2 or 4 antennas | [12%-14%] |  |  |  | FTP 3 with 0.1Mbytes CDRX configuration: 160-100-8 |  |
| MediaTek [3][25] | BWP-based adaptation to reduce RX number for PDCCH monitoring | [15%-20%] |  |  |  | Traffic model: FTP/Video, IM , VoIP | No system performance impact |
| Apple [12][32] | UE is configured with maximum layer of 2 | [30%] |  |  |  | Continuous PDCCH monitoring | No system performance impact |

- UE power saving scheme in achieving reducing PDCCH Monitoring

The UE power consumption can be reduced when the number of UE PDCCH monitoring occasions and/or the number of PDCCH blind decoding is reduced.

The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows,

  - Triggering of PDCCH monitoring – dynamic trigger through L1 signal/signaling

    - Power saving signal triggering PDCCH monitoring

    - Go-to-sleep signaling to skip PDCCH monitoring

  - PDCCH skipping -

    - DCI based indication  for PDCCH skipping (e.g., indication in DCI content, new SFI state).

    - L1 signal/signaling (other than DCI) based triggering  -

  - Multiple CORESET/search space configurations

    - Configuration of different PDCCH periodicities with dynamic signaling

    - Adaptation of CORESET/search space configuration – DCI/timer/HARQ-ACK based indication

    - Dynamic/semi-persistent CORESET/search space ON/OFF

    - Adaptation between DRX ONduration timer and inactivitytimer

    - Separated PDCCH monitoring of DL and UL

  - L1 signaling triggering to assist  UE in reducing the number of PDCCH blind decoding

  - Reduced PDCCH monitoring on SCell (including cross carrier scheduling)

  - Network assistance –  RS is dynamically transmitted based on the need to assist UE performing synchronization, channel tracking, measurements and  channel estimations before PDCCH decoding

Proposal:

- The following table is subject to further update, particularly regarding evaluation results/assumptions

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vivo [24] | (1)Dynamic switching of PDCCH monitoring periodicities by DCI (2)DCI explicit indication of PDCCH monitoring occasions within a periodicity (3)UE monitoring of PDCCH search space for retransmission is triggered by decoding failure of SPS PDSCH | [2%-93%] | [(1) 27%-91% (for gaming), 2%-93% (for FTP3); (2) 57%-85% (for gaming), 2%-93% (for FTP3); (3) 20%] |  |  | numerical simulation Traffic model: Gaming and FTP3 for case (1)(2), VoIP for case (3) | The Rel-15 baseline schemes are: PDCCH monitoring with and without C-DRX for case (1)(2) UE monitoring of PDCCH search space for retransmission is configured by RRC for case (3) |
| MediaTek [3][25] | Before data arrival - Switching on/off PDCCH monitoring for next DRX on duration - Adapt PDCCH monitoring period to the data traffic - Adapt scheduling delays, including K0 and K2, to the data traffic - Additional adaptation to DRX After data arrival - Adapt PDCCH monitoring period during data inactivity - Adapt scheduling delays, including K0 and K2, during data inactivity - Combined processing time adaptation | [5%-44%] |  |  |  | Traffic model: FTP/Video, IM , VoIP | Background activity window to optimize the sleep time and power consumption by aggregating periodic CSI and power saving signal monitoring around SS burst can improve power saving gain. |
| ZTE [4][26] | Search space skipping | [15%-89%] |  |  |  | DRX configuration (40-10-4, 40-25-4, 160-40-8, 160-100-8, 320-80-10, 320-200-10) |  |
| CATT[6] | Power saving signal triggering PDCCH monitoring with on-demand PS-RS | [3%-35%] |  |  |  | DRX cycle (40,160, 320) ms, FTP3 traffic model with traffic load (0.1,0.8, 3.2) |  |
| Sony[7] | Reduced PDCCH monitoring while the inactivity timer is running | [21%-37%] |  |  |  | DRX configuration (160,100, 8) with FTP3 traffic model, file size 0.1Mbyte |  |
| LG[31] | Different PDCCH periodicity during DRX ON with inactivity timer | [22%-36%] |  |  |  | Traffic model: FTP3 with file size 0.5 Mbyte |  |
| QC[15][33] | PDCCH skipping | [9%-43%(FR1) 19%-60% (FR2)] |  |  |  | Method: SLS No C-DRX Traffic model: FTP3 |  |
| Samsung [19] | 1-bit Power Saving signal triggering PDCCH monitoring adaptation on search space configuration Reduction on PDCCH candidate in achieving PDCCH blind decoding reduction Power saving signal triggering SPS PDSCH reception | [41% 0% - 80% Up to 30% 29%-31%] |  |  |  | Traffic model: FTP3 with file size 0.1 Mbyte baseline: T_PDCCH = 0.5ms, D = 0.5 ms |  |
| Nokia, NSB [36][22] | Reduce PDCCH blind decoding | [3-24%] |  |  |  | DRX configuration (160, 40, 8) Trafic model FTP3 |  |
| OPPO[5][27] | PDCCH skipping | [49.3% and 75.52%] |  |  |  | System level simulation. The referrence Power configuration is DRX as agreed in previous meetings. Traffic model: FTP3 and IM |  |

- Assistance information for UE adaptation for further study

The UE assistance information for any UE power saving scheme is for UE to provide the assistance information to the network in configuration for UE adaptation to achieve power saving gain. UE assistance information reported by UE for power saving is a UE recommendation and the network shall make the final decision on whether and how to use that information.

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

Offline Proposals:

- Capture in the TR:

New section 5.2.3 “Additional RS used for UE power saving”  in TR

Additional RS  is the RS provided  by gNB to assist UE in performing e.g.,fine synchronization, channel/beam tracking, and/or CSI/RRM measurements in addition to the existing RS in Rel-15.  The additional RS was proposed for study  for power saving schemes for e.g, UE adaptation to the DRX operation,  BWP switching, fast SCell activation, reducing PDCCH monitoring, and/or  RRM measurements.   RS design is assumed to reuse Rel-15 waveform.  Power saving signal could be used to meet the purpose of additional RS.

- Reference

- R1-1900040	Power saving schemes	Huawei, HiSilicon

- R1-1900145	Discussion on UE adaptation to the traffic and UE power consumption characteristics		vivo

- R1-1900192	Adaptation designs for NR UE power saving		MediaTek Inc.

- R1-1900226	On adaptation aspects for NR UE power consumption reduction		ZTE Corporation

- R1-1900305	UE Adaptation to the Traffic and UE Power Consumption Characteristics		OPPO

- R1-1900344	UE Power saving schemes with adaptation		CATT

- R1-1900380	Power consumption adaptation for NR	sony

- R1-1900421	Discussion on UE power saving schemes with adaption to UE traffic	CMCC

- R1-1900508	UE adaptation to the traffic and UE power consumption characteristics	Intel Corporation

- R1-1900599	Discussion on UE adaptation to UE power consumption characteristics	LG Electronics

- R1-1900719	Discussion on UE adaptation to the traffic and UE power consumption characteristics	Spreadtrum Communications

- R1-1900753	Power saving techniques based on UE adaptation	Apple Inc.

- R1-1900813	On power saving techniques 	InterDigital, Inc.

- R1-1900818	PDSCH resource allocation restriction for power saving	ASUSTEK COMPUTER (SHANGHAI)

- R1-1900911	UE Adaptation to the Traffic and UE Power Consumption Characteristics	Qualcomm Incorporated

- R1-1900947	UE power saving schemes	Motorola Mobility, Lenovo

- R1-1900980	Discussion on UE adaptation to the traffic and UE power consumption characteristics	NTT DOCOMO, INC.

- R1-1900992	Power saving during scheduling opportunity is sparse	NEC Corporation

- R1-1901087	On UE adaptation schemes	Samsung

- R1-1901118	Discussion on UE traffic adaptation and power consumption characteristics	PANASONIC

- R1-1901166	Adaptation aspects of NR UE power saving	Ericsson

- R1-1901188	On UE adaptation to the traffic	Nokia, Nokia Shanghai Bell

- R1-1900039	Remaining issues for evaluation methodology for UE power saving	Huawei, HiSilicon

- R1-1900144	Evaluation Results and remaining issues on evaluation methodology for UE power saving		vivo

- R1-1900191	Evaluation of NR power saving designs	MediaTek Inc.

- R1-1901282	Evaluation results of UE power saving	 (Revision of R1-1900225) 	ZTE Corporation

- R1-1900304	Evaluation methodology for UE power saving	OPPO

- R1-1900343	Evaluation Results of UE Power Saving Schemes	CATT

- R1-1900379	Evaluation results for power saving study	sony

- R1-1900507	Evaluation results of UE power saving signals	Intel Corporation

- R1-1900598	Numerical results for UE power saving schemes	LG Electronics

- R1-1900752	Evaluation of UE power saving techniques	Apple Inc.

- R1-1900910	Evaluation Methodology and Results for UE Power Saving	Qualcomm Incorporated

- R1-1901086	Evaluation results for UE power savings	Samsung

- R1-1901165	Evaluations and modeling of UE power consumption	Ericsson

- R1-1901187	Initial evaluation results and consideration on DRX settings	Nokia, Nokia Shanghai Bell

- R1-1900041	Power saving signal/channel/procedure for triggering adaptation  Huawei, HiSilicon

- R1-1901245	Analysis and evaluation of go-to-sleep signaling scheme Huawei, HiSilicon.

- R1-1901246	Analysis and evaluation of wake up signaling scheme Huawei, HiSilicon.

- R1-1900754    Power Saving Techniques based on Explicit Network Indication, Apple

- R1-1900381	Power Consumption Triggering for NR, Sony