3GPP TSG-RAN WG2 #100                                                            	R2-1712414

Reno, USA, 27th November – 1st December 2017

Source:	CATT

Title:	Capturing of default configuration of SRB.

Agenda Item:	10.4.1.3.4

Document for:	Discussion and Decision

Introduction

In RAN2 #99bis meeting, the following agreements about SRB3 and split SRBs were made:

Agreements

1:	The LCID of SRB1S/SRB2S should be the same with that of NR SRB1/SRB2 which can be 1/2 to align with the SRB ID. The LCID of the SRB3 should be 3.

2:	The default configurations of SRB1S/SRB2S should be the same with that of NR SRB1/SRB2. The default configurations of NR SRB1 and SRB2 should be same except for the priority. The default configurations of the SRB3 should be the same with that of SRB1S.

In the agreement, UE can be configured with default configuration for SRB3 and SRB1S/SRB2S in EN-DC and can be configured with default configuration for SRB1 and SRB2 in SA. The LCH IDs for SRBs and split SRB are fixed and align with SRB identity.

Email discussion 99bis #16 provides a combined TP for 38.331, however the above agreements are not included in this TP.

This contribution, we discuss how to capture the above agreements in 38.331.

Discussion

LCH-Config and associated RB

The structure of the LCH-Config in the latest 38.331 TP is:

LCH-Config ::=								SEQUENCE {

logicalChannelIdentity						LogicalChannelIdentity		OPTIONAL,	-- Cond LCH-SetupOnly

-- Associate the logical channel with an SRB or a DRB:

servedRadioBearer ::=						CHOICE {

srb-Association								SRB-Identity,

drb-Association								DRB-Identity

}																			OPTIONAL,	-- Cond LCH-SetupOnly

ul-SchedulingRestriction					UL-SchedulingRestriction		OPTIONAL,	-- Cond LCH-Setup

reestablishRLC								ENUMERATED{true}		OPTIONAL, 	-- Need ON

rlc-Config									RLC-Config      				OPTIONAL,	-- Cond LCH-Setup

mac-LogicalChannelConfig					LogicalChannelConfig			OPTIONAL	-- Cond LCH-Setup

}

LCH ID of SRBs are fixed in the specification to 1, 2 or 3 and LCH ID of SRB aligns with SRB ID.  When the SRB is being setup or modified, only either LCH ID or SRB ID should be present to address which SRB is being configured. In section 5.3.5.4.4 of the 38.331 TP, the network uses LCH ID to address the modified RB. Only the LCH ID should be present when SRB is being setup or modified, the SRB ID should not be present. This means that the IE of servedRadioBearer should only be associated with DRB ID as the association between SRB ID and LCH is fixed in the specification.

LCH ID is used as the index to address the configured RB at setup and modification. Hence the LCH ID should be always present to associate the LCH configuration with the RB. LCH ID should be mandatory.

For DRB ID, in section 5.3.5.4.4, “An already configured LCH must not be re-associated with another radio bearer. Hence servedRadioBearer must not be present in this case (RB modification)” is specified.  In this rule, the DRB ID can only be present when the DRB is being setup, i.e. the LCH is being setup which doesn’t equal to1, 2 and 3 otherwise it is not present.

Proposal 1: In LCH-Config field, IE logicalChannelIdentity should be present always and used to address configured RB. The servedRadioBearer IE should only be associated with DRB ID and it is present only when the logical channel is being setup which doesn’t equal to 1, 2 and 3 considering that the association between LCH ID and SRB is fixed in the specification.

Default configuration for rlc-Config and mac-LogicalChannelConfig

The UE can be configured to use default configuration or specified configuration of rlc-Config or mac-LogicalChannelConfig for SRB. Current ASN.1 of LCH-Config can be used to setup and modify a SRB or DRB with specified configuration when the IE rlc-Config and mac-LogicalChannelConfig present. But the structure can’t be implemented to inform UE to configure SRB with default configuration.

In order to configure UE with default configuration, there are two options to modify the current ASN.1 of LCH-Config.

- Option1: retain current structure, specify the present condition

Specify the present condition of rlc-Config and mac-LogicalChannelConfig such that when LCH ID is 1/2/3, if the LCH is being setup and the IE is not present, the default configuration is used for the associated SRB. If the LCH has been configured, if the IE is not present, no action relating to these IEs (i.e. SRB/DRB is modified).  When LCH ID doesn’t equal to 1, 2 and 3, if the logical channel is being setup, rlc-Config and mac-LogicalChannelConfig must present. This method should explicitly specify the actions of the UE according to the LCH ID when the rlc-Config or mac-LogicalChannelConfig IE is not present.

- Option2: introduce choice branch including defaultValue and explicitValue

The IE of rlc-Config or mac-LogicalChannelConfig is choice type, which can be 1) explicitValue for explicit configuration or 2) defaultValue to inform UE to configure SRB with default configuration. The default configuration can only be used for SRB, i.e. the logicalChannelIdentity is 1, 2 or 3. The explicitValue can be used with any LogicalChannelIdentity. If the IE is not present, no action is needed.

Option1 can save one bit overhead compared with Option2, but description of present condition is not straightforward and confusing.   Option2 is clearer than option 1. Therefore option 2 is preferred.

Proposal 2: rlc-Config and mac-LogicalChannelConfig should be choice type which can be configured to defaultValue when LCH ID is 1, 2 or 3 or configured to explicitValue for any LCH ID.

ASN.1 for LCH-Config

According to the above discussion, the ASN.1 for LCH-Config should be modified as following:

LCH-Config ::=								SEQUENCE {

logicalChannelIdentity						LogicalChannelIdentity

-- Associate the logical channel with a DRB:

servedRadioBearer								DRB-Identity                OPTIONAL,	-- Cond DRBLCH-SetupOnly

ul-SchedulingRestriction					UL-SchedulingRestriction		OPTIONAL,	-- Cond LCH-Setup

reestablishRLC								ENUMERATED{true}		OPTIONAL, 	-- Need ON

rlc-Config							CHOICE {

explicitValue						RLC-Config,

defaultValue						NULL

}		OPTIONAL,	-- Cond LCHSetupCond

mac-LogicalChannelConfig				CHOICE {

explicitValue						LogicalChannelConfig,

defaultValue						NULL

}		OPTIONAL,   -- Cond LCHSetupCond

}

| Conditional presence | Explanation |
| --- | --- |
| DRBLCH-SetupOnly | The field is optionally present, need ON, if the logical channel which is associated with one DRB is being setup (i.e. logicalChannelIdentity doesn’t equal to 1, 2 and 3 ), it is present; otherwise it is not present. |
| LCH-Setup | The field is optionally present, need ON, and the filed should be mandatory present when the logical channel setup. |
| LCH-SetupCond | This filed is optionally present, need ON, and the filed should be mandatory present when logical channel setup, and the defaultValue can only be chosen when LCH ID is 1,2 or 3, the explictValue can be chosen for any LCH ID. |

A TP is provided in Annex capturing above changes.

Proposal 3: agree on the TP (provided in Annex) in capturing the above changes.

Conclusion

In this contribution, we discuss the IEs in LCH-Config and their present conditions for SRB considering default configuration and propose:

Proposal 1: In LCH-Config field, IE logicalChannelIdentity should be present always and used to address configured RB. The servedRadioBearer IE should only be associated with DRB ID and  it is present only when the logical channel is being setup which doesn’t equal to 1, 2 and 3 considering that the association between LCH ID and SRB is fixed in the specification.

Proposal 2: rlc-Config and mac-LogicalChannelConfig should be choice type which can be configured to defaultValue when LCH ID is 1, 2 or 3 or configured to explicitValue for any LCH ID.

Proposal 3: agree on the TP (provided in Annex) in capturing the above changes.

Reference

[1]  RAN2#99 bis, Chairman notes.

[2] email discussion 99bis#16 NR TS 38.331

[3] TS 36.331

Annex : Text Proposal for TS 38.331

- –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entites and of a primary cell (PCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELL-GROUP-CONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig	::= 						SEQUENCE {

cellGroupId									CellGroupId														OPTIONAL,	-- Cond SCG

-- Logical Channel configuration and association with radio bearers:

logicalChannel-ToAddModList 					SEQUENCE (SIZE(1..maxLCH)) OF LCH-Config					OPTIONAL,

logicalChannel-ToReleaseList					SEQUENCE (SIZE(1..maxLCH)) OF LogicalChannelIdentity		OPTIONAL,

-- Parameters applicable for the entire cell group:

mac-CellGroupConfig							MAC-CellGroupConfig												OPTIONAL,	-- Need ON,

rlf-TimersAndConstants						RLF-TimersAndConstants											OPTIONAL,	-- Need ON

-- Serving Cell specific parameters (PCell and SCells)

pCellConfig									PCellConfig														OPTIONAL, 	-- Need ON

sCellToAddModList							SCellToAddModList												OPTIONAL,	-- Need ON

sCellToReleaseList							SCellToReleaseList												OPTIONAL	-- Need ON

}

CellGroupId ::=								INTEGER (1.. maxSCellGroups)

-- Configuration of one logical channel:

LCH-Config ::=								SEQUENCE {

logicalChannelIdentity						LogicalChannelIdentity

-- Associate the logical channel with a DRB:

servedRadioBearer								DRB-Identity                OPTIONAL,	-- Cond DRBLCH-SetupOnly

ul-SchedulingRestriction					UL-SchedulingRestriction	OPTIONAL,	-- Cond LCH-Setup

reestablishRLC								ENUMERATED{true}		OPTIONAL, 	-- Need ON

rlc-Config							CHOICE {

explicitValue						RLC-Config,

defaultValue						NULL

}		OPTIONAL,	-- Cond LCHSetupCond

mac-LogicalChannelConfig				CHOICE {

explicitValue						LogicalChannelConfig,

defaultValue						NULL

}		OPTIONAL,   -- Cond LCHSetupCond

}

LogicalChannelIdentity ::= 					INTEGER (1..FFS)

-- CHECK: Keep scheduling restriction here (inside the logical channel of the cell group) or move it to PDCP?

UL-SchedulingRestriction ::= 				CHOICE {

-- The UE shall not serve the PDCP UL TX entity of this DRB via this LCH.

ul-Disabled									BOOLEAN,

-- The UE shall serve the PDCP entity of this DRB via this LCH only if the amount of data available in PDCP exceeds the given threshold.

ul-SplitThreshold							ENUMERATED { b0, b100, b200, b400, b800, b1600, b3200, b6400,

b12800, b25600, b51200, b102400, 	b204800, b409600, b819200, spare1},

-- The UE shall use this LCH for data duplication, i.e., the UL PDCP entity shall provide duplicates

-- of the PDCP PDUs to this Logical Channel if duplication is activated on MAC level for this logical channel

ul-Duplication								BOOLEAN

}																											OPTIONAL	-- Need ON

-- Serving cell specific MAC and PHY parameters for a PCell or PSCell:

PCellConfig ::=						SEQUENCE {

-- Parameters for the synchronous reconfiguration to the target PCell/PSCell:

synchronousReconfiguration 			SEQUENCE {

pCellConfigCommon					ServingCellConfigCommon,

newUE-Identity						C-RNTI,

t304								ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000-v1310},

rach-ConfigDedicated				RACH-ConfigDedicated													OPTIONAL,	-- Need N

}																												OPTIONAL,	-- Cond HO

pCellConfigDedicated				ServingCellConfigDedicated													OPTIONAL,	-- Need M

}

SCellToReleaseList ::=				SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex

SCellToAddModList ::=				SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig

SCellConfig ::=						SEQUENCE {

sCellIndex							SCellIndex,

sCellConfigCommon					ServingCellConfigCommon														OPTIONAL,	-- Cond SCellAdd

sCellConfigDedicated				ServingCellConfigDedicated													OPTIONAL,	-- Cond SCellAddMod

}

-- TAG-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| DRBLCH-SetupOnly | The field is optionally present, need ON, if the logical channel which is associated with one DRB is being setup (i.e. logicalChannelIdentity doesn’t equal to 1,2 and 3 ), it is present; otherwise it is not present. |
| LCH-Setup | The field is optionally present, need ON, and the filed should be mandatory present when the logical channel setup. |
| LCH-SetupCond | This filed is optionally present, need ON, and the filed should be mandatory present when logical channel setup, and the defaultvalue can only be chosen when LCH ID is 1,2 or 3, the explictvalue can be chosen whatever the LCH id is. |