3GPP TSG-RAN WG2 #100	Tdoc R2-1712533

Reno, Nevada, USA, Nov 27th – Dec 1st 2017	(Revision of R2-1710480)

Agenda Item:	10.4.1.8

Source:	Ericsson

Title:	Establishment causes for NR

Document for:	Discussion, Decision

# Introduction

This contribution is an update of R2-1710480 [1] (only minor changes).

In a response [2] to a RAN2 LS sent to CT1 on unified access control [3] CT1 asks RAN2 for information related to expected changes to establishment causes and call types:

While doing the assessment of the feasibility to map an access attempt to an access category, the following additional questions for RAN2 were raised:

Question 3: Will RAN2 still require NAS to provide RRC establishment cause or the Call Type or both.

Question 4: If the answer to Question 3 is Yes, does RAN2 expect that there will be changes to the existing RRC establishment cause and call types defined for E-UTRA?

At the SA1#79 meeting, a CR [5] on requirements for unified access control for 5G to TS 22.261 [6] was agreed and SA1 sent a reply LS [4]  to e.g. RAN2 to take appropriate actions.

Further, at the CT1#106 meeting, CT1 sent an LS [10] to SA1 and RAN2, about requirements on unified access control and their implications.

This contribution further elaborates on information included in a connection request and in particular aspects of access category, call type and establishment cause in NR.

# Discussion

## Use of establishment cause and call type in LTE

In E-UTRAN and over RRC, establishment causes are used in RRC Connection Request or Resume Request to indicate the reason for establishment. The main purpose of this is to allow differentiation by the network, dependent on cause. What RAN needs to know is how important it is to admit that UE at that point to allocate resources between RA and Initial Context Setup. For example, the network may choose to treat certain establishment causes with higher priority than others. Lower priority accesses may be rejected in case of congestion.

The UE NAS will, when requesting (or requesting to resume) a signaling connection establishment to lower layers, provide an RRC establishment cause and a call type. The RRC establishment cause is then included in Msg3 and the call type is used for purposes of access control.

For RRC Connection Request and RRC Connection Resume Request, the establishment/resume causes specified for EUTRAN are: Emergency, High Priority Access, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare (For NB, it is mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, spare3, spare2, spare1)

The call types, (3GPP TS 24.301) are: "originating signalling", “emergency calls”, "originating MMTEL voice" , "originating MMTEL video", "originating SMSoIP", "originating SMS", "terminating calls", "originating calls", "mobile originating CS fallback".

The call types are input from NAS to RRC to perform barring checks as described in 36.331. The call types and the establishment causes seems to be largely the same, but there is no real 1-1 mapping (for example, delayTolerantAccess is not a call-type but EAB “call type” serves same purpose) The purpose of them are however both to allow for a differentiated handling, through either prevention or prioritization of an access or resource request. For NR, there seems to be opportunities to align these two mechanisms.

## Access Categories in Unified Access Control

In the agreed SA1 CR [5] on access control, a number of access categories are defined as can be seen in Table 1:

Table 1: Access Categories, from [5]

| Access category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 (NOTE 1) | All | MO signalling resulting from paging |
| 1 (NOTE 2) | One or some of Access Classes 11-15 are set. At least one of them is valid in the registered PLMN and justified its priority handling by the registered PLMN with regards to access control. | All |
| 2 (NOTE 3) | UE is configured for delay tolerant service and subject to access control for access category 2, which is judged based on relation of UE’s HPLMN and the registred PLMN. | All |
| 3 | All except for the cases of access categories 1-2. | Emergency |
| 4 | All except for the cases of access categories 1-2. | MO signalling |
| 5 | All except for the cases of access categories 1-2. | MMTEL voice |
| 6 | All except for the cases of access categories 1-2. | MMTEL video |
| 7 | All except for the cases of access categories 1-2. | SMS |
| 8 | All except for the cases of access categories 1-2. | MO data that do not belong to any other access categories |
| 9-31 |  | Reserved standardized access categories |
| 32-63 | All except for the cases of access categories 1-2 and except for roaming-UEs | Based on operator classification |
| NOTE 1: Access category 0 is not barred. NOTE 2: Access Classes 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Classes 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. If the barring control information contains flag for “unbarred” for at least one of these valid Access Classes, all access attempts from the UE require priority handling and fall into access category 1. Otherwise the UE does not require priority handling with regards to access control and other access categories apply. Access category 1 is not barred. NOTE 3: The barring parameter for access category 2 is accompanied with information on whether the access control applies to UEs registered in UE’s HPLMN/EHPLMN, the most preferred VPLMN, or other PLMNs. |  |  |

The above list seems to include also aspects similar to establishment cause and call type, at least for standardized categories. We can make the following observation:

Establishment causes and call types may be a subset of what can be used to determine new unified access category

Based on this, it seems reasonable to consider if the access category determination, i.e., what events on higher layers that should map to what access categories, is controlled by NAS and signalled to RRC we think that at least the call type can be directly replaced with the access category. This would also allow that future services, slices or whatever distinctions desired can be applied without changing the framework, by simply changing the rules for what access category to use.

For an NR UE, a request from NAS including new access category removes the need of a call type indication.

The establishment cause value is, as mentioned above, also indicated by higher layers in LTE.

According to TS 24.301, for a single NAS event and call type, there could be different mappings to establishment causes. For example, for NAS event Tracking Area Update, there could be one and the same call type “originating signalling” but the establishment cause can differ. See below extract from 3GPP TS 24.301 on Tracking Area update:

| If the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [17]) and there is a pending request from upper layers to transmit user data related to an exceptional event, the RRC establishment cause shall be set to MO exception data. | "originating signalling" |
| --- | --- |
| If the UE is requesting resources for V2X communication over PC5, the RRC establishment cause shall be set to MO signalling. (See Note 1) | "originating signalling" |

We believe that many of these rules can be referred to an operator-configured access category rather than specified with specific mappings between NAS events, call types and establishment causes. One reason why this would be attractive would be that the input priority indication for transmitting an access (barring, i.e., priority to use the PRACH resources) would be the same as the priority indication for allocating resources for further signaling (SRB). We don’t really see why they should be different. For example, the use of not blocking an access (use of PRACH) is questionable if the request will anyway be rejected after Msg3 and vice versa. If the same input (access category) is used both as barring input and for establishment cause in the RRC message (Msg3) to the network, then a consistent handling can be supported. Including an access category in Msg3 also provide for a future-proof solution, where operators easier can incorporate new rules and priorities for access (both to prevent access and to allow resources for SRB), simply by defining how to configure and map, e.g., NAS events to access categories. If an access category can be configured with, e.g., operator-controlled rules, virtually any access category can be created, and treatment, both in barring and in the network when an access request is received, will be fully configurable.

A framework where access category is replacing both call type and establishment cause will be simpler, future-proof and more flexible

With all the above it seems reasonable to further study if access category can replace or be used as input for both access barring and admission/response to Msg3.

RAN2 should consider access category both for access barring purposes and as establishment cause.

Continuing, we note that at least the following topics should be addressed:

- When multiple access categories may apply for a given access request

- Msg3 size

- How to set the establishment cause when accessing 5GC via E-UTRAN

- Dependency on CT1 progess

These topics are further discussed below.

## When multiple access categories may apply for a given access request

Several access attempts can be performed in parallel - e.g. MMTEL voice call can be established in parallel with SMSoIP application sending an SMS. In other words, when requesting a NAS signalling connection, there can be several ongoing, already authorized, access attempts of possibly different access categories.

When an RRC connection establishment is triggered by NAS, multiple services and applications may be running and consequently more than one access attempt (and thus possibly more than one access category) may apply.

The UE needs to perform the unified access control barring check whether the access attempt is authorized for each of those access attempts separately. In order to deal with these aspects of access control, we think that the following principles are to be used when NAS requests to setup a connection (also discussed in [7]):

- At the moment when NAS requests setup of NAS signalling connection, RRC can assume that all the barring checks were already done and passed.

- NAS shall determine the “most applicable access category”. How this is done should be specified by CT1.

- NAS layer provides this single access category to RRC when requesting a NAS signalling connection.

NAS layer provides a single access category to RRC when requesting a NAS signalling connection.

## Establishment cause and Msg3 size

Establishment cause is currently, in LTE, only a 3-bit field, i.e., less than 10% of the total size of Msg3. If a new unified access category should be included in Msg3 instead of establishment cause, more bits would be required. The SA1 CR [5] defines 64 access categories (32 standardized and 32 operator-defined).

As we propose above, NAS provides a single access category to RRC when requesting a NAS signalling connection.

RRC uses this single access category when setting the RRC establishment cause.

For NR, RAN2 should discuss how to set the establishment cause based on access category.

A straightforward way to set the establishment cause is to set it to the actual value of the access category. This would however result in that at least six bits would be needed in msg3 (if also operator-defined categories arew possible to use as establishment cause).

If access category should replace establishment cause for NR, more bits are needed in Msg3.

An alternative way is to specify a mapping from access category to establishment cause. This may reduce the amount of bits needed in msg3.

If the RRC layer sets the establishment cause based on a specified mapping from the access category, this may reduce the amount of bits needed in msg3.

## How to set the establishment cause when accessing 5GC via E-UTRA

In LTE, the codepoints for the establishment cause is already specified. According to stage-1 requirements and RAN2 agreements, unified access control is applied also when accessing 5GC via LTE. Therefore NAS would provide an access category to RRC also in this case. We propose to use the same principle as we propose above for NR, that NAS provides a single access category when requesting setup of the connection. The LTE RRC layer would then need to set the establishment cause based on the access category value. Since there are less codepoints for the LTE establishment cause then the access category, some mapping would be needed.

When accessing 5GC via E-UTRA, the E-UTRA RRC layer should set the establishment cause based on a specified mapping from the unified access control category.

## Dependencies to progress in CT1

Assuming that the access category determination is a NAS responsibility, RAN2 cannot by itself specify how UE NAS would provide priority indications (as proposed, access categories) to AS.

It is proposed that RAN2 send an LS to CT1 to inquire about their view on replacing both call type and establishment cause with a configurable access category indication, used both for purposes of barring as well as for inclusion in a connection request

It is proposed that RAN2 send an LS to CT1 to get CT1 view of replacing call type and establishment cause with access category.

In [9] we provide a draft LS to CT1.

# Conclusion

In section 2 we made the following observations:

Observation 1	Establishment causes and call types may be a subset of what can be used to determine new unified access category

Observation 2	For an NR UE, a request from NAS including new access category removes the need of a call type indication.

Observation 3	A framework where access category is replacing both call type and establishment cause will be simpler, future-proof and more flexible

Observation 4	When an RRC connection establishment is triggered by NAS, multiple services and applications may be running and consequently more than one access attempt (and thus possibly more than one access category) may apply.

Observation 5	If access category should replace establishment cause for NR, more bits are needed in Msg3.

Observation 6	If the RRC layer sets the establishment cause based on a specified mapping from the access category, this may reduce the amount of bits needed in msg3.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 should consider access category both for access barring purposes and as establishment cause.

Proposal 2	NAS layer provides a single access category to RRC when requesting a NAS signalling connection.

Proposal 3	For NR, RAN2 should discuss how to set the establishment cause based on access category.

Proposal 4	When accessing 5GC via E-UTRA, the E-UTRA RRC layer should set the establishment cause based on a specified mapping from the unified access control category.

Proposal 5	It is proposed that RAN2 send an LS to CT1 to get CT1 view of replacing call type and establishment cause with access category.

# References

R2-1710480, Establishment causes for NR, Ericsson, RAN2 WG2#99bis, Prague, Czech Republic, 9th – 13th October 2017

R2-1704007 / C1-171965, LS on unified Access Control for 5G NR (Response to C1-171293/R2-1702441 on LS on Access Control for NR from RAN2), 3GPP CT WG1 #103, Spokane, (WA), USA, April 3-7, 2017

R2-1702441, LS on Access Control for NR, 3GPP RAN WG2 #97, Athens, Greece, February 13-17, 2017

R2-1710062 / S1-173552, Reply LS on unified Access Control for 5G NR, 3GPP SA1, RAN2 WG2#99bis, Prague, Czech Republic, 9th – 13th October 2017

S1-173548, Clarification on access control requirement, NTT DOCOMO, Ericsson, 3GPP TSG-SA WG1 Meeting #79, Guilin, China, 21 - 25 August 2017

3GPP TS 22.261, Service requirements for the 5G system, v15.2.0, 2017-09

R2-1712531, Access control for NR, Ericsson, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017

R2-1712532, Access control for RRC-initiated Access Attempts, Ericsson, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017

R2-1712535, DRAFT LS on Establishment Causes in NR, Ericsson, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017

R2-1712105 / C1-174626, LS on requirements on unified access control for 5GS, 3GPP CT1, RAN2 WG2#100, Reno, Nevada, USA, Nov 27th – Dec 1st 2017